| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0130 |
| --- | --- |

| Title: | Inter-view SAO process in 3DV coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taesup Kim Jin Heo Moonmo Koo Sehoon Yea #221, Yangjae-dong, Seocho-gu, Seoul, 137-130, Korea | Tel: Email: | +82-10-2440-6393 taesup.kim@lge.com jin78.heo@lge.com moonmo.koo@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

- In the current 3DV coding scheme [1], SAO process is modeled to independently implement on each view. Although some common properties between neighbored views could be observed, each view’s SAO parameters are derived and signaled without any sharing action between neighbored views. To understand and utilize these correlations between neighbored views on SAO process in the 3DV coding, this contribution presents an approach for SAO process in the 3DV coding to remove SAO parameters’ redundancies by sharing information between neighbored views that reportedly shows around 1.5% gains on dependent views.

# Introduction

In the 3DV coding, one of the main purposes is to efficiently code neighbored views by making use of the correlations between neighbored views to remove redundancies over them. Yet SAO process in the current 3DV coding doesn’t consider any redundancy different to other 3DV coding schemes.

SAO process consists of a number of parameters that each partition (LCU) of slices has merging left or up flag and SAO type index with corresponding offset values [2]. These parameters are derived by RD optimization under certain structures or modes, which are LCU-based (interleaving mode) and picture-based (quad-tree). During RD optimization, neighbored partitions could be merged or clustered into a single partition sharing a SAO type and offset values. This means neighbored partitions, which could be regarded as similar partitions having common texture properties, have somewhat greater chances to reduce redundancies on SAO parameters. Moreover, there still are efforts in the HEVC to discover correlations between color components, and make it useful to obtain some more gains [3, 4, 5].

# Proposed Method

To reduce redundancies between neighbored views in the 3DV texture coding, this contribution proposes a method to share SAO parameters between neighbored views. In this proposed method, only one set of SAO parameters is used and transmitted for all 2 or 3 views. The shared SAO parameters are exclusively derived from the base view, which is assumed to cover the most common regions over all views. Moreover, shared SAO parameters are used on dependent views without any modification.

In the following, this contribution presents two types of inter-view SAO process in 3DV coding, which aims to reduce redundancies on SAO process in 3DV coding.

## Simple Inter-view SAO Process

In this proposed method, it is assumed that SAO parameters could be shared between neighbored views without any additional process. As depicted on Figure 1, SAO parameters are only derived and transmitted from the base view (view 0), and dependent views (view 1, 2) only reuse the base view’s SAO parameters without any additional process.

Figure 1. SAO parameters are only derived and transmitted from the base view, and dependent views just reuse the base view's SAO parameters.

Therefore, dependent views don’t parse any parameter for SAO parameters on slice header and APS, and these syntax changes are described on Table 1, 2.

Table 1. Proposed syntax change on slice header to share SAO parameters.

Flags related to SAO process are only parsed for the base view.

| parseSliceHeader() { | Descriptor |
| --- | --- |
| … |  |
| if (viewIdx==0){ |  |
| slice_sao_interleaving_flag | u(1) |
| slice_sao_flag | u(1) |
| if (slice_sao_interleaving_flag && slice_sao_flag){ |  |
| slice_sao_flag_cb | u(1) |
| slice_sao_flag_cr | u(1) |
| } |  |
| … |  |
| } |  |

Table 2. Proposed syntax change on APS to share SAO parameters.

| parseAPS(viewIdx) { | Descriptor |
| --- | --- |
| … |  |
| if (viewIdx==0){ |  |
| aps_sao_interleaving_flag | ae(v) |
| if (aps_sao_interleaving_flag){ |  |
| aps_sao_flag | ae(v) |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

## Inter-view SAO Process with Global Disparities

Since there exists somewhat differences between base view and dependent views due to the disparity, this second proposed method additionally aligns dependent views with the base view to appropriately implement SAO process with the shared base view’s SAO parameters by using global disparity information as shown in Figure 2. Global disparity information is derived from the encoder side with a block-based SAD matching method, and only horizontal disparity value is considered. By aligning the dependent views by global disparity values, only common regions between base view and dependent views are filtered by shared SAO parameters as depicted by faded regions in Figure 2.

Figure 2. Proposed method with 1) deriving SAO parameters from the base view, 2) calculating global disparity between base view and dependent view, 3) aligning views by referring global disparities, and 4) applying SAO process with shared base view’s SAO parameters.

Instead of coding the whole SAO parameters for dependent view, the bit-rate could be reduced by only transmitting global disparity information on dependent views. Moreover, the delta global disparity value (difference global disparity between consecutive POCs) is coded rather than the raw value on the slice header as described in Table 3, and the global disparity is properly updated on the decoder side using parsed delta global disparity.

Table 3. Proposed syntax change on slice header to share

SAO parameters and parse global disparity.

| parseSliceHeader() { | Descriptor |
| --- | --- |
| … |  |
| if (viewIdx==0){ |  |
| slice_sao_interleaving_flag_base_view | u(1) |
| slice_sao_flag_base_view | u(1) |
| if (slice_sao_interleaving_flag_base_view && slice_sao_flag_base_view){ |  |
| slice_sao_flag_cb_base_view | u(1) |
| slice_sao_flag_cr_base_view | u(1) |
| } |  |
| }else if (viewIdx!=0){ |  |
| delta_global_disparity | se(v) |
| } |  |
| … |  |
| } |  |

# Simulation Results

## Simple Inter-view SAO Process

### Compression Performance

The proposed method 1(simple inter-view SAO process) was implemented and evaluated on its coding performance in comparison with the anchor HTM4.0.1. The simulation was performed based on the common test conditions. The result is shown in Table 4. All dependent views appeared with consistent gains on all sequences, and the average gains were 1.7% and 1.6% on video 1, 2, respectively. Moreover, 0.4% gains were reported for overall videos.

Table 4. Rate-distortion results on 3-view coding with depth of ‘simple inter-view SAO process’

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.5% | -1.6% | -0.5% | -0.5% | -0.5% |
| Kendo | 0.0% | -1.8% | -1.4% | -0.5% | -0.5% | -0.5% |
| Newspapercc | 0.0% | -1.4% | -1.1% | -0.3% | -0.4% | -0.3% |
| GhostTownFly | 0.0% | -2.5% | -2.3% | -0.4% | 0.0% | -0.1% |
| PoznanHall2 | 0.0% | -1.4% | -2.4% | -0.6% | -0.7% | -0.6% |
| PoznanStreet | 0.0% | -2.1% | -1.1% | -0.1% | -0.4% | -0.3% |
| UndoDancer | 0.0% | -1.2% | -1.2% | -0.1% | -0.3% | -0.2% |
| 1024x768 | 0.0% | -1.6% | -1.4% | -0.4% | -0.5% | -0.4% |
| 1920x1088 | 0.0% | -1.8% | -1.8% | -0.3% | -0.3% | -0.3% |
| average | 0.0% | -1.7% | -1.6% | -0.4% | -0.4% | -0.4% |

### Complexity Analysis

Table 5 shows the ratios of encoding, decoding and rendering time of the proposal in comparison with the anchor. The results indicate the running times decreased a bit compared to the anchor’s running time.

Table 5. Running time ratio of ‘simple inter-view SAO process’

|  | enc time | dec time | ren time |
| --- | --- | --- | --- |
| Balloons | 100.1% | 92.1% | 101.5% |
| Kendo | 100.1% | 101.2% | 98.8% |
| Newspapercc | 100.4% | 95.1% | 99.0% |
| GhostTownFly | 99.6% | 97.8% | 105.0% |
| PoznanHall2 | 99.9% | 108.9% | 105.4% |
| PoznanStreet | 99.9% | 98.1% | 105.6% |
| UndoDancer | 99.5% | 96.1% | 107.2% |
| 1024x768 | 100.2% | 96.1% | 99.8% |
| 1920x1088 | 99.7% | 100.1% | 105.8% |
| Average | 99.9% | 98.4% | 103.2% |

## Inter-view SAO Process with Global Disparities

### Compression Performance

This proposed method was also implemented and evaluated on its coding performance in comparison with the anchor HTM4.0.1. The result is shown in Table 6. All dependent views appeared with consistent gains on all sequences, and the average gains were 1.7% and 1.5% on video 1, 2, respectively. Moreover, 0.3% gains were reported for overall videos.

Table 6. Rate-distortion results on 3-view coding with depth of ‘inter-view SAO process with global disparities’

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.5% | -1.4% | -0.4% | -0.4% | -0.4% |
| Kendo | 0.0% | -1.7% | -1.3% | -0.5% | -0.5% | -0.4% |
| Newspapercc | 0.0% | -1.3% | -0.9% | -0.2% | -0.3% | -0.2% |
| GhostTownFly | 0.0% | -2.2% | -2.2% | -0.4% | 0.0% | -0.1% |
| PoznanHall2 | 0.0% | -1.8% | -2.6% | -0.7% | -0.7% | -0.7% |
| PoznanStreet | 0.0% | -1.8% | -0.8% | 0.0% | -0.3% | -0.2% |
| UndoDancer | 0.0% | -1.4% | -1.3% | -0.2% | -0.2% | -0.2% |
| 1024x768 | 0.0% | -1.5% | -1.2% | -0.4% | -0.4% | -0.3% |
| 1920x1088 | 0.0% | -1.8% | -1.7% | -0.3% | -0.3% | -0.3% |
| average | 0.0% | -1.7% | -1.5% | -0.3% | -0.3% | -0.3% |

### Complexity Analysis

Table 7 shows the ratios of encoding, decoding and rendering time of the proposal in comparison with the anchor. It seems that the running times increased a bit due to the derivation of global disparities.

Table 7. Running time ratio of ‘inter-view SAO process with global disparities’

|  | enc time | dec time | ren time |
| --- | --- | --- | --- |
| Balloons | 101.4% | 104.1% | 103.7% |
| Kendo | 101.3% | 104.1% | 102.0% |
| Newspapercc | 101.5% | 97.1% | 100.0% |
| GhostTownFly | 101.8% | 102.0% | 109.0% |
| PoznanHall2 | 102.1% | 105.9% | 103.7% |
| PoznanStreet | 102.1% | 104.9% | 98.7% |
| UndoDancer | 102.1% | 102.0% | 106.8% |
| 1024x768 | 101.4% | 101.7% | 101.9% |
| 1920x1088 | 102.0% | 103.7% | 104.5% |
| Average | 101.8% | 102.8% | 103.4% |

# Conclusion

In this contribution, it reports that SAO parameters could be shared between neighbored views without transmitting any SAO parameters for dependent views. Though the global disparity could be useful in some cases, simply sharing SAO parameters without any process resulted in consistent gains on dependent views.

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 1”, ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-A1005, Jul. 2012

[2] I. Kim, K.McCann, K. Sugimoto, B. Bross, W. Han, “HM7: High Efficiency Video Coding (HEVC) Test Model 7 8 Encoder Description”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-JI1002, Jul. 2012

[3] G. Laroche, T. Poirier, C. Gisquet, P. Onno, “Non-CE1: On SAO parameters reduction for Chroma”, ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCTVC-I0183, Apr. 2012

[4] K. Minoo, D. Baylon, “Coding of SAO merge left and merge up flags”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J0355, Jul. 2012

[5] E. Alshina, T. Lee, J. Park, G. Laroche, C. Gisquet, P. Onno, “AhG6: On SAO type sharing between U and V components”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- J0045, Jul. 2012

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).