| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0133 |
| --- | --- |

| Title: | 3D-CE5.h Software fix of disparity derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jaewon Sung, Moonmo Koo, Sehoon Yea 221, Yangjae-dong, Seocho-gu, Seoul 137-130, Korea | Tel: Email: | +82-10-434-5259 {jw.sung, moonmo.koo, sehoon.yea}.lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In HTM 4.0.1, a flag is used to indicate whether a block is DV-MCP block or not. The flag is saved with other motion parameters such as motion vector and reference index. In HTM4.0.1 there is a bug which through the flag can be copied to neighbour blocks, which is not desirable. This contribution presents the effect of the bug and reports the simulation results after bug fixing. As the bug does not affect to the coding performance, experiment results show exactly same results with those of HTM4.0.1 anchor coding case.

# Description of bug fix

In the HTM4.0.1, neighbour block disparity vector (NBDV) scheme searches a disparity vector from neighbour blocks, where the disparity vector is used for inter-view motion/residual prediction. The disparity vector is obtained from neighbour disparity compensated prediction (DCP) blocks and DV-MCP blocks [1]. The DV-MCP block is a motion compensated prediction (MCP) block whose motion vector is predicted by inter-view predicted temporal motion parameter candidate in the AMVP, MERGE, and SKIP modes.

To indicate whether a block is a DV-MCP block or not, one bit flag, say IsDvMcpFlag, is saved with other motion parameters such as motion vector and reference index in the current implementation. When the AMVP, MERGE, and SKIP mode is used, each motion parameter candidate is generated by copying the motion parameters including the IsDvMcpFlag of corresponding neighbour block.

In that case, a MCP block whose motion vector is copied from left block can be marked as a DV-MCP block when its left block is a DV-MCP block. To prevent the propagation of the DV-MCP flag to neighbour blocks, the IsDvMcpFlag in the motion parameter candidates should be explicitly set to false after constructing the motion parameter candidate list, except for the inter-view predicted temporal motion parameter candidate. However, in the HTM4.0.1, the IsDvMcpFlag in a temporal motion parameter candidate for AMVP is not explicitly set to false. The proposed fix added two lines to fix it.

The proposed fix does not affect to the coding performance of HTM4.0.1. The bug causes non DV-MCP block predicted by AMVP to be marked as DV-MCP block. However, current disparity vector derivation process uses only SKIP coded DV-MCP block.

Although the bug does not affect to any other coding tools in CTC, it can cause some problems when somebody want to use the DV-MCP flag of non SKIP coded block. So, we provided the fixed version of software and recommended to CE5.h participants.

# Experiment results

The proposed fixe is implemented on top of the HTM4.0.1. For experiment, common test condition for coding 3 view texture and depth coding was used. Table 1 shows the coding performance of bug fixed HTM4.0.1. The fix does not change any number in the table under CTC.

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | Enc time | Dec time | Ren Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 101.2% | 108.2% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 101.8% | 104.3% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 95.7% | 96.0% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.7% | 103.3% | 112.9% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 101.0% | 103.4% | 102.5% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.8% | 102.9% | 98.8% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.9% | 103.9% | 109.2% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 99.5% | 102.7% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.8% | 103.4% | 105.7% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.5% | 101.7% | 104.4% |

# References

- Jaewon Sung, Moonmo Koo, Sehoon Yea, "3D-CE5.h Simplification of disparity vector derivation for HEVC-based 3D video coding", ISO/IEC JTC1/SC29/WG11 JCT3V/A0126, July 2012, Stockholm, Sweden.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).