| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0134 |
| --- | --- |

| Title: | Results on Weighted Prediction in 3D Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiwook Jung, Hongbin Liu Jaewon Sung, Jie Jia, Sehoon Yea 221, Yangjae-Dong, Seocho-Gu, Seoul 137-130, Korea | Tel: Email: | +82-2-6912-6477 jiwook.jung@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

Weighted Prediction is known for its benefit coding fading in/out sequences. In 3D Video Coding, weighted prediction has not been considered yet. In this contribution, two methods for weighted prediction on dependent view were tested. Basically, the weighted prediction for inter-view coding was disabled. Because, the weighted prediction scheme of HM is not appropriate for inter-view coding.

At first, the performance of weighted prediction is tested when weighted prediction parameters are calculated independently. Second, the inheritance of weighted prediction parameter is tested. WP parameters on dependent view are inherited from parameters on independent view because temporal weighted prediction parameters on different views might be same or similar.

# Introduction

Weighted Prediction(WP) is known for its benefit coding fading in/out sequences. In 3D Video Coding, WP has not been considered yet. In this contribution, two methods for weighted prediction on dependent view were tested.

In HTM-4.0.1, WP has not been maintained to work appropriately. WP on HM has been integrated for weighted prediction test in[1]. However, the weighted prediction for inter-view coding was disabled. Because, the weighted prediction scheme of HM is not appropriate for inter-view coding. For example, the regions captured by cameras are not the same in the different view positions. Therefore, estimated WP parameters can be unsuitable for inter-view coding.

Additionally, inheritance of weighted prediction parameter on dependent view was tested. In [3], it was suggested WP parameters of base layer are reused on enhancement layer in Scalable video coding. In this method, WP parameters on dependent view are inherited from parameters on independent view because temporal weighted prediction parameters on different views might be similar. In this method, a flag indicating whether WP parameters on the dependent view was inherited from the independent view should be on slice header. The method has coding gain because coding WP parameters on dependent views are not coded.

# Proposed Algorithm

## Disabling weighted prediction for inter-view coding (Test A)

The performance of the weighted prediction used in HM was tested with disabling weighted prediction for inter-view coding.

## Inheritance of weighted prediction parameter on dependent view(Test B)

Figure 2.1 shows the concept of the inheritance method. WP table is estimated on the independent view temporally as the same manner of HEVC. On the dependent views, WP table can be inherited from the WP table of the independent view. In this case, the slice header flag which indicate whether inheritance is used will be turned on.

Figure 2.1 The concept of weighted prediction parameter inheritance on dependent view

Table2.1 Syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| … |  |
| if( !entropy_slice_flag ) { |  |
| … | … |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) { |  |
| if( !base_view && !is_depth && ) { |  |
| base_pred_weight_table_flag | u(1) |
| if( base_pred_weight_table_flag == 0 ) |  |
| pred_weight_table( ) |  |
| } |  |
| … |  |
| } |  |

# Test sequences

Fading in/out sequences are generated as the same manner of HEVC. In JCTVC-E041[2], there are an enclosed software that it can create linear fading sequences.

In this experiment, we use same conditions as in [2], [3]: a linear fade is applied to the first 2 seconds of the 3DV sequences and the experiments are done with the 2-second sequences. A linear fade-out is applied during the first half (0~1 second) and a linear fade-in is applied during the second half(1~2 second) (Error: Reference source not found). The fade strength is the range [0.25, 1]. In this experiments, only texture video are encoded.

Figure 3.1 fading in/out strength

# Results

Table 4.1 shows the performance of the method that WP is disabled on inter-view coding. The anchor is HTM-4.0.1

Table 4.1 BDR performance of Test A

|  | video 0 | video 1 | video 2 | video only | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | -36.0% | -19.5% | -17.7% | -29.6% | 156.7% | 100.0% |
| Kendo | -18.6% | -4.9% | -3.1% | -13.5% | 192.4% | 101.6% |
| Newspapercc | -33.0% | -22.4% | -24.1% | -29.4% | 139.5% | 94.2% |
| GhostTownFly | -29.6% | -3.2% | -3.4% | -24.8% | 162.1% | 104.9% |
| PoznanHall2 | -32.1% | -10.8% | -15.5% | -24.9% | 166.4% | 99.4% |
| PoznanStreet | -27.5% | -15.6% | -14.9% | -24.5% | 136.2% | 102.0% |
| UndoDancer | -37.2% | -20.0% | -24.1% | -33.3% | 155.9% | 102.4% |
| 1024x768 | -29.2% | -15.6% | -15.0% | -24.2% | 161.4% | 98.5% |
| 1920x1088 | -31.6% | -12.4% | -14.5% | -26.9% | 154.7% | 102.2% |
| average | -30.6% | -13.8% | -14.7% | -25.7% | 157.5% | 100.6% |

Table 4.2 shows the performance of the inheritance of weighted prediction parameter on dependent view. The anchor is HTM-4.0.1.

Table 4.2 BDR performance of Test B

|  | video 0 | video 1 | video 2 | video only | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | -36.0% | -26.3% | -25.1% | -32.0% | 160.8% | 99.7% |
| Kendo | -18.6% | -12.9% | -11.9% | -16.4% | 193.2% | 101.0% |
| Newspapercc | -33.0% | -28.2% | -29.2% | -31.3% | 143.1% | 93.4% |
| GhostTownFly | -29.6% | -11.6% | -12.2% | -26.2% | 163.4% | 104.3% |
| PoznanHall2 | -32.1% | -18.6% | -19.9% | -27.0% | 168.0% | 98.4% |
| PoznanStreet | -27.5% | -19.1% | -19.4% | -25.5% | 141.2% | 101.2% |
| UndoDancer | -37.2% | -23.8% | -26.7% | -33.9% | 158.0% | 102.1% |
| 1024x768 | -29.2% | -22.5% | -22.1% | -26.6% | 164.4% | 98.0% |
| 1920x1088 | -31.6% | -18.3% | -19.6% | -28.2% | 157.3% | 101.5% |
| average | -30.6% | -20.1% | -20.7% | -27.5% | 160.3% | 100.0% |

# Conclusion

In this proposal, the utility of weighted prediction for coding fading in/out sequences is reported. Also, it is presented that inheritance of weighted prediction parameter have benefits. Weighted prediction should be considered in 3D Video coding for coding fading in/out sequences.

There are chances to apply weighted prediction for inter-view coding and the method for the inheritance of WP parameters.

It is recommended that weighted prediction method is adopted in 3D Video Coding.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Hongbin Liu, Jiwook Jung, Jaewon Sung, Jie Jia, Sehoon Yea, “3D-CE2.h : Results of Illumination Compensation for Inter-View Prediction”, JCT3V-B0045, Shanghai, Oct. 2012.

- Philippe Bordes, “Weighted Prediction ", JCTVC-E041, Geneva, March 2011.

- Philippe Bordes, “Weighted Prediction ", JCTVC-F265, Torino, July 2011.

- Peng Yin, Jill Boyce, Purvin Pandit, “Weighted Prediction in SVC ", JVT-P064, Poznan, July 2005.

- Heiko Schwarz, Dmytro Rusanovskyy, "Common Test Conditions for 3DV experimentation”, ISO/IEC JTC1/SC29/WG11, JCT3V-A1100, Stockholm, July 2012.