| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0136 |
| --- | --- |

| Title: | 3D-CE5.h Related: Support of parallel merge in disparity vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo, Jaewon Sung, Sehoon Yea 221, Yangjae-dong, Seocho-gu, Seoul 137- 130, Korea | Tel: Email: | +82-10-3054-8642 {moonmo.koo, jw.sung, Sehoon.yea}.lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In HTM anchors so far, parallel merge scheme has not been activated. Thus, the A0126 proposal adopted in the last meeting did not present experimental results with the scheme. To reduce complexity for deriving availabilities of neighbor blocks and dispatching motion information of them and to maintain parallelism enabled by the scheme, we should also consider parallel merge in disparity vector derivation process. This proposal shows experimental results for parallel merge to be added to the A0126 proposal, and the benefits of inter-view motion prediction are still preserved even with the parallel merge.

# Introduction

In the current HEVC specification draft document [1], parallel merge is considered together during the derivation of merge candidate list. The CU block size with the parallel merge applied is signaled through log2_parallel_merge_level_minus2 syntax element in a picture parameter set. While constructing merge candidate lists for PU blocks in a CU block of which size is the same as the one designated by log2_parallel_merge_level_minus2. The disparity vector derivation process described in the A0126 proposal [2] utilizes the information of the same neighborhood blocks as the ones for HEVC except for disparity vectors of DV-MCP blocks. If parallel merge is not applied in the same manner as the baseline HEVC during disparity vector derivation, parallelization of PU blocks cannot be realized and algorithm complexity can be increased because availability information of neighbor blocks becomes different from those of HEVC. Therefore, it is necessary for applying the parallel merge to prediction of a disparity vector according to log2_parallel_merge_level_minus2 value that also affects the baseline HEVC.

# Proposed modifications

In the current HTM 4.0.1 anchor software, parallel processing cases due to the parallel merge cannot occur because log2_parallel_merge_level2 value is fixed to be zero (log2_parallel_merge_level2 is set by LOG2_PARALLEL_MERGE_LEVEL_MINUS2 macro defined in TypeDef.h file). Hence, parallel merge has never been engaged in the CE5.h. To expose the parallel merge in the software, first, we varied log2_parallel_merge_level_minus2 value in the range of 1 (8 x 8 level), 2 (16 x 16 level), 3 (32 x 32 level), and 4 (64 x 64 level). In addition, when log2_parallel_merge_level_minus2 becomes more or equal to 1, a merge candidate list for an 8 x 8 CU block must be shared with the PU blocks in the CU block according to [1], which is indicated by the singleMCLFlag variable in [1] and by CU_BASED_MRG_CAND_LIST macro defined in the TypeDef.h.

We added code fragments to the software function referring DCP vectors of neighbor blocks or predicted disparity vectors of neighbor DV-MCP blocks in the same way as the parallel merge scheme described in [1].

# Experimental results

The experimental results for all of parallel merge levels are as follows. Basically, we compared the results with HTM 4.0.1 anchor ones.

Table 1  Experimental result for 8 x 8 parallel merge level (log2_parallel_merge_level_minus2 = 1)

Table 2  Experimental result for 16 x 16 parallel merge level (log2_parallel_merge_level_minus2 = 2)

Table 3  Experimental result for 32 x 32 parallel merge level (log2_parallel_merge_level_minus2 = 3)

Table 4  Experimental result for 64 x 64 parallel merge level (log2_parallel_merge_level_minus2 = 4)

As shown in Table 1, 2, 3, and 4, the overall performance is degraded as expected due to the parallel merge, as parallel merge level increases. Table 5 presents performance differences between off and on cases of inter-view motion prediction (MultiviewMVPred was set to be zero in encoder.cfg file).

Table 5  Performance comparison of the cases whether inter-view motion prediction is turned on or not

| Parallel merge level | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| 8 x 8 | 0.0% | -11.9% | -11.6% | -5.2% | -3.8% | -4.1% |
| 16 x 16 | 0.0% | -12.2% | -11.7% | -5.3% | -3.9% | -4.2% |
| 32 x 32 | 0.0% | -12.9% | -12.4% | -5.6% | -4.1% | -4.4% |
| 64 x 64 | 0.0% | -13.9% | -13.1% | -6.0% | -4.5% | -4.8% |

We confirmed that the effectiveness on the inter-view motion prediction was still observed even if parallel merge was applied during disparity vector derivation process as shown in Table 5. Consequently, we recommend the adoption of this proposal because it seems reasonable to add description on the parallel merge to the previous A0126 proposal [2] in terms of performance and consistency with the baseline HEVC.

# Conclusion

We modified the HTM 4.0.1 software for parallel merge to be also considered during inter-view motion prediction, and run our modified software with parallel merge supported for all parallel merge levels. Also, we confirmed that the benefit of inter-view motion prediction was still maintained even with parallel merge in disparity vector derivation. For complexity reduction and parallelization in sync with the baseline HEVC, we strongly recommend this proposal should be adopted for making the A0126 proposal [2] in the last meeting more suitable for implementation.

# References

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, and Thomas Wiegand, “High efficiency video coding (HEVC) text specification draft 8”, ISO/IEC JTC1/SC29/WG11/JCT-VC/J1003_d7, July 2012, Stockholm, Sweden.

- Jaewon Sung, Moonmo Koo, Sehoon Yea, "3D-CE5.h Simplification of disparity vector derivation for HEVC-based 3D video coding", ISO/IEC JTC1/SC29/WG11 JCT3V/A0126, July 2012, Stockholm, Sweden.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).