| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT2-B0146 |
| --- | --- |

| Title: | 3D-AVC results on improved inside view motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kwan-Jung Oh, Jaejoon Lee, Du-Sik Park | Email: | kwanjung.oh@samsung.com jaejoon1.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

Modifications on IVMP (Inside View Motion Prediction, M22583) are proposed to improve coding gains. IVMP availability conditions and motion prediction method are changed. Other parts including syntaxes are not changed at all.

# Proposed modifications

## IVMP Availability

For the mixed resolution 3D video cases, the existing IVMP is enabled when all of the corresponding four texture macroblocks are coded as skip or inter 16x16 or inter 16x8 or inter 8x16. In the proposed modification, IVMP is allowed always even though all corresponding texture macroblocks are intra coded. Thus, we solve the parsing dependency problem between texture and depth at the decoder side.

## Motion Prediction

For the mixed resolution 3D video cases, the four texture macroblocks are corresponds to one depth macroblock. In IVMP, motion information of the one texture macroblock is mapped to 8x8 sub-block of one depth macroblock as shown in Fig. 1.

Fig.1. Macroblock correspondence of texture and depth for mixed resolution 3D video

The motion prediction is carried out by 8x8 block unit and its sub-block type and motion information depend on corresponding texture macroblock. For example, the sub-block type of A’ is 8x4 if mb_type of A is P16x8. The H.264/AVC has reference index for each 8x8 block and motion vector for each 4x4 block. It means that we have to decide representative motion information when corresponding texture macroblock has multiple reference indexes. In that case, the sub-block type is set as 8x8 and the motion information of the 4x4 block that has motion vector of maximum magnitude is used. If the corresponding texture macroblock is intra coded, the sub-block type is set as 8x8 and motion information is derived from the adjacent neighboring inter coded block. If all corresponding texture macroblocks are intra coded, the zero vectors are used.

# Coding experiments

Results of the coding experiment using a 3D-ATM version 5.1r2 including the proposed modifications for the three view case are given in Table 1 and Table 2. It can be seen that run time is almost same with anchor and a rate reduction of about 0.1% can be achieved. Additional results can be found in the attached excel sheet. The method 1 uses mb_type information for decision of representative motion information when corresponding texture macroblock is intra coded. In addition, method 1 does not support IVMP when all corresponding texture macroblocks are intra coded. The method 2 is same with above algorithm description.

Table 1: coding results for proposed modifications (Method 1)

|  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  | Complexity estimate (ratio to anchor) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | Encoder Time, % | Decoder Time, % |
| PoznanHall2 | 0.06 | 0.00 | 0.15 | 0.00 | 100.17 | 99.57 |
| PoznanStreet | -0.07 | 0.00 | -0.07 | 0.00 | 100.24 | 99.40 |
| UndoDancer | -0.10 | 0.00 | -0.11 | 0.00 | 100.09 | 99.51 |
| GhostTownFly | -0.16 | 0.01 | -0.15 | 0.01 | 100.04 | 100.87 |
| Kendo | -0.03 | 0.00 | -0.01 | 0.00 | 99.80 | 99.68 |
| Balloons | -0.07 | 0.00 | -0.06 | 0.00 | 100.45 | 100.24 |
| Newspaper | -0.05 | 0.00 | 0.00 | 0.00 | 100.17 | 100.54 |
| Average | -0.06 | 0.00 | -0.04 | 0.00 | 100.14 | 99.97 |

Table 2: coding results for proposed modifications (Method 2)

|  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  | Complexity estimate (ratio to anchor) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | Encoder Time, % | Decoder Time, % |
| PoznanHall2 | 0.00 | 0.00 | 0.08 | 0.00 | 100.23 | 99.28 |
| PoznanStreet | -0.06 | 0.00 | -0.06 | 0.00 | 100.74 | 99.30 |
| UndoDancer | -0.12 | 0.00 | -0.24 | 0.01 | 100.32 | 98.92 |
| GhostTownFly | -0.15 | 0.01 | -0.17 | 0.01 | 99.51 | 101.53 |
| Kendo | -0.04 | 0.00 | -0.06 | 0.00 | 101.14 | 100.26 |
| Balloons | -0.07 | 0.00 | -0.05 | 0.00 | 100.33 | 99.92 |
| Newspaper | -0.05 | 0.00 | -0.02 | 0.00 | 99.97 | 100.54 |
| Average | -0.07 | 0.00 | -0.08 | 0.00 | 100.32 | 99.96 |

# Conclusion

Modifications on IVMP have been suggested. Coding results for these modifications have been presented. It achieves about 0.1% coding gain without run time increase.

# Patent right declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).