| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0147r1 |
| --- | --- |

| Title: | MVC+D: coding of unpaired MVD data |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Payman Aflaki Dmytro Rusanovskyy, Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution proposes modifications to the MVC+D specification text, which enable coding of unpaired texture and depth views, so called un-paired MVD format. The proposed changes are limited to high-level syntax and semantics.

In a typical use case of unpaired MVD, a reduced number of depth views compared to the number of texture views are coded. This reduces encoding/decoding complexity and memory requirement while still provides the 3DV functionality for many use scenarios. This proposal evaluates two examples of such use cases. The simulation results show that for the tested use cases, the proposed un-paired MVD format outperforms the anchor MVD format by 7% of dBR for coded data and by 1-5% of dBR for coded/synthesized data.

Revision 1 of the contribution contained a few bug fixes in the proposed syntax and semantics of the sequence parameter set 3DVC extension syntax structure.

# Introduction

MVC+D bitstreams are required to contain a depth view for each texture view. However, there are use cases where unpaired texture or depth views would be reasonable, such as:

- In a typical depth through disparity estimation process having a stereoscopic video as input, a disparity search is performed in a one-directional manner, i.e. for each sample in a first view component (e.g. a picture of the left view), the corresponding sample in a second view component (e.g. a picture of the right view) is searched. The resulting disparity pictures are equivalent to depth pictures for areas that are covered by the field of view of both views and are not occluded in either view. Hence, disparity search along the other direction may be omitted for example to save computational resources in the encoding end.

- Depth information may be generated using a specific depth sensor rather than generated in a per-pixel depth estimation process based on texture views. In such a camera setup, the depth sensor(s) can be either collocated with some of utilized image sensor(s) or located at a different position than the color camera(s), hence representing a different view. A visualization of this concept is shown below:

Figure 1. Stereo camera with a single ranging device.

- In some use cases stereo baseline adjustment would be required in relatively close proximity to a decoded texture view, whereas another decoded texture view would be taken as a base view. Therefore a single depth view can be sufficient as input for view synthesis. A visualization of this concept is shown below:

Figure 2. Visualization of depth perception adjustment by view synthesis in close proximity from a one decoded view

- In some cases decoded multiview content is expected to be subject to view extrapolation rather than interpolation. For example, an optimal disparity for two-view autostereoscopic displays for handheld use is typically wider than that for living-room polarized or shutter-glass displays. Depth-enhanced stereoscopic content for handheld devices could therefore have two texture views and only one depth view, as the extrapolation quality would remain the same as that for two texture and depth views. A visualization of this concept is shown below:

Figure 3. Visualization of depth perception adjustment by view synthesis to an extrapolated view

- The decoded picture buffer and memory access bandwidth requirements can be reduced if the number of depth views is smaller than the number of texture views.

Depth views in MVC+D are coded independently of texture views. Hence, no changes in the decoding process are needed to support the unpaired MVD format. This contribution proposes syntax and semantics modifications to the MVC+D specification text, which enable coding of unpaired texture and depth views. The contribution also proposes constraints for the Multiview Depth High profile in order to enable forward view synthesis with a texture view component and a depth view component of the same view as inputs.

# Overview of the Proposal

As a consequence of the number of depth views potentially differing from the number of texture views, the following modifications are proposed to the MVC+D specification text:

- Signalling of unpaired texture and depth views.

A 3DVC sequence parameter set is modified to contain only the sequence parameter set 3DVC extension syntax structure. The sequence parameter set 3DVC extension syntax structure still includes the same information as in the previous version of the MVC+D specification. It is also modified to contain the same information as present in the sequence parameter set MVC extension syntax structure. Additionally, it is proposed to contain flags whether a depth view is present for a certain view_id and whether a texture view is present for a certain view_id.

- DPB support for unpaired texture and depth views.

The DPB size is indicated separately for texture view component and depth view components, which enables efficient memory usage when the number of depth views is unequal to the number of texture views. Similarly, the DPB is operated separately for texture view component and depth view components. However, in the “bumping” process, a decoded depth view component, if any, is output next to the respective texture view component in order to have the same output order than in the output timing conformance.

# Use Case Study and Simulation Results

To demonstrate the usefulness of the unpaired MVD format, we evaluated its performance in the 2-view (C2) scenario. The performance was evaluated using two view synthesis schemes, differing in the complexity and memory requirements. Scheme 1 targeted low-complexity encoder and decoder operations with reduced memory requirements, whereas Scheme 2 assumed a more advanced decoder. In our scenarios, stereo baseline adjustment was provided in a proximity to one decoded texture view, whereas another decoded texture view was utilized as a second view of the displayed stereo pair, see Figure 2.

Please note that we did not evaluate the performance of unpaired MVD for the extrapolation use case presented in Figure 3, since unpaired MVD has identical view synthesis quality of extrapolated views compared to that of the anchor but has a smaller bitrate.

## Scheme 1

The flowchart of the tested 3DV scheme is shown in Figure 4.

The proposed scheme follows all conditions and settings of CTC for C2, except of the following:

- Depth component D1 is not coded

- Components T1 and D1 do not participate in DIBR

- DIBR (1D Fast VSRS) operates in the “extrapolation mode”

- Synthesized views are adjusted to the use case scenario as follows:

  - Views are synthesised in 50% of the input camera baseline

  - Views are synthesized in the proximity of T0/D0: 0.1 and 0.2 of the input baseline

- A stereo pair is formed by 1 coded and 1 synthesised views, e.g. (0.2-1), (0.5-1)

- A stereo pair is cropped at the left border by 16 pixels for each view to exclude the impact of the global disocclusion.

Comparisons were performed vs. the anchor MVC+D scheme executed under CTC/C2, with stereo pairs created as specified in this section.

Figure 4. Experimental framework on unpaired MVD in the C2 3DV scenario, Scheme 1

A summary of the simulation results of Scheme 1 is shown in Table 1. The simulation results show that

- The proposed scheme outperformed the anchor for coded data by 7% of dBR on average.

- Scheme 1 outperformed the anchor by 4% of dBR for coded/synthesised stereo pairs with view adjustment of 10% of the input views baseline.

- With view adjustment 20%, the proposed scheme showed a slightly better performance relative to the anchor: 1% dBR reduction on average and 5% dBR reduction for GT_Fly.

- With view adjustment 50% of input baseline, the proposed scheme suffered a significant degradation in the quality of synthesized views, and underperformed against the anchor by about 20% of dBR.

Table 1. Summary of simulation results for the tested unpaired MVD scheme, Scheme 1.

|  | Total (Coded PSNR) |  | Stereo pair (Base + 0.9) |  | Stereo pair (Base + 0.8) |  | Stereo pair (Base + 0.5) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB |
| S01 | -5.90 | 0.22 | -3.15 | 0.10 | 3.55 | -0.14 | 32.59 | -1.22 |
| S02 | -4.97 | 0.16 | -5.37 | 0.18 | -2.70 | 0.09 | 14.88 | -0.49 |
| S03 | -2.28 | 0.08 | -4.05 | 0.15 | -0.55 | 0.02 | 24.09 | -0.74 |
| S04 | -3.18 | 0.13 | -5.22 | 0.20 | -4.45 | 0.17 | 4.96 | -0.19 |
| S05 | -14.10 | 0.79 | -5.13 | 0.22 | -0.98 | -0.01 | 19.75 | -1.05 |
| S06 | -9.06 | 0.50 | -4.76 | 0.23 | 0.17 | -0.05 | 20.52 | -1.09 |
| S08 | -9.14 | 0.42 | -0.38 | -0.01 | 12.22 | -0.53 | 55.54 | -1.80 |
| Average | -6.95 | 0.33 | -4.01 | 0.15 | 1.04 | -0.07 | 24.62 | -0.94 |

### Scheme 2

Considering the significant penalty against the anchor provided by Scheme 1 with view adjustment 0.5 of the baseline, we evaluated how much improvement can be introduced with additional post-processing. An example of such post-processing could be execution of the DIBR in traditional, interpolation mode. To enable such mode with unpaired MVD coding, a virtual depth map D1 should be created at the decoder side and utilized together with T0/D0 and T1 in the view synthesis.

The flowchart of the tested 3DV Scheme 2 is shown in Figure 5. The utilized experiment framework is identical to Scheme 1 except of the following changes:

- DIBR (1D Fast VSRS) operates in the “interpolation mode” as given in CTC

- Virtual depth D1* is produced by projection of D0

- Components T1/D1*  and T0/D0 are utilized in DIBR

Figure 5. Experimental framework on unpaired MVD in the C2 3DV scenario, Scheme 2

A summary of simulation results of Scheme 2 are shown in Table 2. The simulation results show that

- The proposed scheme outperformed the MVD anchor for all tested scenarios.

- With view adjustment 10% of input baseline, the tested unpaired MVD Scheme outperformed the MVD anchor by 5.5% of dBR on average, for view adjustment of 20% and 50%, the dBR improvement was about 4% and 1% of dBR on average, respectively.

Table 2. Summary of simulation results for tested unpaired MVD scheme, Scheme 2.

|  | Total (Coded PSNR) |  | Stereo pair (Base + 0.9) |  | Stereo pair (Base + 0.8) |  | Stereo pair (Base + 0.5) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB |
| S01 | -5.90 | 0.22 | -5.51 | 0.18 | -3.59 | 0.12 | -16.07 | 0.60 |
| S02 | -4.97 | 0.16 | -4.65 | 0.15 | -4.18 | 0.14 | -2.39 | 0.08 |
| S03 | -2.28 | 0.08 | -2.75 | 0.10 | 0.34 | -0.01 | 11.08 | -0.36 |
| S04 | -3.18 | 0.13 | -3.19 | 0.12 | -2.96 | 0.11 | -1.50 | 0.06 |
| S05 | -14.10 | 0.79 | -11.58 | 0.59 | -10.48 | 0.52 | -7.26 | 0.33 |
| S06 | -9.06 | 0.50 | -7.77 | 0.41 | -6.58 | 0.34 | -3.25 | 0.14 |
| S08 | -9.14 | 0.42 | -2.62 | 0.09 | 0.62 | -0.06 | 13.09 | -0.58 |
| Average | -6.95 | 0.33 | -5.44 | 0.23 | -3.83 | 0.16 | -0.90 | 0.04 |

# Proposed Specification Text Changes

Text proposed to be removed is marked with red strikethroughs and text proposed to be added is marked with red font.

## Signaling of Unpaired Texture and Depth Views

- 7.3.2.1.3	Subset sequence parameter set RBSP syntax

| subset_seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_data( ) | 0 |  |
| if( profile_idc = = 83 ¦ ¦ profile_idc = = 86 ) { |  |  |
| seq_parameter_set_svc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| svc_vui_parameters_present_flag | 0 | u(1) |
| if( svc_vui_parameters_present_flag = = 1 ) |  |  |
| svc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 118 ¦ ¦ profile_idc = = 128 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| mvc_vui_parameters_present_flag | 0 | u(1) |
| if( mvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } |  |  |
| if( profile_idc = = 138 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| seq_parameter_set_3dvc_extension( ) | 0 |  |
| } |  |  |
| additional_extension3_flag | 0 | u(1) |
| if( additional_extension3_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| additional_extension3_data_flag | 0 | u(1) |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

- I.7.3.2.1.5	Sequence parameter set 3DVC extension syntax

| seq_parameter_set_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| num_views_minus1 | 0 | ue(v) |
| for( i = 0, NumDepthViews = 0; i <= num_views_minus1; i++ ) { |  |  |
| view_id[ i ] | 0 | ue(v) |
| depth_view_present_flag[ i ] | 0 | u(1) |
| NumDepthViews += depth_view_present_flag[ i ] |  |  |
| texture_view_present_flag[ i ] | 0 | u(1) |
| } |  |  |
| for( i = 1; i <= num_views_minus1; i++ ) { |  |  |
| if( depth_view_present_flag[ i ] ) { |  |  |
| num_anchor_refs_l0[ i ] | 0 | ue(v) |
| for( j = 0; j < num_anchor_refs_l0[ i ]; j++ ) |  |  |
| anchor_ref_l0[ i ][ j ] | 0 | ue(v) |
| num_anchor_refs_l1[ i ] | 0 | ue(v) |
| for( j = 0; j < num_anchor_refs_l1[ i ]; j++ ) |  |  |
| anchor_ref_l1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| } |  |  |
| for( i = 1; i <= num_views_minus1; i++ ) { |  |  |
| if( depth_view_present_flag[ i ] ) { |  |  |
| num_non_anchor_refs_l0[ i ] | 0 | ue(v) |
| for( j = 0; j < num_non_anchor_refs_l0[ i ]; j++ ) |  |  |
| non_anchor_ref_l0[ i ][ j ] | 0 | ue(v) |
| num_non_anchor_refs_l1[ i ] | 0 | ue(v) |
| for( j = 0; j < num_non_anchor_refs_l1[ i ]; j++ ) |  |  |
| non_anchor_ref_l1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| } |  |  |
| num_level_values_signalled_minus1 | 0 | ue(v) |
| for( i = 0; i <= num_level_values_signalled_minus1; i++ ) { |  |  |
| level_idc[ i ] | 0 | u(8) |
| num_applicable_ops_minus1[ i ] | 0 | ue(v) |
| for( j = 0; j <= num_applicable_ops_minus1[ i ]; j++ ) { |  |  |
| applicable_op_temporal_id[ i ][ j ] | 0 | u(3) |
| applicable_op_num_target_views_minus1[ i ][ j ] | 0 | ue(v) |
| for( k = 0; k <= applicable_op_num_target_views_minus1[ i ][ j ]; k++ ) |  |  |
| applicable_op_target_view_id[ i ][ j ][ k ] | 0 | ue(v) |
| applicable_op_num_texture_views_minus1[ i ][ j ] | 0 | ue(v) |
| applicable_op_num_depth_views[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| } |  |  |
| 3dvc_vui_parameters_present_flag | 0 | u(1) |
| if( 3dvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) |  |  |
| texture_vui_parameters_present_flag | 0 | u(1) |
| if( texture_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) |  |  |
| } |  |  |

- I.7.4.1.2.1	Order of 3DVC sequence parameter set RBSPs and picture parameter set RBSPs and their activation

- …

- The active texture 3DVC sequence parameter set RBSPs for different values of VOIdx may be the same 3DVC sequence parameter set RBSP. The active 3DVC sequence parameter set RBSP and an active texture 3DVC sequence parameter set RBSP for a particular value of VOIdx may be the same 3DVC sequence parameter set RBSP. An active view 3DVC sequence parameter set RBSP for a particular value of VOIdx and an active texture 3DVC sequence parameter set RBSP for a particular value of VOIdx may be the same 3DVC sequence parameter set RBSP.

- ...

- For any active 3DVCMVC sequence parameter set or active view 3DVCMVC sequence parameter set, part of the syntax elements in the MVC sequence parameter set extension applies only to the depth views referring to this sequence parameter set, while the some other parts of the syntax elements in the 3DVCMVC sequence parameter set extension collectively apply to both the depth views referring to this sequence parameter set and the corresponding texture views. More specifically, the view dependency information of the 3DVCMVC sequence parameter set extension applies only to the depth views, and the level definitions collectively apply to operation points, each of which contains both depth views and their corresponding texture views. Moreover, the mvc_vui_parameters_extension( ) included when 3dvc_vui_parameters_present_flag is equal to 1 applies collectively to both the depth views referring to this 3DVC sequence parameter set and the corresponding texture views. The vui_parameters( ) included in the sequence parameter set data syntax structure, if present, apply collectively to both the depth views referring to this sequence parameter set and the corresponding texture views, except for the aspect ratio information and the bitstream restriction information, if present, which apply only to the depth views referring to this 3DVC sequence parameter set. The aspect ratio information and the bitstream restriction information for the texture views may be present in the vui_parameters( ) syntax structure included in an MVC sequence parameter set.

- I.7.4.2.1.4	Sequence parameter set MVC extension semantics

The semantics specified in subclause H.7.4.2.1.4 apply with the substitution of texture view component or depth view component for view component

- I.7.4.2.1.5	Sequence parameter set 3DVC extension semantics

The semantics specified in subclause H.7.4.2.1.4 apply with the substitution of texture view component or depth view component for view component and with the following additions:

depth_view_present_flag[ i ] equal to 0 specifies that there is no depth view having a view_id equal to view_id[ i ]. depth_view_present_flag[ i ] equal to 1 specifies that there is a depth view having a view_id equal to view_id[ i ].

texture_view_present_flag[ i ] equal to 0 specifies that there is no texture view having a view_id equal to view_id[ i ]. texture_view_present_flag[ i ] equal to 1 specifies that there is a texture view having a view_id equal to view_id[ i ]. If depth_view_present_flag[ i ] is equal to 0, texture_view_present_flag[ i ] shall be equal to 1.

num_anchor_refs_l0[ i ], anchor_ref_l0[ i ][ j ], num_anchor_refs_l1[ i ], anchor_ref_l1[ i ][ j ], num_non_anchor_refs_l0[ i ], non_anchor_ref_l0[ i ][ j ], num_non_anchor_refs_l1[ i ], and non_anchor_ref_l1[ i ][ j ] apply to depth view components.

applicable_op_num_texture_views_minus1[ i ][ j ] plus 1 specifies the number of texture views required for decoding the target output views corresponding to the j-th operation point to which the level indicated by level_idc[ i ] applies. The number of texture views specified by applicable_op_num_views_minus1 includes the texture views of the target output views and the texture views that the target output views depend on. The value of applicable_op_num_texture_views_minus1[ i ][ j ] shall be in the range of 0 to 1023, inclusive.

applicable_op_num_depth_views_minus1[ i ][ j ] plus 1 specifies the number of depth views required for decoding the target output views corresponding to the j-th operation point to which the level indicated by level_idc[ i ] applies. The number of depth views specified by applicable_op_num_depth_views_minus1 includes the depth views of the target output views and the depth views that the depth views of the target output views depend on as specified by sub-bitstream extraction process in I.8.5 with tIdTarget equal to applicable_op_temporal_id[ i ][ j ] and viewIdTargetList equal to the list of applicable_op_target_view_id[ i ][ j ][ k ] for all k in the range of 0 to applicable_op_num_target_views_minus1[ i ][ j ], inclusive, as inputs. The value of applicable_op_num_depth_views_minus1[ i ][ j ] shall be in the range of 0 to 1023, inclusive.

All sequence parameter set 3DVC extensions that are included in the active view 3DVC sequence parameter set RBSPs of one coded video sequence shall be identical.

- I.8	3DVC decoding process

…

The target output views are either specified by external means not specified in this Specification, or, when not specified by external means, there shall be one target output view which is the base view. Let OutputVOIdxList be the list of VOIdx values, in increasing order of VOIdx, of all the target output views in one access unit. The list OutputVOIdxList shall not change within a coded video sequence.

NOTE – The association of VOIdx values to view_id values according to the decoding process of clause I.8 may differ from that of the decoding process of clause H.8.

For profiles specified in Annex I, a target output view includes either only a texture view, when no depth view with the same value of view_id is present, or both the texture view and the depth view, which have the same view_id value VOIdx. [Ed. Consider moving this sentence to I.10, as it is a profile constraint.]

## DPB Support for Unpaired Texture and Depth Views

Annex C

…

The CPB size (number of bits) is CpbSize[ SchedSelIdx ]. The DPB size (number of frame buffers) is Max( 1, max_dec_frame_buffering ). When the coded video sequence conforms to one or more of the profiles specified in Annex Error: Reference source not found and the decoding process specified in Annex Error: Reference source not found is applied, the DPB size is specified in units of view components. When the coded video sequence conforms to one or more of the profiles specified in Annex I and the decoding process specified in Annex I is applied, the DPB is operated separately for texture view components and depth view components and terms texture DPB and depth DPB are used respectively. The texture DPB size is specified in units of texture view components and the depth DPB size is specified in units of depth view components the DPB size is specified in units of texture view components and depth view components.

…

- C.2	Operation of the decoded picture buffer (DPB)

The decoded picture buffer contains frame buffers. When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, each of the frame buffers may contain a decoded frame, a decoded complementary field pair or a single (non-paired) decoded field that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures). When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, each frame buffer may contain a decoded frame, a decoded complementary field pair, a single (non-paired) decoded field, a decoded reference base frame, a decoded reference base complementary field pair or a single (non-paired) decoded reference base field that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures). When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, each of the frame buffers may contain a decoded frame view component, a decoded complementary field view component pair, or a single (non-paired) decoded field view component that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures) or is held as reference for inter-view prediction (inter-view only reference components). When a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I, each of the frame buffers of the texture DPB may contain a decoded texture frame view component, a decoded depth frame view component, a decoded complementary texture field view component pair, or a single (non-paired) decoded texture field view component that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures) or is held as reference for inter-view prediction (inter-view only reference components). When a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I, each of the frame buffers of the depth DPB may contain a decoded depth frame view component, a decoded complementary depth field view component pair, or a single (non-paired) decoded depth field view component that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures) or is held as reference for inter-view prediction (inter-view only reference components).

…

- C.4.1	Operation of the output order DPB

The decoded picture buffer contains frame buffers. When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in clauses Error: Reference source not found-Error: Reference source not found, each of the frame buffers may contain a decoded frame, a decoded complementary field pair or a single (non-paired) decoded field that is marked as "used for reference" or is held for future output (reordered pictures). When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, each frame buffer may contain a decoded frame, a decoded complementary field pair, a single (non-paired) decoded field, a decoded reference base frame, a decoded reference base complementary field pair or a single (non-paired) decoded reference base field that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures). When a coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found is decoded by applying the decoding process specified in Annex Error: Reference source not found, each of the frame buffers may contain a decoded frame view component, a decoded complementary field view component pair, or a single (non-paired) decoded field view component that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures) or is held for inter-view prediction (inter-view only reference components). When a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I, each of the frame buffers of the texture DPB may contain a decoded texture frame view component, a decoded depth frame view component, a decoded complementary texture field view component pair, a single (non-paired) decoded texture field view component that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures) or is held for inter-view prediction (inter-view only reference components). When a coded video sequence conforming to one or more of the profiles specified in Annex I is decoded by applying the decoding process specified in Annex I, each of the frame buffers of the depth DPB may contain a decoded depth frame view component, a decoded complementary depth field view component pair, or a single (non-paired) decoded depth field view component that is marked as "used for reference" (reference pictures) or is held for future output (reordered or delayed pictures) or is held as reference for inter-view prediction (inter-view only reference components).

…

- C.4.5.3	"Bumping" process

When the decoding process specified in Annex Error: Reference source not found is applied, the process specified in this subclause is invoked for a particular view with view order index VOIdx, with "picture" being replaced by "view component", "frame" being replaced by "frame view component", and "field" being replaced by "field view component". During the invocation of the process for a particular view, only view components of the particular view are considered and frame buffers containing view components of other views are not emptied. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain texture view components and depth view components of the current view and other views.

When the decoding process specified in Annex I is applied, the process specified in this subclause for Annex Error: Reference source not found is invoked for particular texture view and depth view with view order index VOIdx, with each "view component" being replaced by "texture view component" or "depth view component", "frame view component" being replaced by "texture frame view component" or "depth frame view component", and "field view component" being replaced by "texture field view component". During the invocation of the process for a particular texture view, only the texture view components of the particular view are considered while respective depth view components may be cropped and output too. During the invocation of the process for a particular depth view, only the depth view components of the particular view are considered and frame buffers containing view components of other views are not emptied. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain view components of other views. The DPB fullness represents the total number of non-empty frame buffers, including frame buffers that contain view components of other views, for the texture DPB or the depth DPB depending on whether the process is invoked for a texture view or a depth view, respectively.

The "bumping" process is invoked in the following cases.

–	There is no empty frame buffer (i.e., DPB fullness is equal to DPB size) and an empty frame buffer is needed for storage of an inferred "non-existing" frame, as specified in subclause Error: Reference source not found.

–	The current picture is an IDR picture and no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, as specified in subclause Error: Reference source not found.

–	The current picture has memory_management_control_operation equal to 5, as specified in subclause Error: Reference source not found.

–	There is no empty frame buffer (i.e., DPB fullness is equal to DPB size) and an empty frame buffer is needed for storage of a decoded (non-IDR) reference picture or a reference base picture, as specified in subclause Error: Reference source not found.

–	There is no empty frame buffer (i.e., DPB fullness is equal to DPB size) and the current picture is a non-reference picture that is not the second field of a complementary non-reference field pair and the current picture has OutputFlag equal to 1 and there are pictures in the DPB that are marked as "needed for output" that precede the current non-reference picture in output order, as specified in subclause Error: Reference source not found, so an empty buffer is needed for storage of the current picture.

–	There is no empty frame buffer (i.e., DPB fullness is equal to DPB size) and the current picture is a non-reference picture that is not the second field of a complementary non-reference field pair and the current picture has StoreInterViewOnlyRefFlag equal to 1, as specified in subclause Error: Reference source not found, so an empty buffer is needed for storage of the current picture.

The "bumping" process consists of the following ordered steps:

- The picture or complementary reference field pair that is considered first for output is selected as follows:

- The frame buffer is selected that contains the picture having the smallest value of PicOrderCnt( ) of all pictures in the DPB marked as "needed for output".

- Depending on the frame buffer, the following applies:

–	If this frame buffer contains a complementary non-reference field pair with both fields marked as "needed for output" and both fields have the same PicOrderCnt( ), the first of these two fields in decoding order is considered first for output.

–	Otherwise, if this frame buffer contains a complementary reference field pair with both fields marked as "needed for output" and both fields have the same PicOrderCnt( ), the entire complementary reference field pair is considered first for output.

NOTE – When the two fields of a complementary reference field pair have the same value of PicOrderCnt( ), this "bumping" process will output these pictures together, although the two fields have different output times from a decoder that satisfies output timing conformance criteria (as specified in subclause Error: Reference source not found).

–	Otherwise, the picture in this frame buffer that has the smallest value of PicOrderCnt( ) is considered first for output.

- Depending on whether a single picture or a complementary reference field pair is considered for output, the following applies:

–	If a single picture is considered first for output, this picture is cropped, using the cropping rectangle specified in the active sequence parameter set for the picture, the cropped picture is output, and the picture is marked as "not needed for output".

–	Otherwise (a complementary reference field pair is considered first for output), the two fields of the complementary reference field pair are both cropped, using the cropping rectangle specified in the active sequence parameter set for the pictures, the two fields of the complementary reference field pair are output together, and both fields of the complementary reference field pair are marked as "not needed for output".

- If there is a single depth view component or a complementary depth view component pair having the same values of view_id and PicOrderCnt( ) as the single picture or the complementary reference field pair considered for output, the single depth view component or the complementary depth view component pair are output as in step 2.

- The frame buffer that included the picture or complementary reference field pair that was cropped and output is checked, and when any of the following conditions are true, the frame buffer is emptied and the DPB fullness is decremented by 1:

–	The frame buffer contains a non-reference non-paired field.

–	The frame buffer contains a non-reference frame.

–	The frame buffer contains a complementary non-reference field pair with both fields marked as "not needed for output".

–	The frame buffer contains a non-paired reference field marked as "unused for reference".

–	The frame buffer contains a reference frame with both fields marked as "unused for reference".

–	The frame buffer contains a complementary reference field pair with both fields marked as "unused for reference" and "not needed for output".

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).