| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0152 |
| --- | --- |

| Title: | 3DV-ATM: On disparity map coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulu Chen (USTC) Dmytro Rusanovskyy Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

In this document, we propose to enable in 3DV-ATM a mixed depth map/disparity map coding. Simulation results show proposed that this type of coding is beneficial some of test sequences in CTC set, gain of 0.3% of dBR on average for synthesized results is observed.  Changes required to support this mode are limited to the encoder side algorithm and to high level of decoding operations.

# Introduction

As a result of the adopted contribution JCT3V-A0140, 4 types of ranging information can be used in MVC+D bitstreams, see Table 1.

Table 1. Types of ranging information supported in MVC+D

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each luma pixel value in coded frame of depth views represents an inverse of Z value normalized in range from 0 to 255 |
| 1 | Each luma pixel value in coded frame of depth views represents disparity normalized in range from 0 to 255 |
| 2 | Each luma pixel value in coded frame of depth views represents Z value normalized in range from 0 to 255 |
| 3 | Each luma pixel value in coded frame of depth views represents nonlinearly mapped disparity, normalized in range from 0 to 255. |

Due to extensive use of depth information for coding of texture, enabling support for that plurality of types in 3DV-ATM may be not desirable. However, two types of ranging information, namely depth map and disparity, have a special interest due it their common representation.

# Proposal

In this document, we propose a concept of mixed depth map/disparity coding for 3DV-ATM.

This proposal is based on the assumption, that depth map to disparity map conversion is performed as specified in JCT3V-B0150. Conversion is implemented through a linear model of correspondence between depth map value d and disparity D as it shown in (1):

D = (w * d + o ) >> n,					(1)

where d is a depth sample value, w is a scale factor, o is an offset value , and n is a shift parameter that depends on the required accuracy of the disparity vectors. Parameters of conversion w and o are transmitted within a bitstream with conventional variable length coding.  For every pair of views (source view and target view) utilized in joint coding an independent set of parameters w and o are transmitted. In addition to this, parameters of w and o can be re-send for every coded frame if parameters of depth map are varying, e.g. GT_Fly sequence.

With this representation, a depth map becomes a special case of the disparity (1) with w=1 and o = n = 0.

Thus, depth map and disparity map coding support in 3DV-ATM is straightforward under the assumption that disparity is represented with 255 level of quantization.

However, study of CTC test material, show that parameters of disparity are temporally varying and for some frames it cannot be represented with 8 bits.

We propose that an optimal representation of the ranging data is selected at the encoder side for every frame, e.g. depth map or disparity map of a particular accuracy. Following the selection, an image of input depth map data may be converted to a disparity image and coded as a disparity map. The type of utilized depth map representation is signaled to the decoder. Decoder up-dates the parameters w and o utilized for in-loop disparity conversion and for output, based on the type of coded data.

Proposed changes are limited to the encoder side algorithm and to logic related for look up table production with equation (1).

# Simulation results

The proposed scheme was implemented in 3DV-ATM v5.1r2.

In the tested scheme, encoder analyzes input depth map data and selects for it a disparity map representation, if the dynamical range of the resulting disparities belongs to 0…127 range (disparity limited to a half pixel accuracy). If disparity range exits the predefined range 0…127, input ranging data is coded as a depth map.

Table 1 Simulation results with mixed depth/disparity coding, disparity map range 0…127.

|  | Texture Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 0.10 | 0.00 | -0.63 | 0.02 | 0.85 | -0.03 |
| S02 | 0.44 | -0.01 | -1.59 | 0.05 | 0.10 | 0.00 |
| S03 | 0.73 | -0.02 | 0.20 | -0.01 | 4.41 | -0.13 |
| S04 | 0.48 | -0.02 | -1.45 | 0.05 | -0.08 | 0.00 |
| S05 | 0.58 | -0.03 | -7.62 | 0.36 | -4.67 | 0.20 |
| S06 | 0.59 | -0.03 | -5.15 | 0.25 | -2.05 | 0.09 |
| S08 | 0.23 | -0.01 | -5.38 | 0.23 | -0.64 | 0.02 |
| Average | 0.45 | -0.02 | -3.09 | 0.14 | -0.30 | 0.02 |

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).