| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0153 |
| --- | --- |

| Title: | 3D-CE5.a related results on temporal motion vector prediction in dependent view |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Kwan-Jung Oh, Jaejoon Lee, and Du-Sik Park | Emil: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Depth-based motion vector prediction (M24824, JCT2-A0045, and JCT2-A0046) was integrated on the current 3DV-ATM. This proposal introduces a temporal motion vector prediction method in dependent view. It is shown that the proposed method achieves the coding gain of -0.11% in only texture and overall coding. In some sequence, the bit rate is saved up to -0.19%.

# Algorithm Description

For the current temporal prediction, temporal motion vectors of the adjacent blocks around the current block are employed. If a temporal motion vector is unavailable, a zero vector is used for the unavailable block. On the contrary, inter-view motion vectors of the neighboring blocks are used for the inter-view prediction. If an inter-view motion vector is unavailable, a max disparity derived from the corresponding depth block is used (M24824 and JCT2-A0046).

In the proposed design, a concept of an inter-view motion vector in Fig. 1 (JCT2-A0045) is applied to the temporal motion vector prediction.

Fig. 1. Inter-view motion vector derivation

In the temporal prediction, when the temporal motion vector is unavailable in the neighboring block A, B, or C, the proposed method employs the inter-view motion vector for the unavailable blocks instead of the zero vector as displayed in Fig. 2. A max disparity which is associated with Cb is used for the inter-view motion vector derivation. The max disparity can be calculated from a max depth value among four corner depth samples (JCT2-A0040 and JCT2-A0046).

Fig. 2. Temporal motion vector prediction

# Test Results

Test results about the temporal motion vector prediction method in the dependent view are summarized in Table 1. In the proposed method, about -0.11% coding performance is obtained for the only texture and overall coding, respectively. Especially, -0.19% coding gain is achieved in some sequence.

Table 1. Summary results

# Conclusion

The temporal motion vector prediction was proposed. By integrating the proposed method on the current 3DV-ATM, it achieved the coding gain. Since the modifications are very simple but improve the coding performance, it is recommended to adopt the proposed method to the next 3DV-ATM.

# Patent Rights Declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).