| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0160 |
| --- | --- |

| Title: | Enabling CU early termination(ECU) at depth coding in common test conditions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ilsoon Lim Jaejoon Lee 14-1 Nongseo-dong Giheung-gu Yongin-si Gyeonggi-do 446-712 South Korea | Tel: Email: | +82 31 280 6688 ilsoon.lim@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes to enable CU early termination (ECU) [1], which is adopted in HM and 3DV-HTM, for depth coding in the common test conditions of 3DV-HTM. The simulation results show that to enable CU early termination (ECU) brings coding gain and encoding time reduction at once.

# Introduction

Choi et. al, have already proposed a fast CU decision method (encoder only) for HEVC by early determination of the CU size based on coding tree pruning. ECU option currently is disabled in texture coding and depth coding. Although ECU in texture coding results in the coding loss for the benefit of the encoding time reduction, it brings not only the encoding time reduction but also coding gain in depth coding. Because, the trade-offs between encoding time and coding gain differs each other in texture coding and depth coding. This CU early termination is the existing module and the encoder issue only, and thus we can obtain the coding gain or encoding time reduction without any endeavour. Experimental results shows -2.21% (decoded view) and 1.16% (rendered view) BD-rate for 3 view case, and the ratio of encoding time was no more than 68.5% compared to anchor (3DV-HTM ver.4.0.1).

# Test results

Tests were based on the common test condition for the 3view configuration [2]. The results are summarized in Table 1 and Table 2.

Table 1 BD rate results for 3-view case

|  | video 0 | video 1 | video 2 | video only | coded only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -2.1% | 1.1% | 0.1% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -2.1% | 1.3% | 0.2% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | -2.7% | 2.2% | 0.5% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -2.4% | 1.7% | 0.3% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | -2.7% | -0.1% | -0.9% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | -1.5% | 0.3% | -0.3% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | -2.0% | 1.7% | 0.5% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -2.3% | 1.5% | 0.2% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -2.1% | 0.9% | -0.1% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -2.21% | 1.16% | 0.05% |

Table 2 Complexity results for 3-view case

|  | enc time | dec time | ren time |
| --- | --- | --- | --- |
| Balloons | 67.8% | 98.0% | 100.0% |
| Kendo | 71.0% | 107.7% | 100.0% |
| Newspapercc | 67.6% | 99.1% | 100.0% |
| GhostTownFly | 67.7% | 101.9% | 100.0% |
| PoznanHall2 | 69.2% | 101.8% | 100.0% |
| PoznanStreet | 66.4% | 102.0% | 100.0% |
| UndoDancer | 70.1% | 94.1% | 100.0% |
| 1024x768 | 68.8% | 101.5% | 100.0% |
| 1920x1088 | 68.3% | 99.9% | 100.0% |
| average | 68.5% | 100.6% | 100.0% |

# Conclusion

In this document, we conduct the simulation for CU early termination at depth coding in 3DV-HTM. Experimental results shows -2.21% (decoded view) and 1.16% (rendered view) BD-rate for 3 view case, compared to anchor and the encoding time was reduced no less than 30%. We recommend to enable CU early termination at common test conditions in 3DV-HTM

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. doesn’t have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Kiho Choi, Sang-Hyo Park, Euee S. Jang, “Coding tree pruning based CU early termination,” JCTVC-F092, JCT-VC 6th Meeting: Torino, IT, 14-22 July, 2011

- “Common Test Conditions for 3DV experimentation,” JCT2-A1100, JCT-3V 1st Meeting , Stockholm, SE, 16-20 July 2012.