| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0161 |
| --- | --- |

| Title: | [Depth deblocking filter for HEVC based 3D video coding] |  |  |
| --- | --- | --- | --- |
| Status: | [Input document] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Kwan-Jung Oh, Jaejoon Lee, Du-Sik Park | Email: | kwanjung.oh@samsung.com jaejoon1.lee@samsung.com dusikpark@samsung.com |
| Source: | [Samsung Electronics Co., Ltd] |  |  |

_____________________________

# Abstract

Modifications on boundary strength and threshold table for depth deblocking filter are proposed to improve coding efficiency. It achieves up to 0.5% and average 0.2 % coding gain.

# Proposed modifications

The current deblocking filter is designed for texture, thus it is not good for depth coding. The current HEVC based 3D uses VSO to optimize the relation between depth bit-rate and rendering quality. Thus, excessive deblocking filter can degrades rendering quality and increase the depth bit-rate. In this proposal, the depth deblocking filter is enabled with simple modification. The depth deblocking filter becomes weak by changing the boundary strength and threshold table for filtering process.

- if ( (pcCUQ->getCbf( ) == 0 && pcCUP->getCbf() == 0) )

uiBs = 0;

- iTc  = (tctable_8x8[iIndexTC]*iBitdepthScale)/k;

iBeta = (betatable_8x8[iIndexB]*iBitdepthScale)/k;

# Coding experiments

Results of the coding experiment are given in Table  and Table 2. It can be seen that run time is slightly increased and a rate reduction of about 0.2% and 0.1% can be achieved. The results for the convectional deblocking filter for depth are given in Table 3. Additional results can be found in the attached excel sheet.

Table 1: coding results for proposed modifications (Bs modification)

|  | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.2% | -0.1% | 100.4% | 103.4% |
| Kendo | 0.0% | -0.1% | -0.1% | 98.2% | 104.7% |
| Newspapercc | 0.0% | -0.2% | -0.2% | 101.9% | 102.4% |
| GhostTownFly | 0.0% | 0.1% | 0.0% | 100.7% | 103.4% |
| PoznanHall2 | 0.0% | 0.1% | 0.1% | 102.8% | 102.5% |
| PoznanStreet | 0.0% | -0.2% | -0.1% | 99.4% | 103.9% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 99.0% | 102.9% |
| 1024x768 | 0.0% | -0.2% | -0.1% | 100.1% | 103.5% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 100.5% | 103.2% |
| average | 0.0% | -0.1% | -0.1% | 100.3% | 103.3% |

Table 2: coding results for proposed modifications (Thershold, k=3)

|  | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.3% | -0.2% | 100.4% | 102.3% |
| Kendo | 0.0% | -0.4% | -0.3% | 100.2% | 102.6% |
| Newspapercc | 0.0% | -0.5% | -0.4% | 100.2% | 101.2% |
| GhostTownFly | 0.0% | -0.1% | -0.1% | 102.1% | 102.8% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 100.5% | 102.6% |
| PoznanStreet | 0.0% | -0.3% | -0.2% | 100.5% | 102.4% |
| UndoDancer | 0.0% | 0.5% | 0.3% | 100.6% | 102.2% |
| 1024x768 | 0.0% | -0.4% | -0.3% | 100.3% | 102.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 100.9% | 102.5% |
| average | 0.0% | -0.2% | -0.1% | 100.6% | 102.3% |

Table 3: coding results for proposed modifications (Depth deblocking On)

|  | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.2% | 0.1% | 101.5% | 104.2% |
| Kendo | 0.0% | -0.4% | -0.3% | 101.1% | 105.6% |
| Newspapercc | 0.0% | 0.8% | 0.5% | 100.9% | 105.0% |
| GhostTownFly | 0.0% | 0.6% | 0.4% | 101.9% | 104.4% |
| PoznanHall2 | 0.0% | 1.1% | 0.7% | 100.9% | 103.8% |
| PoznanStreet | 0.0% | 0.1% | 0.1% | 100.5% | 104.9% |
| UndoDancer | 0.0% | 3.5% | 2.5% | 100.4% | 104.8% |
| 1024x768 | 0.0% | 0.2% | 0.1% | 101.2% | 104.9% |
| 1920x1088 | 0.0% | 1.4% | 0.9% | 100.9% | 104.5% |
| average | 0.0% | 0.9% | 0.6% | 101.0% | 104.7% |

# Conclusion

Modification on depth deblocking filter has been suggested. It achieves about 0.2% and 0.1% coding gains with slight run time increase.

# Patent right declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).