| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0162 |
| --- | --- |

| Title: | 3DV-AVC HLS: On depth acquisition type |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seok Lee, Seungsin Lee, Hocheon Wey, Jaejoon Lee San 14, Nongseo-dong, Giheung-gu, Yongin-si, Gyeonggi-do | Tel: Email: | +82-31-280-6696 lee.seok@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a modification on the depth acquisition information SEI message and a definition of depth acquisition type which specifies depth map generation method. The proposed depth acquisition type includes three kinds of acquisition method: extraction from 3D graphics, estimation from texture data and measurement using depth camera. Since the characteristics of depth information are different according to how it was generated, this contribution proposes to define depth acquisition type as a part of the SEI message in [1] [2]. The usefulness of proposed information is examined by hybrid post filtering for depth [3].

# Depth Characteristics

The depth data estimated using color images or measured from depth sensors such as TOF cameras have different characteristics from the graphics depth data. For graphics data, the exact depth information can be extracted from 3D models and edge correlation between texture and depth images is perfect. On the other hand, the estimated and measured depth contains noise and accuracy can be low depending on the sequence and algorithm, and the edge is not guaranteed to be matched with corresponding texture data. In addition, the depth estimation is generally implemented based on disparity whose degree of accuracy is usually half pel or quarter pel. The lack of accuracy of the depth estimation results in discrete changes of depth values even in flat regions unlike graphics depth data which smoothly varies in non edge regions. Fig. 1 is the example of graphics depth (Undo Dancer) and estimated depth (Kendo) and corresponding histograms. In (b) the shape of human and background objects is not clear compared to (a). Figure (c) and (d) show the edge correlation between depth and texture information. Red lines are edge map of each depth (a) and (b), and it is perfectly aligned in (c), whereas depth edges in (d) are not matched to texture edges and even exist in wrong regions such as flat background. (e) and (f) are histograms of each depth and in the case of graphics (e), the depth value is distributed widely from 0 to 140, on the other hand pixel intensity is concentrated on several values in (f).

(a) Graphics depth (Undo Dancer)                      (b) Estimated depth (Kendo)

(c) Edge alignment with texture                        (d) Edge alignment with texture

(e) Histogram for (a)                                  (f) Histogram for (b)

Fig. 1 Example of graphics and estimated depth

For these different characteristics of depth data, it is helpful to synthesize higher quality virtual views if the type of transferred depth map is known.

# Hybrid Depth Post Filtering using Proposed SEI Message

In this section, experimental result for hybrid post filtering method using proposed acquisition type information is described. There is dilation based post filter in current ATM software [4], but in this experiment different filtering approaches are applied according to the type of depth map [3]. Gray scale dilation filter is applied only to graphics depth data to directly remove artifact in synthesized view, and segmentation based filter is applied to estimated depth data to enhance edge and to reduce the compression noise. Result in table 1 using ATM v5.0 shows that the -1.296% BD-rate gain in terms of synthesized PSNR is obtained for 7 test sequences.

The results suggest that the hybrid post filtering can improve the performance with depth type identification.

Table 1 Test results for ATM 5.0 with proposed SEI message

| Sequences | BD-rate for synthesized views (%) | BD-PSNR for synthesized views (dB) |
| --- | --- | --- |
| Poznan Hall2 | -2.265 | 0.077 |
| Poznan Street | 0.078 | -0.003 |
| Undo Dancer | 0.000 | 0.000 |
| GT Fly | 0.000 | 0.000 |
| Kendo | -1.448 | 0.063 |
| Balloons | -2.333 | 0.108 |
| Newspaper | -3.105 | 0.119 |
| Average | -1.296 | 0.052 |

# Proposed Syntax and Semantics

The proposed syntax and its semantics are presented in below.

      - Depth acquisition information SEI message syntax

| depth_acquisition_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_acquisition_type | 5 | ue(v) |
| all_views_equal_flag | 5 | u(1) |
| if( all_views_equal_flag = = 0 ) { |  |  |
| num_views_minus1 | 5 | ue(v) |
| numViews = num_views_minus1 + 1 |  |  |
| } else { |  |  |
| numViews = 1 |  |  |
| num_frames | 5 | ue(v) |
| } |  |  |
| z_near_flag | 5 | u(1) |
| z_far_flag | 5 | u(1) |
| d_min_flag | 5 | u(1) |
| d_max_flag | 5 | u(1) |
| focal_length_x_flag | 5 | u(1) |
| focal_length_y_flag | 5 | u(1) |
| principal_point_x_flag | 5 | u(1) |
| principal_point_y_flag | 5 | u(1) |
| translation_flag | 5 | u(1) |
| rotation_flag | 5 | u(1) |
| for( i = 0; i < numViews; i++ ) { |  |  |
| if( z_near_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 0, 7, ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |  |
| if( z_far_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |  |
| if( d_min_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |  |
| if( d_max_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |  |
| if( focal_length_x_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 1, 6, FocalLengthXSign, FocalLengthXExp, FocalLengthXMantissa, FocalLengthXManLen ) |  |  |
| if( focal_length_y_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, FocalLengthYSign, FocalLengthYExp, FocalLengthYMantissa, FocalLengthYManLen ) |  |  |
| if( principal_point_x_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, PrincipalPointXSign, PrincipalPointXExp, PrincipalPointXMantissa, PrincipalPointXManLen ) |  |  |
| if( principal_point_y_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, PrincipalPointYSign, PrincipalPointYExp, PrincipalPointYMantissa, PrincipalPointYManLen ) |  |  |
| if( translation_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, TXSign, TXExp, TXMantissa, TXManLen ) |  |  |
| if( rotation_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, YawSign, YawExp, YawMantissa, YawManLen ) |  |  |
| } |  |  |
| } |  |  |

      - Depth acquisition information SEI message semantics

depth_acquisition_type specifies the depth map generation method as specified in Table I-XX.

Table I-XX Definition of depth_acquisition_type

| depth_acquisition_type | Interpretation |
| --- | --- |
| 0 | Each frame of depth views extracted from 3D graphics data |
| 1 | Each frame of depth views estimated from texture data |
| 2 | Each frame of depth views measured by depth cameras |
| Other values | Reserved for future use |

# Conclusion

Additional depth acquisition information of depth acquisition type which represents depth map generation method is proposed for the purpose of using at post processing, i.e. view synthesis. We recommend adopting the proposed modification on depth acquisition information SEI message.

# Patent rights declaration(s)

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- ISO/IEC JTC1/SC29/WG11, “MVC Extension for Inclusion of Depth Maps Draft Text 4”, ISO/IEC JTC1/SC29/WG11 JCT3V-A1001, July 2012, Stockholm, SE.

- ISO/IEC JTC1/SC29/WG11, “3D-AVC Draft Text 3”, ISO/IEC JTC1/SC29/WG11 JCT3V-A1002, July 2012, Stockholm, SE.

- Seungsin Lee, Seok Lee, Hocheon Wey and Jaejoon Lee, "3D-CE3.a Related results on depth hybrid post filter ", ISO/IEC JTC1/SC29/WG11 JCT3V-A0039, July 2012, Stockholm, SE.

- Seok Lee, Seungsin Lee, Hocheon Wey and Jaejoon Lee, "3D-CE3.a results on dilation filter for depth post processing ", ISO/IEC JTC1/SC29/WG11 JCT3V-A0038, July 2012, Stockholm, SE.