| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0177 |
| --- | --- |

| Title: | 3D-CE1.h cross-check on View Synthesis Prediction (JCT3V-B0102) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Lin Chang +886-3-5670766 ext. 25314 yulin.chang@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-check result for the proposal of NTT and MERL on view synthesis prediction in 3D-HEVC. The test case c and d were cross-checked in this contribution. The verification tasks were done successfully, and the results match those provided by NTT and MERL.

# Verification

NTT and MERL provided the HTM source codes of their proposed codec described in JCT3V-B0102 [1]. There are five test cases in JCT3V-B0102 as show in Table 1. Case c and case d were cross-checked in this contribution. The verification is carried out by setting the following macros in the provided code:

Case c:

#define VSP_MV_ZERO 0

#define VSP_FRAME_INTERVAL 2

Case d:

#define VSP_MV_ZERO 0

#define VSP_FRAME_INTERVAL 1

Table 1: Test cases summary

|  | VSP including VSP skip | VSP used | MV info for VSP | VSP for depth coding | VSO setting |
| --- | --- | --- | --- | --- | --- |
| Anchor | N/A | N/A | N/A | N/A | CTC |
| Case a | X | ref only | No motion info | X | CTC |
| Case b | X | ref and non-ref | No motion info | X | CTC |
| Case c | X | ref only | With motion info | X | CTC |
| case d | X | ref and non-ref | With motion info | X | CTC |
| case h | X | ref only | With motion info |  | CTC |

The simlations are conducted on the common test condiciton [2].

# Results

The BD-rates from our experimental results match those provided by NTT and MERL. The encoding time/decoding time of the anchor and the tests were measured independently, so they can only show a trend of the computational complexity. The detailed results can be seen in JCT3V-B0177_TEST_C.xls and JCT3V-B0177_TEST_D.xls.

Table 2 Experiments results

- Case c (Except for non-reference frames, coding vector info., both texture and depth, CTC VSO)

- Case d (All frames, coding vector info., both texture and depth, CTC VSO)

# Conclusion

The results presented by NTT and MERL in JCT3V-B0102 are confirmed.

# References

- F. Zou, D. Tian, A. Vetro, S. Shimizu, S. Sugimoto, “CE1.h: Results on View Synthesis Prediction”, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B0102, Oct. 2012, Shanghai.

- Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, “Common Test Conditions of 3DV Core Experiments”,  Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-A1100, July 2012, Stockholm.