| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0178r2 |
| --- | --- |

| Title: | MVC+D: Enhanced 3DVC reference displays information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao and Lu Yu Zhejiang University #38 Zheda Road, Hangzhou, China Andrey Nokin Ericsson Färögatan 6, 164 80 Stockholm, Sweden | Tel: Email: Tel: Email: | +86 571 87953107 {zhaoyin,yul}@zju.edu.cn +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Zhejiang University and Ericsson |  |  |

_____________________________

# Abstract

The 3DVC reference displays information SEI message, adopted in JCT3V-A0163, specifies the baseline for one or more reference displays. This proposal suggest further including view information that explicitly defines a pair of views corresponding to the reference baseline, which helps 1) avoid window violation and 2) maintain image quality of the presented stereo-pair.

# Problem Setting

The 3DVC reference displays information SEI message, adopted in JCT3V-A0163, specifies the baseline for one or more reference displays, which is a minimum set for indicating the depth sensation from a recommended stereo-pair. In other words, any stereo-pair corresponding to the signalled reference baseline is considered to have suitable depth sensation for the reference display.

However, window violation that a popping-out object hit display margin may occur in one stereo-pair, but may be avoided by using another stereo-pair (with the same baseline) consisting of two other viewpoints, as shown in Fig. 1(a). Besides, when the views are coded asymmetrically, one stereo pair including the higher-quality view can have better image quality (thus better 3D visual quality), as shown in Fig. 1(b). Therefore, it is beneficial to include the view information in addition to the already adopted baseline information.

(a)						(b)

Fig. 1. (a) Illustration of window violation by a stereo pair of two sub-optimal views. The object appears at the left margin of the display when stereo-pair B is presented, while stereo-pair A does not have this problem. (b) Adding view information may benefit the display side by informing a stereo pair of better image quality in asymmetric coding: stereo-pair C is better than stereo-pair D in terms of visual quality.

# Proposal

It is proposed to further signal the left and right reference view information of a reference stereo pair that corresponds to the reference baseline. With the reference view information, the display side can explicitly know which two views should be selected to form the presented stereo-pair, instead of guessing (or figuring out) one stereo-pair of the reference baseline when the reference view information is absent. With explicit view information, the window violation may be avoided while the same image quality expected by the content provider can be guaranteed at the decoder side in the case of asymmetric coding of multiview texture sequences.

The enhanced syntax supports two methods to signal the left and right reference view information of a reference stereo-pair:

Method 1: the left view id and the right view id. (Note: in two-view case, the left and right views are the views of VOIdx = 0 and VOIdx = 1 by default, which need not be signalled).

Method 2: one view id + baseline + whether the signalled view is on the left of the other view of the reference stereo-pair which is inferred from the baseline. (Note: in two-view MVD case, the view id can be replaced by a flag indicating one of the two views of VOIdx = 0 and VOIdx = 1)

The former is straightforward for a reference stereo pair consisting of two coded views, and the latter is suitable for a reference stereo pair consisting of a coded view and a synthesized virtual view.

A show case is prepared to demonstrate the benefit from adding the proposed view information to the adopted 3DVC reference displays information SEI message, although the benefit of avoiding window violation and maintaining image quality is quite obvious.

# Specification text

The proposed specification text changes are indicated with green highlights on top of the MVC Extension for Inclusion of Depth Maps Draft Text 4 (JCT3V-A1001). Besides, a few text improvement is indicated with yellow highlights.

J.13.1.5 3DVC reference displays information SEI message syntax

| 3d_reference_displays_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| two_view_flag | 5 | u(1) |
| prec_ref_baseline | 5 | ue(v) |
| prec_ref_display_width | 5 | ue(v) |
| ref_viewing_distance_flag | 5 | u(1) |
| if( ref_viewing_distance_flag ) |  |  |
| prec_ref_viewing_dist | 5 | ue(v) |
| num_ref_displays_minus1 | 5 | ue(v) |
| numRefDisplays = num_ref_displays_minus1 + 1 |  |  |
| for( i = 0; i < numRefDisplays; i++ ) { |  |  |
| ref_stereo_pair_mode_flag[ i ] | 5 | u(1) |
| if (ref_stereo_pair_mode_flag[ i ] ) { |  |  |
| if(!two_view_flag) { |  |  |
| left_ref_view_id[ i ] | 5 | ue(v) |
| right_ref_view_id[ i ] | 5 | ue(v) |
| } |  |  |
| }else{ |  |  |
| exponent_ref_baseline[ i ] | 5 | u(6) |
| mantissa_ref_baseline[ i ] | 5 | u(v) |
| preferred_view_flag[ i ] | 5 | u(1) |
| if (preferred_view_flag[ i ] ){ |  |  |
| if (!two_view_flag) |  |  |
| preferred_view_id[ i ] | 5 | ue(v) |
| else |  |  |
| preferred_view_selection_flag[ i ] | 5 | u(1) |
| preferred_view_is_left_ref_view_flag[ i ] | 5 | u(1) |
| } |  |  |
| } |  |  |
| exponent_ref_display_width[ i ] | 5 | u(6) |
| mantissa_ref_display_width[ i ] | 5 | u(v) |
| if( ref_viewing_distance_flag ) { |  |  |
| exponent_ref_viewing_distance[ i ] | 5 | u(6) |
| mantissa_ref_viewing_distance[ i ] | 5 | u(v) |
| } |  |  |
| additional_horizontal_shift_present_flag[ i ] | 5 | u(1) |
| if (additional_horizontal _shift_present_flag[ i ]) |  |  |
| num_pixel_shift_plus128[ i ] | 5 | u(8) |
| } |  |  |
| } |  |  |

When present, this SEI message shall be associated with an IDR access unit. A reference displays information message contains information about the reference display width(s) and reference viewing distance(s) as well as information about the corresponding baseline distance(s) and additional horizontal image shift(s), which form a stereo-pair for the reference display width and the reference viewing distance. This information may also contain the left and right reference view information of a reference stereo-pair corresponding to the reference baseline for each reference display. This information enables a view renderer to produce a proper stereo-pair for the target screen width and the viewing distance. The reference display width and viewing distance values are signaled in units of centimeters. The reference baseline values shall be signaled in the same units as the x component of the translation vector in the SEI message depth acquisition information that is valid for the same access unit. When a reference displays information SEI message is present in an access unit, the depth acquisition information SEI message shall also be present in the same access unit. The baseline and shift information signaled for the reference display is valid for all access units they associated with and until the next IDR access unit or the next access unit containing depth acquisition information SEI message or reference displays information SEI message.

two_view_flag equal to 1 indicates the bitstream is a two-view bitstream, containing two coded views. two_view_flag equal to 0 indicates the bitstream is not a two-view bitstream, i.e., the number of views in the bitstream is not equal to 2.

ref_stereo_pair_mode_flag[ i ] equal to 1 indicates that the reference stereo-pair corresponding to the reference baseline of the i-th reference display consists of two coded views, namely the left and right reference views, and the reference baseline is the baseline between the two coded views. ref_stereo_pair_mode_flag[ i ] equal to 0 indicates that the reference stereo-pair corresponding to the reference baseline of the i-th reference display consists of a preferred coded view and a synthesized (virtual) view, or consists of two unspecified views. The baseline between the two views of the reference stereo-pair, i.e., the reference baseline, is signaled by a floating-point number consisting of three parts: precision, exponent and mantissa.

left_ref_view_id[ i ] indicates the view_id of the left reference view of the reference stereo-pair corresponding to the i-th reference display.

right_ref_view_id[ i ] indicates the view_id of the right reference view of the reference stereo-pair corresponding to the i-th reference display.

preferred_view_flag[ i ] equal to 1 indicates that the information about a preferred view of the reference stereo-pair corresponding to the i-th reference display is present in the bitstream. preferred_view_flag[i] equal to 0 indicates that the information about a preferred view of the reference stereo-pair corresponding to i-th reference display is not present in the bitstream.

preferred_view_id[ i ] indicates the view_id of the preferred view corresponding to the i-th reference display.

preferred_view_selection_flag[ i ] equal to 1 indicates that the view of VOIdx = 0 is the preferred view corresponding to the i-th reference display. preferred_view_selection_flag[ i ] equal to 0 indicates that the view of VOIdx = 1 is the preferred view corresponding to the i-th reference display.

preferred_view_is_left_ref_view_flag[ i ] equal to 1 indicates that the preferred view is the left reference view of the reference stereo-pair corresponding to the i-th reference display. preferred_view_is_left_ref_view_flag[ i ] equal to 0 indicates that the preferred view is the right reference view of the reference stereo-pair corresponding to the i-th reference display.

additional_horizontal_shift_present_flag[ i ] equal to 1 indicates that the information about additional horizontal shift of the left and right views for the i-th reference display is present in the bitstream. additional_horizontal_shift_present_flag[ i ] equal to 0 indicates that the information about additional horizontal shift of the left and right views for the i-th reference display is not present in the bitstream.

Note: the syntax elements in A0163 are a special case when ref_stereo_pair_mode_flag[i] = 0 and preferred_view_flag[i] = 0.

The simplified syntax without special support for the 2-view 3DV is provided in the Appendix.

# Suggested clean-up of introductory clause.

It is proposed to modify the introductory clause of the 3D reference displays SEI to message as in the following. The proposed modification explains the intended use of the 3D reference displays SEI to message in a more clear way. An additional change (which broadens the way of using this SEI message) is to allow sending the 3D reference displays SEI message not only for the IDR access units but also for the access unit corresponding to a scene cut. This functionaly would allow changing the intended reference setup at the scene cut.

      - 3D reference displays information SEI message semantics

When present, this SEI message should be associated with an IDR access unit or with an access unit corresponding to the scene-cut (scene shange). A reference displays information message contains information about the reference display width(s) and reference viewing distance(s) as well as information about the corresponding baseline distance(s) and additional horizontal image shift(s), which form a stereo-pair for the reference display width and the reference viewing distance. This information enables a view renderer to produce a proper stereo-pair for the target screen width and the viewing distance. The reference display width and viewing distance values are signaled in units of centimeters. The reference baseline values shall be signaled in the same units as the x component of the translation vector in the SEI message depth acquisition information that is valid for the same access unit. When a reference displays information SEI message is present in an access unit, the depth acquisition information SEI message shall also be present in the same access unit. The baseline and shift information signaled for the reference display is valid for all access units they associated with and until the next IDR access unit or the next access unit containing depth acquisition information SEI message or reference displays information SEI message.

NOTE – The reference displays information SEI message specifies display parameters for which the 3D sequence was optimized and the corresponding reference parameters. Each reference display (i.e. a reference display width and possibly a corresponding viewing distance) is associated with one reference baseline distance.

The following formulas can be used for calculating the baseline distance and horizontal shift for the receiver's display when the ratio between the receiver's viewing distance and the reference viewing distance is the same as the ratio between the receiver screen width and the reference screen width:

baseline = ref_baseline * ( ref_display_width ÷ display_width )

shift = ref_shift * ( ref_display_width ÷ display_width )

In the provided formulas, the width of the visible part of the display used for showing the video sequence should be understood under "display width". The same formulas can also be used for choosing the baseline distance and horizontal shift in cases when the viewing distance is not scaled proportionally to the screen width compared to the reference display parameters. In this case, the effect of applying these formulas would be to keep the perceived depth in the same proportion to the viewing distance as in the reference setup. The reference shift ref_shift denotes the sum of a difference between the principal point coordinate X of the two cameras located at the reference baseline distance and the additional shift signaled in this SEI message.

When camera parameters are updated by a depth acquisition information SEI message in a following access unit and the baseline between the views used in the view synthesis process in the following access unit changes relative to that in the access unit which the reference displays information SEI belongs to, the reference baseline and the reference horizontal shift in the following access unit should be recalculated. The baseline and the shift for the receiver's display in the following access unit are therefore updated based on the new values of the reference baseline and reference shift using the equations of this note. Let the scaling factor sb be equal to the ratio of the baseline between two views in the following access unit and the baseline between the same two views in the access unit which the reference displays information SEI message belongs to, where the two views are used in the view synthesis process. Then the reference baseline distance in the following access unit is obtained by multiplying with the scaling factor sb the reference baseline distance in the access unit which the reference displays information SEI belongs to. Let the scaling factor sh be equal to the ratio of the difference of the principal point coordinate X of the two views in the following access unit and the difference of the principal point coordinate X of the same two views in the access unit which the reference displays information SEI message belongs to. The reference horizontal shift in the following access unit is obtained by multiplying with the scaling factor sh the component of the reference shift obtained from the difference of the principal point coordinate X in the access unit which the reference displays information SEI belongs to and by keeping the additional horizontal shift signaled in this SEI constant.

# Conclusion

The proposed reference view information help reconstruct the 3D experience (including depth sensation, visual comfort, image quality) expected by content producer at the display side. We recommend adopting the enhanced syntax elements on reference view information.

It is also proposed to modify the semantics of the introductory clause of the SEI message to make them clearer and to allow sending the SEI message not only for an IDR access unit but also for an access unit corresponding to a scene-cut.

# Appendix

The syntax without special support for the 2-view 3DV is provided in this Appendix.

J.13.1.5 3DVC reference displays information SEI message syntax

| 3d_reference_displays_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| prec_ref_baseline | 5 | ue(v) |
| prec_ref_display_width | 5 | ue(v) |
| ref_viewing_distance_flag | 5 | u(1) |
| if( ref_viewing_distance_flag ) |  |  |
| prec_ref_viewing_dist | 5 | ue(v) |
| Num_ref_displays_minus1 | 5 | ue(v) |
| numRefDisplays = num_ref_displays_minus1 + 1 |  |  |
| for( i = 0; i < numRefDisplays; i++ ) { |  |  |
| ref_stereo_pair_mode_flag[ i ] | 5 | u(1) |
| if (ref_stereo_pair_mode_flag[ i ] ) { |  |  |
| left_ref_view_id[ i ] | 5 | ue(v) |
| right_ref_view_id[ i ] | 5 | ue(v) |
| }else{ |  |  |
| exponent_ref_baseline[ i ] | 5 | u(6) |
| mantissa_ref_baseline[ i ] | 5 | u(v) |
| preferred_view_flag[ i ] | 5 | u(1) |
| If (preferred_view_flag[ i ] ){ |  |  |
| preferred_view_id[ i ] | 5 | ue(v) |
| preferred_view_is_left_ref_view_flag[ i ] | 5 | u(1) |
| } |  |  |
| } |  |  |
| exponent_ref_display_width[ i ] | 5 | u(6) |
| mantissa_ref_display_width[ i ] | 5 | u(v) |
| if( ref_viewing_distance_flag ) { |  |  |
| exponent_ref_viewing_distance[ i ] | 5 | u(6) |
| mantissa_ref_viewing_distance[ i ] | 5 | u(v) |
| } |  |  |
| additional_horizontal_shift_present_flag[ i ] | 5 | u(1) |
| if (additional_horizontal _shift_present_flag[ i ]) |  |  |
| num_pixel_shift_plus128[ i ] | 5 | u(8) |
| } |  |  |
| } |  |  |

When present, this SEI message shall be associated with an IDR access unit. A reference displays information message contains information about the reference display width(s) and reference viewing distance(s) as well as information about the corresponding baseline distance(s) and additional horizontal image shift(s), which form a stereo-pair for the reference display width and the reference viewing distance. This information may also contain the left and right reference view information of a reference stereo-pair corresponding to the reference baseline for each reference display. This information enables a view renderer to produce a proper stereo-pair for the target screen width and the viewing distance. The reference display width and viewing distance values are signaled in units of centimeters. The reference baseline values shall be signaled in the same units as the x component of the translation vector in the SEI message depth acquisition information that is valid for the same access unit. When a reference displays information SEI message is present in an access unit, the depth acquisition information SEI message shall also be present in the same access unit. The baseline and shift information signaled for the reference display is valid for all access units they associated with and until the next IDR access unit or the next access unit containing depth acquisition information SEI message or reference displays information SEI message.

ref_stereo_pair_mode_flag[ i ] equal to 1 indicates that the reference stereo-pair corresponding to the reference baseline of the i-th reference display consists of two coded views, namely the left and right reference views, and the reference baseline is the baseline between the two coded views. ref_stereo_pair_mode_flag[ i ] equal to 0 indicates that the reference stereo-pair corresponding to the reference baseline of the i-th reference display consists of a preferred coded view and a synthesized (virtual) view, or consists of two unspecified views. The baseline between the two views of the reference stereo-pair, i.e., the reference baseline, is signaled by a floating-point number consisting of three parts: precision, exponent and mantissa.

left_ref_view_id[ i ] indicates the view_id of the left reference view of the reference stereo-pair corresponding to the i-th reference display.

right_ref_view_id[ i ] indicates the view_id of the right reference view of the reference stereo-pair corresponding to the i-th reference display.

preferred_view_flag[ i ] equal to 1 indicates that the information about a preferred view of the reference stereo-pair corresponding to the i-th reference display is present in the bitstream. preferred_view_flag[i] equal to 0 indicates that the information about a preferred view of the reference stereo-pair corresponding to i-th reference display is not present in the bitstream.

preferred_view_id[ i ] indicates the view_id of the preferred view corresponding to the i-th reference display.

preferred_view_is_left_ref_view_flag[ i ] equal to 1 indicates that the preferred view is the left reference view of the reference stereo-pair corresponding to the i-th reference display. preferred_view_is_left_ref_view_flag[ i ] equal to 0 indicates that the preferred view is the right reference view of the reference stereo-pair corresponding to the i-th reference display.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).