| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0178r1 |
| --- | --- |

| Title: | MVC+D: Enhanced 3DVC reference displays information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao and Lu Yu Zhejiang University #38 Zheda Road, Hangzhou, China Andrey Nokin Ericsson Färögatan 6, 164 80 Stockholm, Sweden | Tel: Email: Tel: Email: | +86 571 87953107 {zhaoyin,yul}@zju.edu.cn +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Zhejiang University and Ericsson |  |  |

_____________________________

# Abstract

The 3DVC reference displays information SEI message, adopted in JCT3V-A0163, specifies the baseline for one or more reference displays. This proposal suggest further including view information that explicitly defines a pair of views corresponding to the reference baseline, which helps 1) avoid window violation and 2) maintain image quality of the presented stereo-pair.

# Problem Setting

The 3DVC reference displays information SEI message, adopted in JCT3V-A0163, specifies the baseline for one or more reference displays, which is a minimum set for indicating the depth sensation from a recommended stereo-pair. In other words, any stereo-pair corresponding to the signalled reference baseline is considered to have suitable depth sensation for the reference display.

However, window violation that a popping-out object hit display margin may occur in one stereo-pair, but may be avoided by using another stereo-pair (with the same baseline) consisting of two other viewpoints, as shown in Fig. 1(a). Besides, when the views are coded asymmetrically, one stereo pair including the higher-quality view can have better image quality (thus better 3D visual quality), as shown in Fig. 1(b). Therefore, it is beneficial to include the view information in addition to the already adopted baseline information.

(a)						(b)

Fig. 1. (a) Illustration of window violation by a stereo pair of two sub-optimal views. The object appears at the left margin of the display when stereo-pair B is presented, while stereo-pair A does not have this problem. (b) Adding view information may benefit the display side by informing a stereo pair of better image quality in asymmetric coding: stereo-pair C is better than stereo-pair D in terms of visual quality.

# Proposal

It is proposed to further signal the left and right reference view information of a reference stereo pair that corresponds to the reference baseline. With the reference view information, the display side can explicitly know which two views should be selected to form the presented stereo-pair, instead of guessing (or figuring out) one stereo-pair of the reference baseline when the reference view information is absent. With explicit view information, the window violation may be avoided while the same image quality expected by the content provider can be guaranteed at the decoder side in the case of asymmetric coding of multiview texture sequences.

The enhanced syntax supports two methods to signal the left and right reference view information of a reference stereo-pair:

Method 1: the left view id and the right view id. (Note: in two-view case, the left and right views are the views of VOIdx = 0 and VOIdx = 1 by default, which need not be signalled).

Method 2: one view id + baseline + whether the signalled view is on the left of the other view of the reference stereo-pair which is inferred from the baseline. (Note: in two-view MVD case, the view id can be replaced by a flag indicating one of the two views of VOIdx = 0 and VOIdx = 1)

The former is straightforward for a reference stereo pair consisting of two coded views, and the latter is suitable for a reference stereo pair consisting of a coded view and a synthesized virtual view.

A show case is prepared to demonstrate the benefit from adding the proposed view information to the adopted 3DVC reference displays information SEI message, although the benefit of avoiding window violation and maintaining image quality is quite obvious.

# Specification text

The proposed specification text changes are indicated with green highlights on top of the MVC Extension for Inclusion of Depth Maps Draft Text 4 (JCT3V-A1001). Besides, a few text improvement is indicated with yellow highlights.

J.13.1.5 3DVC reference displays information SEI message syntax

| 3d_reference_displays_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| two_view_flag | 5 | u(1) |
| prec_ref_baseline | 5 | ue(v) |
| prec_ref_display_width | 5 | ue(v) |
| ref_viewing_distance_flag | 5 | u(1) |
| if( ref_viewing_distance_flag ) |  |  |
| prec_ref_viewing_dist | 5 | ue(v) |
| num_ref_displays_minus1 | 5 | ue(v) |
| numRefDisplays = num_ref_displays_minus1 + 1 |  |  |
| for( i = 0; i < numRefDisplays; i++ ) { |  |  |
| ref_stereo_pair_mode_flag[ i ] | 5 | u(1) |
| if (ref_stereo_pair_mode_flag[ i ] ) { |  |  |
| if(!two_view_flag) { |  |  |
| left_ref_view_id[ i ] | 5 | ue(v) |
| right_ref_view_id[ i ] | 5 | ue(v) |
| } |  |  |
| }else{ |  |  |
| exponent_ref_baseline[ i ] | 5 | u(6) |
| mantissa_ref_baseline[ i ] | 5 | u(v) |
| preferred_view_flag[ i ] | 5 | u(1) |
| if (preferred_view_flag[ i ] ){ |  |  |
| if (!two_view_flag) |  |  |
| preferred_view_id[ i ] | 5 | ue(v) |
| else |  |  |
| preferred_view_selection_flag[ i ] | 5 | u(1) |
| preferred_view_is_left_ref_view_flag[ i ] | 5 | u(1) |
| } |  |  |
| } |  |  |
| exponent_ref_display_width[ i ] | 5 | u(6) |
| mantissa_ref_display_width[ i ] | 5 | u(v) |
| if( ref_viewing_distance_flag ) { |  |  |
| exponent_ref_viewing_distance[ i ] | 5 | u(6) |
| mantissa_ref_viewing_distance[ i ] | 5 | u(v) |
| } |  |  |
| horizontal_shift_present_flag[ i ] | 5 | u(1) |
| if (horizontal _shift_present_flag[ i ]) |  |  |
| Num_pixel_shift_plus128[ i ] | 5 | u(8) |
| } |  |  |
| } |  |  |

When present, this SEI message shall be associated with an IDR access unit. A reference displays information message contains information about the reference display width(s) and reference viewing distance(s) as well as information about the corresponding baseline distance(s) and additional horizontal image shift(s), which form a stereo-pair for the reference display width and the reference viewing distance. This information may also contain the left and right reference view information of a reference stereo-pair corresponding to the reference baseline for each reference display. This information enables a view renderer to produce a proper stereo-pair for the target screen width and the viewing distance. The reference display width and viewing distance values are signaled in units of centimeters. The reference baseline values shall be signaled in the same units as the x component of the translation vector in the SEI message depth acquisition information that is valid for the same access unit. When a reference displays information SEI message is present in an access unit, the depth acquisition information SEI message shall also be present in the same access unit. The baseline and shift information signaled for the reference display is valid for all access units they associated with and until the next IDR access unit or the next access unit containing depth acquisition information SEI message or reference displays information SEI message.

two_view_flag equal to 1 indicates the bitstream is a two-view bitstream, containing two coded views. two_view_flag equal to 0 indicates the bitstream is not a two-view bitstream, i.e., the number of views in the bitstream is not equal to 2.

ref_stereo_pair_mode_flag[ i ] equal to 1 indicates that the reference stereo-pair corresponding to the reference baseline of the i-th reference display consists of two coded views, namely the left and right reference views, and the reference baseline is the baseline between the two coded views. ref_stereo_pair_mode_flag[ i ] equal to 0 indicates that the reference stereo-pair corresponding to the reference baseline of the i-th reference display consists of a preferred coded view and a synthesized (virtual) view, or consists of two unspecified views. The baseline between the two views of the reference stereo-pair, i.e., the reference baseline, is signaled by a floating-point number consisting of three parts: precision, exponent and mantissa.

left_ref_view_id[ i ] indicates the view_id of the left reference view of the reference stereo-pair corresponding to the i-th reference display.

right_ref_view_id[ i ] indicates the view_id of the right reference view of the reference stereo-pair corresponding to the i-th reference display.

preferred_view_flag[ i ] equal to 1 indicates that the information about a preferred view of the reference stereo-pair corresponding to the i-th reference display is present in the bitstream. preferred_view_flag[i] equal to 0 indicates that the information about a preferred view of the reference stereo-pair corresponding to i-th reference display is not present in the bitstream.

preferred_view_id[ i ] indicates the view_id of the preferred view corresponding to the i-th reference display.

preferred_view_selection_flag[ i ] equal to 1 indicates that the view of VOIdx = 0 is the preferred view corresponding to the i-th reference display. preferred_view_selection_flag[ i ] equal to 0 indicates that the view of VOIdx = 1 is the preferred view corresponding to the i-th reference display.

preferred_view_is_left_ref_view_flag[ i ] equal to 1 indicates that the preferred view is the left reference view of the reference stereo-pair corresponding to the i-th reference display. preferred_view_is_left_ref_view_flag[ i ] equal to 0 indicates that the preferred view is the right reference view of the reference stereo-pair corresponding to the i-th reference display.

horizontal_shift_present_flag[ i ] equal to 1 indicates that the information about additional horizontal shift of the left and right views for the i-th reference display is present in the bitstream. horizontal_shift_present_flag[ i ] equal to 0 indicates that the information about additional horizontal shift of the left and right views for the i-th reference display is not present in the bitstream.

Note: the syntax elements in A0163 are a special case when ref_stereo_pair_mode_flag[i] = 0 and preferred_view_flag[i] = 0.

The simplified syntax without special support for the 2-view 3DV is provided in the Appendix.

# Conclusion

The proposed reference view information help reconstruct the 3D experience (including depth sensation, visual comfort, image quality) expected by content producer at the display side. We recommend adopting the enhanced syntax elements on reference view information.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

The syntax without special support for the 2-view 3DV is provided in this Appendix.

J.13.1.5 3DVC reference displays information SEI message syntax

| 3d_reference_displays_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| prec_ref_baseline | 5 | ue(v) |
| prec_ref_display_width | 5 | ue(v) |
| ref_viewing_distance_flag | 5 | u(1) |
| if( ref_viewing_distance_flag ) |  |  |
| prec_ref_viewing_dist | 5 | ue(v) |
| Num_ref_displays_minus1 | 5 | ue(v) |
| numRefDisplays = num_ref_displays_minus1 + 1 |  |  |
| for( i = 0; i < numRefDisplays; i++ ) { |  |  |
| ref_stereo_pair_mode_flag[ i ] | 5 | u(1) |
| if (ref_stereo_pair_mode_flag[ i ] ) { |  |  |
| left_ref_view_id[ i ] | 5 | ue(v) |
| right_ref_view_id[ i ] | 5 | ue(v) |
| }else{ |  |  |
| exponent_ref_baseline[ i ] | 5 | u(6) |
| mantissa_ref_baseline[ i ] | 5 | u(v) |
| preferred_view_flag[ i ] | 5 | u(1) |
| If (preferred_view_flag[ i ] ){ |  |  |
| preferred_view_id[ i ] | 5 | ue(v) |
| preferred_view_is_left_ref_view_flag[ i ] | 5 | u(1) |
| } |  |  |
| } |  |  |
| exponent_ref_display_width[ i ] | 5 | u(6) |
| mantissa_ref_display_width[ i ] | 5 | u(v) |
| if( ref_viewing_distance_flag ) { |  |  |
| exponent_ref_viewing_distance[ i ] | 5 | u(6) |
| mantissa_ref_viewing_distance[ i ] | 5 | u(v) |
| } |  |  |
| horizontal_shift_present_flag[ i ] | 5 | u(1) |
| if (horizontal _shift_present_flag[ i ]) |  |  |
| num_pixel_shift_plus128[ i ] | 5 | u(8) |
| } |  |  |
| } |  |  |

When present, this SEI message shall be associated with an IDR access unit. A reference displays information message contains information about the reference display width(s) and reference viewing distance(s) as well as information about the corresponding baseline distance(s) and additional horizontal image shift(s), which form a stereo-pair for the reference display width and the reference viewing distance. This information may also contain the left and right reference view information of a reference stereo-pair corresponding to the reference baseline for each reference display. This information enables a view renderer to produce a proper stereo-pair for the target screen width and the viewing distance. The reference display width and viewing distance values are signaled in units of centimeters. The reference baseline values shall be signaled in the same units as the x component of the translation vector in the SEI message depth acquisition information that is valid for the same access unit. When a reference displays information SEI message is present in an access unit, the depth acquisition information SEI message shall also be present in the same access unit. The baseline and shift information signaled for the reference display is valid for all access units they associated with and until the next IDR access unit or the next access unit containing depth acquisition information SEI message or reference displays information SEI message.

ref_stereo_pair_mode_flag[ i ] equal to 1 indicates that the reference stereo-pair corresponding to the reference baseline of the i-th reference display consists of two coded views, namely the left and right reference views, and the reference baseline is the baseline between the two coded views. ref_stereo_pair_mode_flag[ i ] equal to 0 indicates that the reference stereo-pair corresponding to the reference baseline of the i-th reference display consists of a preferred coded view and a synthesized (virtual) view, or consists of two unspecified views. The baseline between the two views of the reference stereo-pair, i.e., the reference baseline, is signaled by a floating-point number consisting of three parts: precision, exponent and mantissa.

left_ref_view_id[ i ] indicates the view_id of the left reference view of the reference stereo-pair corresponding to the i-th reference display.

right_ref_view_id[ i ] indicates the view_id of the right reference view of the reference stereo-pair corresponding to the i-th reference display.

preferred_view_flag[ i ] equal to 1 indicates that the information about a preferred view of the reference stereo-pair corresponding to the i-th reference display is present in the bitstream. preferred_view_flag[i] equal to 0 indicates that the information about a preferred view of the reference stereo-pair corresponding to i-th reference display is not present in the bitstream.

preferred_view_id[ i ] indicates the view_id of the preferred view corresponding to the i-th reference display.

preferred_view_is_left_ref_view_flag[ i ] equal to 1 indicates that the preferred view is the left reference view of the reference stereo-pair corresponding to the i-th reference display. preferred_view_is_left_ref_view_flag[ i ] equal to 0 indicates that the preferred view is the right reference view of the reference stereo-pair corresponding to the i-th reference display.