| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0187 |
| --- | --- |

| Title: | Level limits in MVC+D |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anthony Vetro Mitsubishi Electric 201 Broadway, 8th Floor Cambridge, MA, 02139 USA | Tel: Email: | +1-617-621-7591 avetro@merl.com |
| Source: | Mitsubishi Electric |  |  |

_____________________________

# Abstract

This document observes that the level limits in the current specification of MVC plus depth are underspecified, and proposes limits that should be considered for the final specification.

# Introduction

The current specification of levels for the Multiview Depth Profile as defined in subclause I.10.2 of [1] simply references the specification of levels associated with MVC profiles defined in Annex H. While the texture and depth views may indeed be separately encoded MVC bitstreams, they are wrapped together with high-level syntax and should form distinct conformance points.

Considering the above, the current specification of Levels does not appear adequate for Annex I profiles, for at least the following reasons:

- The value of NumViews specified in Annex H does not appear to be adequate for bitstreams conforming to the Multiview Depth High profile since the number of depth views is not accounted for.

- The resolution and inter-view prediction dependencies of texture and depth components for bitstreams conforming to the Multiview Depth High profile could vary and must be properly accounted for in the provisioning of decoding resources.

- The scale factors that are used to determine limits on MaxDpbFrames or MaxMBPS should be reconsidered in the context of decoding requirements for Annex I.

- It is unclear whether the Annex H constraints on VCL and NAL HRD parameters directly apply.

# Proposal

It is recommended that the above issues be discussed during the 2nd JCT-3V meeting. Initial text suggestions for the specification of Levels in Annex I is provided in the appendix of this document, with suggested changes relative to Annex H shown with red text and constraints that should be reviewed further highlighted in yellow. The main features of the proposed text are outlined as follows:

- Set the value of NumViews considering both texture views and depth views.

- Rather than specify PicSizeInMbs as the number of macroblocks in a single view component, a new variable TotalPicSizeInMbs it is specified as the total macroblocks in a picture, including both texture and depth view components. In this way, the resolution differences between texture and depth are easily absorbed into the level limits where appropriate.

- Clarify that the picture width and height constraints, expressed in terms of macroblocks, apply to each view component.

- Specify MaxDpbFrames in terms of resolution of texture view component. This assumes the texture view component always has a resolution that is greater than or equal to the depth view component. If this is not true, the proposed text should be revised to account for the worst case situation. It may also be desirable to specify a corresponding constraint on the bitstream.

Further examination of the constraints regarding number of slices, bit rates, number of bytes in NAL units should be considered.

# References

[1] T. Suzuki, M. Hannuksela, Y. Chen, S. Hattori, G. Sullivan (Eds.), “MVC Extension for Inclusion of Depth Maps Draft Text 4,” JCT3V-A1001, Stockholm, Sweden, July 2012.

# Patent rights declaration(s)

Mitsubishi Electric does not have any current or pending patent rights relating to the technology described in this contribution.

# Appendix: Proposed text on Levels for Annex I

    - Levels

The following is specified for expressing the constraints in this clause:

–	Let access unit n be the n-th access unit in decoding order with the first access unit being access unit 0.

–	Let picture n be the primary coded picture or the corresponding decoded picture of access unit n.

Let the variable fR be derived as follows:

–	If picture n is a frame, fR is set equal to 1  172.

–	Otherwise (picture n is a field), fR is set equal to 1  (172 * 2).

The value of mvcScaleFactor is set equal to 2.

The value of NumViews indicates the number of views, including texture views and depth views, which are required for decoding the target output views corresponding to the j-th operation point for level_idc[ i ] as signalled in the subset sequence parameter set, and is set equal to applicable_op_num_depth_views_minus1[ i ][ j ] + applicable_op_num_depth_views_minus1[ i ][ j ] + 2.

The value of PicWidthInMbs and FrameHeightInMbs refer to the width and height of each view component, while the value of TotalPicSizeInMbs indicates the total number of macroblocks in the texture view components and depth view components of a picture.

      - Level limits for the Multiview Depth High profile

Bitstreams conforming to the Multiview Depth High profile at a specified level shall obey the following constraints:

- The nominal removal time of access unit n (with n > 0) from the CPB as specified in clause Error: Reference source not found, satisfies the constraint that tr,n( n ) − tr( n − 1 ) is greater than or equal to Max( NumViews * TotalPicSizeInMbs  ( mvcScaleFactor * MaxMBPS ), fR ), where MaxMBPS is the value specified in Error: Reference source not found that applies to picture n − 1, and TotalPicSizeInMbs is the total number of macroblocks in the texture view components and depth view components of picture n – 1.

- The difference between consecutive output times of pictures from the DPB as specified in clause Error: Reference source not found, satisfies the constraint that to,dpb( n ) >= Max( NumViews * TotalPicSizeInMbs  ( mvcScaleFactor * MaxMBPS ), fR ), where MaxMBPS is the value specified in Error: Reference source not found for picture n, and TotalPicSizeInMbs is the total number of macroblocks in the texture view components and depth view components of picture n, provided that picture n is a picture that is output and is not the last picture of the bitstream that is output.

- PicWidthInMbs * FrameHeightInMbs <= MaxFS, where MaxFS is specified in Error: Reference source not found.

- PicWidthInMbs <= Sqrt( MaxFS * 8 ), where MaxFS is specified in Error: Reference source not found.

- FrameHeightInMbs <= Sqrt( MaxFS * 8 ), where MaxFS is specified in Error: Reference source not found.

- max_dec_frame_buffering <= MaxDpbFrames, where MaxDpbFrames is equal to Min( mvcScaleFactor * MaxDpbMbs / ( PicTextureWidthInMbs * FrameTextureHeightInMbs ), Max( 1, Ceil( log2( NumViews ) ) ) * 16 ) and MaxDpbMbs is specified in Error: Reference source not found and TextureWidthInMbs and TextureHeightInMbs are the width and height in terms of macroblocks for a single texture view component of picture 0, respectively.

- The vertical motion vector component range does not exceed MaxVmvR in units of luma frame samples, where MaxVmvR is specified in Error: Reference source not found.

- The horizontal motion vector range does not exceed the range of −2048 to 2047.75, inclusive, in units of luma samples.

- Let setOf2Mb be the set of unsorted pairs of macroblocks that contains the unsorted pairs of macroblocks (mbA, mbB) of a coded video sequence for which any of the following conditions are true:

–	mbA and mbB are macroblocks that belong to the same slice and are consecutive in decoding order,

–	separate_colour_plane_flag is equal to 0, mbA is the last macroblock (in decoding order) of a slice, and mbB is the first macroblock (in decoding order) of the next slice in decoding order,

–	separate_colour_plane_flag is equal to 1, mbA is the last macroblock (in decoding order) of a slice with a particular value of colour_plane_id, and mbB is the first macroblock (in decoding order) of the next slice with the same value of colour_plane_id in decoding order.

NOTE 1 – In the two above conditions, the macroblocks mbA and mbB can belong to different pictures.

For each unsorted pair of macroblocks (mbA, mbB) of the set setOf2Mb, the total number of motion vectors (given by the sum of the number of motion vectors for macroblock mbA and the number of motion vectors for macroblock mbB) does not exceed MaxMvsPer2Mb, where MaxMvsPer2Mb is specified in Error: Reference source not found. The number of motion vectors for each macroblock is the value of the variable MvCnt after the completion of the intra or inter prediction process for the macroblock.

NOTE 2 – When separate_colour_plane_flag is equal to 0, the constraint specifies that the total number of motion vectors for two consecutive macroblocks in decoding order must not exceed MaxMvsPer2Mb. When separate_colour_plane_flag is equal to 1, the constraint specifies that the total number of motion vectors for two consecutive macroblocks with the same value of colour_plane_id in decoding order must not exceed MaxMvsPer2Mb. For macroblocks that are consecutive in decoding order but are associated with a different value of colour_plane_id, no constraint for the total number of motion vectors is specified.

- The number of bits of macroblock_layer( ) data for any macroblock is not greater than 128 + RawMbBits. Depending on entropy_coding_mode_flag, the bits of macroblock_layer( ) data are counted as follows:

–	If entropy_coding_mode_flag is equal to 0, the number of bits of macroblock_layer( ) data is given by the number of bits in the macroblock_layer( ) syntax structure for a macroblock.

–	Otherwise (entropy_coding_mode_flag is equal to 1), the number of bits of macroblock_layer( ) data for a macroblock is given by the number of times read_bits( 1 ) is called in clauses Error: Reference source not found and Error: Reference source not found when parsing the macroblock_layer( ) associated with the macroblock.

- The removal time of access unit 0 shall satisfy the constraint that the number of slices in picture 0 is less than or equal to mvcScaleFactor * ( Max( PicSizeInMbs, fR * MaxMBPS ) + MaxMBPS * ( tr( 0 ) − tr,n( 0 ) ) ) ÷ SliceRate, where MaxMBPS and SliceRate are the values specified in Tables Error: Reference source not found and Error: Reference source not found, respectively, that apply to picture 0 and PicSizeInMbs is the number of macroblocks in a single texture view component of picture 0.

- The removal time of access unit 0 shall satisfy the constraint that the number of slices in each view component of picture 0 is less than or equal to ( Max( PicSizeInMbs, fR * MaxMBPS ) + MaxMBPS * ( tr( 0 ) − tr,n( 0 ) ) ) ÷ SliceRate, where MaxMBPS and SliceRate are the values specified in Tables Error: Reference source not found and Error: Reference source not found, respectively, that apply to picture 0 and PicSizeInMbs is the number of macroblocks in a single view component of picture 0.

- The difference between consecutive removal times of access units n and n − 1 with n > 0 shall satisfy the constraint that the number of slices in picture n is less than or equal to mvcScaleFactor * MaxMBPS * ( tr( n ) − tr( n − 1 ) ) ÷ SliceRate, where SliceRate is the value specified in Error: Reference source not found that applies to picture n.

- The difference between consecutive removal times of access units n and n − 1 with n > 0 shall satisfy the constraint that the number of slices in each view component of picture n is less than or equal to MaxMBPS * ( tr( n ) − tr( n − 1 ) ) ÷ SliceRate, where SliceRate is the value specified in Error: Reference source not found that applies to picture n.

- MVC sequence parameter sets shall have direct_8x8_inference_flag equal to 1 for the levels specified in Error: Reference source not found.

- The value of sub_mb_type[ mbPartIdx ] with mbPartIdx = 0..3 in B macroblocks with mb_type equal to B_8x8 shall not be equal to B_Bi_8x4, B_Bi_4x8, or B_Bi_4x4 for the levels in which MinLumaBiPredSize is shown as 8x8 in Error: Reference source not found.

- For the VCL HRD parameters, BitRate[ SchedSelIdx ] <= cpbBrVclFactor * MaxBR and CpbSize[ SchedSelIdx ] <= cpbBrVclFactor *MaxCPB for at least one value of SchedSelIdx, where cpbBrVclFactor is equal to 1250. With vui_mvc_vcl_hrd_parameters_present_flag[ i ] being the syntax element, in the MVC VUI parameters extension of the active MVC sequence parameter set, that is associated with the VCL HRD parameters that are used for conformance checking (as specified in Annex Error: Reference source not found), BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are given as follows:

–	If vui_mvc_vcl_hrd_parameters_present_flag equal to 1, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are given by Equations Error: Reference source not found and Error: Reference source not found, respectively, using the syntax elements of the hrd_parameters( ) syntax structure that immediately follows vui_mvc_vcl_hrd_parameters_present_flag.

–	Otherwise (vui_mvc_vcl_hrd_parameters_present_flag equal to 0), BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are inferred as specified in clause Error: Reference source not found for VCL HRD parameters.

MaxBR and MaxCPB are specified in Error: Reference source not found in units of cpbBrVclFactor bits/s and cpbBrVclFactor bits, respectively. The bitstream shall satisfy these conditions for at least one value of SchedSelIdx in the range 0 to cpb_cnt_minus1, inclusive.

- For the NAL HRD parameters, BitRate[ SchedSelIdx ] <= cpbBrNalFactor * MaxBR and CpbSize[ SchedSelIdx ] <= cpbBrNalFactor *MaxCPB for at least one value of SchedSelIdx, where cpbBrNalFactor is equal to 1500. With vui_mvc_nal_hrd_parameters_present_flag[ i ] being the syntax element, in the MVC VUI parameters extension of the active MVC sequence parameter set, that is associated with the NAL HRD parameters that are used for conformance checking (as specified in Annex Error: Reference source not found), BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are given as follows:

–	If vui_mvc_nal_hrd_parameters_present_flag equal to 1, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are given by Equations Error: Reference source not found and Error: Reference source not found, respectively, using the syntax elements of the hrd_parameters( ) syntax structure that immediately follows vui_mvc_nal_hrd_parameters_present_flag.

–	Otherwise (vui_mvc_nal_hrd_parameters_present_flag equal to 0), BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are inferred as specified in clause Error: Reference source not found for NAL HRD parameters.

MaxBR and MaxCPB are specified in Error: Reference source not found in units of cpbBrNalFactor bits/s and cpbBrNalFactor bits, respectively. The bitstream shall satisfy these conditions for at least one value of SchedSelIdx in the range 0 to cpb_cnt_minus1, inclusive.

- The sum of the NumBytesInNALunit variables for access unit 0 is less than or equal to 384 * mvcScaleFactor * ( Max( PicSizeInMbs, fR * MaxMBPS ) + MaxMBPS * ( tr( 0 ) − tr,n( 0 ) ) ) ÷ MinCR, where MaxMBPS and MinCR are the values specified in Error: Reference source not found that apply to picture 0 and PicSizeInMbs is the number of macroblocks in a single (texture?) view component of picture 0.

- The sum of the NumBytesInNALunit variables for the VCL NAL units of each view component of access unit 0 is less than or equal to 384 * ( Max( PicSizeInMbs, fR * MaxMBPS ) + MaxMBPS * ( tr( 0 ) − tr,n( 0 ) ) ) ÷ MinCR, where MaxMBPS and MinCR are the values specified in Error: Reference source not found that apply to picture 0 and PicSizeInMbs is the number of macroblocks in a single view component of picture 0.

- The sum of the NumBytesInNALunit variables for access unit n with n > 0 is less than or equal to 384 * mvcScaleFactor * MaxMBPS * ( tr( n ) − tr( n − 1 ) ) ÷ MinCR, where MaxMBPS and MinCR are the values specified in Error: Reference source not found that apply to picture n.

- The sum of the NumBytesInNALunit variables for the VCL NAL units of each view component of access unit n with n > 0 is less than or equal to 384 * MaxMBPS * ( tr( n ) − tr( n − 1 ) ) ÷ MinCR, where MaxMBPS and MinCR are the values specified in Error: Reference source not found that apply to picture n.

- When PicSizeInMbs is greater than 1620, the number of macroblocks in any coded slice shall not exceed MaxFS / 4, where MaxFS is specified in Error: Reference source not found.

- max_num_ref_frames shall be less than or equal to MaxDpbFrames / mvcScaleFactor for each view component, where MaxDpbFrames is specified in item f).

- 3DVC sequence parameter sets shall have frame_mbs_only_flag equal to 1 for the levels specified in Error: Reference source not found.

Error: Reference source not found specifies the limits for each level. A definition of all levels identified in the "Level number" column of Error: Reference source not found is specified for the Multiview Depth High profile. Error: Reference source not found specifies limits for each level that are specific to bitstreams conforming to the Multiview Depth High profile. Each entry in Tables Error: Reference source not found and Error: Reference source not found indicates, for the level corresponding to the row of the table, the absence or value of a limit that is imposed by the variable corresponding to the column of the table, as follows:

–	If the table entry is marked as "-", no limit is imposed by the value of the variable as a requirement of bitstream conformance to the profile at the specified level.

–	Otherwise, the table entry specifies the value of the variable for the associated limit that is imposed as a requirement of bitstream conformance to the profile at the specified level.

For coded video sequences conforming to the Multiview Depth High profile, the level_idc value is specified as follows:

–	If level_idc is not equal to 0, level_idc indicates the level that applies to the coded video sequence operating with all the views being target output views.

NOTE 3 – A level_idc value that is not equal to zero may indicate a higher level than necessary to decode the coded video sequence operating with all the views being target output views. This may occur when a subset of views or temporal subsets are removed from a coded video sequence according to the sub-bitstream extraction process specified in clause I.8.5.3Error: Reference source not found, and the level_idc value is not updated accordingly.

–	Otherwise (level_idc is equal to 0), the level that applies to the coded video sequence operating with all the views being target output views is unspecified.

NOTE 4 – When profile_idc is equal to 118 or 128 and level_idc is equal to 0, there may exist a level indicated by level_idc[ i ] that is applicable to the coded video sequence operating with all the views being target output views. This may occur when a subset of views or temporal subsets are removed from a coded video sequence according to the sub-bitstream extraction process specified in clause I.8.5.3Error: Reference source not found, and a particular value of level_idc[ i ] corresponds to the resulting coded video sequence.

In bitstreams conforming to the Multiview Depth High profile, the conformance of the bitstream to a specified level is indicated by the syntax element level_idc or level_idc[ i ] as follows:

–	If level_idc or level_idc[ i ] is equal to 9, the indicated level is level 1b.

–	Otherwise (level_idc or level_idc[ i ] is not equal to 9), level_idc or level_idc[ i ] is equal to a value of ten times the level number (of the indicated level) specified in Error: Reference source not found.