| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0188 |
| --- | --- |

| Title: | 3D-CE4.h: Crosscheck of LGE JCT3V-B0131 on depth distortion metric with a weighted depth fidelity term |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Dong Tian 201 Broadway Cambridge, MA 02139 | Tel: Email: | +1-617-621-7556 tian@merl.com |
| Source: | Mitsubishi Electric Research Labs (MERL) |  |  |

_____________________________

# Abstract

This document reports the outcome of cross-verification of JCT3V-B0131, "CE4.h Results on Depth distortion metric with a weighted depth fidelity term", performed by Mitsubishi Electric Research Labs (MERL). The simulation results obtained for this cross-verification basically match those provided by the proponents.

# Introduction

In contribution JCT3V-B0131 [1], a depth-fidelity term is proposed to the depth distortion metric function, referred as WVSO. Comparison results with WVSO off and on are performed.

# Performance of proposed method

The coded and synthsized results are basically matched with the results from the proponent. In the results, WVSO off serves as the reference, and WVSO on is compared against WVSO off.

In a first test, VSRS 3.5 is used as the rendering engine. The results obtained by MERL are shown in Table 1. The bitrate rate saving in terms of synthesis view is 2.6%, even higher than the reported 2.4% in [1]. The mismatch should come from minor difference in VSRS settings. Since our synthesis results are better than LGE's results, the basis of the LGE proposal is basically fine. At least the mismatch would not give any negative impacts on LGE's proposal.

In a second test, VSRS-1D-Fast is used as the rendering engine. As shown in Table 2, the coded and synthesized results match those obtained by the proponent. The coding time ratio to also very close to the ratio reported by the proponent.

Table 1. WVSO off vs. on, using VSRS 3.5 rendering engine

Table 2. WVSO off vs. on, using VSRS-1D-Fast rendering engine

# Conclusions

The results obtained during this cross verification basically match those provided by the proponents.

# References

- Jiwook Jung, Sehoon Yea, "CE4.h Results on depth distortion metric with a weighted depth fidelity term", JCT3V-B0131, Shanghai, October 2012