| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0194 |
| --- | --- |

| Title: | Cross-check of motion data buffer reduction for 3D-HEVC (JCT3V-B0091) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitomo Takahashi Sally Hattori Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 yoshitomo.takahashi@jp.sony.com Shinobu.Hattori@jp.sony.com ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results on JCT3V-B0091 proposed by the proponent.

In JCT3V-B0091 [1], the motion data buffer is reduced. The numbers of MVs are compressed in two parts, which are the time after decoding a picture of each view and the time after decoding the pictures of all views. In the first part, only the MVs of the block where the top-left sample is a multiple of 8 are stored. In the second part, the only MVs of the first 4x4 block in each 16x16 block is stored.

It is reported that the cross-checking results match with the ones provided from proponent. The cross-checking reports show 0.0% and 0.3% BD-rate increase in video1 and video2 respectively and 0.1% BD-rate increase in the total of coding views and synthesized views.

It is commented that JCT3V-B0091 is the same method as second approach in JCT3V-B0038.

# Cross-checking

The software based on HTM-4.0.1 and the experimental results were provided by the proponent. The software was compiled fine and the cross-checking was performed on 64bit linux environment. The simulation has been conducted under common test conditions [2] for 3 view case.

The results of proposal were generated with the following macros,

#define MTK_PROGRESSIVE_MV_COMPRESSION	1

# Results

The cross-checking results are summarized in the following table. The detailed results are shown in the attached excel-file.

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 102.4% | 107.6% |
| Kendo | 0.0% | 0.0% | 0.2% | 0.0% | 0.2% | 0.2% | 102.9% | 106.0% |
| Newspapercc | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 103.2% | 104.0% |
| GhostTownFly | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% | 100.7% | 107.0% |
| PoznanHall2 | 0.0% | 0.1% | 0.8% | 0.2% | 0.1% | 0.1% | 102.9% | 106.3% |
| PoznanStreet | 0.0% | -0.2% | 0.3% | 0.0% | 0.1% | 0.1% | 103.0% | 105.4% |
| UndoDancer | 0.0% | 0.1% | 0.3% | 0.1% | 0.1% | 0.1% | 101.3% | 104.0% |
| 1024x768 | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 102.8% | 105.9% |
| 1920x1088 | 0.0% | 0.0% | 0.4% | 0.1% | 0.1% | 0.1% | 102.0% | 105.7% |
| average | 0.0% | 0.0% | 0.3% | 0.1% | 0.1% | 0.1% | 102.3% | 105.8% |

# Comment

JCT3V-B0113 can reduce the motion data buffer with no significant coding loss. It is possible to reduce the motion data memory to the one-fourth at most compared with the current method in the case of the 4x4 PU.

JCT3V-B0091 is the same method as second approach in JCT3V-B0038.

# Conclusion

In this contribution, the cross-checking results on JCT3V-B0091 are reported. It is confirmed that the results match with the ones provided by the proponent.

# Reference

[1] ISO/IEC JTC1/SC29/WG11, “motion data buffer reduction for 3D-HEVC”, Doc. JCT3V-B0091, Shanghai, CN, Oct. 2012.
[2] ISO/IEC JTC1/SC29/WG11, “Common Test Conditions of 3DV Core Experiments”, Doc. JCT3V-A1100, Stockholm, SE, July 2012.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).