| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0197 |
| --- | --- |

| Title: | 3D-CE5.h related: Cross-check of restricted motion vector coding for inter-view prediction (JCT3V-B0113) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitomo Takahashi Sally Hattori Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 yoshitomo.takahashi@jp.sony.com Shinobu.Hattori@jp.sony.com ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results on JCT3V-B0113 proposed by the proponent. 

In JCT3V-B0113 [1], the vertical component of inter-view vector is restricted. A flag related to inter-view vector restriction is defined in SPS (or PPS). When the flag is equal to 0, the coding scheme is not changed from the anchor. When the flag is equal to 1, the vertical component of inter-view vector is restricted. In the encoder, when the reference picture is for the inter-view prediction, the vertical search of motion estimation is skipped, and the vertical component of inter-view vector is always set to 0. In addition, the syntax of vertical component of inter-view vector is not encoded. In the decoder, when the reference picture is for the inter-view prediction, the syntax of vertical component of inter-view vector is not decoded, and inferred to 0.

It is reported that the cross-checking results match with the ones provided from the proponent. The cross-checking reports show 6.0% and 6.6% BD-rate increase in video1 and video2 respectively and 2.0% BD-rate increase in the total of coding views and synthesized views.

# Cross-checking

The software based on HTM-4.0.1 and the experimental results were provided by the proponent. The software was compiled fine and the cross-checking was performed on 64bit linux environment. The simulation has been conducted under common test conditions [2] for 3 view case.

This report includes two cross-checking results. The first one is the cross-checking results for the proposal. The results were generated with the following macros,

#define SH_RESTRICT_DV                    1
#define SH_DISP_MVDY_ZERO                 1
#define DISP_MVDY_ZERO_FLAG_IN_SPS        1

The second one is the cross-checking results for the additional experiment conducted by the proponent. In this experiment, the results are shown when the vertical component of the inter-view vector is restricted only in the encoder. The results were generated with the following macros,

#define SH_RESTRICT_DV                    1

#define SH_DISP_MVDY_ZERO                 0

# Results

The cross-checking results are summarized in the following table. The detailed results are shown in the attached excel-file. Table 1 shows the results of the proposal and Table2 shows the results of additional experiment conducted by the proponent.

Table 1:Results of the proposal

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 5.2% | 2.7% | 1.8% | 1.2% | 1.4% | 90.0% | 104.6% |
| Kendo | 0.0% | 4.8% | 2.8% | 1.7% | 1.2% | 1.3% | 89.4% | 101.6% |
| Newspapercc | 0.0% | 4.8% | 10.9% | 3.4% | 2.3% | 2.5% | 89.2% | 99.1% |
| GhostTownFly | 0.0% | 0.7% | 0.8% | 0.4% | 0.4% | 0.4% | 89.4% | 105.7% |
| PoznanHall2 | 0.0% | 11.4% | 8.2% | 4.7% | 3.4% | 3.7% | 88.3% | 104.5% |
| PoznanStreet | 0.0% | 22.8% | 28.0% | 8.8% | 6.5% | 7.0% | 92.5% | 102.6% |
| UndoDancer | 0.0% | 0.7% | 0.7% | 0.4% | 0.6% | 0.5% | 92.1% | 104.2% |
| 1024x768 | 0.0% | 4.9% | 5.5% | 2.3% | 1.6% | 1.7% | 89.6% | 101.7% |
| 1920x1088 | 0.0% | 8.9% | 9.4% | 3.6% | 2.7% | 2.9% | 90.6% | 104.2% |
| average | 0.0% | 7.2% | 7.7% | 3.0% | 2.2% | 2.4% | 90.1% | 103.1% |

Table 2: Results of additional experiment

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 4.4% | 2.1% | 1.5% | 1.0% | 1.1% | 90.9% | 103.1% |
| Kendo | 0.0% | 4.0% | 1.9% | 1.4% | 0.9% | 1.0% | 88.5% | 99.9% |
| Newspapercc | 0.0% | 3.6% | 10.0% | 3.0% | 1.9% | 2.1% | 88.2% | 97.9% |
| GhostTownFly | 0.0% | -0.9% | -0.5% | 0.0% | 0.0% | 0.0% | 89.6% | 105.3% |
| PoznanHall2 | 0.0% | 9.9% | 7.0% | 4.1% | 2.9% | 3.2% | 88.4% | 103.5% |
| PoznanStreet | 0.0% | 21.2% | 26.1% | 8.2% | 6.0% | 6.5% | 92.6% | 103.8% |
| UndoDancer | 0.0% | -0.5% | -0.6% | -0.1% | 0.2% | 0.1% | 94.1% | 103.7% |
| 1024x768 | 0.0% | 4.0% | 4.6% | 1.9% | 1.2% | 1.4% | 89.2% | 100.2% |
| 1920x1088 | 0.0% | 7.4% | 8.0% | 3.0% | 2.3% | 2.4% | 91.1% | 104.1% |
| average | 0.0% | 6.0% | 6.6% | 2.6% | 1.8% | 2.0% | 90.3% | 102.4% |

# Comment

JCT3V-B0113 can reduce the encoding time since the vertical search of motion estimation is skipped when the reference picture is for inter-view prediction. The cross-checking results show some 10% encoding time reduction when compared to the anchor. 

JCT3V-B0113 can improve the coding efficiency of CG sequences, but degrades the coding efficiency of other sequences. It may be effective in the vertically well-aligned sequences such as CG that the syntax of vertical component of inter-view vector is not coded and always inferred to 0.

# Conclusion

In this contribution, the cross-checking results on JCT3V-B0113 are reported. It is confirmed that the results match with the ones provided by the proponent.

# Reference

[1] ISO/IEC JTC1/SC29/WG11, “3D-CE5.h related: Restricted motion vector coding for inter-view prediction”, Doc. JCT3V-B0113, Shanghai, CN, Oct. 2012.
[2] ISO/IEC JTC1/SC29/WG11, “Common Test Conditions of 3DV Core Experiments”, Doc. JCT3V-A1100, Stockholm, SE, July 2012.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).