| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0204 |
| --- | --- |

| Title: | CE5.h cross-check of results on pruning process for the inter-view candidate (JCT3V-B0086) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Tadashi Uchiumi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution reports a cross-check result of the pruning proposal for AMVP (JCT3V-B0086). In this cross-check Test4 “an AMVP change” and Test 7 “a combination of merge change (test3) and the test4” were conducted. The experimental results perfectly match with those provided by the proponent and the software was also confirmed that it follows the description. It is reported that the change of AMVP (test 4) causes no loss (0.0%) and the AMVP + merge change (test 7) achieves 0.2 % gain. The gain of test 7 is considered from the additions of pruning in merge mode.

# Verification

|  | HTM4.0.1 | Test (JCT3V-B0086) |
| --- | --- | --- |
| Test 4 (AMVP change) | A B InterCand Full pruning T | A B (added if B!=A) InterCand (added if InterCand!=A) // no pruning T |
| Test 3 (Merge change) | InterCand A0 A1 (if A1 is not A0) B0 B1 (if B1 is not B0) B2 | InterCand A0 A1 (if A1 is not A0 or InterCand) B0 B1 (if B1 is not B0 or InterCand) B2 |
| Test 7 (Test 3 + Test4) | HM4.0.1 | Test 3 + Test4 (described above) |

# Experimental results

The experiment by cross-checker results are shown in Table 1.

Table 1: Experiment result (Test 4)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 102.2% | 100.4% |
| Kendo | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 99.9% | 100.3% | 100.7% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 99.4% | 100.3% |
| GhostTownFly | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 99.1% | 97.8% | 99.1% |
| PoznanHall2 | 0.0% | 0.4% | 0.4% | 0.2% | 0.1% | 0.1% | 100.1% | 98.2% | 98.4% |
| PoznanStreet | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | 0.0% | 97.9% | 94.8% | 96.6% |
| UndoDancer | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 99.1% | 95.7% | 98.1% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 100.6% | 100.5% |
| 1920x1088 | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 99.0% | 96.6% | 98.0% |
| average | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 99.5% | 98.3% | 99.1% |

Table 2: Experiment result (Test 7)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.3% | -0.5% | -0.2% | -0.2% | -0.2% | 101.4% | 101.4% | 100.7% |
| Kendo | 0.0% | -0.5% | -0.4% | -0.2% | -0.2% | -0.2% | 99.8% | 101.9% | 100.6% |
| Newspapercc | 0.0% | -0.4% | -0.5% | -0.2% | -0.1% | -0.1% | 99.8% | 101.4% | 99.8% |
| GhostTownFly | 0.0% | -0.1% | -0.2% | -0.1% | 0.0% | -0.1% | 101.9% | 98.8% | 102.0% |
| PoznanHall2 | 0.0% | 0.1% | -0.5% | -0.2% | -0.1% | -0.1% | 101.2% | 102.0% | 101.8% |
| PoznanStreet | 0.0% | -0.4% | -0.6% | -0.2% | -0.2% | -0.2% | 101.1% | 98.1% | 101.0% |
| UndoDancer | 0.0% | -0.4% | -0.5% | -0.2% | -0.2% | -0.2% | 101.5% | 99.8% | 101.4% |
| 1024x768 | 0.0% | -0.4% | -0.5% | -0.2% | -0.1% | -0.2% | 100.3% | 101.6% | 100.4% |
| 1920x1088 | 0.0% | -0.2% | -0.4% | -0.2% | -0.1% | -0.1% | 101.5% | 99.7% | 101.6% |
| average | 0.0% | -0.3% | -0.4% | -0.2% | -0.1% | -0.2% | 101.0% | 100.5% | 101.0% |

# Conclusion

The experimental results perfectly match with those provided by the proponent. It is reported that the total gain of Test 4 and Test 7 is 0.0 % and 0.2%.

# References

[1] J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei , “CE5.h results on pruning process for the inter-view candidate, “ JCT3V-B0086, JCT3V, 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).