| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0207 |
| --- | --- |

| Title: | Cross-check report for JCT3V-B0160 on Enabling CU early termination(ECU) at depth coding in common test conditions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Eunyong Son, Jiwook Jung, Sehoon Yea 19, Yangjae-daero 11gil Seocho-gu, Seoul 137-130, Korea | Tel: Email: | 82-2-6912-6477 eunyong.son@lge.com jiwook.jung@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Introduction

This document reports the cross check results on JCT3V-B0160 Enabling CU early termination(ECU) at depth coding in common test conditions by Samsung[1]. Performance evaluation results reported by the proponent were verified.

# Test results

The tests were finished on a 64bit Red Hat Enterprise Linux 5.3 cluster system, based on the common test conditions [2]. Table 1 shows the test results of JCT3V-B0160. The BD-rate from our experimental results matches exactly those provided by Samsung. The encoding/decoding time is not matched because of the different simulation environment.

Table 1. Test results under CTC

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 1.1% | 0.1% | 72.2% | 100.2% | 100.1% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 1.3% | 0.2% | 72.3% | 100.2% | 101.2% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 2.2% | 0.5% | 69.8% | 99.0% | 99.6% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 1.7% | 0.3% | 70.4% | 99.4% | 99.5% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.9% | 71.7% | 99.9% | 102.9% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.3% | -0.3% | 70.5% | 99.5% | 101.4% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 1.7% | 0.5% | 73.6% | 99.5% | 100.2% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 1.5% | 0.2% | 71.4% | 99.8% | 100.3% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.9% | -0.1% | 71.5% | 99.6% | 101.0% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 1.2% | 0.1% | 71.5% | 99.7% | 100.7% |

# Conclusion

The cross check results on Enabling CU early termination(ECU) at depth coding in common test conditions by Samsung were matched to those provided by the proponent.

# References

[1] I. Lim, J. Lee, “Enabling CU early termination(ECU) at depth coding in common test conditions,” ISO/IEC JTC1/SC29/WG11, JCT3V-B0160, Shanghai, China, Oct 2012.

[2] Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, “Common Test Conditions for 3DV Core Experiments,” ISO/IEC JTC1/SC29/WG11 JCT2-A1100, Stockholm, Sweden, July 2012.