| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0211 |
| --- | --- |

| Title: | 3D-CE5.h related: Cross check of JCT3V-B0089 on Improvement of MV candidates in 3DVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | J. Jung, E. Mora 38 Rue Général Leclerc 92130 Issy-les-Moulineaux France | Tel: Email: | +33145295366 joelb.jung@orange.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

The document reports cross-check results for the contribution JCT3V-B0089. It has been verified that common conditions are followed and that results provided by the proponent (MediaTek) are complete and match with the results reported in this document. The source code has also been checked, the software has been compiled and no issue is reported.

# Introduction

In Inter View Motion Prediction in 3DV-HTM, a DV estimation process is launched for each block to establish the interview candidate in both AMVP and Merge, wherein neighboring causal blocks are checked, in order, to see whether they are coded with a disparity vector, or whether they are coded with a motion vector which was predicted using a disparity vector (DV-MCP). The DV of the first block that satisfies either condition is retained for the current block. If no DV is found, then DV is set as (0,0).

In AMVP, filling up the AMVP list in case a disparity vector is currently predicted (reference frame is interview) means checking all neighboring blocks for DVs to make spatial candidates. This search is repeated twice, once here and once in the DV estimation process so there is a useless redundancy. Furthermore the disparity vector candidate that comes from the DV estimation process will be equal to one of its spatial predecessors except it will have a zero vertical component so it will be inefficient.

In Merge, the interview candidate is always placed in the first position, whether it is temporal or Multiview. However, putting it in first position means it is considered reliable which isn’t generally the case for Multiview.

# Proposal verification

An improvement on MV candidates for 3DVC is proposed.

First, if no DV is found in the DV estimation process, it is not set to (0,0). The interview candidate in this case simply does not exist.

Second, in AMVP, the DV estimation process is simply used to fill the AMVP candidate list (in case a disparity vector is being predicted of course). Not only the first DV found in the DV estimation process is added but rather all of them. This eliminates the redundancy described above. Plus, the interview candidate itself, which contains a zero vertical component, is removed. This allows DV from DV-MCP blocks to act as predictors as well.

Third, in the Merge candidate list, in case the interview candidate turned out to be Multiview, it is added at the 4th position in the list, and no longer at first.

This cross-check correspond to the test were all three modifications were activated.

Anchor: HTM4.0.1 with common conditions.

Tested: anchor with proposed method

The software has been verified and compiled. It does not raise any issue.

Results obtained by MediaTek and Orange Labs are reported in the following table.

Results obtained by MediaTek

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | -0,4% | -0,4% | -0,2% | -0,2% | -0,2% | 92,6% | 93,2% | 96,5% |
| Kendo | 0,0% | -0,2% | 0,0% | -0,1% | 0,0% | 0,0% | 97,8% | 97,5% | 92,4% |
| Newspapercc | 0,0% | -0,7% | -0,7% | -0,3% | -0,3% | -0,3% | 107,2% | 96,7% | 105,5% |
| GhostTownFly | 0,0% | -0,3% | -0,2% | -0,1% | -0,1% | -0,1% | 94,1% | 83,4% | 81,7% |
| PoznanHall2 | 0,0% | -0,3% | -0,4% | -0,2% | -0,3% | -0,3% | 98,4% | 97,8% | 100,2% |
| PoznanStreet | 0,0% | -1,1% | -1,2% | -0,4% | -0,4% | -0,4% | 99,4% | 98,1% | 99,9% |
| UndoDancer | 0,0% | -0,4% | -0,5% | -0,2% | -0,2% | -0,2% | 91,9% | 87,7% | 89,8% |
| 1024x768 | 0,0% | -0,5% | -0,4% | -0,2% | -0,2% | -0,2% | 99,0% | 95,8% | 98,0% |
| 1920x1088 | 0,0% | -0,5% | -0,6% | -0,2% | -0,2% | -0,2% | 95,9% | 91,5% | 92,6% |
| average | 0,0% | -0,5% | -0,5% | -0,2% | -0,2% | -0,2% | 97,2% | 93,3% | 94,9% |

Results obtained by Orange Labs

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | -0,4% | -0,4% | -0,2% | -0,2% | -0,2% | 98,3% | 99,7% | 97,8% |
| Kendo | 0,0% | -0,2% | 0,0% | -0,1% | 0,0% | 0,0% | 98,2% | 98,9% | 92,4% |
| Newspapercc | 0,0% | -0,7% | -0,7% | -0,3% | -0,3% | -0,3% | 101,1% | 92,3% | 106,0% |
| GhostTownFly | 0,0% | -0,3% | -0,2% | -0,1% | -0,1% | -0,1% | 103,1% | 97,9% | 102,1% |
| PoznanHall2 | 0,0% | -0,3% | -0,4% | -0,2% | -0,3% | -0,3% | 90,4% | 104,3% | 87,4% |
| PoznanStreet | 0,0% | -1,1% | -1,2% | -0,4% | -0,4% | -0,4% | 106,3% | 98,2% | 98,8% |
| UndoDancer | 0,0% | -0,4% | -0,5% | -0,2% | -0,2% | -0,2% | 103,7% | 98,4% | 95,1% |
| 1024x768 | 0,0% | -0,5% | -0,4% | -0,2% | -0,2% | -0,2% | 99,2% | 96,9% | 98,6% |
| 1920x1088 | 0,0% | -0,5% | -0,6% | -0,2% | -0,2% | -0,2% | 100,7% | 99,7% | 95,7% |
| average | 0,0% | -0,5% | -0,5% | -0,2% | -0,2% | -0,2% | 100,0% | 98,5% | 96,9% |

Perfect match was achieved.

# Conclusion

This verification contribution confirms the results obtained by MediaTek.