| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0212 |
| --- | --- |

| Title: | 3D-CE4.h: Crosscheck on Depth Distortion Metric with a weighted depth fidelity term(JCT3V-B0131) |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Li Wang, Lu Yu Zhejiang University | Tel: Email: | +86-571-87953107 wangli@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Introduction

A depth-fidelity term is proposed to the depth distortion metric function, introduced in JCT3V-B0131 [1], referred as WVSO. This document reports the outcome of cross-verification of WVSO. Using VSRS3.5 as the renderer of synthesized views, comparison results with WVSO off and on are shown in Table 1. And the results of the cross verification basically match those provided by the proponents.

# Test results

The coded and synthesized results are basically matched with the results from the proponent. In the test, WVSO_off serves as the anchor. Table 1 shows the results when using VSRS 3.5 as the renderer. Bit saving in terms of synthesis view is 2.6%, higher than the reported 2.4% in [1]. The mismatch should come from the difference of camera parameters used in the synthesising (Un-coded camera parameters were used in the cross-check).

Table 1. WVSO vs. VSO(WVSO_off), using VSRS 3.5 as renderer

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.8% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 1.4% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | -3.7% | -1.8% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -4.1% | -2.5% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.6% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% | -0.1% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | -10.4% | -7.3% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -1.2% | 0.1% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -3.7% | -2.3% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -2.6% | -1.3% |

# Conclusions

The results obtained in this cross verification basically match those of the proponents.

# References

- Jiwook Jung, Sehoon Yea, "CE4.h Results on depth distortion metric with a weighted depth fidelity term", JCT3V-B0131, Shanghai, October 2012