| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0213 |
| --- | --- |

| Title: | 3D-CE1.a related: Results on joint proposal of Qualcomm and MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Li Zhang and Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA Chi-Ling Wu, Yu-Lin Chang, Yu-Pao Tsai, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: Tel: Email: | 86-10-5776-0696 xinzhao@qti.qualcomm.com 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com Shawmin Lei +886-3-5670766 ext. 25555 {chiling.wu, yulin.chang, yupao.tsai, shawmin.lei}@mediatek.com |
| Source: | Qualcomm Incorporated and MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results on joint proposal of Qualcomm and MediaTek. In JCT3V-B0054, generalized view synthesis prediction (GVSP) method is proposed by Qualcomm, and in JCT3V-B0094, an interview skip mode with sub-partition scheme is also proposed by MediaTek. This contribution combines the Qualcomm and MediaTek schemes on top of ATM v5.1. Under common test condition, the joint solution achieves 0.60% BD-rate gain in terms of decoded texture PSNR with 5% decoding time reduction compared to the 3DV-ATM v5.1.

# Introduction

In JCT3V-B0054 [1], Qualcomm proposes to generalize the concept of view synthesis prediction (VSP) with block-level signaling of VSP. With GVSP, a VSP reference picture is never inserted into the reference picture list and block-based VSP (BVSP) is always used to perform VSP prediction. One flag is used to signal whether the current MB partition is predicted using B-VSP.

In JCT3V-B0094 [2], MediaTek proposes an inter-view Skip mode to eliminate full frame view synthesis for reducing the computational complexity, memory bandwidth, and saving reference frame buffer.

In this contribution, both two proposals are combined for the improvement of VSP in terms of both coding performance and decoding complexity.

# Test results

Test results are summarized in Table 1 and Table 2. These tables show BD-rate gain in terms of PSNR of decoded views over texture + depth bit rate and BD-rate gain in terms of PSNR of synthesized views over texture + depth bit rate, respectively when Qualcomm’s GVSP and MediaTek’s interview skip mode with sub-partition scheme are enabled. The ATM v5.1r1 result with common test condition (CTC) [3] is used as anchor.

Table 1 Test results of the proposed joint scheme

|  | Texture Coding |  | Depth Coding |  | Total (Coded) |  | Total (Synthesed) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dPSNR,dB | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -0.91 | 0.03 | 0.00 | 0.00 | -0.82 | 0.03 | -0.86 | 0.03 |
| S02 | -0.38 | 0.01 | 0.00 | 0.00 | -0.35 | 0.01 | -0.32 | 0.01 |
| S03 | -0.56 | 0.02 | 0.00 | 0.00 | -0.52 | 0.02 | -0.41 | 0.01 |
| S04 | -0.90 | 0.03 | 0.00 | 0.00 | -0.85 | 0.03 | -0.78 | 0.03 |
| S05 | -0.62 | 0.03 | 0.00 | 0.00 | -0.53 | 0.02 | -0.52 | 0.02 |
| S06 | -0.56 | 0.03 | 0.00 | 0.00 | -0.51 | 0.02 | -0.46 | 0.02 |
| S08 | -0.29 | 0.01 | 0.00 | 0.00 | -0.26 | 0.01 | -0.21 | 0.01 |
| Average | -0.60 | 0.02 | 0.00 | 0.00 | -0.55 | 0.02 | -0.51 | 0.02 |

Table 2 Computational time of the proposed joint scheme

|  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- |
|  | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 111.60 | 93.35 | 104.10 |
| S02 | 110.27 | 90.29 | 100.89 |
| S03 | 112.81 | 99.28 | 101.04 |
| S04 | 111.21 | 97.53 | 101.51 |
| S05 | 112.67 | 91.91 | 106.77 |
| S06 | 113.14 | 94.16 | 100.83 |
| S08 | 112.22 | 98.55 | 101.01 |
| Average | 111.99 | 95.01 | 102.31 |

# Conclusion

In this contribution, a joint proposal of Qualcomm and MediaTek was integrated on 3D-ATM v5.1 and performance evaluation was reported. Under common test condition, it is reported that the joint solution achieves -0.60% BD-rate gain in terms of decoded texture PSNR, -0.51% BD-rate gain in terms of synthesized PSNR and 5% decoding time reduction.

# References

- X. Zhao, L. Zhang, and Y. Chen, “3D-CE1.a: Generalized view synthesis prediction (GVSP) mode,” JCT3V-B0054, Shanghai, CN, 13–19 Oct. 2012.

- C.-L. Wu, Y.-L. Chang, Y.-P. Tsai, S. Lei, “3D-CE1.a: interview skip/direct mode with sub-partition scheme,” JCT3V-B0094, Shanghai, CN, 13–19 Oct. 2012.

- H. Schwarz and D. Rusanovskyy, “Common Test Conditions for 3DV experimentation,” JCT3V-A1100, Stockholm, SE, 16–20 July 2012.

# Patent rights declaration (s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).