| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0217 |
| --- | --- |

| Title: | 3D-CE5.h related: Cross-check of JCT3V-B0051 on Advanced residual prediction for multiview coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng | Tel: Email: | zhengxiaozhen@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution document reports the cross verification results of proposal B0051. The source code was implemented based on HTM-4.0. We compiled, inspected, and ran the code of these cases with CTC configurations. The cross-check has been completed successfully and the results match those provided by proponents of JCT3V-B0051.

# Test description and crosscheck results

This document reports the cross-check results of B0051. Two test cases are proposed by B0051. The settings of the two test cases are set as follows

Proposed #1:

#define LGE_DVMCP                         1

#define QC_ARP      1      ////< advanced residual prediction

Proposed #2:

#define LGE_DVMCP                         0

#define QC_ARP      1      ////< advanced residual prediction

The encoder and decoder reconstructed YUV files generated from the above settings are matched. The experimental results of the rate-distortion are illustrated in Error: Reference source not found. These results match the BD-rate results provided by proponents of JCT3V-B0051. Please notice that the encoding and decoding time are un-reliable.

Table 1: Cross-check results of B0051 (Proposed#1)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -2.5% | -2.6% | -0.9% | -0.7% | -0.7% |  |  |  |
| Kendo | 0.0% | -3.3% | -3.4% | -1.2% | -0.9% | -0.9% |  |  |  |
| Newspapercc | 0.0% | -1.8% | -1.6% | -0.6% | -0.3% | -0.4% |  |  |  |
| GhostTownFly | 0.0% | -1.8% | -2.2% | -0.5% | -0.3% | -0.3% |  |  |  |
| PoznanHall2 | 0.0% | -1.0% | -0.4% | -0.3% | -0.4% | -0.4% |  |  |  |
| PoznanStreet | 0.0% | -2.8% | -2.9% | -0.8% | -0.7% | -0.7% |  |  |  |
| UndoDancer | 0.0% | -2.3% | -2.6% | -0.6% | -0.4% | -0.4% |  |  |  |
| 1024x768 | 0.0% | -2.5% | -2.5% | -0.9% | -0.7% | -0.7% |  |  |  |
| 1920x1088 | 0.0% | -2.0% | -2.0% | -0.5% | -0.4% | -0.5% |  |  |  |
| average | 0.0% | -2.2% | -2.2% | -0.7% | -0.5% | -0.5% |  |  |  |

Table 1: Cross-check results of B0051 (Proposed#2)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -3.2% | -3.0% | -1.1% | -0.8% | -0.9% |  |  |  |
| Kendo | 0.0% | -3.7% | -4.3% | -1.5% | -1.1% | -1.1% |  |  |  |
| Newspapercc | 0.0% | -2.7% | -1.9% | -0.8% | -0.5% | -0.6% |  |  |  |
| GhostTownFly | 0.0% | -1.6% | -1.9% | -0.4% | -0.3% | -0.3% |  |  |  |
| PoznanHall2 | 0.0% | -1.3% | -1.2% | -0.5% | -0.6% | -0.6% |  |  |  |
| PoznanStreet | 0.0% | -3.5% | -4.0% | -1.1% | -0.9% | -1.0% |  |  |  |
| UndoDancer | 0.0% | -2.6% | -2.6% | -0.7% | -0.4% | -0.5% |  |  |  |
| 1024x768 | 0.0% | -3.2% | -3.1% | -1.1% | -0.8% | -0.9% |  |  |  |
| 1920x1088 | 0.0% | -2.3% | -2.4% | -0.7% | -0.6% | -0.6% |  |  |  |
| average | 0.0% | -2.7% | -2.7% | -0.9% | -0.7% | -0.7% |  |  |  |