| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0219 |
| --- | --- |

| Title: | BoG report on conformance testing of MVC+D |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
| Source: | BoG on conformance testing of MVC+D |  |  |

_____________________________

# Agenda

- Reviewing of related contributions

  - JCT3V-B0032 and B0067

- Bitstream design, features, and the assignment of volunteers

- Plan

- BoG recommendations

# Review of contributions

JCT3V-B0032 AHG12: Proposal on the interlace texture plus interlace depth conformance stream [Ching-Chieh Lin, Fang-Chu Chen (ITRI)]

This document proposes a conformance test stream related to interlace texture plus interlace depth for MV+D extension of AVC. Interlace depth was generated using Undo_Dacer (m25069). The software supporting interlace texture and depth was included in JCT3V-A0144. The proponent can provide the interlace sequences proposed in m25069. But the copyright of Undo_Dancer needs to be confirmed.

JCT3V-B0067 AHG12: On conformance testing of the MVC plus depth extension of AVC [T. Suzuki, S. Hattori (Sony), D. Rusanovskyy (Nokia)]

This contribution proposes the initial idea of conformance testing of Multiview plus depth extension of AVC. The features to be tested by conformance testing are proposed as a starting point of the discussion. The volunteers for missing features should be identified during Shanghai meeting. The initial draft of the conformance testing is also attached to this contribution.

BoG agreed to use this draft specification as a starting point.

# Conformance bitstreams

| Categories | Bitstream features | Volunteers | File Name | Multi-view Depth High | Level | Frame Rate (Frame/Sec) |
| --- | --- | --- | --- | --- | --- | --- |
| Depth Resolution | Ballons (d0.5) | Nokia |  | X | 3 and higher | 30 |
|  | Street (d0.5) | Nokia |  | X | 4 and higher | 25 |
|  | Ballon (d1.0) | Sony |  | X | 3 and higher | 30 |
|  | Street (d1.0) | Sony |  | X | 4 and higher | 25 |
|  | Various combination of texture/depth resolutions |  |  | X |  |  |
|  |  |  |  | X |  |  |
| Views Configuration | Ballon (C2) | Sony |  | X | 3 and higher | 30 |
|  | Street (C2) | Sony |  | X | 4 and higher | 25 |
|  | Ballon (C3) | Nokia |  | X | 3 and higher | 30 |
|  | Street (C3) | Nokia |  | X | 4 and higher | 25 |
| Inter View Prediction / temporal inter prediction | Street (PIP) | Nokia |  | X | 4 and higher | 25 |
|  | Ballon (IBP) | Sony |  | X | 3 and higher | 30 |
|  | temporal inter prediction (IPP) | MERL, Qualcomm |  | X |  |  |
|  | temporal inter prediction (IBP) | MERL, Qualcomm |  | X |  |  |
|  | Various combinations |  |  | X |  |  |
|  |  |  |  | X |  |  |
|  | IPP | MERL, Qualcomm |  | X |  |  |
|  | IBP | MERL, Qualcomm |  | X |  |  |
| Interlaced Coding Tools | Interlace view + progressive depth | ITRI, Sony |  | X |  |  |
|  | Interlace view + interlace depth | ITRI |  | X |  |  |
| Corner case (This aspects should be considered to generate bitstreams) | maximum number of view/depth in DPB |  |  |  |  |  |

# Plan

## Procedure

- Repository for this activity is necessary (using AVC conformance/bitstream exchange site ?)

  - The volunteer uploads the followings to the repository

    - bitstream

    - short description

    - yuv file

    - trace file (optional: useful for debugging. Recommend to include short trace. full trace is optional)

- Software

  - Encoder: any encoders can be used. (There are two reference software implementations, JMVC and ATM at the moment. The HLS in the current JMVC is not updated. The software needs to be updated before this activity).

  - Decoder: we should clarify which decoder is used. (At the next meeting)

- Acceptance of bitstream

  - Uploaded bitstreams should be verified

    - Verify if it can be decoded by MVC+D decoder

      - It is better to confirmed if part of bitstream can be decoded by AVC, MVC decoder.

    - Confirm bitstream feature are correctly included by checking trace file.

- Confirm copy right of test materials. Use only the sequences allowed for this purpose.

  - Balloon and Street can be used.

  - Need to check Undo_dancer is allowed to use for this purpose.

## Time line

- When should we start ? The activity should be started after this meeting as the first exercise.

  - Update the software first, based on the text (output document at this meeting)

  - Start to generate bitstreams (about one month after this meeting)

  - Then revise the bitstreams when FDAM based software is produced in Jan 2013.

  - All bitstreams must be ready by FDAM of conformance spec (maybe one year later)

# Recommendations

BoG recommends,

- to use the draft specification proposed in JCT3V-0067 as a starting point

- to set up the repository for this activity

- to update the reference software based on the latest draft specification

- to approve the work plan proposed by the BoG