| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 October 2012 | Document: JCT3V-B0222 |
| --- | --- |

| Title: | Subjective Test Results on QuadTree limitation and predictive coding (JCT3V-B0068) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | J. Jung, E. Mora 38 Rue Général Leclerc 92130 Issy-les-Moulineaux France | Tel: Email: | +33145295366 joelb.jung@orange.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

During the #2 JCT-3V meeting, quadtree limitation + predictive coding was proposed in JCT3V-B0068. For a given CTU, the quadtree of the depth is linked to the collocated CTU quadtree in the texture. A given CU of the depth cannot be split more than its collocated CU in the texture. The quadtree of the depth is predictively encoded relatively to the quadtree of the texture. Experimental results exhibit some losses on synthetized views (PSNR losses), while gains are obtained for coded+synthesized views. It is reported that this PSNR loss is related to the smoothing of wrong edges in the depth. The goal of this subjective session is consequently to verify if any new artifact is introduced by this method, from a subjective point of view.

# Introduction

Subjective viewing tests took place during the meeting. Two viewing sessions were organized. A total of 9 people has taken part, from 6 different companies or universities. It has been verified by a non-proponent that the results reported in this document match with the scoring sheets.

# Test settings

## Sequences

Two sequences have been selected, Newspaper and Balloons given that these two sequences where providing the highest objective losses on synthesized views. Additionally, QP 25 has been selected because it provides the highest loss among other QPs, and QP 35 selected to provide the viewers with a more degraded version.

## Viewing session

Reference and proposed method are presented as sequences A and B (as a blind test) in the following order:

| TEST NUMBER | Seq A | Display Seq A | Seq B | Display Seq B | Seq A | Display Seq A | Seq B | Display Seq B | Vote |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For each test, a score between -3 and +3 is requested, using the following scale:

| Sequence A is | +3: largely better than +2: better than +1: slightly better than 0: same as -1: slightly worse than -2 worse than -3: largely worse than | Sequence B |
| --- | --- | --- |

## Tests

# Two tests have been conducted: one on 3D, one on 2D.

# 3D test: a stereo view has been constructed from two synthesized views, corresponding to the two views that are the closest to the center view.

2D test: the closest synthesized view on the left of the center view has been used. This 2D test is designed in case the 3D test does not allow seeing any new artifact.

# Results

## 3D test

Coding scores: negative scores represent an improvement resulting from the proposed method

| Test | AVG | Viewer |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| Balloons 25 | -0,6 | 0 | -1 | -1 | -1 | 0 | 0 | -1 | -1 | 0 |
| Newspaper 25 | 0,1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| Balloons 35 | 0,4 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| Newspaper 35 | -0,2 | 0 | 0 | -1 | 0 | 0 | 0 | 0 | -1 | 0 |
| AVG | -0,1 |  |  |  |  |  |  |  |  |  |

Observations:

- No score above 1 or below -1 is reported. It is confirmed that no new annoying artifact is noticeable.

- Results are particularly consistent. For each sequence, there was either no difference, or one of the method claimed as slightly better. Said differently, for one given experiment, it is not possible to find a score of +1 and -1 simultaneously.

- The average score is -0.1 which is slightly in favor of the proposed method.

## 2D test

Coding scores: negative scores represent an improvement resulting from the proposed method

| Test | AVG | Viewer |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| Balloons 25 | 0,2 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| Newspaper 25 | -0,7 | -1 | -1 | -1 | -1 | 0 | 0 | -1 | 0 | -1 |
| Balloons 35 | 0 | 0 | 0 | 0 | 1 | -1 | -1 | 0 | 1 | 0 |
| Newspaper 35 | -0,3 | 0 | -1 | 0 | -1 | -1 | 1 | 0 | -1 | 0 |
| AVG | -0,2 |  |  |  |  |  |  |  |  |  |

Observations:

- No score above 1 or below -1 is reported. It is confirmed that no new annoying artifact is noticeable.

- 2D scores show more clearly the slight gain resulting from the proposed method, compared to the 3D scores.

- The average score is -0.2 which is in favor of the proposed method.

# Conclusion

# This subjective testing reports that no new artifact is introduced by the quadtree limitation + predictive coding as proposed in JCT3V-B0068, experiment H3.