| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0224 |
| --- | --- |

| Title: | 3DV-ATM: Working draft text for B-VSP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

In this document, we provide working draft text description for B-VSP scheme which was adopted to 3DV-ATM software at the 1st JCT-3V meeting.

# Changes to 3D-AVC Working Draft document

The following are proposed changes to the Working draft of 3D-AVC specification.

- J.8.3	3DVC inter prediction, inter-view prediction, and view synthesis prediction process

..

Subclause H.8.4 is applied with the following the following changes.

–	The following paragraph is added after the paragraph starting with “Outputs of this process are …”:

When nal_unit_type is equal to 21 and DepthFlag is equal to 0, subclause  is invoked to decode the sample arrays for each synthetic reference picture included in RefPicListX for X equal to 0 and, if the current slice is a B slice, 1.

–	When nal_unit_type is equal to 21, DepthFlag is equal to 0, and dmvp_flag is equal to 1, subclause Error: Reference source not found is invoked instead of subclause 8.4.1.

–	When nal_unit_type is equal to 21, DepthFlag is equal to 1, and mb_ivmp_flag is equal to 1, subclause Error: Reference source not found is invoked instead of subclause 8.4.1.

–	The following additional step applied between steps 3 and 4 in subclause 8.4.

–	When (nal_unit_type is equal to 21 and depth_weighted_pred_flag is equal to 1 and (slice_type % 5) is equal to 0 or 3) or (depth_weighted_bipred_flag is equal to 1 and (slice_type % 5) is equal to 1), the derivation process for prediction weights in depth-range-based weighted prediction in subclause Error: Reference source not foundis invoked.

–	The following sentence “The decoding process for Inter prediction samples as specified in subclause 8.4.2 is invoked.” in step 4 of subclause 8.4 is replaced by:

If refIdxL0 indicates a synthetic reference component, the derivation process for luma and chroma samples predicted from synthetic reference component in subclause J.8.3.2  is invoked.

Inputs to this process are: [Ed. match the inputs with those of J.8.3.2.]

–	a macroblock partition mbPartIdx,

–	a sub-macroblock partition subMbPartIdx,

–	variables specifying partition width and height for luma and chroma (if available), partWidth, partHeight, partWidthC (if available), and partHeightC (if available),

–	luma motion vectors mvL0 and mvL1 and when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags predFlagL0 and predFlagL1,

–	variables for weighted prediction logWDC, w0C, w1C, o0C, o1C with C being replaced by L and, when ChromaArrayType is not equal to 0, Cb and Cr.

Outputs of this process are inter prediction samples (pred); which are a (partWidth)x(partHeight) array predPartL of prediction luma samples and when ChromaArrayType is not equal to 0 two (partWidthC)x(partHeightC) arrays predPartCr, and predPartCb of prediction chroma samples, one for each of the chroma components Cb and Cr.

Otherwise (refIdxL0 does not indicate a synthetic reference component), the decoding process for Inter prediction samples as specified in subclause 8.4.2 is invoked.

The following section replaces the current subclause J.8.3.2

J.8.3.2	Derivation process for luma and chroma samples predicted from synthetic reference component

This process is invoked when nal_unit_type is equal to 21, DepthFlag is equal to 0 and refIdxLX is referencing to a synthetic reference component that appears in the RefPicListX, where X=0,1.

Inputs of this process are:

Motion vector mvLX

the current partition of texture given by its partition index mbPartIdx and its sub-macroblock partition index subMbPartIdx,

the width and height partWidth, partHeight of this partition in luma-sample units,

a decoded depth view component currDepthPic,

a decoded texture view component srcTexturePicY and, if chroma_format_idc is equal to 1 srcTexturePicCb and srcTexturePicCr of the reference texture component,

Outputs of this process are:

– a (partWidth)x(partHeight) array predPartLXL of prediction luma sample values,

– when ChromaArrayType is not equal to 0, two (partWidthC)x(partHeightC) arrays predPartLXCb, and predPartLXCr of prediction chroma sample values.

Let ( xAL, yAL ) be the location given in full-sample units of the upper-left luma sample of the current partition given by mbPartIdx\subMbPartIdx relative to the upper-left luma sample location of the given two-dimensional array of luma samples.

For each luma sample location (0 <= xL < partWidth, 0 <= yL < partHeight) inside the prediction luma sample array predPartLXL, the corresponding prediction luma sample value predPartLXL[ xL, yL ] is derived as specified by the following ordered steps:

- Variable Disparity_x is  derived by:

dX = xAL >> reduced_resolution_flag;

dY = yAL >> reduced_resolution_flag;

depth_sample = currDepthPic[dY][dX]

Disparity_x =dispQPel ( NdrInverse[ d ], ZNear[dps_id, index ], ZFar[dps_id, index ], FocalLengthX[dps_id, index ], AbsTX[ index ] – AbsTX[ refIndex ] , PrincipalPointX[ index] – PrincipalPointX[ refIndex]),

wherein index and refIndex are derived by the following formulas:

index = ViewIdTo3DVAcquisitionParamIndex(view_id of the current view)

refIndex = ViewIdTo3DVAcquisitionParamIndex(ViewId of the texture view component referenced by refIdxLX)

- Variable vsp_mv is derived as:

vsp_mv  = clip( 0, (WidthT<<2)-1, mvLXN[0] + Disparity_x));

where WidthT is a width of the current texture component

- Variables xIntL, yIntL, xFracL, and yFracL are derived by:

xIntL = xAL + (vsp_mv  >> 2 ) + xL

yIntL = yAL

xFracL = vsp_mv & 3

yFracL = 0

- The prediction luma sample value predPartLXL[ xL, yL ] is derived by invoking the process specified in subclause 8.4.2.2.1 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLXL given as input.

When ChromaArrayType is not equal to 0, the following applies.

For each choma sample location (0 <= xL < partWidthC, 0 <= yL < partHeightC) inside the prediction chroma sample array predPartLXc, the corresponding prediction chroma sample value predPartLXc[ xC, yC ] is derived as specified by the following ordered steps:

- Variable Disparity_x is  derived by:

dX = xAL;

dY = yAL;

depth_sample = currDepthPic[y][x]

Disparity_x =dispQPel ( NdrInverse[ d ], ZNear[dps_id, index ], ZFar[dps_id, index ], FocalLengthX[dps_id, index ], AbsTX[ index ] – AbsTX[ refIndex ] , PrincipalPointX[ index] – PrincipalPointX[ refIndex]),

wherein index and refIndex are derived by the following formulas:

index = ViewIdTo3DVAcquisitionParamIndex(view_id of the current view)

refIndex = ViewIdTo3DVAcquisitionParamIndex(ViewId of the input texture view component)

- Variable vsp_mv is derived as:

vsp_mv  = clip( 0, WidthT-1, (mvLXN[0] + Disparity_x)>>1));

where WidthC is a width of the current texture component

Depending on ChromaArrayType, the variables xIntC, yIntC, xFracC, and yFracC are derived as follows:

- Variables xIntC, yIntC, xFracC, and yFracC are derived by:

xIntc = xAc + (vsp_mv  >> 2 ) + xc

yIntc = yAc

xFracc = vsp_mv & 5

yFracc = 0

Otherwise, if ChromaArrayType is equal to 2,

xIntC = ( xAL / SubWidthC ) + ( mvCLX[ 0 ] >> 2 ) + xC

yIntC = ( yAL / SubHeightC ) + ( mvCLX[ 1 ] >> 1 ) + yC

xFracC = mvCLX[ 0 ] & 5

yFracC = ( mvCLX[ 1 ] & 1 ) << 1

– Otherwise (ChromaArrayType is equal to 3),

xIntC = xAL + ( mvLX[ 0 ] >> 1 ) + xC

yIntC = yAL + ( mvLX[ 1 ] >> 1 ) + yC

xFracC = ( mvCX[ 0 ] & 1 )

yFracC = ( mvCX[ 1 ] & 1 )

- Depending on ChromaArrayType, the following applies:

– If ChromaArrayType is not equal to 3, the following applies:

– The prediction sample value predPartLXCb[ xC, yC ] is derived by invoking the process specified in

subclause 8.4.2.2.2 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLXCb given as input.

– The prediction sample value predPartLXCr[ xC, yC ] is derived by invoking the process specified in

subclause 8.4.2.2.2 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLXCr given as input.

– Otherwise (ChromaArrayType is equal to 3), the following applies:

– The prediction sample value predPartLXCb[ xC, yC ] is derived by invoking the process specified in

subclause 8.4.2.2.1 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLXCb given as input.

– The prediction sample value predPartLXCr[ xC, yC ] is derived by invoking the process specified in

subclause 8.4.2.2.1 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLXCr given as input.

For C being replaced by L, Cb (if available), or Cr (if available), the array predPartC of the prediction samples of component C is derived by invoking the process specified in subclause Error: Reference source not found with the current partition specified by mbPartIdx and subMbPartIdx, the prediction utilization flags predFlagL0 and predFlagL1, the arrays predPartL0C and predPartL1C, and the variables for weighted prediction logWDC, w0C, w1C, o0C, o1C given as input.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).