| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0225 |
| --- | --- |

| Title: | Report: Subjective testing results on comparing synthesis quality of IDWR and VSRS 1D Fast |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Nikolce Stefanoski | Email: | stefanos@disneyresearch.com |
| Source: | Disney Research Zurich |  |  |

_____________________________

# Abstract

According to proposal JCT3V-B0077, the relative synthesis quality between IDWR and VERS 1D Fast was assessed in a formal subjective test conducted during the meeting. In the testing, 10 subjects participated. Test data was used, which was generated by synthesizing new views with both synthesis methods using original uncoded data. The subjective testing showed that a significant majority of 75%-100% considered the quality provided by both synthesis methods either comparable or preferred IDWR. This confirms that the warp data, which was provided in JCT3V-B0074, is of sufficiently high quality to be used as test data for warp coding experiments and further analysis in the scope of the JCT-3V activity.

# Introduction

In this document, subjective testing results are reported, which compare the synthesis quality of IDWR and VSRS 1D Fast. Testing was performed according to the description in JCT3V-B0077 in the scope of mandate 2 of the AhG8 on Alternative 3D Formats.

In the testing, 10 subjects participated. During the testing, two versions of each sequence where shown on a stereoscopic display, which were synthesized with the respective synthesis methods. Subjects had to assess the relative quality by voting according to the following voting scale:

Quality

0 – The same

1 – First version is slightly better

11 – First version is much better

2 – Second version is slightly better

22 – Second version is much better

Two repetitions where conducted in a randomized order. Thus, 20 statistically independent votes where obtained from 10 subjects for each pair of stereo sequences to be compared.

# Results

Figure 1 shows the voting results. Detailed voting results are reported in attached excel file. Histograms are shown according to the voting scale. Two histograms are shown for each sequence. The histogram indicated as “original votes”, shows the histogram obtained by all original votes which were obtained in 2 voting passes. The histogram indicated as “adjusted votes”, shows the histogram of all votes again but after removing voting inconsistencies. Voting inconsistencies could be detected due to the double voting for each pair of stereo sequences (2 voting passes in a randomized order). Votes where adjusted only in the case where for the same pair of stereo sequences participants preferred once IDWR and once VSRS 1D Fast as a synthesis method. Such inconsistencies where removed by counting the corresponding voting results as a voting for same quality.

Figure 1. Histograms of votes according to the voting scale. “Original votes” shows the histogram over the originally obtained votes. “Adjusted votes” shows the histogram after removing voting inconsistencies, which could be detected due to the 2 pass voting procedure.

Table 1 shows a summary of the voting results. For 5 sequences out of 7 (excluding Dancer and Newspaper) a significant majority voted that both synthesis methods provide the same quality. In the case of the sequence Dancer 45% voted for same quality and another 45 % preferred IDWR, while in the case of sequence Newspaper a significant majority of 70 % preferred IDWR.

A major preference for a synthesis method (“much better” vote) is only observed in the case of the sequences Dancer, GT_Fly and Newspaper, while in case of the other four sequences the quality was perceived as the same or only a slight preference for a synthesis method was expressed. A major preference for IDWR was expressed in the case of Dancer and Newspaper with 15% and 20%, respectively, while in the case of GT_Fly, a major preference for VSRS 1D Fast was expressed with 5%.

Table 1: Summary of voting results.

To be able to verify if the warp data provided in JCT3V-B0074 (this data was used for the synthesis with IDWR) allows a synthesis quality which is comparable or better to a synthesis with depth, corresponding voting results are summarized in Table 2. Obviously a significant majority in the range between 75%-100% depending on the sequence supports this hypothesis.

Table 2: Voting results for the event “same quality or IDWR is better”.

# Conclusion

The subjective testing showed that a significant majority of 75%-100% considered the quality provided by both synthesis methods either comparable or preferred IDWR. This confirms that the warp data, which was provided in JCT3V-B0074, is of sufficiently high quality to be used as test data for warp coding experiments and further analysis in the scope of the JCT-3V activity.

# Patent rights declaration(s)

Disney Research Zurich may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).