| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B0227 M27270 |
| --- | --- |

| Title: | AHG8: Proposal on Signaling of Alternative 3D Format in ISO/IEC 14496-10 (ITU-T H.264) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Nukui-Kitamachi 4-2-1, Koganei, Tokyo 184-8795, Japan Masayuki Tanimoto, Zehn 101, Kawana-cho 2-64, Showa-ku, Nagoya, 466-0856, Japan | Tel: Email: | +81-42-327-7262 senoh@nict.go.jp +81-52-223-6639 tanimoto@nisri.jp |
| Source: | National Institute of Information and Communications Technology(NICT) Nagoya Industrial Science Research Institute(NISRI) |  |  |

_____________________________

# Abstract

This contribution proposes a solution for a mandate for AHG8, which is

- to investigate possible signaling methods required to identify the type of 3D data format (e.g. global view/depth, warp) and to interpret such decoded 3D data by non-normative processing after decoding.

The solution proposes a new depth_type in Video Usability Information (VUI) for ISO/IEC 144996-10 (ITU-T H.264). (option 1)

Another option is a new depth_type in SEI messagefor ISO/IEC 144996-10 (ITU-T H.264). (optin 2)

These depth_type signal alternative 3D format to an informative view synthesis process after decoding.

# Introduction

At the 1st JCT-3V meeting (July 2012), AHG8 was established. One of its mandates is to investigate possible signaling methods required to identify the type of 3D data format (e.g. global view/depth, warp) and to interpret such decoded 3D data by non-normative processing after decoding. This contribution proposes a solution for that mandate based on ISO/IEC 144996-10 (ITU-T H.264) by specifying a new depth_type. This signaling will be an amendment for ISO/IEC 144996-10 (ITU-T H.264). The following text is prepared for such amendment for ISO/IEC 144996-10 (ITU-T H.264).

# Syntax and Semantics (normative)

# (option 1)

In 0.6, add the following paragraph after the last paragraph that starts with “An extension of Multiview video coding”:

A signaling method that supports the alternative 3D format is specified in Annex E: Video usability information and associated non-normative view synthesis process is specified in Annex J allowing the construction of bitstreams that represent alternative 3D format based on the ISO/IEC 14496-10 specifications. Since the alternative 3D format represents both Global View and Depth format and Warp format, a depth_type is specified in the video usability information to identify the decoded data for view synthesis process.

In 0.7, add the following paragraph after the paragraph that starts with “Annex I specifies”:

Annex J specifies an informative view synthesis process according to the signaling of alternative 3D format specified in Annex E. The reader is referred to Annex J for the entire view synthesis process for Alternative 3D format, which is specified there with references being made to clauses 2-9 and Annexes A-I.

- In E.1.1, replace the syntax table with:

    - VUI parameter syntax

| - vui_parameters( ) { | C | Descriptor |
| --- | --- | --- |
| - aspect_ratio_info_present_flag | - 0 | - u(1) |
| - if( aspect_ratio_info_present_flag ) { |  |  |
| - aspect_ratio_idc | - 0 | - u(8) |
| - if( aspect_ratio_idc = = Extended_SAR ) { |  |  |
| - sar_width | - 0 | - u(16) |
| - sar_height | - 0 | - u(16) |
| - } |  |  |
| - } |  |  |
| - overscan_info_present_flag | - 0 | - u(1) |
| - if( overscan_info_present_flag ) |  |  |
| - overscan_appropriate_flag | - 0 | - u(1) |
| - video_signal_type_present_flag | - 0 | - u(1) |
| - if( video_signal_type_present_flag ) { |  |  |
| - video_format | - 0 | - u(3) |
| - video_full_range_flag | - 0 | - u(1) |
| - colour_description_present_flag | - 0 | - u(1) |
| - if( colour_description_present_flag ) { |  |  |
| - colour_primaries | - 0 | - u(8) |
| - transfer_characteristics | - 0 | - u(8) |
| - matrix_coefficients | - 0 | - u(8) |
| - } |  |  |
| - } |  |  |
| - chroma_loc_info_present_flag | - 0 | - u(1) |
| - if( chroma_loc_info_present_flag ) { |  |  |
| - chroma_sample_loc_type_top_field | - 0 | - ue(v) |
| - chroma_sample_loc_type_bottom_field | - 0 | - ue(v) |
| - } |  |  |
| - timing_info_present_flag | - 0 | - u(1) |
| - if( timing_info_present_flag ) { |  |  |
| - num_units_in_tick | - 0 | - u(32) |
| - time_scale | - 0 | - u(32) |
| - fixed_frame_rate_flag | - 0 | - u(1) |
| - } |  |  |
| - nal_hrd_parameters_present_flag | - 0 | - u(1) |
| - if( nal_hrd_parameters_present_flag ) |  |  |
| - hrd_parameters( ) | - 0 |  |
| - vcl_hrd_parameters_present_flag | - 0 | - u(1) |
| - if( vcl_hrd_parameters_present_flag ) |  |  |
| - hrd_parameters( ) | - 0 |  |
| - if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ) |  |  |
| - low_delay_hrd_flag | - 0 | - u(1) |
| - pic_struct_present_flag | - 0 | - u(1) |
| - bitstream_restriction_flag | - 0 | - u(1) |
| - if( bitstream_restriction_flag ) { |  |  |
| - motion_vectors_over_pic_boundaries_flag | - 0 | - u(1) |
| - max_bytes_per_pic_denom | - 0 | - ue(v) |
| - max_bits_per_mb_denom | - 0 | - ue(v) |
| - log2_max_mv_length_horizontal | - 0 | - ue(v) |
| - log2_max_mv_length_vertical | - 0 | - ue(v) |
| - num_reorder_frames | - 0 | - ue(v) |
| - max_dec_frame_buffering | - 0 | - ue(v) |
| - } |  |  |
| - depth_type_info_present_flag | - 0 | - u(1) |
| - if(depth_type_info_present_flag) { |  |  |
| depth_type | 0 | u(3) |
| } |  |  |
| - } |  |  |

Add the following text after the paragraph on max_dec_fram_buffering in E.2.1:

depth_type_info_present_flag equal to 1 specifies that signaling of alternative 3D format by depth_type is present. Depth_type_info_present_flag equal to 0 specifies that the signaling is not present.

depth_type specifies the view synthesis type for Alternative 3D format in MVC+Depth extension (Annex I). The depth_type = 0 is reserved. The depth_type = 1 signals view synthesis by global depth without residual depth. The depth_type = 2 signals view synthesis by global depth with residual depth. The depth_type = 3 signals view synthesis by two global depth without residual depth. The depth_type = 4 signals view synthesis by two short global depth without residual depth.

The associated view synthesis process is specified in Annex J.

- Table E-7 – Meaning of depth_type

| - depth_type | - Meaning |
| --- | --- |
| - 0 | - reserved |
| - 1 | - Global Depth without residual depth |
| - 2 | - Global Depth with residual depth |
| - 3 | - Two Global Depths without residual depth |
| - 4 | - Two short Global Depths without residual depth |
| - 5…8 | - unspecified |

# (option 2)

In 0.6, add the following paragraph after the last paragraph that starts with “An extension of Multiview video coding”:

A signaling method that supports the alternative 3D format is specified in Annex D: Supplemental Enhancement Information and associated non-normative view synthesis process is specified in Annex J allowing the construction of bitstreams that represent alternative 3D format based on the ISO/IEC 14496-10 specifications. Since the alternative 3D format represents both Global View and Depth format and Warp format, a depth_type is specified in the SEI message to identify the decoded data for view synthesis process.

In 0.7, add the following paragraph after the paragraph that starts with “Annex I specifies”:

Annex J specifies an informative view synthesis process according to the signaling of alternative 3D format specified in SEI message. The reader is referred to Annex J for the entire view synthesis process for Alternative 3D format, which is specified there with references being made to clauses 2-9 and Annexes A-I.

In Table D.1, add the following after “depth_representation_information ( payloadSize )”:

| else if( payloadType = = 48 ) |  |  |
| --- | --- | --- |
| 3dv_scalable_nesting( payloadSize ) |  |  |
| else if ( payloadType = = 49 ) |  |  |
| 3d_view_scalability_info( payloadSize ) |  |  |
| else if (payloadType = = 50 ) |  |  |
| depth_acquisition_info ( payloadSize ) |  |  |
| else if (payloadType = = 51 ) |  |  |
| depth_representation_information ( payloadSize ) |  |  |
| else if (payloadType = = 52 ) |  |  |
| depth_type_info ( payloadSize ) |  |  |

In  the following Table)”:

    - Depth type information SEI message syntax

| depth_type_info ( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_type | 5 | u (3) |
| } |  |  |

Add Annex J:

- Annex J

- View Synthesis process for alternative 3D format
(informative)

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies the view synthesis process for Global View and Depth in Alternative 3D Format for 3DVC.

  - Scope

This document specifies the associated view synthesis process for global view and depth in alternative 3D format, which is signaled by depth_type in video usability information in Annex. E.

  - Normative references

The specifications in clause Error: Reference source not found apply.

  - Definitions

For the purpose of this annex, the following definitions apply additionally.

    - Alternative 3D format: is an alternative 3D video format for multi-view and depth video coding which is specified in Annex I.

    - Global view and depth: is an alternative 3D format which synthesizes multi-view images base on global depth.

  - Abbreviations

The specifications in clause Error: Reference source not found apply.

  - Conventions

The specifications in clause Error: Reference source not found apply.

  - Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specifications in clause Error: Reference source not found apply.

  - Syntax and semantics

Syntax and semantics are completely specified in clauses 2-9 and Annexes A-I.

  - Decoding process

- Decoding process is completely specified in clauses 2-9 and Annexes A-I.

  - View synthesis process for Alternative 3D Format

    - View synthesis process for global view and depth

This sub-clause specifies view synthesis process based on global view and depth.

        - View synthesis process for global depth without residual depth

          - View synthesis process for 3 view case

- Target views including view_2 (left view) and view_3 (right view) are synthesized from decode view_1 (center view: base view), decoded depth_1 (center depth: global depth) decoded view_2 (residual view), extracted camera_parameter and provided target view ID. Figure J.9.1 illustrates the view synthesis process.

- Figure J.9.1.1.1.1 View synthesis process for global depth without residual depth (3-view case)

- First, decoded depth_1 (global depth at center view) is up-sampled. During the up-sampling, if the difference of contiguous two depth samples are smaller than th (=10, etc.), the average value is inserted between them. Otherwise, a larger sample is repeated. After the up-sampling, 2D median filter of 3x3 and 5x5 are applied to smooth the shape of depth edge. Then, each depth pixel D(x,y) in the global depth map is projected to left target depth pixel D(xL, y) and right target depth pixel D(xR, y). The corresponding address xL, xR in the target views are given by the following equation. Where, f is focal length of camera, BL and BR are baseline lengths between the target views and global depth, D(x,y) is depth value of global depth pixel (x,y).

δK=fBK255(1Znear−1Zfar)D(x,y), (K=L, or R)xL=x+δL, xR=x−δR	(J.9.1.1.1.1)

- After the projection, holes are inpainted with a smaller existing sample of left or right edge of the hole.

- Then, decoded view_1 (global view at center view), which is base view, is projected to left and right target views by the left and right target depth maps. Before the projection, Chroma components are up-sampled to the same size as Luma component. View projection of sub-pixel accuracy is done by averaging two contiguous pixels weighted by the mantissa of disparity value calculated from the depth value. The corresponding pixel address (x, y) in the base view is calculated from the equation (J.9.1) by solving it for x.

- During the projection, occlusion holes are detected by checking the target depth map by the following equation. Here, k=L,C,R denotes left target view, base view (center view) and right target view. Bk denotes the baseline length between the target view and base view.

- if (Dk(x+δ,y)−Dk(x,y))≥th, (x,y) is in occlusion hole.th=255fBk(1Znear−1Zfar)δ, (0≤δ≤fBkZnear−fBkZfar)	(J.9.1.1.1.2)

Figure J.9.1.1.1.2 illustrates a case for left target view (k=L).

Figure J.9.1.1.1.2 Occlusion hole detection for left target view

- If a neighbour depth value DL(x+δ, y) is larger than the currently checked depth value DL(x, y) by th, the current pixel (x, y) is in an occlusion hole, where th is the depth difference value of the disparity difference δ.

- Then, in order to refine the occlusion area, a disparity differenceεis re-calculated from the depth difference DL(x+δ, y)- DL(x, y) as follows.

- ε=fBL255(1Znear−1Zfar)DL(x+δ,y)−DL(x,y)	(J.9.1.1.1.3)

- If DL(x+ε, y)-DL(x, y) ≧k*th, the pixel (x, y) is in an occlusion hole. Where, k=0.6 is a margin for a gap between depth map and texture.

- Since an occlusion hole exists in the left side of foreground objects when the base view resides in the right-hand side of the depth map view, only right neighbour depth values are checked. If the base view resides in the left-hand side, the left-side neighbour depth values are checked by changing the sign of δ. The search area of δ is 0 to the maximum disparity difference given by fBL(1/Znear - 1/Zfar). The detected hole areas are registered to left and right base view hole masks.

On the other hand, decoded view_2 (residual view) is separated to decoded left and right residuals. Then, they are up-sampled to up-sampled left and right residuals of the original size with simple two-tap (0.5, 0.5) filter vertically and horizontally. The up-sampled left and right residuals are projected to the target views by left and right target depth maps as same as the base view projection. During the residual projection, occlusion holes are also detected and registered to left and right residual hole masks as well.

- Then, the left and right base view hole masks are dilated by two pixels in order to ease the mismatch between the depth map and the texture. Projected residuals are over-written in the hole area of left and right target views according to the left and right base view hole masks. Then, common hole masks are created by ANDing the left and right base view hole masks and the left and right residual hole masks. Common holes in the left and right target views are inpainted according to these common hole masks. Then, horizontal two pixels on the both sides of base hole mask edge are filtered by three-tap filter(0.25, 0.5, 0.25) to reduce the hole edge contour if their difference is smaller than threshold (=20). If the difference is equal or larger than threshold, it is left as the object edge. Finally, the target views are output as synthesized target views.

          - View synthesis process for 2 view case

- To be specified.

        - View synthesis process for global depth with residual depth

To be specified.

    - View synthesis process for other alternative 3D format

- To be specified.

  - Encoding process for alternative 3D format

    - Encoding process for global view and depth

- The encoding process for global depth without residual depth is described in JCT2-A00072 | ISO/IEC JTC1/SC29/WG11 M26104 and ISO/IEC JTC1/SC29/WG11 M22573.

- The encoding process for global depth with residual depth is described in JCT3V-B0075| ISO/IEC JTC1/SC29/WG11 M26809.

  - Informative references

- JCT2-A00072 | ISO/IEC JTC1/SC29/WG11 M26104, Proposal on Simple Test Model for 3DV Coding based on CE7.

- ISO/IEC JTC1/SC29/WG11 M22573, Technical description of 3DV proposal P-27

- JCT3V-B0075| ISO/IEC JTC1/SC29/WG11 M26809

# Patent rights declaration(s)

NICT (National Institute of Information and Communications Technology), NISRI (Nagoya Industrial Science Research Institute) and DRZ (Disney Research Zurich) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).