| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B1003 |
| --- | --- |

| Title: | 3D-AVC Test Model 4 |  |  |
| --- | --- | --- | --- |
| Status: | Draft Output Document Approved by the JCT-3V |  |  |
| Purpose: | Test model description |  |  |
| Editors: | Miska M. Hannuksela Ying Chen Teruhiko Suzuki | Email: | miska.hannuksela@nokia.com cheny@qualcomm.com teruhikos@jp.sony.com |
| Source: | JCT-3V |  |  |

_____________________________

INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 MPEG2012/Nxxxxx

October 2012

| Source | JCT-3V |
| --- | --- |
| Status | Draft |
| Title | 3D-AVC Test Model 4 |
| Editors | Miska M. Hannuksela, Ying Chen, Teruhiko Suzuki |

_____________________________

Editing status

Summary of adoptions for 3D-AVC (from JCT3V-B_Notes_d7) and their status in this document:

| Contribution | Status in TM doc |
| --- | --- |
| JCT3V-B0031: CE2.A results on inter-view coding with adaptive luminance compensation [M. Mishurovskiy, A. Fartukov, I. Kovliga, J. Lee (Samsung)] | TBD |
| JCT3V-B0054: 3D-CE1.a: Generalized view synthesis prediction (GVSP) mode (Qualcomm), (partial adoption of sub-block signalling) (TM only) | Need not be done yet? |
| JCT3V-B0057: 3D-CE3.a related: On inside-view motion prediction for 3D-AVC (Qualcomm), (removal of dependency in IVMP) | TBD |
| JCT3V-B0081: 3D-CE5.a: Unification of the depth to DV conversion [Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei (Mediatek)] (partial search, 4x4 block size, not in CTC) | TBD |
| JCT3V-B0149: CE5.a results on inter-view motion vector derivation using max disparity in skip and direct modes [J. Y. Lee, J. Lee, D.-S. Park (Samsung)] (partial search) | TBD |
| JCT3V-B0079: 3D-CE5.a related: Draft text for the adopted simplified disparity vector derivation proposed in JCT3V-A0046 [Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, S. Lei (Mediatek)] | TBD |
| JCT3V-B0150: 3DV-ATM: Simplified Calculations of Disparity [Lulu Chen (USTC), Dmytro Rusanovskyy, Miska M. Hannuksela (Nokia)] | TBD |
| JCT3V-B0151: 3D-CE5.a related results on median-based skip and direct motion vector prediction [J. Y. Lee, J. Lee, D.-S. Park (Samsung)] | TBD |
| JCT3V-B0153: 3D-CE5.a related results on temporal motion vector prediction in dependent view [J. Y. Lee, K.-J. Oh, J. Lee, D.-S. Park (Samsung)] | TBD |
| JCT3V-B0033: AHG7 3D-AVC: Loss Detection of Depth Parameter Sets [M. M. Hannuksela (Nokia)] (SEI message) | Included |
| JCT3V-B0224: 3DV-ATM: Working draft text for B-VSP [D. Rusanovskyy, M.M. Hannuksela (Nokia)] | Included |
| JCT3V-B0094 4x4 motion compensation for VSP (non CTC) | TBD |

Summary of adoptions from earlier meetings that have not been included in this document:

| Contribution | Status in TM doc |
| --- | --- |
| JCT2-A0045/m25916 3D-CE5.a results on motion vector competition-based Skip/Direct mode with explicit signaling (MediaTek) | TBD |
| JCT2-A0008 3D-AVC HLS: Depth Range Parameter Set for AVC Compatible 3D Video Coding (ZTE) | TBD |
| JCT2-A0046/m25917 3D-CE5.a related: Simplification on the disparity vector derivation for AVC-based 3D video coding (MediaTek) | TBD |

Foreword

This document describes the test model for AVC-based 3D video coding (3D-AVC). The document serves as a source of general tutorial information on the coding tools in “MVC Extension for Inclusion of Depth Maps” (JCT3V-B1001) and 3D-AVC (JCT3V-B1002). The document also provides an encoder-side description of 3D-AVC Test Model (3DV-ATM) software as well as a description of decoder-side post-processing tools.

3DV-ATM includes two sets of coding configurations:

- “3D High” corresponding to “MVC extension for inclusion of depth maps”.

This configuration is characterized in MPEG N12557 as follows. The main target of this work item is to enable 3D enhancements while maintaining MVC stereo compatibility.  The method is invoking an independent second stream for the representation of a depth map as if it were monochrome video data, as well as high-level syntax signaling of the necessary information to express the interpretation of the depth data and its association with the video data. Macroblock-level changes to the AVC or MVC syntax, semantics and decoding processes are not considered.

- “3D Extended High” corresponding to the standardization track called “AVC compatible video-plus-depth extension” as outlined in MPEG N12557, also known as 3D-AVC.

The 3D Extended High configuration is suitable for applications that only require monoscopic AVC compatibility. A significant compression gain is achieved, as the syntax and decoding process for non-base texture views and depth information changes at the block level, compared to AVC/MVC.

The 3D Extended High configuration is a superset of the 3D High configuration. That is, a decoder supporting 3D Extended High configuration can also decode bitstreams generated for the 3D High configuration.

Table of Contents

1	Introduction	5

1.1	General Architecture of 3DV-ATM	5

1.2	Elementary coding units and coding order	6

1.3	Coding tools	7

2	Baseline depth coding	8

2.1	Non-linear depth representation (NDR)	8

3	Common tools for enhanced depth and texture coding	9

3.1	Slice header prediction	9

4	Enhanced depth coding	10

4.1	Introduction	10

4.2	Depth Range Based Weighted Prediction (DRWP)	10

4.3	In-loop Joint inter-View Depth Filtering (JVDF)	13

4.4	Motion prediction from texture to depth	14

4.5	Depth Intra Prediction	15

4.5.1	Depth Intra Skip Prediction	15

4.5.2	Plane Segmentation based Intra Prediction (PSIP)	16

5	Enhanced texture coding	18

5.1	Introduction	18

5.2	In-loop view synthesis-based inter-view prediction (VSP)	18

5.3	Depth-based Motion Vector Prediction (D-MVP)	20

6	Supplemental enhancement information	21

6.1	Gradual View Refresh (GVR)	21

6.2	Loss detection of depth parameter sets	22

7	Encoding algorithms	22

7.1	Rate-distortion optimization through view synthesis distortion	22

8	Post processing	24

8.1	Depth dilation filter	24

Annex A: List of Contributors	25

- Introduction

  - General Architecture of 3DV-ATM

3DV-ATM codec is configurable for two coding modes, which are called “3D High” and “3D Extended High” configurations.

The “3D High” configuration provides H.264/MVC compatibility for texture views and implies that texture and depth map data is coded independently into a single coded bitstream. The difference of this configuration from H.264/MVC specification is limited to high level syntax and respective semantics as well as high-level decoder operation .

The “3D Extended High” configuration provides H.264/AVC compatible solution for a texture view and implies joint coding of texture and depth map data into a single bitstream. In this configuration, the texture data of base views (H.264/AVC compatible) is utilized for coding of the associated depth map data, and depth map data of enhancement views is utilized for coding of associated texture views.

High level flow charts of 3DV-ATM encoder and decoder are presented in Figures 1 and 2 respectively. On the figures below, solid lines depict general data flow and dashed lines show control information signaling.

Figure 1. High-level flow chart of 3DV-ATM encoder

Figure 2. High-level flow chart of 3DV-ATM decoder

  - Elementary coding units and coding order

3DV-ATM specifies that all texture and depth map view components which describe 3D scene at a particular moment of time form an access unit. A coded view component is represented by one or more Network Abstraction Layer (NAL) units similarly to AVC/MVC. The data of a coded view component is not interleaved by any other coded view component, and the data for an access unit is not interleaved by any other access unit in the bitstream/decoding order. See Figure 3 for an example, where access unit t consisting of texture and depth view components (T0t,T1t ,D0t,D1t) precede in bitstream and decoding order access unit  t+1 consisting of texture and depth view components (T0t+1,T1t+1 ,D0t+1,D1t+1).

Figure 3. Definition and coding order of access units

Each texture view component for the AVC/MVC-compatible views is coded before the respective depth view component. Each texture view component of enhanced texture views is coded after the respective depth view component. The texture and depth view components of the same access units are coded in view dependency order. Texture and depth view components can be ordered in any order with respect to each other as long as the ordering obeys the mentioned constraints. Examples of coding order for an access unit include but are not limited to the following:

- T0,T1,D0, D1,… (two AVC/MVC compatible texture views)

- T0, D0, T1, D1…(two AVC/MVC compatible texture views)

- T0, D0,D1, T1… (one AVC compatible texture view, one enhanced texture view)

The respective order of texture and depth view components of the same view_id is indicated in the sequence parameter set. This indicated order controls the presence of certain syntax elements, such as flags for turning on/off texture-based coding tools for depth (e.g. IVMP).

  - Coding tools

The 3D High and 3D Extended High configurations include the following baseline depth coding tools (for further information, see Section 2):

- Non-linear depth representation

- Reduced-resolution depth coding

The 3D Extended High configuration additionally includes the following coding tools which can be used for both enhanced depth views and enhanced texture views (for further information, see Section 3):

- Slice header prediction

The 3D Extended High configuration additionally includes the following enhanced depth coding tools (for further information, see Section 4):

- Depth-range-based weighted prediction (DRWP) for depth coding

- In-loop Joint inter-View Depth Filtering (JVDF)

- Motion prediction from texture to depth (a.k.a. Inside View Motion Prediction, IVMP)

- Depth intra prediction: intra skip and plane segmentation based intra prediction

The 3D Extended High configuration additionally includes the following enhanced texture coding tools (for further information, see Section 5):

- In-loop view synthesis prediction (VSP)

- Depth-based motion vector prediction (D-MVP)

Additionally, the following non-normative tools are supported by the 3DV-ATM software and may be used with both 3D High and 3DV Extended High configurations (for further information, see Sections 6, 7 and 8):

- Gradual view refresh (GVR) for texture and depth coding

- Rate-distortion optimization through view synthesis distortion

- Depth dilation post-filter

- Baseline depth coding

  - Non-linear depth representation (NDR)

The idea of NDR tool is to represent closer objects more accurately than the distant ones. This is attained by encoding of non-linearly mapped disparity, normalized in range 0..255, instead of generic depth map values.

If NDR is turned on, prior to the encoding the depth is nonlinearly mapped with respect to forward lookup table.

Afterwards, before the outputting from the decoder and prior to feeding to the renderer, the decoded depth is transformed, with inverse lookup table, back to linear disparity in range 0..255.

The exact shape of forward and inverse transforms is defined by means of line-segment-approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first (0, 0) and the last (255, 255) nodes of the curve are predefined (Figure 4). Positions of additional nodes are defined in form of deviations (NonlinearDepthPoints[i]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to 255 diagonal, with spacing depending on the number of points (NonlinearDepthNum).

Figure 4. Piecewise linear segments for mapping of depth values in Non-linear Depth Representation tool. Arrows mark deviations from uniformly distributed points along straight mapping, which are signalled in the bitstream.

In “3D High” (profile_idc=138) configuration this tool is non-normative and is signalled through “depth_representation” SEI message.

In “3D Extended High” (profile_idc=139) configuration usage of NDR impacts the behaviour of other tools that interpret the depth values. Therefore this tool is normative and is signalled inside SPS of the depth component. The coded depth is internally transformed back with inverse lookup table inside the loop of codec. For the sake of optimization, in some cases the mapping has been incorporated into existing LUT tables (view synthesis etc).

- Common tools for enhanced depth and texture coding

  - Slice header prediction

This tool can be applied only for enhanced texture and depth view components.

There may be redundancy between the slice headers of depth view components and texture view components. This coding tool is used to avoid repetitive coding of the same syntax element values in the view components of the same access unit.

The slice header prediction mechanism enables copying values for the syntax elements in the following categories:

- Reference picture list related syntax elements

- Prediction weight table related syntax elements

- Decoded reference picture marking related syntax elements

- All other syntax elements of the slice header except first_mb_in_slice, slice_type, pic_parameter_set_id and slice_qp_delta

The syntax elements in each mentioned category can be selectively copied from an earlier slice header of the same access unit or included in the current slice header. The source of the syntax element values can be selected separately for each category from the following:

- The previous view component of the same type (texture or depth)

- The view component of an opposite type having the same view_id

- The first view component of the same type (texture or depth)

- Enhanced depth coding

  - Introduction

Similarly, Figure 5 shows processing flow for depth map coding for the non-AVC/MVC compatible depth views in 3DV-ATM. The processing modules which are marked in red indicate the novel coding tools of 3DV-ATM. In the Motion Compensated Prediction (MCP) block, the Depth Range Based Weighted Prediction (DRWP) coding tool is included.

Figure 5. Flow chart of depth map coding in 3DV-ATM

  - Depth Range Based Weighted Prediction (DRWP)

DRWP performs a non-linear compensation of the depth map inconsistency which is caused by use of different Znear/Zfar values in the conversion from the depth values z to the respective depth map samples v. This compensation process is described as follows.

For each depth map value v1 (was produced with Z1near/Z1far) we compute corresponding z depth value:

(1)

Following this, we compute depth map value v2 representation for current depth value z and with new depth range (Z2near/Z2far):

(2)

To enable DRWP in 3DV-ATM, the compensation process was implemented as a form of weighted prediction as follows. Let us choose two sampling points of the remapping function whose difference is a power of two in the input sample values. The input sample values of these sampling points are denoted a and a + 2d, where d is a positive integer. The output sample values y1 and y2, respectively, for these sampling points should be as close as possible to integer values to avoid quantization error.

The output sample value y then becomes a linear function of the input sample value x:

(3)

where round is a function returning the closest integer.

This function can be counted with integer arithmetic as follows:

(4)

where >> denotes a right bit-shift operation.

When offset o is defined as

(5)

and weight w is defined as

(6)

this function becomes identical to explicit weighted prediction:

(7)

If values y1 and y2 are indicated as fixed-point values Qf, where f represents the number of fractional bits, the quantization error in the depth map quantization rescaling may be reduced. The function then becomes

(8)

The encoder and the decoder derive the values of a, d, y1, and y2 from the camera and view synthesis parameters included in the bitstream.

In 3DV-ATM, the use of DRWP is automatically enabled at the encoder and decoder sides when there is a difference in depth map range appearing in two consequent frames.

In 3DV-ATM, linear function of weighted prediction is expressed through a pair of values gradient (w) and offset (o), which computed as:

,       		(9)

The computed values of w and o are used as weights in AVC/MVC weighted prediction process. The encoder signals usage of the DRWP in the slice header.

In order to avoid potential rounding errors resulting from floating point arithmetic, the values of wX and oX (where X is 0 or 1, indicating reference picture list 0 or 1, respectively) are derived with the following fixed-point arithmetic version of the calculations above.

  - Initialize intermediate variables with default values

scale_w=8;

  - Calculate variable W_factora:

temp1 = ZF2 - ZN2;

temp2 = ZF2;

x = ( temp1 + ( temp2 >> 1 ) ) / temp2;

sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1;

W_factora = (x<<scale_w);

W_factora += ( ((temp1 - x * temp2) << scale_w) + sign * (temp2 >> 1) ) / temp2;

  - Calculate variable W_factorb:

temp1 = ZF1;

temp2 = ZF1 – ZN1;

x = ( temp1 + ( temp2 >> 1 ) ) / temp2;

sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1;

W_factorb = (x<<scale_w);

W_factorb += ( ((temp1 - x * temp2) << scale_w) + sign * (temp2 >> 1) ) / temp2;

  - Calculate variable W_factorc:

temp1 = ZN1;

temp2 = ZN2;

x = ( temp1 + ( temp2 >> 1 ) ) / temp2;

sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1;

W_factorc = (x<<scale_w);

W_factorc += ( ((temp1 - x * temp2) << scale_w) + sign * (temp2 >> 1) ) / temp2;

  - Calculate variable wX:

wX= (W_factora*W_factorb*W_factorc + (1<<(scale_w*3- logWD-1)) ) >> (scale_w*3- logWD);

wX=Clip(-127,128,wX);

Variable oX is calculated through the following derivation process.

- Initialize intermediate variable with default value:

scale_o=8;

- Calculate variable O_factora:

O_factora=( (ZN1<<(scale_o))+( ZF2>>1) ) / ZF2;

- Calculate variable O_factorb:

temp1 = ZF1- ZF2;

temp2 = ZF1- ZN1;

sign= (temp1 < 0) ? -1 : 1;

x = ( temp1 + sign * ( temp2 >> 1 ) ) / temp2;

sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1;

O_factorb = (x<<scale_o);

O_factorb += ( ((temp1 - x * temp2) << scale_o) + sign * (temp2 >> 1) ) / temp2;

- Calculate variable o0:

oX=( O_factora*O_factorb + (1<<(scale_o*2-8-1)) ) >> (scale_o*2-8);

oX =Clip(-127,128, oX);

  - In-loop Joint inter-View Depth Filtering (JVDF)

Depth map images of available views are filtered jointly. The depth map of the currently processed view Vc is converted into the depth space (Z-space) as it shown in (1). Following this, depth map images of other available views (Va1, Va2) are converted to the depth space and projected to  the currently processed view Vc. Projections create several estimates of the real depth value, which are averaged in order to produce a denoised estimate of the real depth value. Filtered depth value  of current view Vc is produced through a weighted average with depth estimate values  projected from an available views Va to a currently processed view Vc.

(10)

where {w1, w2} are weighting factors or filter coefficients for the depth values of different views or view projections.

Filtering is applied if depth value estimates belong to a certain confidence interval, in other words, if the absolute difference between estimates is below a particular threshold (Th):

If |za→c – zc |<Th	, w1=w2=0.5				(11)

Otherwise, w1 = 1, w2 = 0

Figure 6 shows the coding of two depth map views with in-loop implementation of JVDF. The AVC coding algorithm is depicted within a dashed line box, marked in black color. The JVDF is depicted in the solid-line box marked in red color. The parameter Th is transmitted to the decoder within a sequence parameter set extension.

Figure 6. In-loop JVDF of 2-view depth map coding.

  - Motion prediction from texture to depth

This tool can be applied only for depth view components for AVC/MVC compatible texture views.

Since the texture view component and its associated depth view component have similar object silhouette, they may have similar object movement, thus there is redundancy in their motion fields. Therefore, motion prediction from a texture view component to the associated depth view component is enabled as a new mode in the proposed codec. An Inside View Motion Prediction (IVMP) mode is enabled for an Inter coded MB only in depth view components. In IVMP mode, the motion information, including mb_type, sub_mb_type, reference indices and motion vectors of the co-located MB in texture view component is reused by the depth view component of the same view. A flag is signaled in each MB to indicate whether it uses the IVMP mode. As shown in Figure 7, the flag is true for the highlighted MB in the 4-th picture of the depth view and the motion vector of the co-located MB in the 4-th picture of the texture view (in red) is reused for the highlighted MB in the depth view component. Note that, in current implementation, IVMP mode applies only to non-anchor pictures.

Figure 7. Motion prediction from texture to depth.

  - Depth Intra Prediction

Following two depth intra prediction tools are applied for depth view components.

    - Depth Intra Skip Prediction

Since the depth images have high spatial correlation, efficient high priority intra prediction can be useful for depth image coding. The depth intra skip prediction (DISP) is designed based on conventional intra 16x16 prediction. However, it uses the estimated prediction direction both at the encoder and decoder sides as well as does not encode any residual data. For the estimation of prediction direction, adjacent above and left lines are used. The number of changes (NOC) value of left line (NOC(L)) is calculated by

(12)

NOC(A) is calculated in the same way. From the calculated NOC(L) and NOC(A), prediction direction is derived as follows.

Figure 8. Flow diagram of estimation of prediction direction.

Rule 1 and rule 2 in Figure 8 are defined as follows:

<Rule 1>

if (left MB = ver I16x16 mode & above MB = ver I16x16 mode)

: vertical direction

else if(left MB = hor I16x16 mode & above MB = hor I16x16 mode)

: horizontal direction

else if(left[0] < up[0])

: vertical direction

else if(left[0] > up[0])

: horizontal direction

else

: DC direction

<Rule 2>

if (left MB = ver I16x16 mode & above MB = ver I16x16 mode)

: vertical direction

else if(left MB = hor I16x16 mode & above MB = hor I16x16 mode)

: horizontal direction

else if (left MB ≠ hor I16x16 mode & above MB = ver I16x16 mode)

: vertical direction

else if (left MB = hor I16x16 mode & above MB ≠ ver I16x16 mode)

: horizontal direction

else if (NOC(L) < NOC(A))

: vertical direction

else if (NOC(L) > NOC(A))

: horizontal direction

else if( abs(above[0] – above[15]) > abs(left[0] – left[15])

: vertical direction

else if( abs(left[0] – left[15]) > abs(above[0] – above[15])

: horizontal direction

else

: DC direction

DISP is only applied for depth views and a flag to indicate the DISP mode is signaled in each macroblock. In addition, boundary strength (Bs) of deblocking filter is adjusted when current macroblock is coded as DISP mode.

    - Plane Segmentation based Intra Prediction (PSIP)

Since it is important to keep the quality of sharp edge in depth map, a new intra prediction scheme is designed to keep sharp edge boundary by assigning more bits for edge. The idea of PSIP is to segment each depth block into two regions, and apply different prediction for each segmented region.

For each N×N depth map block, as shown in Figure 9, it first segments its upper and left available pixels into two groups, and determine two representative values, P1, P2 by averaging each group of pixels. Then, each target pixels, Ci, are predicted by its closest representative value, i.e.,

(13)

where  is predicted pixel value.

Figure 9. Example of k-region segmentation (k=2)

Figure 10. Example of prediction map for Figure 9.

Figure 11. Example of pattern table for 4×4 block.

During PSIP prediction, it is additionally required to encode the edge information, say, prediction map, which is NxN binary information as shown in Figure 10. In PSIP, each 1xN row (or 1xN column) is coded according to the pre-determined pattern table as shown in Figure 11, where  patterns with more than two bit-transitions are excluded. Finally, the difference between adjacent rows (or columns) is encoded, since difference value will be closed to 0. The difference is computed as

(14)

where MOD indicates modulo operator, and si represents ith row (or column) pattern code (s0 means a pattern code for neighbourhood pixels).

- Enhanced texture coding

  - Introduction

Figure 12 shows enhanced texture coding in 3DV-ATM. The processing modules which are marked in red indicate the novel coding tools of 3DV-ATM when compared to AVC/MVC.

Figure 12. Flow chart of enhanced texture coding in 3DV-ATM.

To enable View Synthesis Prediction (VSP) and Depth-based Motion Vector Prediction (D-MVP), 3DV-ATM specifies in-loop resolution normalization for depth map data to the resolution of the texture data. The encoder specifies and signals in the bitstream the spatial resolution of output pictures to which both texture and depth images are resampled in the decoder. Rescaling of depth map data up to the spatial resolution of texture is performed with bi-linear resampling.

  - In-loop view synthesis-based inter-view prediction (VSP)

In-loop View Synthesis Prediction (VSP) is supported in 3DV-ATM for enhanced texture coding. VSP utilizes backward warping, as illustrated below in Figure 13. In the example the following coding order is utilized: (T0, D0, D1, T1). Texture component T0 is a base view and T1 is dependent view coded with the VSP. Depth map components D0 and D1 are the depth maps associated with T0 and T1, respectively.

In the dependent view T1, sample values of the currently coded block Cb are predicted from the reference area R(Cb) that consists of sample values of the base view T0. The displacement vector (motion vector) between coded and reference samples is found as a disparity between T1 and T0 from a depth map value associated with a currently coded texture sample. In such implementation, B-VSP is comparable with coventional Motion Compensated Prediction (MCP) where displacement (motion) vectors are replaced with disparity vectors.

Figure 13. Visualization of view synthesis prediction based on backward warping.

The process of conversion may be performed for example with following equations:

ZCbj,i=1dCbj,i255⋅1Znear−1Zfar+1Zfar;DCbj,i=f⋅bZCbj,i; 	(15)

where j and i are local spatial coordinates within Cb, d(Cb(j,i))  is a depth map value in depth map image of a view #1, Z is its actual depth value, and D  is a disparity to a particular view #0. The parameters f, b, Znear and Zfar are parameters specifying the camera setup; i.e. the used focal length (f), camera separation (b) between view #1 and view #0i  and depth range (Znear,Zfar) representing parameters of depth map conversion.

In the 3DV-ATM software equation (1) is precomputed for every depth map value (0…255) and stored as a look up-table.

To avoid in-loop depth map resampling, the depth map of the dependent view D1 is processed at the its coded resolution. Therefore, if depth maps have a quarter resolution compared to luma texture, a single disparity value is calculated for a 2x2 block of luma texture samples of T1. The 3DV-ATM software can also be configured to perform VSP with 4x4 blocks.

A synthesized picture resulting from VSP is included in the initial reference picture lists List0 and List1 following temporal and inter-view reference frames. However, reference picture list modification syntax (i.e., RPLR commands) are extended to support VSP reference pictures, thus any ordering of reference picture lists is allowed.

VSP skip/direct provides another way to utilize VSP frames as references when there exist VSP frames in the reference picture list. In order to employ the VSP skip/direct, two additional flags are additionally defined. First, a VSP skip flag is signaled to distinguish the VSP skip mode from the conventional skip one. The VSP skip flag can be placed ahead of or behind the conventional skip flag based on mode information of the neighboring blocks around the current block Second, if an MB is signaled as the conventional or VSPdirect mode, a direct type flag is further signaled whether a VSP frame is forced to be used as reference.

  - Depth-based Motion Vector Prediction (D-MVP)

Depth-based motion vector prediction (D-MVP) is a coding tool which takes in use available depth map data and utilizes it for coding/decoding of the associated with depth map texture data.

This coding tool is enabled for enhanced texture coding and requires depth map data to be coded prior to the texture data.

The D-MVP tool consists of two parts, direction-separated MVP and disparity-based Skip and Direct modes, which are described next.

A.	Direction-separated MVP

Conventional median MVP of H.264/AVC is restricted to identical prediction directions of motion vector candidates. All available neighboring blocks are classified according to the direction of their prediction (temporal or inter-view). If the current block Cb, see Figure 14, uses an inter-view reference picture, all neighboring blocks which do not utilize inter-view prediction are marked as not-available for MVP. Motion vectors of the neighboring blocks marked as not-available are replaced with the max disparity associated with Cb instead of a zero motion vector and then considered in the median MVP of H.264/AVC. The max disparity is converted from the largest pixel value among pixels within the current blocks. Vice versa, if Cb uses temporal prediction, neighboring blocks that used inter-view reference frames are marked as not-available for MVP. The flowchart of this process is depicted in Figure 14.

Figure 14. Flow chart of direction-separated MVP.

B.	Disparity-based Skip and Direct modes

[Text to be provided]

- Supplemental enhancement information

  - Gradual View Refresh (GVR)

When decoding is started from a GVR point, a subset of the views can be accurately decoded, while the remaining views can be approximately reconstructed using view synthesis. Perfect reconstruction of all views can be reached at a subsequent random access point.

While no inter prediction is applied in IDR and anchor access units, GVR access units are coded in a manner that inter prediction is selectively enabled and hence compression improvement compared to IDR and anchor access units may be reached. The encoder selects which views are refreshed in a GVR access units and codes these view components in the GVR access unit without inter prediction, while the remaining non-refreshed views may use both inter and inter-view prediction. The selection of refreshed views is done in a manner that each view becomes refreshed within a reasonable period, which may depend on the targeted application but is typically up to few seconds at most. To allow decoders to detect the possibility of starting decoding, GVR access units may be indicated in the NAL unit header or slice header similarly to anchor access units.

Figure 15 presents a configuration of using GVR access units at every other random access point. It is assumed in Figure 15 that the frame rate is 30 Hz and random access points are coded every half a second. GVR access units refresh the base view only, while the non-base views are refreshed once per second with anchor access units.

a)							b)

Figure 15. Examples of GVR access units (picture order count 15 and 45) coded at every other random access point; a) Two-view coding, b) Three-view coding with PIP inter-view prediction hierarchy.

When decoding is started from a GVR access unit, the texture and depth view components which do not use inter prediction are decoded, and the remaining “non-refreshed” views are approximately reconstructed using view synthesis (a.k.a. depth-image-based rendering, DIBR). Decoding of the non-refreshed views can be started at subsequent IDR, anchor, or GVR access units. Figure 16 presents an example of the decoder side operation when decoding is started at a GVR access unit of the bitstream presented in Figure 15a.

Figure 16. Decoder operation when starting decoding from GVR access unit at picture order count 15.

The GVR functionality can be realized by including a recovery point SEI message into a 3DVC scalable nesting SEI message, in which the views that are refereshed are indicated.

  - Loss detection of depth parameter sets

Depth parameter sets convey information for the closest and farthest depth as well as parameters related to view synthesis prediction. While parameter sets enable loss-resilient transmission of parameters, for example through retransmission possibility, it is not possible to detect a loss of a depth parameter set without external means (e.g. loss detection provided by the systems layer) or using the constrained depth parameter set identifier SEI message. If the depth_parameter_set_id value of a lost depth parameter set NAL unit has been used previously by another depth parameter set NAL unit, the decoder would continue decoding using the previously received depth parameter set NAL unit. After that any reference to the parameter set ID value used in the lost parameter set NAL unit points to the previous parameter set NAL unit using the same ID value. Therefore wrong syntax element values would be used in the decoding process, which would be likely to result into clearly visible artifacts.

When present, the constrained depth parameter set identifier SEI message is associated with an IDR access unit and its semantics are valid for a coded video sequence. In the constrained depth parameter identifier SEI message the encoder specifies the value range of depth_parameter_set_id values with the max_dps_id syntax element. In other words, the value of depth_parameter_set_id included in the depth parameter set RBSP shall be in the range of 1 to max_dps_id, inclusive. Value 0 of depth_parameter_set_id is inferred to refer to the depth parameters provided in the active sequence parameter set.

The encoder also specifies a range of depth_parameter_set_id values that are considered “used” and indicates that range to the decoder in max_dps_id_diff. The range is relative to the greatest depth_parameter_set_id (in modulo sliding-window arithmetic) among activated depth parameter set NAL units and hence specifies a kind of a sliding window of valid depth_parameter_set_id values. Depth parameter set NAL units that have an depth_parameter_set_id value outside the sliding-window range are considered “unused” and a new depth parameter set NAL unit with the same depth_parameter_set_id value may be transmitted. Depth parameter set NAL units having an depth_parameter_set_id value in the “used” range must repeat the content of the previous depth parameter set NAL unit of the same depth_parameter_set_id value. It is recommended that encoders increment the depth_parameter_set_id value of activated depth parameter sets by 1. As depth_parameter_set_id values may wrap over, modulo arithmetic is used in determining the depth_parameter_set_id values within the sliding-window range. The depth_parameter_set_id constraint is “reset” at IDR access units.

- Encoding algorithms

As the 3DV-ATM software is built on top of the JM reference software of the H.264/AVC, the encoding algorithms utilized in 3DV-ATM are identical to those implemented in the JM. Encoding side algorithms which are solely serving 3DV coding are described below.

  - Rate-distortion optimization through view synthesis distortion

The coding distortion in depth map does not linearly affect the synthesis distortion, and the impact of depth map distortions varies according to the corresponding texture information. For example, the same depth distortions on textured and textureless regions lead to different synthesis distortions. For efficient rate-distortion optimization of depth-enhanced 3D video coding,  a new distortion function measuring the synthesized view distortion by analyzing view warping / rendering process is required.

In a conventional video coding system, one commonly used distortion function is the sum of squared differences (SSD), which is defined between original and encoded depth block as

(16)

where D and  indicate the original and reconstructed depth map, respectively, and (x, y) means the pixel position in a (macro-) block B. However, the conventional SSD metric does not reflect the synthesized view distortion. Instead, the following new view synthesis distortion (VSD) metric is used in 3DV-ATM:

(17)

where indicates the reconstructed texture, and α is proportional coefficient determined by the following equation

(18)

where f  is the focal length, L is the baseline between the current and the rendered view, Znear and Zfar are the values of the nearest and farthest depth of the scene, respectively.

If the reconstructed texture is not available e.g. depth map is coded prior to texture view as in the enhanced texture coding of 3DV-ATM, then the original texture image is used instead of the reconstructed one. Since the original texture usually contains more details, the VSD function is modified as

(19)

where ρ is the ratio between original and decoded texture in base view as

(20)

where CI represents the texture image in base-view (I-view).

When depth map is half resolution of texure, it is required to adjust the resolution to apply the above-mentioned VSD function. For more accurate distortion measure, each depth block candidate is up-sampled, by which VSD function in (20) and (22) can be used without any modification

If the information of the synthesized view position is available at the encoder, VSD can be more accurately calculated by considering the rendering distortion for each view position, and averaging them as

(21)

where wi indicates the weight factor describing how much the current view contributes the target rendered view, e.g., closer target view takes larger weight factor. Then, the objective cost function with VSD will be

(22)

where lambda (λV) is adjustable value, since VSD takes different distortion domain. Finally, objective cost function for R-D mode decision will be obtained by the following equation

(23)

where β is provided as encoder configuration. In this way, the depth RDO is enhanced by considering the synthesis distortions. As can be seen from the presented equations, the new RDO method takes into account that the depth distortions in high-texture areas are generally more harmful than those in low-texture areas.

- Post processing

  - Depth dilation filter

After decoding process of depth data, depth is upsampled when the size of depth is smaller than texture resolution and then grey scale dilation filter is applied to the upsampled result (Figure 17). If resolution of decoded depth is same with texture resolution, dilation filter is applied without upsampling. Grey scale dilation can be done by taking maximum value of neighbouring pixels. Most synthesis artifacts are occurred in edge region where foreground and background are met.  By applying dilation filter, foreground region of depth is extended to background regions, and synthesis artifact occurs out of boundary edge. Thus objective and subjective quality of view synthesis result is improved.

Figure 17. Depth dilation filtering in post process.

- Annex A: List of Contributors

The following people, given in alphabetical order, have contributed to this document:

Ying Chen (Qualcomm)

Deliang Fu (Zhejiang University)

Miska M. Hannuksela (Nokia)

Jaejoon Lee (Samsung)

Jin Young Lee (Samsung)

Seok Lee (Samsung)

Byung Tae Oh (Samsung)

Dmytro Rusanovskyy (Nokia)

Olgierd Stankiewicz (Poznan University of Technology)

Wenyi Su (University of Science and Technology of China)

Dong Tian (Mitsubishi)

Lu Yu (Zhejiang University)

Li Zhang (Qualcomm)

Yin Zhao (Zhejiang University)