| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN , 13–19 Oct. 2012 | Document: JCT3V-B1005_d0 |
| --- | --- |

| Title: | 3D-HEVC Test Model 2 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-3V |  |  |
| Purpose: | Draft of 3D-HEVC Test Model Description |  |  |
| Author(s) or Contact(s): | Gerhard Tech Fraunhofer HHI Krzysztof Wegner Poznan University of Technology Ying Chen Qualcomm Incorporated Sehoon Yea LG Electronics | Email: Email: Email: Email: | gerhard.tech@hhi.fraunhofer.de kwegner@multimedia.edu.pl cheny@qti.qualcomm.com sehoon.yea@lge.com |
| Source: | Editor |  |  |

_____________________________

Abstract

Draft 2 of 3D-HEVC Test Model Description

Ed. Notes (WD2) (based on JCT3V-A1005)

Accepted changes and marked delta to base spec

(3DC-GT2) Editorial improvements, small corrections

(3DC-CY) Editorial improvements, small corrections

(MVS-02/JCT3V-B0046) Treatment of inter-view pictures as long term- reference pictures

(3DE-11) Revised text related to 3Dn-01

(3Dn-01/m23639) Results on motion parameter prediction

(3DE-12) Revised text related residual prediction

(3DE-10) Revised text Related to Illumination compensation.

(3DN-01/JCT3V-B0045) Illumination compensation for inter-view prediction.

(3Dn-02/m24766) Restricted Inter-View Residual Prediction

(3DE-09) Revised text related to depth intra: Edge Intra

(3DE-09) Revised text related to depth intra: SDC

(3DE-09) Revised text related to depth intra: DMMs

(3DO-01/JCT3V-B0131) Depth distortion metric with a weighted depth fidelity term

(3DN-12/JCT3V-B0036) Simplified Depth Coding with an optional Depth LUT

(3DN-13/JCT3V-B0039) Simplified Wedgelet search for DMM modes 1 and 3

(3DN-03/JCT3V-B0083) Unconstrained motion parameter inheritance

(3DE-08) Incorporated context tables for SDC

(3DE-07) Improved MPI text.

(3DN-02/JCT3V-B0068) Incorporated Depth Quadtree Prediction.

(3DE-06) Incorporated parsing process, including tables for DMMs.

(3DE-05) Added missing initialization of invalid motion/disparity parameters

(3DC-03) Added missing pruning of collocated merge candidate due to number of total candidates.

(3DE-04) Moved pruning of spatial merge candidate B2 due to number of total candidates.

(3DE-03) Moved derivation of disparity one level higher in process hierarchy.

(3DE-02) Inserted "Derivation process for motion vector components and reference indices" from base spec

(3DC-02) Fixed storage of IvpMvFlagLX and IvpMvDisp.

(3DN-09-10-11/JCT3V-B0048,B0069,B0086) Modification inter-view merge candidates

(3DC-01) Fixed derivation of inter-view merge candidates.

(3DE-01) Revised derivation of disparity from temporal candidates

(3DN-04/JCT3V-B0047) Improvements for disparity vector derivation)

(3DN-08/JCT3V-B0136) Support of parallel merge in disparity vector derivation

(3DN-05/JCT3V-B0135) Modified disparity vector derivation process for memory reduction

(3DN-04/JCT3V-B0111) Decoupling inter-view candidate for AMVP

(3DN-07/JCT3V-B0096) Removal of dependency between multiple PUs in a CU for DV-derivation

(3DC-GT) Small corrections, editorial improvements

Ed. Notes (WD1) (based on N12744)

(3D08/JCT3V-A0126) (T,N) Simplified disparity derivation

(3D16) Moved 3D-tool related flags from SPS to VPS, removal camera parameters

(3D09/JCT3V-A0049) (N) Inter-view motion prediction modification

(3D13/JCT3V-A0119) (T) VSO depth fidelity

(3D07/JCT3V-A0070) (T,N) Region boundary chain coding for depth maps

(3D06/JCT3V-A0087) (T) RDO selection between Non-Zero Residual and All-Zero Residual Intra

(3D12) (T) Depth Quadtree Prediction

(3D15) (N) Fix references

(3D11) (T,N) Improvement of text of already adopted tools

(3D10/JCT3V-A0097) (T;N) Disparity vector generation

(3D02) (N) Removed MV-Part and update to Annex F

(3D03) (T) Labelling of tools not in CTC/Software. Removal?

(3D05/JCT3V-A0093) (T) VSO early skip

(3D04/JCT3V-A0033) (T) VSO model based estimation

(3D14) (N) Update of low level specification to match HEVC text specification 8(d7)

(3D01) (N): Removed HEVC text specification

Contents

Abstract	1

Contents	3

List of Figures	5

List of Tables	6

1.	Data Format and System Description	8

2.	Coding Algorithm	9

2.1	Coding of the Independent View	11

2.2	Coding of Dependent Views	11

2.2.1	Disparity-compensated prediction	11

2.2.1.1	Reference list construction and modification	12

2.2.2	View synthesis based inter-view prediction [Not in SW]	13

2.2.2.1	Post processing in-loop filtering [Not in SW]	13

2.2.3	Inter-view motion prediction	14

2.2.3.1	Derivation of Disparity Vectors	14

2.2.3.2	Usage of Inter-View Motion Parameter Prediction	20

2.2.3.3	Derivation of co-located motion vector candidate	21

2.2.4	Depth-based motion parameter prediction [Not in SW]	22

2.2.5	Inter-view residual prediction	22

2.2.5.1	Availability of reference residual signal	23

2.2.5.2	Availability of residual prediction	24

2.2.6	Illumination compensation (IC)	24

2.2.7	Adjustment of QP of texture based on depth data [Not in SW]	24

2.3	Coding of Depth Maps	25

2.3.1	Disabled chrominance coding [Not in SW]	25

2.3.2	Non-linear depth representation [Not in SW]	25

2.3.3	Z-near z-far compensated weighted prediction  [Not in SW]	25

2.3.4	Modified motion compensation and motion vector coding	26

2.3.5	Disabling of in-loop filtering	26

2.3.6	Depth modelling modes	26

2.3.6.1	Mode 1: Explicit Wedgelet Signalization	27

2.3.6.2	Mode 2: Intra-predicted Wedgelet Partitions	28

2.3.6.3	Mode 3: Inter-component prediction of Wedgelet partitions	28

2.3.6.4	Mode 4: Inter-component prediction of Contour partitions	29

2.3.6.5	Constant partition value coding	29

2.3.6.6	Co-located Texture Luma Block intra direction aligned search	30

2.3.6.7	Coarse wedgelet search with refinement step	30

2.3.6.8	Mode selection	31

2.3.6.9	Signalling in the bitstream	31

2.3.7	Region boundary chain coding	32

2.3.7.1	Signalling in the bitstream	32

2.3.8	Simplified depth coding	33

2.3.8.1	Depth Lookup Table	33

2.3.9	Motion parameter inheritance	34

2.3.10	Depth Quadtree Prediction	35

2.4	Encoder Control	36

2.4.1	View Synthesis Optimization	36

2.4.1.1	Synthesized View Distortion Change (SVDC)	37

2.4.1.2	Model based synthesized view distortion estimation without rendering	41

2.4.1.3	Depth fidelity term	41

2.4.1.4	Integration of distortion metrics in the Encoder Control	41

2.4.1.5	Adaptation of the Lagrange Multiplier	41

2.4.2	Zero residual coding for depth intra CUs	42

2.4.3	Optional Encoder Control using a depth quadtree limitation	42

2.4.4	Optional Encoder Control for Renderable Regions in Dependent Views [Not in CTC]	42

2.4.5	Depth edge-based r-d optimization tuning [Not in SW]	43

3.	View Synthesis Algorithms	43

3.1	Fast 1-D View Synthesis (VSRS 1D Fast Mode)	44

3.1.1	Upsampling of input video pictures	44

3.1.2	Warping, interpolation and hole filling	44

3.1.3	Reliability map creation	45

3.1.4	Similarity enhancement	45

3.1.5	Combination	45

3.1.6	Chroma decimation	46

3.2	VSRS (alternative view synthesis algorithm) [Not in CTC]	46

3.2.1	General mode	46

3.2.2	1-d mode	48

4.	Software	51

4.1	Software repository	51

4.2	Build System	51

4.3	Software Structure	51

5.	3D high efficient video coding text specification draft	52

Annex G 	3D high efficient video coding	52

G.1	Scope	52

G.2	Normative references	52

G.3	Definitions	52

G.4	Abbreviations	52

G.5	Conventions	52

G.6	Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships	52

G.7	Syntax and semantics	53

G.7.1	Method of specifying syntax in tabular form	53

G.7.2	Specification of syntax functions, categories, and descriptors	53

G.7.3	Syntax in tabular form	53

G.7.3.1	NAL unit syntax	53

G.7.3.2	Raw byte sequence payloads and RBSP trailing bits syntax	53

G.7.3.3	Profile, tier and level syntax	55

G.7.3.4	Operation point syntax	55

G.7.3.5	Slice header syntax	56

G.7.3.6	Slice data syntax	58

G.7.3.7	Coding tree unit syntax	58

G.7.3.8	Coding quadtree syntax	59

G.7.3.9	Coding unit syntax	59

G.7.3.10	Prediction unit syntax	64

G.7.3.11	Transform tree syntax	64

G.7.3.12	Transform unit syntax	64

G.7.3.13	Residual coding syntax	64

G.7.4	Semantics	65

G.7.4.1	NAL unit semantics	65

G.7.4.2	Raw byte sequence payloads and RBSP trailing bits semantics	65

G.7.4.3	Profile, tier and level semantics	66

G.7.4.4	Operation point semantics	66

G.7.4.5	Slice header semantics	67

G.7.4.6	Slice data semantics	67

G.7.4.7	Coding tree unit semantics	67

G.7.4.8	Coding quadtree semantics	67

G.7.4.9	Coding unit semantics	68

G.7.4.10	Prediction unit semantics	71

G.7.4.11	Transform tree semantics	71

G.7.4.12	Transform coefficient semantics	72

G.7.4.13	Residual coding semantics	72

G.8	Decoding process	73

G.8.1	General decoding process	73

G.8.2	NAL unit decoding process	73

G.8.3	Slice decoding process	73

G.8.3.1	Decoding process for picture order count	73

G.8.3.2	Decoding process for reference picture set	73

G.8.3.3	Decoding process for generating unavailable reference pictures	73

G.8.3.4	Decoding process for reference picture lists construction	73

G.8.3.5	Derivation process for the candidate picture list for disparity vector derivation	74

G.8.3.6	Decoding process for a depth lookup table	74

G.8.4	Decoding process for coding units coded in intra prediction mode	75

G.8.4.1	General decoding process for coding units coded in intra prediction mode	75

G.8.4.2	Derivation process for luma intra prediction mode	75

G.8.4.3	Derivation process for chroma intra prediction mode	77

G.8.4.4	Decoding process for intra blocks	77

G.8.5	Decoding process for coding units coded in inter prediction mode	85

G.8.5.1	Inter prediction process	85

G.8.5.2	Decoding process for prediction units in inter prediction mode	85

G.8.5.3	Decoding process for the residual signal of coding units coded in inter prediction mode	108

G.8.6	Scaling, transformation and array construction process prior to deblocking filter process	108

G.8.7	In-loop filter process	108

G.9	Parsing process	108

G.9.1	Parsing process for Truncated Unary codes	109

G.9.2	Parsing process for 0-th order Exp-Golomb codes	109

G.9.2.1	Mapping process for signed Exp-Golomb codes	109

G.9.3	CABAC parsing process for slice data	109

G.9.3.1	Initialization process	109

G.9.3.2	Binarization process	113

G.9.3.3	Decoding process flow	115

G.9.3.4	Arithmetic encoding process (informative)	117

G.10	Specification of bitstream subsets	117

G.11	Profiles and levels	117

G.12	Byte stream format	117

G.13	3D-HEVC hypothetical reference decoder	117

G.14	3D-HEVC SEI messages	117

G.15	Video usability information	118

List of Figures

Figure 1: Overview of the system structure and the data format for the transmission of 3D video.	8

Figure 2: Access unit structure and coding order of view components.	9

Figure 3: Basic codec structure with inter-component prediction (red arrows).	11

Figure 4: Disparity-compensated prediction as an alternative to motion-compensated prediction.	12

Figure 5: Update of decoded picture buffer.	12

Figure 6: Construction of reference lists (L0, L1).	13

Figure 7: Modification of reference lists (L0, L1).	13

Figure 8: a) The original side view, b) Disocclusion in the side view, and c) CUs selected by the rd-opt for coding in the side view.	13

Figure 9: Basic principle of deriving motion parameters for a block in a current picture based on motion parameters in an already coded reference view and an estimate of the depth map for the current picture.	14

Figure 10: Mapping of a depth map into another view: (left) original depth map; (middle) converted depth map after displacing the original samples; (right) final converted depth map after filling of holes.	15

Figure 11: Generation of an initial depth map estimate after coding the first dependent view of a random access unit.	16

Figure 12: Derivation of a depth map estimate for the current picture using motion parameters of an already coded view of the same access unit.	16

Figure 13: Update of depth map estimate for a dependent view based on coded motion and disparity vectors.	17

Figure 14: Location of spatial neighbour blocks	17

Figure 15: Location of temporal neighbour blocks in temporal candidate pictures	17

Figure 16: Example: DV derivation for the second PU, block A1 is omitted to enable parallel processing	18

Figure 17: The inter-view predicted motion vector of a MCP coded block.	19

Figure 18: Example: For the derivation of the disparity from DV-MCP neighbouring blocks of the current PU, above block B2, B1 and B0 are not used, since they are not located within the current CU.	20

Figure 19: Independent derivation of motion information for each point of encoded CU from corresponding point in reference view.	22

Figure 20: Basic concept for the inter-view residual prediction.	23

Figure 21: Derivation of the location of reference residual block.	23

Figure 22: Wedgelet partition of a block: continuous (left) and discrete signal space (middle) with corresponding partition pattern (right).	27

Figure 23: Contour partition of a block: continuous (left) and discrete signal space (middle) with corresponding partition pattern (right).	27

Figure 24: Intra prediction of Wedgelet partition (blue) for the scenarios that the above reference block is either of type Wedgelet partition (left)or regular intra direction(right).	28

Figure 25: Prediction of Wedgelet (blue) and Contour (green) partition information from texture luma reference.	29

Figure 26: CPVs of block partitions: CPV prediction from adjacent samples of neighbouring blocks (left) and cross section of block (right), showing relation between different CPV types.	29

Figure 27: Reconstructed edge using the relationship between previous and current directions	32

Figure 28: Example of region boundary chain coding	33

Figure 29: Illustration of the concept of motion parameter inheritance.	34

Figure 30: Texture partitions and corresponding possible depth partitions	36

Figure 31: Definition of the SVDC related to the distorted depth data of the block B depicted by the hatched area in the bottom branch; VS denotes the view synthesis step and SSD stands for sum of squared differences.	37

Figure 32: Example for the dependencies between input, intermediate and output signals of the rendering or error calculation step.	39

Figure 33: Example of a CTB QT partitioning for the texture (left), allowed collocated depth CTB QT partitioning (centre), and disallowed collocated depth CTB QT partitioning (right).	42

Figure 34: Rendering from a left camera position to a right camera position using depth maps.	42

Figure 35: Distortion calculation on different tree depths. Renderable samples (gray shaded) are not taken into account.	43

Figure 36: Processing steps of the view synthesis approach.	44

Figure 37: Flow diagram for VSRS general mode.	47

Figure 38: Inpainting: “damaged” image, mask, and result after inpainting.	48

Figure 39: Flow diagram for VSRS 1D mode.	49

List of Tables

Table 1: The availability of the co-located motion vector	21

Table 2: Split flags and partition sizes of depth depending on split flags and partition sizes of texture	36

Table G-1 – Interpretation of dmm_mode	69

Table G-2 – Interpretation of edge_code	69

Table G-3 – Interpretation of sdc_pred_mode	71

Table G-4 – Specification of intra prediction mode and associated names	75

Table G-5 – Specification of divCoeff depending on psDenomDiv	107

Table G-6 – Association of ctxIdx and syntax elements for each initializationType in the initialization process	110

Table G-7 – Values of variable initValue for dmm_flag ctxIdx	111

Table G-8 – Values of variable initValue for dmm_mode ctxIdx	111

Table G-9 – Values of variable initValue for wedge_full_tab_idx ctxIdx	111

Table G-10 – Values of variable initValue for dmm_delta_end_flag and dmm_delta_end_abs_minus1 ctxIdx	111

Table G-11 – Values of variable initValue for dmm_dc_1_abs and dmm_dc_2_abs ctxIdx	111

Table G-12 – Values of variable initValue for res_pred_flag ctxIdx	111

Table G-13 – Values of variable initValue for ic_flag ctxIdx	111

Table G-14 – Values of variable initValue for mvp_l0_idx and mvp_l1_idx ctxIdx	111

Table G-15 – Values of variable initValue for sdc_flag ctxIdx	112

Table G-16 – Values of variable initValue for sdc_residual_flag ctxIdx	112

Table G-17 – Values of variable initValue for sdc_residual_sign_flag ctxIdx	112

Table G-18 – Values of variable initValue for sdc_residual_abs_minus1 ctxIdx	112

Table G-19 – Values of variable initValue for sdc_pred_mode ctxIdx	112

Table G-20 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset	114

Table G-21 – Assignment of ctxIdxInc to syntax elements with context coded bins	116

Table G-22 – Specification of ctxIdxInc using left and above syntax elements	116

Table G-23 – Specification of ctxIdxInc using segment index	117

# Data Format and System Description

3D video is represented using the Multiview Video plus Depth (MVD) format, in which a small number of captured views as well as associated depth maps are coded and the resulting bitstream packets are multiplexed into a 3D video bitstream. After decoding the video and depth data, additional intermediate views suitable for displaying the 3D content on an auto-stereoscopic display can be synthesized using depth-image-based rendering (DIBR) techniques. For the purpose of view synthesis, camera parameters are additionally included in the bitstream. The bitstream packets include header information, which signal, in connection with transmitted parameter sets, a view identifier and an indication whether the packet contains video or depth data. Sub-bitstreams containing only some of the coded components can be extracted by discarding bitstream packets that contain non-required data. One of the views, which is also referred to as the base view or the independent view, is coded independently of the other views and the depth data using a conventional HEVC video coder. The sub-bitstream containing the independent view can be decoded by an unmodified HEVC video decoder and displayed on a conventional 2D display. Optionally, the encoder can be configured in a way that a sub-bitstream representing two views without depth data can be extracted and independently decoded for displaying the 3D video on a conventional stereo display. The codec can also be used for coding multiview video signals without depth data. In that case alternative methods such as Image Domain Warping (IDW) may be used to generate a multiview signal. And, when using depth data, it can be configured in a way that the video pictures can be decoded independently of the depth data.

Figure 1: Overview of the system structure and the data format for the transmission of 3D video.

The basic concept of the system and data format is illustrated in Figure 1. In general the input signal for the encoder consists of multiple views, associated depth maps, and corresponding camera parameters. However, as described above, the codec can also be operated without depth data. The input component signals are coded using a 3D video encoder, which represents an extension of HEVC. At this, the base view is coded using an unmodified HEVC encoder. The 3D video encoder generates a bitstream, which represents the input videos and depth data in a coded format. If the bitstream is decoded using a 3D video decoder, the input videos, the associated depth data, and camera parameters are reconstructed with the given fidelity. For displaying the 3D video on an autostereoscopic display, additional intermediate views are generated by a DIBR algorithm using the reconstructed views and depth data. If the 3D video decoder is connected to a conventional stereo display instead of to an autostereoscopic display, the view synthesizer can also generate a pair of stereo views, in case such a pair is not actually present in the bitstream. At this, it is possible to adjust the rendered stereo views to the stereo geometry of the viewing conditions. One of the decoded views or an intermediate view at an arbitrary virtual camera position can also be used for displaying a single view on a conventional 2D display.

The 3D video bitstream is constructed in a way that the sub-bitstream representing the coded representation of the base view can be extracted by simple means. The bitstream packets representing the base view can be identified by inspecting transmitted parameter sets and the packet headers. The sub-bitstream for the base view can be extracted by discarding all packets that contain depth data or data for the dependent views and, then, the extracted sub-bitstream can be directly decoded with an unmodified HEVC decoder and displayed on a conventional 2D video display.

The encoder can also be configured in a way that the sub-bitstream containing only two stereo views can be extracted and directly decoded using a stereo decoder. The encoder can also be configured in a way that the views can be generally decoded independently of the depth data. It is also possible to synthesize intermediate view using only the stereo sequences as input of the view synthesis.

A detailed description of the coding scheme is given in sec. 2. Depth-image-based rendering algorithms are described in sec. 3.

# Coding Algorithm

In the following, the coding algorithm based on the MVD format, in which each video picture is associated with a depth map, is described. The coding algorithm can also be used for a multiview format without depth maps. The video pictures and, when present, the depth maps are coded access unit by access unit, as it is illustrated in Figure 2. An access unit includes all video pictures and depth maps that correspond to the same time instant. Non-VCL NAL units containing camera parameters may be additionally associated with an access unit. It should be noted that the coding order of access units doesn't need to be identical to the capture or display order. In general, the reconstructed data of already coded access units can be used for an efficient coding of the current access unit. Random access is enabled by so-called random access units or instantaneous decoding refresh (IDR) access units, in which the video pictures and depth maps are coded without referring to previously coded access units. Furthermore, an access unit doesn't reference any access unit that precedes the previous random access unit in coding order. If the picture of the base view in an access unit is an IDR picture, the access unit is an IDR access unit. All pictures in an IDR access unit shall be IDR pictures. The IDR picture type cannot be used in non-IDR access units. For random access, the IDR picture and the clean random access (CRA) picture cannot be simultaneously present in an access unit.

Figure 2: Access unit structure and coding order of view components.

The video pictures and depth maps corresponding to a particular camera position are indicated by a view identifier (viewId). All video pictures and depth maps that belong to the same camera position are associated with the same value of viewId. The view identifiers are used for specifying the coding order inside the access units and detecting missing views in error-prone environments. Inside an access unit, the video picture and, when present, the associated depth map with viewId equal to 0 are coded first, followed by the video picture and depth map with viewId equal to 1, etc. A video picture and depth map with a particular value of viewId are transmitted after all video pictures and depth maps with smaller values of viewId. For the independent view, the video picture is always coded before the associated depth map. For dependent views, the video picture maybe coded before or after the associated depth map (i.e., the depth map with the same value of viewId). It should be noted that the value of viewId doesn't necessarily represent the arrangement of the cameras in the camera array. For ordering the reconstructed video pictures and depth map after decoding, each value of viewId is associated with another identifier called view order index (VOI). The view order index is a signed integer values, which specifies the ordering of the coded views from left to right. If a view A has a smaller value of VOI than a view B, the camera for view A is located left to the camera of view B. In addition, camera parameters required for converting depth values into disparity vectors are included in the bitstream. For the considered linear setup, the corresponding conversion parameters consist of a scale factor and an offset. The vertical component of a disparity vector is always equal to 0. The horizontal component is derived according to

dv = ( s * v + o ) >> n,

where v is the depth sample value, s is the transmitted scale factor, o is the transmitted offset, and n is a shift parameter that depends on the required accuracy of the disparity vectors.

All of the video and depth sequences are associated with a video parameter set. Each video sequence and depth sequence is associated with a separate sequence parameter set and a separate picture parameter set. The picture parameter set syntax, the NAL unit header syntax, and the slice header syntax for the coded slices haven't been modified for including a mechanism by which the content of a coded slice NAL units can be associated with a component signal.

The video parameter set provides the following information about all component sequences:

- the view identifier (indicates the coding order of a view);

- the depth flag (indicates whether video data or depth data are present);

- the view order index (indicates the location of the view relative to other coded views);

For the base view, view identifier and depth flag are not present and inferred to be equal to 0.

The sequence parameter set for all component sequences except the base view has been extended.  These sequence parameter sets contain the following additional parameters:

- an indicator specifying whether camera parameters are present in the sequence parameter set or in the slice headers;

- when camera parameters are present in an sequence parameter set, for each viewId value smaller than the current view identifier, a scale and an offset specifying the conversion of a depth sample of the current view to a horizontal disparity between the current view and the view with viewId;

- when camera parameters are present in an sequence parameter set, for each viewId value smaller than the current view identifier, a scale and an offset specifying the conversion of a depth sample of the view with viewId to a horizontal disparity between the current view and the view with viewId;

The sequence parameter set for the base view doesn't contain the additional parameters. The sequence parameter sets for dependent views include a flag, which specifies whether the camera parameters are constant for a coded video sequence or whether they can change on a picture by picture basis. If this flag indicates that the camera parameters are constant for a coded video sequence, the camera parameters (i.e., the scale and offset values described above) are present in the sequence parameter set. Otherwise, the camera parameters are not present in the sequence parameter set, but instead the camera parameters are coded in the slice headers that reference the corresponding sequence parameter set.

Figure 3: Basic codec structure with inter-component prediction (red arrows).

The basic structure of the 3D video codec is shown in the block diagram of Figure 3. In principle, each component signal is coded using an HEVC-based codec. The resulting bitstream packets, or more accurately, the resulting Network Abstraction Layer (NAL) units, are multiplexed to form the 3D video bitstream. The base or independent view is coded using an unmodified HEVC codec. Given the 3D video bitstream, the NAL units containing data for the base layer can be identified by parsing the parameter sets and NAL unit header of coded slice NAL units (up to the picture parameter set identifier). Based on these data, the sub-bitstream for the base view can be extracted and directly coded using a conventional HEVC decoder.

For coding the dependent views and the depth data, modified HEVC codecs are used, which are extended by including additional coding tools and inter-component prediction techniques that employ already coded data inside the same access unit as indicated by the red arrows in Figure 3. For enabling an optional discarding of depth data from the bitstream, e.g., for supporting the decoding of a stereo video suitable for conventional stereo displays, the inter-component prediction can be configured in a way that video pictures can be decoded independently of the depth data. A detailed description of the added coding tools is given in the following subsections.

## Coding of the Independent View

The independent view, which is also referred to as the base view, is coded using an unmodified HEVC codec.

## Coding of Dependent Views

For the dependent views, the same concepts and coding tools are used as for the independent view. However, additional tools have been integrated into the HEVC codec, which employ already coded data in other views for efficiently representing a dependent view. The additionally integrated tools are described in the following.

### Disparity-compensated prediction

As a first coding tool for the dependent views, the well-known concept of disparity-compensated prediction (DCP), which is also used in MVC, has been added as an alternative to motion-compensated prediction (MCP). At this, MCP refers to an inter-picture prediction that uses already coded pictures of the same view, while DCP refers to an inter-picture prediction that uses already coded pictures of other views in the same access unit, as it is illustrated in Figure 4.

Figure 4: Disparity-compensated prediction as an alternative to motion-compensated prediction.

The macroblock syntax and decoding process haven't been changed for adding DCP, only the high-level syntax has been modified so that already coded video pictures of the same access unit can be inserted into the reference pictures lists (as described in 2.2.1.1). As illustrated in Figure 4, the transmitted reference picture index (R in the figure) signals whether an inter-coded blocks is predicted by MCP or DCP. The motion vector prediction is modified in a way that the motion vectors of motion-compensated blocks are predicted by only using the neighbouring blocks that also use temporal reference pictures, while the disparity vectors of disparity-compensated blocks are predicted by only using the neighbouring blocks that also use inter-view reference pictures.

#### Reference list construction and modification

For motion-/disparity-compensated prediction, the inter-view reference frames as well as the inter-frame reference frames are included in the reference lists (L0, L1). The reference lists are constructed as specified in the following steps.

- Update of decoded picture buffer (DPB)

- The inter-view reference frames, which are already coded frames in the same access unit, are added to the current DPB. The status of the inter-view reference frames are signalled as view dependency in Video Parameter Set (VPS)

- The status of view components present in DPB in a view is signalled by the Reference Picture Set (RPS).

Figure 5: Update of decoded picture buffer.

- Construction of reference lists (L0, L1)

- The inter-frame references are firstly added into the reference lists (L0, L1) in the same manner as the reference list construction in HEVC. After that, the inter-view references are added at the end of the inter-frame references. The inter-view references can be included in L0, L1, or both.

Figure 6: Construction of reference lists (L0, L1).

- Modification of reference lists (L0, L1)

- The constructed reference lists can be modified  based on reference picture list modification syntax table as defined in HEVC.

- One example of the final reference picture lists 0 after modifying the default list is shown in Figure 7.

Figure 7: Modification of reference lists (L0, L1).

### View synthesis based inter-view prediction [Not in SW]

The encoder and the decoder use the same inter-prediction view synthesis algorithm. Basing on all already coded views, a new virtual view is synthesized in the position of the currently processed view. Some regions of newly synthesized image are not available because they were occluded in previously coded views. Those disoccluded regions are identified and marked on a binary map, named availability map, which controls coding and decoding process. Coder and decoder simultaneously use this map to determine, whether given CU is coded or not. Because in a typical case most of the scene is the same in all of views, only small parts are disoccluded in subsequently coded views, and thus only small amount of CUs is coded.

a)                                                b)                                                   c)

Figure 8: a) The original side view, b) Disocclusion in the side view, and c) CUs selected by the rd-opt for coding in the side view.

#### Post processing in-loop filtering [Not in SW]

A final step of view-synthesis prediction is reduction of artefacts in synthesized view. This post-processing consists of Depth-Gradient-based Loopback Filterer (DGLF) and Availability Deblocking Loopback Filter (ADLF).

The first one (DGLF), reduces texture artefacts introduced by DIBR technique in the areas 
of a sudden depth changes. In order to cope that the synthesized image is adaptively filtered with respect to depth gradient strengths. Large depth edges impose strong low-pass filtering of the synthesized texture, while flat depth regions are not filtered at all.

The latter (ADLF), reduces artefacts that are generated as a result of block CU-based coding. Shape of coded region not necessarily matches shape of binary availability map. This discrepancy is a source of artificial edges between those regions (Figure 8 b) and c)) . The ADLF provides smooth transition between coded and synthesized regions by interpolating between them.

### Inter-view motion prediction

The basic concept of the inter-view prediction of motion parameters is illustrated in Figure 9. For the following overview, it is assumed that an estimate of a pixel-wise depth map for the current picture is given. Below, it is described how such an estimate can be derived. For deriving candidate motion parameters for a current block in a dependent view, the maximum depth value d within the associated depth block is converted to a disparity vector. By adding the disparity vector to the sample location x, which is in the middle of the block, a reference sample location xR is obtained. The prediction block in the already coded picture in the reference view that covers the sample location xR is used as the reference block. If this reference block is coded using MCP, the associated motion parameters can be used as candidate motion parameters for the current block in the current view. The derived disparity vector can also be directly used as a candidate disparity vector for DCP.

Figure 9: Basic principle of deriving motion parameters for a block in a current picture based on motion parameters in an already coded reference view and an estimate of the depth map for the current picture.

#### Derivation of Disparity Vectors

The concept of inter-view motion prediction requires a disparity vector to locate a corresponding block of the current PU/CU in an already coded picture of the same time instance. Therefore the codec provides four possibilities to derive a disparity vector. Methods 1 and 2 derive the disparity vector from a depth map. This depth map can be the coded depth map, described as method 1, or the depth map estimate for the current picture, described as method 2. If depth maps are coded when using method 2, the depth map associated with a picture can be coded after the picture. This enables coding techniques that employ the coded pictures for an efficient representation of the depth maps. Methods 3 and 4 derive a disparity estimate from spatial and temporal neighbouring blocks using interview prediction or motion vectors obtained by interview prediction.

In the following, all four methods by which a suitable disparity vector for the current block can be derived based on already transmitted information are described. All methods have been integrated in the codec, and one of the methods can be chosen by configuring the encoder (or by macro switches) accordingly. The used method is signalled in the sequence parameter sets for dependent views. The first method requires the transmission of depth data as part of the bitstream, and by using this method a decoder must decode the depth maps of previously coded views for decoding dependent views. The second method is also applicable if depth maps are not coded inside the bitstream, and if depth maps are coded, the decoding of the video pictures is independent of the depth maps. The third method, namely Neighbouring Blocks based Disparity Vector (NBDV), utilizes disparity motion vectors from spatially and temporally neighbouring blocks. Method 4 simplifies the NBDV method by reducing the number of accesses to temporal reference pictures and acquiring the disparity vectors not only from the disparity-compensated prediction blocks but also motion-compensated prediction blocks.

Method 1: Disparity estimate based on already coded depth map [Not in CTC]

Since the depth map for a reference view is coded before the current picture, the reconstructed depth map is mapped into the coordinate system of the current picture for obtaining a suitable depth map estimate for the current picture. In Figure 10, such a mapping is illustrated for a simple depth map, which consists of a square foreground object and background with constant depth. For each sample of the given depth map, the depth sample value is converted into a sample-accurate disparity vector. Then, each sample of the depth map is displaced by the disparity vector. If two or more samples are displaced to the same sample location, the sample value that represents the minimal distance from the camera (i.e., the sample with the larger value) is chosen. In general, the described mapping leads to sample locations in the target view to which no depth sample value is assigned (black area in the middle picture of Figure 10). These areas represent parts of the background that are uncovered due to the movement of the camera and can be filled using surrounding background sample values. Therefore, a hole filling algorithm, which processes the converted depth map line by line, is used. Each line segment that consists of successive sample location to which no value has been assigned is filled with the depth value of the two neighbouring samples that represents a larger distance to the camera (i.e., the smaller depth value).

Figure 10: Mapping of a depth map into another view: (left) original depth map; (middle) converted depth map after displacing the original samples; (right) final converted depth map after filling of holes.

The disparity vector used for inter-view motion or residual prediction of a block of the current picture is finally derived based on the maximum value within the associated depth block.

Method 2: Depth/Disparity estimate based on coded disparity and motion vectors [Not in CTC]

The above described method 1 is only applicable if depth maps are included in the bitstream, and by using this method, the video pictures (except the base view) cannot be decoded independently of the depth maps. In the following, a method for deriving depth map estimates that only uses data that are available in the coded representations of the video pictures is described. When using this method, one depth sample is derived for a 4x4 block of luma samples. Consequently, the estimated depth maps have 1/4-th of the horizontal and vertical resolution of the luma components. The disparity vector used for inter-view motion or residual prediction of a block of the current picture is finally derived based on the maximum value within the associated depth block.

In random access units, all blocks of the base view picture, are intra-coded. In the pictures of dependent views, most blocks are typically coded using DCP and the remaining blocks are intra-coded. When coding the first dependent view in a random access unit, no depth or disparity information is available. Hence, candidate disparity vectors are derived using a local neighbourhood, i.e., by conventional motion vector prediction. But after coding the first dependent view in a random access unit, the transmitted disparity vectors are used for deriving a depth map estimate, as it is illustrated in Figure 11. Therefore, the disparity vectors used for DCP are converted into depth values and all depth samples that correspond to a disparity-compensated block are set equal to the derived depth value. The depth samples of intra-coded blocks are derived based on the depth samples of neighbouring blocks; the used algorithm is similar to spatial intra prediction. If more than two views are coded, the obtained depth map is mapped into other views using the method described above and used as depth map estimate for deriving candidate disparity vectors. During this mapping, the calculation of the disparity vectors takes into account that the estimated depth maps have 1/4-th of the horizontal and vertical resolution of the luma components.

Figure 11: Generation of an initial depth map estimate after coding the first dependent view of a random access unit.

The depth map estimate for the picture of the first dependent view in a random access unit is used for deriving a depth map for the next picture of the first dependent view. The basic principle of the algorithm is illustrated in Figure 12. After coding the picture of the first dependent view in a random access unit, the derived depth map is mapped into the base view and stored together with the reconstructed picture. The next picture of the base view is typically inter-coded. For each block that is coded using MCP, the associated motion parameters are applied to the depth map estimate. A corresponding block of depth map samples is obtained by MCP with the same motion parameters as for the associated texture block; instead of a reconstructed video picture the associated depth map estimate is used as reference picture. The block of depth samples that is associated with a block of luma samples has 1/4-th of the horizontal and vertical resolution of the luma block. In order to simplify the motion compensation and avoid the generation of new depth map values, the MCP for depth block doesn't involve any interpolation. The motion vectors are rounded to depth-sample-precision (1/4-th of the luma sample precision) before they are used. The depth map samples of intra-coded blocks are again determined on the basis of neighbouring depth map samples. Finally, the depth map estimate for the first dependent view, which is used for the inter-view prediction of motion parameters, is derived by mapping the obtained depth map estimate for the base view into the first dependent view.

Figure 12: Derivation of a depth map estimate for the current picture using motion parameters of an already coded view of the same access unit.

After coding the second picture of the first dependent view, the estimate of the depth map is updated based on actually coded motion and disparity parameters, as it is illustrated in Figure 13. For blocks that are coded using DCP, the depth map samples are obtained by converting the disparity vector into a depth value. The depth map samples for blocks that are coded using MCP are obtained by MCP of the previously estimated depth maps, similar as for the base view.

In an optional configuration, new depth values are determined by adding a depth correction. The depth correction is derived by converting the difference between the motion vectors for the current block and the corresponding reference block of the base view into a depth difference. The depth values for intra-coded blocks are again determined by a spatial prediction. The updated depth map is mapped into the base view and stored together with the reconstructed picture. It is also used for deriving a depth map estimate for other views in the same access unit.

Figure 13: Update of depth map estimate for a dependent view based on coded motion and disparity vectors.

For all following pictures, the described process is repeated. After coding the base view picture, a depth map estimate for the base view picture is determined by MCP using the transmitted motion parameters. This estimate is mapped into the second view and used for the inter-view prediction of motion parameters. After coding the picture of the second view, the depth map estimate is updated using the actually used coding parameters. At the next random access unit, the inter-view motion parameter prediction is not used, and after decoding the first dependent view of the random access unit, the depth map is re-initialized as described above.

Method 3: Disparity vector from neighbouring blocks [Not in CTC]

The disparity vector is derived by searching neighbouring blocks for a motion vector used for DCP, called disparity motion vector). Once a disparity motion vector is identified, the whole disparity vector derivation process terminates. If no disparity motion vector is found from the neighbouring blocks, a zero disparity vector is used for inter-view motion prediction. The neighbouring blocks can be spatial neighbouring blocks and temporal neighbouring blocks.

Which spatial and temporal neighbouring blocks are searched for NBDV is described in the following part, followed by the checking order.

| Figure 14: Location of spatial neighbour blocks | Figure 15: Location of temporal neighbour blocks in temporal candidate pictures |
| --- | --- |

- Spatial Neighbouring Blocks

Five spatial neighbouring blocks are used for the disparity vector derivation. They are: the below-left, left, above-right, above and above-left blocks of current prediction unit (PU), denoted by A0, A1, B0, B1 or B2, as defined in Figure 14.

To enable the DV derivation process to be performed in a parallel way two constraints on searched blocks are applied. The first constraint is that DV are not derived from neighbouring blocks in the same CUs, when the CU contains two PUs. Figure shows an example where for the second PU block A1 is not used for DV derivation .

A1

B1

B2

B0

A0

Temporal DV predictor

(TDVP)

1st PU

2nd PU

Figure 16: Example: DV derivation for the second PU, block A1 is omitted to enable parallel processing

A second constraint is to search only the blocks that are also utilized in the merge scheme in the HEVC base specification, when the DV is derived for the derivation of an inter-view merge candidate.

- Temporal Neighbouring Blocks

Up to two reference pictures from current view are treated as candidate pictures for temporal neighbours. The first candidate picture is the co-located picture as used for Temporal Motion Vector Prediction (TMVP) in HEVC without low delay check. The co-located picture is indicated in a slice header. The second picture is derived in the reference picture lists with the ascending order of reference picture indices, and added into the candidate list, given as follows:

- A random access point (RAP) is searched in the reference picture lists. If found, the RAP is placed into the candidate list for the second picture and the derivation process is completed. In a case that the RAP is not available for the current picture, go to step (2).

- A picture with the lowest temporalID (TID) is searched out and placed into the candidate list of the temporal pictures as the second entry.

- If multiple pictures with the same lowest TID exist, a picture of less POC difference with the current picture is chosen.

As shown in the above description, the second temporal candidate picture is chosen in a way that disparity motion vectors can have more chance to be present in the picture. The derivation process of the second candidate picture can be done in the slice level and be invoked only once per slice.

For each candidate picture two temporal neighbouring blocks Centre and BR as depicted in Figure 15 are searched.

Checking order

Spatial neighbouring blocks are firstly checked, followed by temporal neighbouring blocks. The checking order of the five spatial neighbouring blocks is defined as A1, B1, B0, A0 and B2. For each candidate picture the two temporal neighbour CUs are checked. The checking order is defined as: Centre and BR for the first non-base view or BR, Centre for the second non-base view.

Disparity derivation from MCP coded neighbour blocks

In addition to the DCP coded blocks, blocks coded by motion compensated prediction (MCP) are also used for the disparity derivation process. When a neighbour block is MCP coded block and its motion is predicted by the inter-view motion prediction, as shown in Figure 16, the disparity vector used for the inter-view motion prediction represents a motion correspondence between the current and the inter-view reference picture. This type of motion vector is referred to as inter-view predicted motion vector (IvpMv) and the blocks are referred to as DV-MCP blocks in the sequel. The motion correspondence is used for the disparity derivation process as explained in the following.

V0 (independent view)

(dependent view)

Figure 17: The inter-view predicted motion vector of a MCP coded block.

To indicate whether a blocks is DV-MCP block or not and to save the disparity vector used for the inter-view motion prediction, two variables are used:

- IvpMvFlag

- IvpMvDisparityX.

The block whose motion vector is inter-view predicted is identified when the 0th motion parameter candidate of MERG/SKIP mode is selected. In that case, the IvpMvFlag and IvpMvDisparityX corresponding to the location of current PU are set to 1 and the horizontal component of the disparity vector used for the inter-view motion prediction, respectively.

The disparity vector is derived from SKIP coded DV-MCP blocks. When a block is coded by skip mode, neither mvd (motion vector difference) data nor residual data is signalled, which implies that the disparity vector used for SKIP coded DV-MCP block well describes the motion correspondence than the disparity vector used for DV-MCP blocks that are not SKIP coded.

If DCP coded block is not found in the spatial and temporal neighbour blocks, then disparity derivation process scans the spatial neighbour blocks for DV-MCP compensated in following order: A0, A1, B0, B1, B2. If a neighbour block is a SKIP coded DV-MCP block, then the value of IvpMvDisparityX at the neighbour block is returned as the derived disparity. The vertical component of the disparity vector is set equal to zero.

To reduce the amount of memory required for the derivation of the disparity from DV-MCP blocks, blocks B0, B1 and B2 are only utilized when they are located in the current LCU. An example for this can be seen in Figure 18. Here only spatial neighbour blocks A1 and A0 are utilized.

Figure 18: Example: For the derivation of the disparity from DV-MCP neighbouring blocks of the current PU, above block B2, B1 and B0 are not used, since they are not located within the current CU.

#### Usage of Inter-View Motion Parameter Prediction

In HEVC, two different modes for signalling the motion parameters for a block are specified. In the first mode, which is referred to as adaptive motion vector prediction (AMVP) mode, the number of motion hypotheses, the reference indices, the motion vector differences, and indications specifying the used motion vector predictors are coded in the bitstream. The second mode is referred to as merge mode. For this mode, only an indication is coded, which signals the set of motion parameters that are used for the block. The inter-view motion parameter prediction has been added to both modes, as will be described in the following.

Inter-view motion vector prediction in the AMVP mode

In the adaptive motion vector prediction (AMVP) mode, the number of motion hypotheses, the reference indices specifying the used reference pictures, the motion vector differences, and indexes specifying the used motion vector predictor are transmitted in the bitstream. For each motion hypothesis, a candidate list of motion vector predictors is derived based on the coded reference index. This list includes motion vectors of neighbouring blocks that are associated with the same reference index as well as a motion vector predictor which is derived based on the motion parameters of the co-located block in a temporal reference picture. For including the inter-view motion parameter prediction, the AMVP mode has been extended in a way that an inter-view motion vector predictor is added to the candidate list. It is inserted at the first position of the list. Pruning between the inter-view motion vector predictor and spatial motion vector predictors is not carried out.

To determine the inter-view motion vector prediction, disparity vector and a corresponding block in a reference view are derived as described above. If the reference index for the current block refers to an inter-view reference picture, the inter-view motion vector predictor is set equal to the corresponding disparity vector. If the current reference index refers to a temporal reference picture and the corresponding block uses a motion hypothesis that refers to the same access unit as the current reference index, the motion vector that is associated with this motion hypothesis is used as inter-view motion vector predictor. In all other cases, a zero motion vector is included in the list of motion vector predictor candidates instead of the inter-view motion vector predictor.

Inter-view motion vector prediction in the merge mode (and skip mode)

In the merge mode of HEVC (as well as in the skip mode, which represents the merge mode without coding a residual signal), basically the same motion parameters (number of hypotheses, reference pictures, and motion vectors) as for a neighbouring block are used. If a block is coded in the merge mode, a candidate list of motion parameters is derived, which includes the motion parameters of spatially neighbouring blocks as well as motion parameters that are calculated based on the motion parameters of the co-located block in a temporal reference picture. The chosen motion parameters are signalled by transmitting an index into the candidate list.

Similarly as for the AMVP mode, the candidate list of motion parameters is extended by a motion parameter candidate for MCP (IvMC) that is obtained using inter-view motion prediction. Moreover a motion parameter candidate for DCP constructed from the derived disparity (IvDC) is added. The derivation of both additional candidates is described in the following.

For the derivation of the IvMC and the IvDC candidate, a corresponding block in a view component at the same time instant as the current view component is utilized. The corresponding block is determined by shifting the position of the current block using the disparity vector derived as described above.

If the corresponding block is coded using MCP it is tested for each motion hypothesis of the current block, whether an motion vector and a reference index can be derived for the IvMC candidate. This is the case if a picture is included in the reference picture list belonging to the current slice and motion hypothesis with a picture order count equal to the picture order count of the reference picture of the corresponding block. When such a picture is found the reference index of this picture in the reference picture list belonging to the current slice and motion hypothesis and the motion vector of the hypothesis of the corresponding block are used to derive the IvMC candidate.

To derive the IvDC candidate it is tested for each motion hypothesis of the current block, if the corresponding block is located in an inter-view reference picture that is included in the reference picture list belonging to the current slice and motion hypothesis. If such a reference picture is found, the IvDC candidate is constructed by using the derived disparity as motion vector and the reference index of the found reference picture in the reference picture list of the current slice.

In the case that all motion parameter candidates are available, the position of the IvMC candidate is the first and the position of the IvDC candidate is the fifth in the merge candidate list. Moreover pruning of the list is carried out if the IvMC candidate or the IvDC candidate are equal to one of the motion parameter candidates of the spatially neighbouring blocks A1 and B1.

#### Derivation of co-located motion vector candidate

The availability of co-located vector is specified in Table 1. The co-located motion vector is available for motion vector prediction when the coded motion vector has the same characteristics as the co-located motion vector. The co-located motion vector is not-available for motion vector prediction when the coded motion vector has different characteristics than the co-located motion vector.

Table 1: The availability of the co-located motion vector

| Availability of co-located vector | Coding vector | Co-located vector |
| --- | --- | --- |
| Available | Motion vector | Motion vector |
| Non-available | Motion vector | Inter-view vector |
| Not-available | Inter-view vector | Motion vector |
| Available | Inter-view vector | Inter-view vector |

Inter-view prediction is restricted to reference pictures that have the same POC as the current picture. POC is used to specify the availability of the co-located motion vector. The co-located motion vector is available when one of the following two cases is true. In case 1, both the coded motion vector and co-located motion vector are motion vectors referring to reference pictures inside the current view (temporal motion vectors). In case 2, both the coded motion vector and the co-located motion vector are motion vectors that refer to inter-view reference pictures (disparity vectors).

- (CurrPOC != CurrRefPOC) && (ColPOC != ColRefPOC)

- (CurrPOC == CurrRefPOC) && (ColPOC == ColRefPOC)

The variables in the above conditions are specified as follows:

CurrPOC : POC of the current picture

ColPOC : POC of the co-located picture

CurrRefPOC : POC of the picture that is referenced by the current picture

ColRefPOC : POC of the picture that is referenced by the co-located picture

In case 1, the co-located motion vector is scaled using the difference of POCs as specified in HEVC. In case 2, the co-located motion vector is scaled using the difference of ViewOrderIdx instead of the POC difference. The variable ViewOrderIdx that is associated with each view specifies the order of the view from left to right. The scaling factor is calculated using the following equations:

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )

tx = ( 16384 + Abs( td / 2 ) ) / td

where td and tb are derived as

td = Clip3( -128, 127, ColViewOrderIdx – ColRefViewOrderIdx )

tb = Clip3( -128, 127, CurrViewOrderIdx – CurrRefViewOrderIdx )

The variables in the above equations are specified as follows:

CurrViewOrderIdx :ViewOrderIdx of current picture

ColViewOrderIdx :ViewOrderIdx of co-located picture

CurrRefViewOrderIdx :ViewOrderIdx of the picture that is referenced by the current picture

ColRefViewOrderIdx :ViewOrderIdx of the picture that is referenced by the co-located picture

### Depth-based motion parameter prediction [Not in SW]

Depth-Based Motion Prediction (DBMP) is a new coding tool for multiview video coding which originates from the idea that motion fields of neighbouring views in multiview sequence are highly correlated. DBMP provides an efficient representation of motion data in multiview video bitstreams that carry also depth/disparity maps. The motion information, such as motion vectors and reference indices, for each pixel of encoded coding unit (CU) 
is directly inferred with use of already coded disparity maps from encoded CUs in the neighbouring views at the same temporal instance (Figure 19). This procedure is repeated independently for every pixel of encoded CU. Consequently, motion vectors and reference indices for CU are not transmitted in the bitstream but are obtained from the reference view at the receiving side.

Figure 19: Independent derivation of motion information for each point of encoded CU from corresponding point in reference view.

### Inter-view residual prediction

The basic principle of the inter-view residual prediction is illustrated in Figure 20. Similarly as for the inter-view motion prediction, the inter-view residual prediction is based on a depth map estimate for the current picture. The same depth map estimate as for the inter-view motion prediction is used. Depending on the encoder configuration, the depth map estimate is derived by one of the two methods described in sec. 2.2.3.1. Based on the depth map estimate, a disparity vector is determined for a current block and the residual block in the reference view that is referenced by the disparity vector is used for predicting the residual of the current block.

Figure 20: Basic concept for the inter-view residual prediction.

A more detailed illustration of the concept for deriving a reference block location inside the reference view is given in Figure 21. Inside the current block, a sample location x in the middle of the block is selected and the associated depth value d is converted to a disparity vector. The disparity vector is added to the location of the top-left sample of the current block yielding the location of the top-left sample of the reference block. Then, similar as for motion compensation, the block of residual samples in a reference view that is located at the derived reference location is subtracted from the current residual and only the resulting difference signal is transform coded. If the disparity vector points to a sub-sample location, the residual prediction signal is obtained by interpolating the residual samples of the reference view using a bi-linear filter.

Figure 21: Derivation of the location of reference residual block.

The usage of the inter-view residual prediction can be adaptively selected on a block basis, or more accurately on a coding unit (CU) basis. For that purpose, based on the availability of the reference residual signal (see 2.2.5.1) and the type of the reference residual signal (see 2.2.5.2), a flag indicating the usage of inter-view residual prediction is transmitted as part of the CU syntax. If this flag is equal to 1, the current residual signal is predicted using the potentially interpolated reference residual signal and only the difference is transmitted using transform coding. Otherwise, the residual of the current block is conventionally coded using the HEVC transform coding.

#### Availability of reference residual signal

The inter-view residual prediction consists of two steps. Firstly, the relevant region of residual reference is located. Then the availability of the residual reference is determined based on coded block flag (CBF) and coding mode information of blocks in the relevant region.

The residual reference region is located based on a given disparity vector. Assume the left-top position of the current CU is (x, y) and after taking into consideration of a disparity with a value of dis, which might be fractional-accurate, the co-located area of the reference picture, from which the residue is predicted, is represented by the top-left and bottom-right pixel locations (x0, y0) = (max(floor(x+dis),0), y) and (x1, y1) = (min (ceil(x+dis+CUWidth-1), picWidth-1), y+ CUHeight-1), wherein (CUWidth, CUHeight) is the size of the current CU and picWidth is the width of the picture width.

Afterwards, all transform units (TU) covered or partially covered by the residual reference region are traversed. If any of these TUs is inter-coded and contains a non-zero CBF value (luma CBF or chroma CBF), the related residual reference is marked as available and residual prediction may be applied. Otherwise, IVRP is not applied to this CU. Only when residual prediction may be applied for a CU, an IVRP flag needs to be signalled.

#### Availability of residual prediction

Inter-view residual prediction is only used for temporally predicted PUs. For a CU where inter-view residual prediction flag is true, each PU in it should determine whether to use inter-view residual prediction or not according to the following rule:

- Uni-prediction case

If (CurrViewIdx == RefViewIdx)

Inter-view residual prediction is enabled;

else

Inter-view residual prediction is disabled;

The variables are defined as:

CurrViewIdx: view index of current picture

RefViewIdx: view index of reference picture

- Bi-prediction case

If ( CurrViewIdx == Ref0ViewIdx && CurrViewIdx == Ref1ViewIdx)

Inter-view residual prediction is enabled;

elseif ( CurrViewIdx == Ref0ViewIdx || CurrViewIdx == Ref1ViewIdx )

Inter-view residual prediction is enabled with a weighting factor of 1/2;

else

Inter-view residual prediction is disabled;

Where:

Ref0ViewIdx: view index of reference picture from reference list 0

Ref1ViewIdx: view index of reference picture from reference list 1

When inter-view residual prediction is enabled with a weighting factor of 1/2, the reference residual used in inter-view residual prediction is divided by 2.

Above rule is employed at both encoder and decoder. If inter-view residual prediction is disabled for all PUs in a CU, inter-view residual prediction flag is not encoded/decoded for the CU.

### Illumination compensation (IC)

A linear illumination compensation model is utilized to adapt luminance and chrominance of inter-view predicted blocks to the illumination of the current view. The parameters of the linear model are estimated for each PU using reconstructed neighbouring samples of the current block and of the reference block used for prediction.

[Ed. (GT). Some more information specifying how the linear parameters are computed, which samples are used for computation and how the computation is realized using the LUT would be helpful here.]

To reduce the encoding time, the rate distortion optimized (RDO) selection, whether to enable illumination compensation is not carried out for all inter modes. Instead of this, first a RDO selection between IC on and IC off is performed for the Merge mode with 2Nx2N partitioning. When IC is chosen in this test, IC is switched on when testing the following inter modes.

### Adjustment of QP of texture based on depth data [Not in SW]

In order to improve perceptual quality of coded texture, a tool for bit assignment in the texture layer was developed. The basic idea is to increase texture quality of objects in the foreground and to increase compression factor (decrease texture quality) for objects in the background. The quality is adjusted in coding units (CUs) with use of quantization parameter QP that depends on the corresponding depth values. The QP adjustment is done simultaneously in coder and decoder so that no additional information is send. Described tool is disabled in the base view to preserve HEVC compatibility. The texture QP is modified in the following way:

QP'=QP-2.6+8∙255-maxx,y∈CUdx,y2562

Where QP' is adjusted QP value for a CU with corresponding disparity dx,y.

## Coding of Depth Maps

For the coding of depth maps, basically the same concepts of intra-prediction, motion-compensated prediction, disparity-compensated prediction, and transform coding as for the coding of the video pictures are used. However, some tools have been modified for depth maps, other tools have been generally disabled, and additional tools have been added.

As a first difference to the coding of video pictures, the inter-view motion and residual prediction as described in sec. 2.2.2 and sec. 2.2.4, respectively, are not used for depth coding. Instead, motion parameters are derived based on coded data in the associated video pictures as will be described in sec. 2.3.7 below. The other differences are described in the following subsections.

### Disabled chrominance coding [Not in SW]

Depth maps may be coded in 4:0:0 chroma sampling format.

### Non-linear depth representation [Not in SW]

As alternative representation of depth maps, the depth may be non-linearly scaled as described in the following.

The human perception of depth depends on absolute distance of viewed objects, therefore the internal depth representation is non-linear. Closer objects are represented more accurately than distant ones. Thanks to that, subjective quality of synthesized views is improved.

Internal depth sample values are defined by the following power-law expressions, similar as in the case of well-known gamma correction:

Exponent is automatically chosen by the encoder with use of base QP for the depth and sent to decoder in the encoded bitstream:

exponent=clipQPdepth-30∙0.0125+1.25  ; 1.0  ; 1.66

Depth map samples are represented on increased number of bits with use of IBDI (Internal Bit Depth Increase) tool.

### Z-near z-far compensated weighted prediction  [Not in SW]

Proposed znear-zfar compensation (ZZC) is a new coding tool for multiview video, designed especially for inter-frame depth map coding.

The concept of ZZC exploits the observation that frames from different views and time instances of encoded depth sequence may have different znear and zfar parameters. The mentioned znear and zfar parameters describe range of depths represented in a gray-scale depth map. If znear and zfar parameters are different for two frames, then given depth value is represented with different gray-scale values in those depth maps. Consequently, using one of such depth maps as a reference for the other one will result in a poor prediction.

To overcome this problem, a new ZZC coding tool is proposed. Prior to any inter-frame depth map prediction, each depth map that resides on the codec reference picture list is scaled, so that gray-scale depth values in scaled image and currently coded image refer to the same depth. 
As a result, depth maps with compensated znear and zfar range are used for prediction.

Values used for prediction (instead of the original ones) are calculated as follows:

LT=LS∙zfar S-znear Szfar T-znear T+255∙znear S-znear Tzfar T-znear T

Where LT is compensated disparity in range depth znearT  tozfarT  and LS is original disparity in  depth range znearS and  zfarS.

### Modified motion compensation and motion vector coding

In contrast to natural video, depth maps are characterized by sharp edges and large regions with nearly constant values. The eight-tap interpolation filters that are used for motion-compensated interpolation in HEVC, can produce ringing artefacts at sharp edges in depth maps, which are visible as disturbing components in synthesized intermediate views. For avoiding this issue and for decreasing the encoder and decoder complexity, the motion-compensated prediction (MCP) as well as the disparity-compensated prediction (DCP) has been modified in a way that no interpolation is used. That means, for depth maps, the inter-picture prediction is always performed with full-sample accuracy. For the actual MCP or DCP, a block of samples in the reference picture is directly used as prediction signal without interpolating any intermediate samples. In order to avoid the transmission of motion and disparity vectors with an unnecessary accuracy, full-sample accurate motion and disparity vectors are used for coding depth maps. The transmitted motion vector differences are coded using full-sample instead of quarter-sample precision.

### Disabling of in-loop filtering

The in-loop filters in the HEVC design have been particularly designed for the coding of natural video. For the coding of depth maps, these filters are less useful. In order to decrease the encoder and decoder complexity, the in-loop filters have been disabled for depth coding. This includes the following filters:

- the de-blocking filter;

- the adaptive loop filter (Wiener  filter);

- the sample-adaptive loop filter.

### Depth modelling modes

Depth maps are mainly characterized by sharp edges (which represent object borders) and large areas of nearly constant or slowly varying sample values (which represent object areas). While the HEVC intra prediction and transform coding is well-suited for nearly constant regions, it can result in significant coding artefacts at sharp edges, which are visible in synthesized intermediate views. For a better representation of edges in depth maps, four new intra prediction modes for depth coding are added. In all four modes, a depth block is approximated by a model that partitions the area of the block into two non-rectangular regions, where each region is represented by a constant value. The information required for such a model consists of two elements, namely the partition information, specifying the region each sample belongs to, and the region value information, specifying a constant value for the samples of the corresponding region. Such a region value is referred to as constant partition value (CPV) in the following. Two different partition types are used, namely Wedgelets and Contours, which differ in the way the segmentation of the depth block is derived. The depth modelling modes are integrated as an alternative to the conventional intra prediction modes specified in HEVC. Similar as for the intra prediction modes, a residual representing the difference between the approximation and the original depth signal can be transmitted via transform coding. In the following, the approximation of depth blocks using the four new depth modelling modes is described in more detail.

It is differentiated between Wedgelet and Contour partitioning. For a Wedgelet partition, the two regions are defined to be separated by a straight line, as illustrated in Figure 22, in which the two regions are labelled with P1 and P2. The separation line is determined by the start point S and the end point P, both located on different borders of the block. For the continuous signal space (see Figure 22, left), the separation line can be described by the equation of a straight line. The middle image of Figure 22 illustrates the partitioning for the discrete sample space. Here, the block consists of an array of samples with size uB×vB and the start and end points correspond to border samples. Although the separation line can be described by a line equation as well, the definition of regions P1 and P2 is different here, as only complete samples can be assigned as part of either of the two regions. For employing Wedgelet block partitions in the coding process, the partition information is stored in the form of partition patterns. Such a pattern consists of an array of size uB×vB and each element contains the binary information whether the corresponding sample belongs to region P1or P2. The regions P1and P2 are represented by black and white samples in Figure 22 (right), respectively.

Figure 22: Wedgelet partition of a block: continuous (left) and discrete signal space (middle) with corresponding partition pattern (right).

Unlike for Wedgelets, the separation line between the two regions of a Contour partition of a block cannot be easily described by a geometrical function. As illustrated in Figure 23, the two regions P1 and P2 can be arbitrary shaped and even consist of multiple parts. Apart from that the properties of Contour and Wedgelet partitions are very similar. For employing Contour partitions in the coding process, the partition pattern (see example in Figure 23, right) is derived individually for each block from the signal of a reference block. Due to the lack of a functional description of the region separation line, no pattern lookup lists and consequently no search of the best matching partition are used for Contour partitions.

Figure 23: Contour partition of a block: continuous (left) and discrete signal space (middle) with corresponding partition pattern (right).

Apart from the partition information, either in form of a Wedgelet or a Contour partition, the second information required for modelling the signal of a depth block is the CPV of each of the two regions. For a given partition the best approximation is consequently achieved by using the mean value of the original depth signal of the corresponding region as the CPV.

Four depth-modelling modes, which mainly differ in the way the partitioning is derived and transmitted, have been added:

- Mode 1: Explicit Wedgelet signalling;

- Mode 2: Intra-predicted Wedgelet partitioning;

- Mode 3: Inter-component-predicted Wedgelet partitioning;

- Mode 4: Inter-component-predicted Contour partitioning.

These depth-modelling modes as well as the signalling of the modes and the constant partition values are described in the following four subsections.

#### Mode 1: Explicit Wedgelet Signalization

The basic principle of this mode is to find the best matching Wedgelet partition at the encoder and transmit the partition information in the bitstream. At the decoder the signal of the block is reconstructed using the transmitted partition information.

The Wedgelet partition information for this mode is not predicted. At the encoder, a search over a set of Wedgelet partitions is carried out using the original depth signal of the current block as a reference. During this search, the Wedgelet partition that yields the minimum distortion between the original signal and the Wedgelet approximation is selected. The resulting prediction signal is then evaluated using the conventional mode decision process.

A fast search of the best matching partition is essential for employing Wedgelet models in the depth coding process. This fast search algorithm is further described in section 2.3.6.7.

#### Mode 2: Intra-predicted Wedgelet Partitions

The basic principle of this mode is to predict the Wedgelet partition from data of previously coded blocks in the same picture, i.e. by intra-picture prediction. For a better approximation, the predicted partition is refined by varying the line end position. Only the offset to the line end position is transmitted in the bitstream and at the decoder the signal of the block is reconstructed using the partition information that results from combining the predicted partition and the transmitted offset.

Figure 24: Intra prediction of Wedgelet partition (blue) for the scenarios that the above reference block is either of type Wedgelet partition (left)or regular intra direction(right).

The prediction process of this mode derives the line start position and the gradient from the information of previously coded blocks, i.e. the neighbour blocks left and above of the current block. Note that for some blocks one or both of the neighbouring blocks are not available. In such a case the processing for this mode is carried out with setting the missing information to meaningful default values. As illustrated in Figure 24 two main prediction methods have to be distinguished: The first method covers the case when one of the two neighbouring reference blocks is of type Wedgelet, shown in the example in Figure 24, left. The second method covers the case when the two neighbouring reference blocks are not of type Wedgelet, but of type intra direction, which is the default intra coding type, shown in the example in Figure 24, right.

If the reference block is of type Wedgelet, the prediction process works as follows: The principle of this method is to continue the reference Wedgelet into the current block, which is only possible if the continuation of the separation line of the reference Wedgelet actually intersects the current block. Therefore, it is first checked whether it is possible to continue the reference Wedgelet. In case the check is positive, the start position Spand the end position Ep are predicted by calculating the intersection points of the continued line with block border samples.

If the reference block is of type intra direction, the prediction process works as follows: First, the gradient is derived from the intra prediction direction. As the intra direction is only provided in the form of an abstract index, a mapping or conversion function is defined that associates each intra prediction mode with a gradient. Second, the start position Sp is derived from information that is also available at the decoder, namely the adjacent samples of the left and above neighbouring block, by selecting the sample position with the maximum slope. Finally, the end position Ep is calculated from the start point and the gradient.

The line end position offset for refining the Wedgelet partition is not predicted, but searched within the estimation process at the encoder. For the search, candidate partitions are generated from the predicted Wedgelet partition and an offset value for the line end position Eoff, as illustrated in Figure 24. By iterating over a range of offset values and comparing the distortion of the different resulting Wedgelet partitions, the offset value of the best matching Wedgelet partition is determined using a distortion measure.

#### Mode 3: Inter-component prediction of Wedgelet partitions

The basic principle of this mode is to predict the Wedgelet partition from a texture reference block, namely the co-located block of the associated video picture. This type of prediction is referred to as inter-component prediction. Unlike temporal or inter-view prediction, no motion or disparity compensation is used, as the texture reference picture shows the scene at the same time and from the same perspective. The Wedgelet partition information is not transmitted for this mode and consequently, the inter-component prediction uses the reconstructed video picture as a reference. For efficient processing, only the luminance signal of the reference block is taken into account, as this typically contains the most significant information for predicting the partition of a depth block, i.e. the edges between objects.

Figure 25: Prediction of Wedgelet (blue) and Contour (green) partition information from texture luma reference.

The prediction of a Wedgelet partition pattern from the texture reference is illustrated in the top row of Figure 25. To find a suitable prediction two methods can be applied. The first method is a search over the set of possible Wedgelet partitions as described in section 2.3.6.7. The Wedgelet partition that yields the smallest distortion for the co-located texture block is used for approximating the current depth block. The second method can be used when the co-located texture block is intra coded. In this case the wedgelet search is constraint by the chosen texture intra mode as described in section 2.3.6.6.

#### Mode 4: Inter-component prediction of Contour partitions

The basic principle of this mode is to predict a Contour partition from a texture reference block by inter-component prediction. Like for the inter-component prediction of a Wedgelet partition pattern, the reconstructed luminance signal of the co-located block of the associated video picture is used as a reference, as illustrated in the bottom row of Figure 25. In contrast to Wedgelet partitions, the prediction of a Contour partition is realized by a thresholding method. Here, the mean value of the texture reference block is set as the threshold and depending on whether the value of a sample is above or below the sample position is marked as part of region P1 or P2 in the resulting Contour partition pattern.

#### Constant partition value coding

The method for CPV coding is the same for all four modes introduced above, as it does not distinguish between partition types, but rather assumes that a partition pattern is given for the current depth block. As illustrated in Figure 26, three types of CPVs are differentiated: original, predicted, and delta CPVs.

Figure 26: CPVs of block partitions: CPV prediction from adjacent samples of neighbouring blocks (left) and cross section of block (right), showing relation between different CPV types.

The cross section of the block in Figure 26, right, schematically shows that the original CPVs are calculated as the mean value of the signal covered by the corresponding region. Although these values lead to the best approximation for the given partition, they are not available at the decoder as they require the original signal. Therefore prediction of CPVs is introduced. These predicted CPVs are derived from information that is also available at the decoder, namely the adjacent samples of the neighbouring left and top block, as illustrated in Figure 26, left, where the green and light green line segments highlight the mapping of adjacent samples to the partitions. Again, the predicted CPVs are calculated as the mean value of the corresponding sample values. Depending on the similarity between original signal of the block and adjacent samples, the predicted and original CPVs may differ significantly. This difference is referred to as delta CPVs. By calculating the delta CPVs at the encoder and transmitting them in the bit stream, it is possible to reconstruct the CPVs at the decoder.

Although the distortion of the reconstructed signal is considerably reduced by the delta CPVs, the benefit of this approach is delimited by the additional bit rate required for transmitting the delta CPVs. Therefore, a linear quantization is introduced for the delta CPVs. This method is also used in transform coding and the step size of the quantization is set as a function of the QP. The delta CPVs are linearly quantized at the encoder and de-quantized before reconstruction at the decoder.

In case the distortion is not measured for the original depth, but for synthesized views, the delta CPV derivation process is extended by a minimum distortion search, which iterates over all possible delta CPV combinations for the two partitions. For the sake of efficient processing and signalling the range of tested values is limited. The search results in the combination of delta CPVs that causes the minimum distortion in synthesized views and for transmission these values are finally quantized.

#### Co-located Texture Luma Block intra direction aligned search

For DMM mode 3, the intra direction of the co-located texture luma block can be utilized to constrain the Wedgelet search at the decoder to most probable Wedgelet directions. Basically, the method consists of two parts: first, for each intra direction, a Wedgelet list is initialized with Wedgelet patterns sharing the similar directionality of the intra direction, second, during the Wedgelet search, only the Wedgelet patterns included in the Wedgelet list for the intra direction of co-located texture luma block are considered.

Wedgelet list generation for each intra direction. To generate the Wedgelet list, each Wedgelet pattern index idxW is first mapped to an intra direction widx by applying the following steps:

- For each intra direction i from 2 to 34, Di is calculated as Di = | Vi (Xs-Xe) - Hi (Ye-Ys) |, where Hi and Vi are obtained from Table 1, (Xs, Ys) and (Xe, Ye) indicate the start and end point position of the Wedgelet pattern, respectively.

- The Wedgelet pattern index idxW is mapped to the intra direction widx which minimizes D among all intra directions.

Table 1: Values of H and V for angular intra directions

| intra direction | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| V | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | -2 | -5 | -9 | -13 | -17 | -21 | -26 | -32 |
| intra direction | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |
| H | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |  |
| V | -32 | -32 | -32 | -32 | -32 | -32 | -32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |  |

Then for each intra direction pidx a Wedgelet list is constructed that includes all the Wedgelet patterns with mapped intra direction widx satisfying | widx - pidx | ≤ 1.

Wedgelet search. The intra direction of co-located texture luma block is first derived as pidx, then only the Wedgelet patterns in the Wedgelet list of intra direction pidx is used in the Wedgelet search function as described in section 2.3.6.7, thus large portion of Wedgelet searches are skipped.

#### Coarse wedgelet search with refinement step

The coarse search with refinement step is used at the encoder side for DMM mode 1 and at the decoder side for DMM mode 3. The search method basically consists of three steps: first, the wedgelet pattern list is initialized. In the second step a coarse subset of the Wedgelet pattern list is searched for the minimum distortion partition. The resulting Wedgelet partition is refined in the third step.

Wedgelet pattern list initialization. The Wedgelet pattern lists are generated during encoder and decoder initialization.

For this purpose, the patterns for all possible combinations of start and end point positions are generated and stored in a lookup table for each block size prior to the coding process. The Wedgelet pattern list contains only unique patterns. The resolution for the start and end positions used for generating the Wedgelet patterns depends on the block size. For 16x16 and 32x32 blocks, the possible start and end positions are restricted to locations with an accuracy of 2 samples. For 8x8 blocks, full-sample accuracy is used, and for 4x4 blocks, half-sample accuracy is used

A coarse subset of Wedgelet patterns are marked by a flag during the list generation process. The subset contains the Wedgelets for every second start and end position (see Fig. 1, left). Furthermore, a Wedgelet node is derived for each Wedgelet of the coarse subset, containing up to eight references to refinement Wedgelets (see Fig. 1, right).

Figure 1. Coarse subset of Wedgelet separation line start/end positions (left) and Wedgelet node with refinements (right).

Search function. The following search algorithm is applied to find the Wedgelet within the pattern list, that provides the minimal distortion: In a first step the minimum distortion Wedgelet pattern is searched for the coarse subset by iterating over the Wedgelet node list and testing the referenced pattern. In a second step the Wedgelet partition found in the first step is refined. For this purpose the minimum distortion Wedgelet pattern is searched from the up to eight refinement patterns referenced in the corresponding node.

#### Mode selection

In the encoding process, for an intra-coded CU, one of the described depth modelling modes or one of the conventional intra prediction modes is selected. If a depth modelling mode is selected, the selected mode and the associated prediction data have to be signalled in the bitstream in addition to a syntax element that specifies the usage of a depth modelling mode. The following four depth modelling modes are defined:

- Wedgelet_ModelIntra: Intra modelling of Wedgelet block partition

- Wedgelet_PredIntra: Intra prediction of Wedgelet block partition

- Wedgelet_PredTexture: Inter-component prediction of Wedgelet block partition

- Contour_PredTexture: Inter-component prediction of Contour block partition

Each of the four modes can be applied with or without delta CPVs, resulting in eight different mode_IDs for signalling the decoder, which type of processing has to be applied for prediction and reconstruction of the block.

#### Signalling in the bitstream

The depth modelling modes are implemented as an additional set of block coding modes into the intra path of the 3D video codec. Therefore, an additional flag prior to the mode information is transmitted in the bitstream, signalling whether a block partition mode is used or not. In case this flag is not set, normal intra mode signalling follows. Otherwise, a mode ID is signalled, which specifies the actual block partition mode and if delta CPVs are also transmitted or not. The number of bins required depends on the decision of the mode pre-selection methods, ranging from three bins, if all eight modes are enabled, to one bin, if the number of modes is reduced to two due to pre-selection decisions described in sec. 2.3.6.6.

Mode Wedgelet_ModelIntra: For this mode the Wedgelet partition information is explicitly signalled in the bitstream by the index of the corresponding pattern in the Wedgelet pattern lookup list. The index is signalled with a fixed number of bins. The number of bins used for transmitting the index is given by the size of the list of possible Wedgelet patterns.

Mode Wedgelet_PredIntra: For this mode only the refinement of the Wedgelet partition in terms of the line end position offset is signalled in the bitstream. A first bin indicates whether the offset is zero or not. If the offset is not zero, k+1additional bins follow for signalling offset values in the range ±2k, where the first bin represents the sign and the remaining k bins the absolute value of the offset.k is set equal to 2.

Mode Wedgelet_PredTexture: For this mode no additional signalling regarding the partition information is required.

Mode Wedgelet_PredTexture: For this mode no additional signalling regarding the partition information is required.

Delta CPVs: In case the delta CPVs are transmitted (which is signalled by the transmitted mode ID), the two quantized values are signalled in the bitstream consecutively. For each CPV, a bin string consisting of the absolute value and the sign is transmitted. The sign is coded as a single bin, and the absolute value is coded using a truncated unary code (with 13 bins in the unary part and an exponential golomb code suffix).

### Region boundary chain coding

The region boundary chain coding mode partitions the block into two regions by signalling the region boundaries with chain codes. The region boundary chain coding consists of four steps.

Step 1: Find internal edges

The internal edges inside a depth-map block are calculated in the encoder. The step consists of several procedures.

- Calculate differences between vertically and horizontally adjacent pixels.

- Mark as edge candidates if the difference is greater than the threshold.

- Eliminate the edge candidates which have smaller differences than neighbouring edge candidates.

- Connect unlinked edges if necessary.

- Prune unconnected edges.

- Check whether the block consists of two regions exactly. Otherwise, the method will not be applied further.

Step 2: Code the edges using chain codes

The edges are encoded by using chain codes. First, an edge starts from the block boundaries. Then, the next edge is chosen as an edge connected to the current one. The final edge ends at the boundaries. To construct chain codes, it defines seven traverse types (0, 45, -45, 90, -90, 135, and -135 degree) and their edge patterns. Figure 27 shows the patterns when the previous pixel is the left one (grey arrow). The patterns of other directions differ from the figure but can be constructed in the same manner.

Figure 27: Reconstructed edge using the relationship between previous and current directions

Step 3: Convert the chain codes into bitstream syntax

The region boundaries are expressed as a set of an edge starting point and the traverse codes.

Step 4: Calculate the predictors and fill them into the block

After the regional average values are chosen as the predictors for partitioned regions, the method for constant partition value (CPV) coding described in 2.3.6.5 can be applied to the mean value of the signal covered by the corresponding region.

#### Signalling in the bitstream

The region boundary chain coding modes are implemented as an additional set of block coding modes into the intra path of the 3D video codec. When rem_intra_luma_pred_mode is equal to 31, an additional flag is transmitted in the bitstream, signalling whether a region boundary chain coding mode is used or not.

Two syntax elements (edge_start_left_flag and edge_start_position) are used to determine the starting point. And the traverse codes are signalled with num_edge_codes_minus1 and edge_code as described in Figure 28. The edge_code is constructed to have a different code word length according to the frequency of traverse types.

In case the CPVs are transmitted (which is signalled by edge_dc_flag ), two quantized values are signalled in the bitstream consecutively. For each CPV, a bin string consisting of the absolute value and the sign is transmitted. The sign is coded as a single bin, and the absolute value is coded using a truncated unary code (with 13 bins in the unary part and an exponential golomb code suffix).

Figure 28: Example of region boundary chain coding

### Simplified depth coding

The Simplified Depth Coding (SDC) approach is an alternative intra coding mode. Whether SDC is used is signalled by a SDC flag at CU level. For SDC the depth block is intra predicted by a conventional intra mode or depth modelling mode. The partition size of SDC coded CU is always 2Nx2N and therefore not signalled in the bitstream. Moreover the residual is not coded as quantized transform coefficients but one or two constant residual values are signalled.

In summary following information are signalled for SDC-coded blocks:

- The type of segmentation/prediction of the current block. Possible values are

  - DC (1 segment)

  - DMM Mode 1 – Explicit Wedgelets (2 segments)

  - DMM Mode 2 – Intra-Predicted Wedgelets (2 segments)

  - Planar (1 segment)

- For the two DMM modes, additional prediction information is coded, as described in  section 2.3.6

- For each resulting segment, a residual value (in the pixel domain) is signalled in the bitstream

Before coding, the residual values are mapped to values, which are present in the original, uncompressed depth map by using a Depth Lookup Table (DLT). Consequently, residual values can be coded by signalling only the index into this lookup table, which reduces the bit depth of residual magnitudes. This mapping table is transmitted to the decoder for the inverse lookup from index to valid depth value.

The advantage of using this lookup table is the reduced bit depth of the residual index for sequences with reduced depth value range (e.g. all estimated depth maps where not all depth values are present).

At the encoder side the Residual index iresi to be coded into the bitstream, is given by

iresi=Idorig- I(dpred)

with dorig denoting original depth value, dpred denoting the predicted depth value, and I. denoting the Index Lookup Table and dvalid denoting the number of valid depth values.

The computed residual index iresi is then coded with a significance flag, a sign flag and with log2dvalid bits for the magnitude of the residual index.

#### Depth Lookup Table

The Depth Lookup Table utilizes the property of the depth map, that the full available depth range of 28 values is not utilized. Only a small amount of different depth levels occur due to strong quantization. In the encoder, a dynamic depth lookup-table is constructed by analysing a certain number of frames (e.g. one intra period) of the input sequence. This depth lookup-table is used during the coding process to reduce the effective signal bit-depth of the residual signal.

##### Construction of the depth lookup table

In encoder reads a pre-defined number of frames from the input video sequence to be coded and scans all samples for available depth map values. During this process a mapping table is generated that maps depth values to valid depth values based on the original uncompressed depth map.

The Depth Lookup Table D(.), the index Lookup Table I(.), the Depth Mapping Table M(.) and the number of valid depth values dvalid are derived by the following algorithm, that analyses the depth map Dt

- Initialization

- boolean vector Bd=FALSE for all depth values d

- index counter i=0

- Process each pixel position p in Dt for multiple time instances t:

- Set BDtp=TRUE to mark valid depth values

- Count number of TRUE values in B(d)  dvalid

- For each d with Bd==TRUE:

- Set Di=d

- Set Md=d

- Set Id=i

- i=i+1

- For each d with Bd==FALSE:

- Find d=arg mind-d and Bd==TRUE

- Set Md=d

- Set Id=I(d)

### Motion parameter inheritance

The basic idea behind the motion parameter inheritance (MPI) mode is that the motion characteristics of the video signal and its associated depth map should be similar, since they are both projections of the same scenery from the same viewpoint at the same time instant. Therefore, in order to enable efficient encoding of the depth map data, a new coding mode that allows inheritance of the treeblock subdivision into CUs and PUs and their corresponding motion parameters from the video signal has been introduced. Since the motion vectors of the video signal have quarter-sample accuracy, whereas for the depth map signal only full-sample accuracy is used, in the inheritance process the motion vectors are quantized to their nearest full-sample position. It can be adaptively decided for each block of the depth map, whether the motion data are inherited from the co-located region of the video signal or if new motion data are transmitted (cp. Figure 29). For signalling the MPI coding mode, the merge/skip mode syntax is used. The list of possible merge candidates has been extended in a way that, for depth map coding, the first merge candidate refers to merging with the corresponding block from the associated video signal.

Figure 29: Illustration of the concept of motion parameter inheritance.

Independent of the partitioning of the video picture into its CUs, the MPI mode can be used at any level of the treeblock hierarchy for the depth map. If the MPI mode is indicated at a higher level of the depth map coding tree, corresponding to a CU size that is larger than the CU size that is used for the video signal, the CU/PU subdivision, together with the corresponding motion data, is inherited from the video signal. This makes it possible to specify once for a whole treeblock, typically corresponding to 64 × 64 image samples, that the whole partitioning of this region into its CUs and PUs for the video signal is also applied to the depth map signal. In the other case, if MPI is indicated at a level of the coding tree that corresponds to the same or a smaller CU size than the CU size that is used for the video signal, only the motion data are inherited from the video signal. Since, when using MPI, not only the partitioning and the motion vectors, but also the reference picture indices are inherited from the video signal, it has to be ensured, that the depth maps that correspond to the video reference pictures are also available in the reference picture buffer for the depth map signal. If the region of the video signal, that the motion data and partitioning are to be inherited from includes PUs that are intra coded or not available, the components of the motion vectors and reference indices for these regions are set equal to 0.

### Depth Quadtree Prediction

Depth quadtree prediction performs a prediction of the depth quadtree from the texture quadtree. The partitioning of the depth is limited to the same level as the partitioning of the texture. For a given CTU, the quadtree of the depth is linked to the collocated CTB quadtree in the texture, so that a given CU of the depth cannot be split more than its collocated CU in the texture. Moreover, when the texture is split in 2NxN (or Nx2N), partitioning to 2NxN, Nx2N, or NxN is not performed for depth. The possible partitioning is depicted in

Figure 30. Corresponding split flags and partition sizes for depth depending on the split flags and partition sizes of texture are summarized in Table 2.

Figure 30: Texture partitions and corresponding possible depth partitions

Table 2: Split flags and partition sizes of depth depending on split flags and partition sizes of texture

| Texture SplitFlag | Texture PartSize | Depth SplitFlag | Depth PartSize | Residual Depth SplitFlag | Residual Depth PartSize |
| --- | --- | --- | --- | --- | --- |
| 1 | - | 1 | - | 1 | - |
| 1 | - | 0 | 0, 1, 2 or 3 | 0 | 0, 1, 2 or 3 resp. |
| 0 | 0,1, 2 or 3 | 0 | 0 | - | - |
| - | 0, 1 or 2 | - | 0 | - | - |
| - | 3 | - | 0, 1, 2 or 3 | - | 0,1, 2 or 3 resp. |
| SplitFlag: 0 = no split, 1 = split; PartSize: 0 = 2Nx2N, 1 = Nx2N, 2 = 2NxN, 3 = NxN. |  |  |  |  |  |

## Encoder Control

For mode decision and motion estimation, a Lagrangian technique by which a cost measure D+λ⋅R is determined for each candidate mode or parameter, and the mode or parameter with the smallest cost measure is selected.Dis the distortion that is obtained by coding the considered block in a particular mode or with a particular parameter, R is the number of bits that are required for representing a block in a given mode or that are required for coding a given parameter, and λ is the Lagrangian multiplier that is derived based on the used quantization parameter. As measure for the distortion, the sum of squared differences (SSD) or the sum of absolute differences (SAD) between the original and the reconstructed sample values is used (for the coding of depth maps this measure was modified as described below).

For the coding of depth maps, basically the same decision process is used. However, the distortion measure has been replaced with a measure that considers the distortion in synthesized intermediate views. This technique is described in the following subsection.

### View Synthesis Optimization

The geometry information given by depth data is exploited only indirectly in the rendering process. Hence, the lossy coding of depth data causes distortions in the synthesized intermediate views. The depth map itself is not visible for a viewer. The efficiency of depth coding is improved by considering this property. As a consequence, the distortion measure for the mode decision process for depth maps is modified in a way that a weighted average of the synthesized view distortion and the depth map distortion. To obtain a measure of the synthesized view distortion, two different metrics  are applied in RDO.

The first metric, discussed in section 2.4.1.1, is the synthesized view distortion change (SVDC). The computation of the SVDC requires the usage of rendering functionalities in the encoding process. Since computational complexity is a critical factor in distortion calculation, a method, which is also referred to as renderer model, has been utilized that allows minimal re-rendering of parts of the synthesized view that are affected by a depth distortion. For this, a special renderer is included in the encoder, which supports the basic functionalities, shared by most rendering approaches, like sub-pixel accurate warping, hole filling and view blending.

The second metric, presented in section 2.4.1.2, is a model based synthesized view distortion estimation without rendering. Basic idea of this metric is to derive an estimate for the synthesized view distortion by weighting the depth distortion with a factor derived from the absolute value of the derivation of texture view in horizontal direction.

The integration of both metric in the encoder control is presented in sections 2.4.1.4 and 2.4.1.5.

#### Synthesized View Distortion Change (SVDC)

##### Definition of the SVDC

Since the encoding algorithm operates block-based, the mapping of depth distortion to the synthesized view distortion must be block-based as well. Moreover, the sum of partial distortions (of sub-blocks) must be equal to the overall distortion of a block in order to enable an independent distortion calculation for all partitions of a subdivided block, as hierarchical block structures are used in HEVC.

A relationship between a depth map sDand a synthesized texture sT'is created by the used view synthesis approach. However, disocclusions and occlusions prevent a bijective mapping of the distorted areas in depth maps to distorted areas in the synthesized views. For example, areas in the synthesized view, which depend on depth data of a considered block, can become visible due to the distortions in other depth blocks; or vice versa, the distortion of a depth block has no effect on the synthesized view, since the block is occluded there. Hence, an exact mapping between the distortion of a block of the depth data and an associated distortion in the synthesized view is not possible considering only the depth data within a currently processed block.

For resolving this issue, the change of the overall distortion in a synthesized view depending on the change of the depth data within a block B is determined, while simultaneously also considering depth data outside the block B. For this purpose, the synthesized view distortion change (SVDC) is defined as distortion difference ΔDbetween two synthesized textures sT'and sT,

|  | ΔD=D- D= x,y∈IsTx,y-s'T,Refx,y2-x,y∈IsT'(x,y)-s'T,Refx,y2 | (1) |
| --- | --- | --- |

s'T,Refdenotes a reference texture rendered from original video and depth data. Irepresents the set of all samples in the synthesized view. To illustrate how the textures sT'and sT' are obtained, the SVDC definition from eq. (1) is also depicted in Figure 31. sT'denotes a texture rendered from a depth map sD consisting of encoded depth data in already encoded blocks and original depth data in the other blocks. The current block B, for which the distortion has to be computed, contains original depth data as well. For the synthesis of the texture sT' a depth mapsDis used that differs from the depth map sDin that it contains the distorted depth data also for the current block B.

Figure 31: Definition of the SVDC related to the distorted depth data of the block B depicted by the hatched area in the bottom branch; VS denotes the view synthesis step and SSD stands for sum of squared differences.

The SVDC definition above is motivated by three reasons. First, an exact distortion measure is provided, therefore the overall distortion of the synthesized view and thereby disocclusions and occlusions are considered. Second, the measure is related to a block and third partial distortions are additive. For the latter two reasons, the change of the synthesized view distortion caused by a change of a depth block is employed instead of the total synthesized view distortion itself.

Figure 31 shows the SVDC definition for the extrapolation of virtual views from one input view only. However, the encoder side view synthesis algorithm supports also the interpolation of the texture sT' from a left and a right view. Hence, rendering requires a left sD,l and a right sD,r depth map. To extend the SVDC computation to this two view case, the original depth map of the second view can be used when encoding the first depth map. Subsequently the first already encoded depth map can be utilized for the SVDC computation when encoding the second depth map.

##### Efficient Computation of the SVDC

A straightforward approach to compute the SVDC would be the direct implementation of eq. (1). However, this would require the complete rendering of the synthesized textures sT' and sT' and a rendering of a whole view is computational too complex to be feasible in a rate-distortion optimization process. To overcome this problem, a method which enables a fast computation of the SVDC and is integrated in the encoder.

Renderer Model

The renderer model provides three basic functionalities to the encoder: Initialization, partial re-rendering, and SVDC calculation.

- The initialization of the renderer model is carried out before the encoding of a depth map is started. In the initialization process, the complete synthesized view is rendered using the original input depth maps and the input textures. The input depth maps are stored as the renderer models depth states sD,l and sD,r and the rendered view as the synthesized view state sT'. Intermediate variables used in the rendering process are also stored to enable a fast re-rendering.

- Partial re-rendering is carried out to update the renderer model when the encoding of a block B is finished and the final depth data for the block is known. For this purpose, the reconstructed depth data and the position of block B are signalled to the renderer model. The renderer model changes the block in the depth state sD,l or sD,rfrom original to coded data and re-renders only local parts of the synthesized view state sT' and the intermediate variables that are affected by the change of the depth data. Thus, the renderer model is transferred to a state that is required to compute the SVDC for blocks of the depth data encoded subsequently.

- For the computation of the SVDC, the position and the depth data of a block B to be tested are provided to the renderer model. The renderer model then computes the SVDC as defined in eq. (1). Here, re-rendering followed by the computation of the sum of squared distortions SSD is carried out. However, instead of considering all positions x,y∈I again only positions affected by the depth change are considered. Note that the re-rendering carried out here does not modify any state variables of the renderer model. Hence, the SVDC can be computed for multiple depth candidates successively without the need to re-render with original data in blockB.

Re-Rendering and Error Calculation Algorithm

The main objective of the algorithm is a computational low complex distortion calculation or state transition, hence a low complex re-rendering of the parts of the synthesized view that are affected by a depth change in one of the input depth maps.

Conventional view synthesis consists of multiple steps such as warping of the input samples, interpolation at sub pixel positions, blending with a second view obtained similarly, and hole filling. Typically these steps are executed as independent algorithms that are applied successively using the results of the previous step. To enable fast re-rendering of only parts of the synthesized view, all steps are combined in single algorithm that can be applied pixel-wise to the input depth map. This allows a region-wise processing of the depth map, and thus an update of related regions in the synthesized view.

This process is illustrated in Figure 32 for an example for rendering from a left view to the right. Rendering is applied row wise, hence all depicted signals represent one row of input, intermediate, or output data. The single signals are from bottom to top: the left input texture sT,l, a shifting chart, the texture synthesized from left s'T,lthe texture synthesized from right s'T,r, the blended textures'T, and the reference textures'T,Ref. The arrows denote the relationship between the single samples or sample positions of the signals. Dots shown in the shifting chart represent samples from the input view. Their horizontal position is equal to their position x'in the synthesized view. The vertical position shows their disparities. Since the depth is monotonically decreasing with increasing disparity, the top-most samples in the chart are the samples closest to the camera. Hence, it can be seen from the shifting chart which samples are occluded in the synthesized view.

Figure 32: Example for the dependencies between input, intermediate and output signals of the rendering or error calculation step.

While a conventional view synthesis approach would carry out the single steps depicted from bottom to top for all samples in the intervals (a) to (g), the method supports an interval-wise processing. Hence, all steps are first conducted for interval (a) before continuing with interval (b). Re-rendering and error calculation are carried out by iterating only once over the input depth samples. If only the view synthesis distortion is calculated there is no need to store intermediate results in the state of the renderer model.

The boundaries of an interval in the output view are defined by the warped positions x’s and x’eof two neighboring input view samples at positions xsand xe. For warping, disparities are computed from the depth map as described in the beginning of sec. 2. Subsequently to the calculation of the interval boundaries, processing continues with interpolation, disocclusion handling, or occlusion handling:

- Interpolation is carried out in non-occluded ranges that are not disoccluded, as for example in the intervals (a, c, d, g, h). The accuracy of the warping is higher than the accuracy given by the sampling rate of synthesized view; hence an interpolation at the full sample position x’FPlocated between the interval boundaries x’sand x’eis carried out. For this, samples from an up-sampled version of the input texture sT,l are mapped to the interpolation positions x’FPin the synthesized view s'T,l. The position xin the up-sampled view is derived from the distance of the interpolation position to the interval boundaries:

|  | x=4⋅xFP'- xs'xe'-xs'+xs | (2) |
| --- | --- | --- |

The up-sampled view sT,lis created in the initialization step by interpolating the input texture with quarter-sample accuracy using the FIR-filters specified for motion-compensated interpolation in HEVC.

- Disocclusions: If the width of the warped interval x’e– x’sis greater than two times the width of the sampling distance, as for example for interval (b), a disocclusion is assumed in the synthesized view. Instead of interpolation, hole filling is carried. For this purpose, the samples in the interval are set equal to the value of the sample belonging to the right interval boundary sT,l(xe) (which belongs to the background). If the leftmost full sample position within the interval is close to the left interval border, it is assumed that it belongs to the foreground and it is set equal to the value of the left interval boundary sT,l(xs). Note, that the positions of disoccluded and filled samples are stored as additional information in the a filling map s'F,l.

- Occlusions: Whether an interval is entirely occluded in the synthesized view, as for example interval (f), is determined by detecting if the interval boundaries are reversed (x’e< x’s), hence no complex z-buffering is required. To derive whether other samples left to interval (f) are occluded, the rendering process stores the position of the foreground edge. This stored position is then be utilized when processing the next intervals, for example interval (e), to determine which parts of theses intervals are occluded. If re-rendering does not start at the right image border, the position of the last foreground edge is recovered by carrying out a search to the right of the changed depth samples.

Sample values derived from interpolation or hole filling sT,l', are instantly combined with the texture sample values from a second view sT,r' synthesized the same way and stored as intermediate variable in the renderer model. The result is the sample value that is used in the final synthesized view sT'.

The rendering model supports two different configurations. In the first configuration, a rendering process is considered that renders intermediate views using both surrounding actually coded views. The second configuration considers rendering processes by which an intermediate view is rendered mainly from one coded view; the other coded view is only used for rendering areas that are not present in the preferred coded view.

In the first configuration of the renderer model, the blending process is similar to that implemented in the VSRS software. Note that, although not depicted in Figure 32, a depth map sD,l' is rendered from sD,l, when rendering sT,l', using full sample accuracy. This depth map is used in the blending step. The decision how blending is carried out depends on the filling of sF,l'or sF,r' and the rendered depth maps sD,l'and sD,r'. While sF,l'and sD,l'have been obtained in the rendering process carried out before, sF,r' and sD,r' are stored as intermediate variables in the renderer model. The rules for determining the blended sample value s’T(x,y) from sT,l'(x,y) and sT,r'x,y are specified in the following:

- If the position (x,y) is disoccluded (as indicated by the filling map) in only one view, the sample value from the other view is used.

- Otherwise, if the position (x,y) is disoccluded in both views, the backmost sample value is used.

- Otherwise, if the depth difference retrieved from sD,l'(x,y) and sD,r'(x,y) is greater than a threshold, the front sample is used.

- Otherwise, a weighted average of sT,l'(x,y) and sT,r'x,y, with a higher weight for the view that is closer to the virtual view position, is used.

For the second configuration of the renderer model, the intermediate view is mainly rendered from one view and only holes are filled from the other view. If assuming thatsT,l' is the main view ,the rules to determine the sample value s’T(x,y) from sT,l'(x,y) and sT,r'x,y are specified in the following:

- If sF,l'x,yindicates that there is no disocclusion at sT,l'x,y, the sample valuesT,l'x,y is used.

- Otherwise, if sF,r'x,y indicates that there is a disocclusion at sT,r'x,y, the sample value sT,l'x,y is used.

- Otherwise, the average of sT,r'x,yand sT,l'x,y is used.

If only partial re-rendering is carried out, the result sT' and all intermediate results are stored after the combination step and the processing of the interval is stopped. Otherwise, if the SVDC is determined, the distortion of the calculated value sT' is computed by comparing it to the referencesT,Ref' in the next step.

To obtain the synthesized view distortion change the single intervals are rendered from right to left and the related distortions are summed up continuously. Moreover, and that is actually not depicted in Figure 32, the old per sample distortions of samples in the changed intervals are subtracted.

The renderer model only re-renders those parts of the synthesized view that are affected by the considered depth change. It has to be considered that in some cases not only the intervals related to the changed depth values must be re-rendered, but also some neighbouring intervals. A reason is that neighbouring intervals that are occluded before a depth change can become visible after the depth change. The algorithm detects such cases and continues rendering, until all change samples in the synthesized view are updated. The detection is carried out while warping by also considering the old shifted sample positions as they had been prior to the depth change and storing the left-most old position.

Chroma channels of the synthesized view are rendered together with the luma channel and are stored in the same resolution as luma. For this, up sampled versions of the chroma channels are created in the initialization step, which are later used for interpolation as described above. The sampling rate is increased by a factor of eight in horizontal direction and a factor of two in vertical direction using the interpolation FIR-filters that are specified for motion-compensated prediction in HEVC. However, the total distortion is obtained by a weighted sum of luma SVDC and chroma SVDC with a weight of 1 for luma and a weight of ¼ for each of the two chroma channels.

Early skip of SVDC computation

To increase the processing speed of the VSO algorithm the SVDC calculations is skipped for lines of a block for that the distortion of the disparity vector is zero. This means that if distorted depth and original depth are mapped to the same disparity vector for all pixels in a line of the depth block the SVDC calculation is not carried out and the distortion is assumed to be zero. .

#### Model based synthesized view distortion estimation without rendering

The distortion of depth maps does not linearly affect the synthesis distortion, and the impact of depth map distortions varies according to the corresponding texture information. For example, the same depth distortions on textured and textureless regions lead to different synthesis distortions.

In a conventional video coding system, one commonly used distortion function is the sum of squared differences (SSD), which is defined between original and encoded depth block as

|  | Ddepth=x,y∈BsDx,y-sDx,y2 | (3) |
| --- | --- | --- |

where sDx,y and sDx,y indicate the original and reconstructed depth map, respectively, and (x, y) means the pixel position in a (macro-) block B. However, the conventional SSD metric is not an good estimate of the synthesized view distortion. Instead, the following view synthesis distortion (VSD) metric provides an better estimate by that weighting the depth distortion Ddepth with the sum of absolute horizontal texture gradients:

|  | VSD=x,y∈B12⋅α⋅sDx,y-sDx,y⋅sT(x,y)-sT(x-1,y)+sT(x,y)-sT(x+1,y)2 | (4) |
| --- | --- | --- |

sT indicates the reconstructed texture, and α is proportional coefficient determined by

|  | α=f⋅L255⋅1Znear-1Zfar | (5) |
| --- | --- | --- |

with f denoting the focal length, L denoting the baseline between the current and the rendered view, Znear and Zfar representing the values of the nearest and farthest depth of the scene, respectively.

#### Depth fidelity term

When encoding using the synthesized view distortion change or estimate only, the depth fidelity is strongly distorted. In order to preserve the depth fidelity the distortion measure used in RDO is computed by an weighted average of the synthesized view distortion or the estimated synthesized view distortion and the depth distortion. The distortion D used in RDO for depth maps is given by

|  | D= wsynthDsynth+wdepthDdepth | (6) |
| --- | --- | --- |

with Dsynth denoting the synthesized view distortion change or estimate, Ddepth denoting the distortion of the depth map itself (i.e. SAD or SSD), and wsynth and wdepth denoting the weights for the two distortion terms.

#### Integration of distortion metrics in the Encoder Control

To enable rate-distortion optimization using the SVDC, the described renderer model is integrated in the encoding process for depth data. For this, the conventional distortion computation is replaced with computation of the weighted average of depth distortion and SVDC in distortion computation steps related to the mode decision, coding unit (CU) partitioning, motion parameter inheritance and merging. Note that for updating the renderer model used for the SVDC calculation re-rendering is carried out when a final decision on the coding mode is taken by the encoder control.

In order to reduce the computational complexity the weighted average of SVDC and depth distortion in not used for all encoding decisions. A weighted average of VSD and depth distortion is used for intra-mode pre-selection and residual quadtree partitioning. For motion estimation and rate-distortion optimized quantization the conventional SSD of depth data is used.

#### Adaptation of the Lagrange Multiplier

The usage of the synthesized view distortion in the rate-distortion decisions requires the adaptation of the Lagrange multiplier  to obtain optimized coding results. This adaptation is carried out by adjusting the Lagrange multiplier using an additional scaling factor ls depending on the QP of the coded video. The factor enables an adjustment of video/depth rate allocation. As alternative a constant factor can be used.

The computation of rate-distortion cost  has been modified to

|  | J=D+ls⋅λ⋅R=D+λl⋅R | (7) |
| --- | --- | --- |

with D denoting the weighted average of depth and synthesized view distortion, lsdenoting a scaling factor, and R denoting the rate for the current coding mode.

### Zero residual coding for depth intra CUs

In the rate distortion optimized coding of inter blocks a decision between coding with and without residual is carried out. For depth coding this principle is extended to intra coded blocks that are not part of slices using intra prediction only. Therefore, the residual is set to zero by the encoder. No additional signalling is used.

### Optional Encoder Control using a depth quadtree limitation

In the encoding process a given CTB is split into smaller CUs, based on RD optimized decisions. A corresponding quadtree (QT) is obtained for the texture, and another one for the depth. This tool prevents the encoder from making full investigation of every possible QT configuration for the depth.

The tool forces the encoder to limit the partitioning of the depth at the same level as the partitioning of the texture. For a given CTU, the quadtree of the depth is linked to the collocated CTB quadtree in the texture, so that a given CU of the depth cannot be split more than its collocated CU in the texture.

This encoder restriction results in encoder runtime saving for the depth.

Figure 33: Example of a CTB QT partitioning for the texture (left), allowed collocated depth CTB QT partitioning (centre), and disallowed collocated depth CTB QT partitioning (right).

Figure 33 illustrates this principle. On the left a CTB QT partitioning for the texture is represented. In the centre, the collocated CTB in the depth is represented. This QT partitioning is allowed because it is, CB by CB, coarser than the corresponding texture CB. On the right, another example of possible collocated CTB in the depth is represented. This QT partitioning is disallowed because one CB is more partitioned than the texture (red lines).

### Optional Encoder Control for Renderable Regions in Dependent Views [Not in CTC]

As an optional encoding technique, a mechanism is integrated by which regions in dependent views that can be rendered based on the transmitted independent view and the associated depth maps are identified. These regions are encoded by employing a modified cost measure, which mainly considers the required bit rates. After decoding, the renderable regions can be identified in the same way as in the encoder and replaced by rendered versions.

Figure 34: Rendering from a left camera position to a right camera position using depth maps.

The encoder identifies regions in the current frame that can be rendered from frames of the same time instance in a reference view based on the reconstructed depth maps of the reference view (see Figure 34). During the encoding process, the encoder checks for every CU, if all samples within that CU can be rendered. If all samples can be rendered, no residual is transmitted for this CU. In our HEVC-based codec, this means that for inter prediction the no_residual_data_flag for the CU is set equal to 1 or for intra-prediction the coded block flag of the TUs within the CU is set equal to 0. It should be noted that no syntax change is applied; only the encoder decision is modified.

Due to the quadtree structure in HEVC, the rate-distortion (RD-)costs are compared between different granularities of possible block subdivisions for the R-D optimization. Rendering artefacts have a different impact on the subjective image/video quality perception than coding artefacts and cannot be compared using conventional measurements, such as MSE or PSNR. Samples in renderable regions are not taken into account for calculating the distortion term in the R-D optimized encoder decisions. In Figure 35, the right image shows a block subdivision that is one level deeper than the ones in the left image. The grey area labels the samples that can be rendered and that are therefore not considered in the calculation of the distortion. Thus, for example, the upper left block in the right image is not considered at all. Hence, the costs being compared are D0+λR0(left block subdivision) against D1+λ R1+ D2+λ+D3+λ R3+D4+λ R4 (right block subdivision), where the distortions are only calculated based on the white shaded samples. E.g., the distortion of block B1is D1=0. By this modification, blocks for which a subblock can be rendered are not automatically split, but also the entire block may be coded using a conventional coding mode if this improves the overall coding efficiency.

Figure 35: Distortion calculation on different tree depths. Renderable samples (gray shaded) are not taken into account.

For renderable blocks, the Lagrange multiplier λ is scaled by a factors>1 and the calculation of the R-D costs is changed from C=D+λR to C=D+sλR.

### Depth edge-based r-d optimization tuning [Not in SW]

As alternative or in addition to the r-d optimization for depth maps as described in sec. 2.4.1, the following r-d optimization for depth coding is included. The purpose of this additional rd-opt is to reduce edge ringing artefacts in depth maps.

Normally cost is calculated as SAD between original and distorted (reconstructed) samples:

Ex,y=originalx,y-distortedx,y

C=x,yEx,y

Cdepth=x,yEx,y+αx,yHEx,y2

Where Ex,y is distortion,  is original distortion cost, Cdepthis modified distortion cost for depth, and H∙is a horizontal filter {1,-1} that detects vertical edges.

# View Synthesis Algorithms

In the following, two view synthesis algorithms are described. Sec. 3.1 describes the fast 1-dimensional view synthesis algorithm that is part of the HEVC-based 3DV software. It is also referred to as "VSRS 1D fast mode". In sec. 3.2, an alternative view synthesis algorithm is described. This algorithm is also referred to as "VSRS" and was developed during the 3DV exploration experiments.

## Fast 1-D View Synthesis (VSRS 1D Fast Mode)

An overview of the view synthesis method is depicted in Figure 36. The method supports the interpolation of a synthesized view form a left sT,l and right sT,r texture with corresponding depth maps sD,l and sD,r. For this, two texture sT,l' and sT,r' are extrapolated from the left and the right view at the position of the virtual view. Subsequently, the similarity of sT,l' and sT,r' is enhanced before combining them to synthesized output view sT'. The single processing steps are discussed in the following. Without the loss of generality steps carried out independently for both, the left and the right view, are discussed for the left view only.

Figure 36: Processing steps of the view synthesis approach.

Similarly as the renderer model used in the encoder control (cp. sec. 2.4.1), the view synthesis algorithm supports two configurations. In the first configuration, which is referred to as interpolative rendering, an intermediate view is synthesized using both surrounding coded views. In the second configuration, which is referred to as non-interpolative rendering, an intermediate view is rendered mainly from one coded view; the other coded view is only used for rendering areas that are not present in the preferred coded view.

### Upsampling of input video pictures

The luma channel of input texture sT,l is upsampled by a factor of four in horizontal direction. Chroma channels are upsampled by a factor of eight in horizontal direction and two in vertical direction. For upsampling, the FIR filters specified in HEVC for the purpose of motion-compensated interpolation are used. The resulting upsampled texture is denoted as sT,l.

### Warping, interpolation and hole filling

Warping, interpolation and hole filling are carried out in a combined step. For warping disparities are computed as described in the beginning of sec. 2. Warping, interpolation and hole filling is carried out line wise and within a line interval wise. Processing direction is from left to right. An interval in the output view is defined by the warped positions x’s and x’eof two neighboring input view samples at positions xsand xe. Subsequently to the calculation of the interval boundaries, processing continues depending on the width of the interval.

- Interpolation is applied if the width of the warped interval x’e– x’s is less than or equal to two times the sampling distance. An interpolation at the full sample position x’FPlocated between the interval boundaries x’sand x’eis carried out. For this, samples from the up-sampled version of the input texture sT,l are mapped to the interpolation positions x’FPin the synthesized view s'T,l. The position xin the up-sampled view is derived from the distance of the interpolation position to the interval boundaries:

|  | x=4⋅xFP'- xs'xe'-xs'+xs | (8) |
| --- | --- | --- |

- Disocclusions: If the width of the warped interval x’e– x’sis greater than two times the width of the sampling distance a disocclusion is assumed in the synthesized view. Instead of interpolation hole filling is carried. For this purpose samples in the interval are set to the value of sample belonging to the right interval boundary sT,l(xe) (which belongs to the background). If the leftmost full sample position within the interval is close to the left interval border it is assumed that it belongs to the foreground and it is set to the value of the left interval boundary sT,l(xs). Disoccluded and filled sample position are stored in the filling map s'F,l.

- Occlusions: If the boundaries of an interval are reversed (x’e< x’s) the interval is occluded in the synthesized view. Rendering at a full sample position close to xe' might be carried out, if the next interval is not occluded and xe' belongs to a foreground object. Moreover, the algorithm uses the property that occluded background intervals are automatically overwritten by foreground objects in the synthesized views'T,l, due to the processing direction from left to right.

Chroma channels of the synthesized view are rendered together with luma channel and stored in the same resolution as luma. Moreover, if interpolative rendering is used, also a depth map sD,l' is extrapolated with full sample accuracy from the input depth map sD,l within the steps described above.

### Reliability map creation

In this step the filling map sF,l' is converted to the reliability map sR,l'. If interpolative rendering is used, positions marked as disocclusions in sF,l' are mapped to a reliability of 0. In areas located right to a disocclusion with a width of six samples the reliability is linearly increased from 0 to 255 from left to right in horizontal direction. All other samples are assigned with a reliability of 255. If non-interpolative rendering is used, positions marked as disocclusions in sF,l' are mapped to a reliability of 0. All other samples are assigned with a reliability of 255.

### Similarity enhancement

In this step the histogram of s'T,l is adapted to the histogram of s'T,r. For this purpose a look up table (LUT) realizing a function f is created, that is subsequently applied to map the samples of s'T,l to adapt their values.

The function f and the corresponding LUT are obtained by approximately solving

|  | hfsT,l'= hsT,r' | (9) |
| --- | --- | --- |

whereh[…] denotes the histogram only regarding samples at positions (x,y)with reliabilities sR,l'x,yand sR,r'x,yof 255. Chroma channels are treated in the same way.

### Combination

s'T,lands'T,r are combined to obtain the synthesized output view in this step.

In the interpolative rendering mode is used, the decision how blending is carried out depends on the reliability maps sR,l'or sR,r' and the rendered depth maps sD,l'and sD,r'. The rules for determining the blended sample value s’T(x,y) from sT,l'(x,y) and sT,r'x,y are given in the following:

- If position (x,y) is disoccluded (reliability of 0) in only one view, the sample value from the other view is used.

- Otherwise, if position (x,y) is disoccluded in both views, the backmost sample value is used.

- Otherwise, if the depth difference retrieved form sD,l'(x,y) and sD,r'(x,y) is above a threshold, the front sample is used.

- Otherwise, if one sample is not reliable with a value of 255, a weighted average with the given reliabilities as weights is used.

- Otherwise, a weighted average of sT,l'(x,y) and sT,r'x,ywith a higher weight for the view that is closer to the virtual view position is used.

If the non-interpolative rendering mode is used, the intermediate view is mainly rendered from one view are utilized and only holes are filled from the other view. Assuming sT,l' is the main view,the rules for determining the sample value s’T(x,y) from sT,l'(x,y) and sT,r'x,y are given in the following:

- If sR,l'x,yis equal to 255 or sR,r'x,y is equal 0, the sample valuesT,l'x,y is used.

- Otherwise, if sR,l'x,y is equal to 0, the sample value sT,r'x,y is used.

- Otherwise, a weighted average with the given reliabilities as weights is used.

### Chroma decimation

To convert the 4:4:4 YUV representation obtained by rendering to the required 4:2:0 output, chroma channels are decimated by a factor of two in horizontal and vertical direction using the FIR filter (1;2;1).

## VSRS (alternative view synthesis algorithm) [Not in CTC]

The VSRS algorithm was developed during the MPEG 3DV Exploration Experiments. VSRS takes two reference views and two depth maps as input to generate a synthesized virtual view. The intrinsic and extrinsic camera parameters are required and 1D parallel and non-parallel camera setups are supported.

The software has two main modes referred to as “General mode” and “1D mode”. The reference views are reprojected to the target viewpoint using pixel-by-pixel mapping based on 3D warping in “General mode”, or horizontal pixel shifting in “1D mode”.

### General mode

In the general mode, virtual views are generated by a technique referred to as “3D warping”. This process involves two steps. At first the original view (reference view) is projected into 3D world space using the corresponding reference depth map. Then the 3D space points are projected into the image plane of the “virtual” view. For this, the intrinsic camera parameters A, and extrinsic camera parameters E=[R|t] are required. The intrinsic matrix A, transforms the 3D camera coordinates to its 2D image coordinates. The extrinsic matrix E=[R|t] transforms the world coordinates to camera coordinates, which is composed of rotation matrix R and translation vector t. The two-step warping can be formulated in two equations as in eq. (10) and (12). First a pixel (ur, vr) in the reference view is warped to the world coordinates (Xw, Yw, Zw), using the depth of the reference view:

(10)

where subscript r indicates the reference view and zr is the depth value in the reference view at location (ur, vr) calculated from

(11)

where v is an 8-bit intensity of the depth map value. It is noted that the values z, Znear, and Zfar are assumed to be either all positive or all negative values.

Then the 3D point is mapped to the virtual view:

(12)

where subscript v refers to the virtual view.

The general mode is based on a "reverse warping" algorithm. Instead of forward warping the left and right reference views to the virtual location, the left and right depth maps are warped to the virtual view location. Then after filtering, these depth maps are used to warp the reference views to the virtual view. This results in a higher rendering quality of the final synthesized view. Figure 37 depicts the flow diagram of the general mode.

Figure 37: Flow diagram for VSRS general mode.

The steps of VSRS general mode are briefly described below:

- First, the two depth maps are mapped to the target viewpoint. E.g. the left reference depth is warped to the virtual view location using eq. (10) and (12). If multiple pixels warp to the same location in the virtual view, then the pixel closest to the camera wins, so foreground pixels will occlude background pixels. The right depth map is also warped in a similar way. We denote these warped depth maps as DL’ and DR’, respectively.

- The mapped depth maps DL’ and DR’ may contain small holes. Small holes which are caused by rounding to integer coordinates are filled by a series of median filtering. Furthermore, binary masks for each side are maintained to indicate larger holes, for example caused by occlusions that remain after filtering. During the following steps, these binary masks are used and updated if necessary (for example during hole filling in step ).

- Next, the left and right texture reference views are mapped to the target viewpoint using the filtered depth map DL’ and DR’. So two texture images at the target viewpoint are obtained, one generated from the left reference view and the other from the right reference view. We denote them here as VL’ and VR’, respectively. Note that DL’ is used to warp the left reference, and DR’ is used to warp the right reference.

- Hole areas in the mapped texture images VL’ and VR’, which are caused by occlusion, are filled by pixels from the other mapped texture image. So holes in VL’ are filled from non-hole areas in VR’ and vice versa.

- Next, these two virtual images are blended. The general mode has two modes of blending: Blending-on and Blending-off. The Blending-on mode is a weighted blending based on the baseline distance. So pixels from the reference camera which is closer to the virtual view are assigned a higher weight, based on the baseline ratio. In Blending-off mode, all pixels visible in the closer reference view are copied to the virtual view, and only hole areas are filled from the farther reference view. During this step, the binary masks are merged to form one mask indicating remaining holes which are inpainted in the next step.

- Any remaining holes after blending are filled by an inpainting algorithm using the binary mask. Inpainting algorithms can be used to reconstruct damaged portions of images. Generally a mask is used to indicate which image regions need to be inpainted. Next, colour information is propagated inward from the region boundaries, i.e., the known image information is used to fill in the missing areas. An inpainting example is show in Figure 38.

Additionally, VSRS contains a Boundary Noise Removal algorithm. In this mode, the binary maps indicating holes caused by occlusion, are used to identify object boundaries. After identifying the background side of the holes based on the depth, the holes are expanded into the background. Then these areas in VL’ and VR’ are filled from the opposite reference view. This reduces noise around object boundaries, where foreground pixels are falsely projected into background objects due to depth errors.

Figure 38: Inpainting: “damaged” image, mask, and result after inpainting.

### 1-d mode

VSRS provides a second synthesis mode other than the general "3D warping" as described above: 1D mode. This mode is implemented with assumptions that the optical axes of camera are in parallel and the views are rectified such that no vertical disparities exist. Under the assumption of 1D mode, formulations can be simpler than in the general case:

- The rotation matrix for every camera is identical to each other.

- The translation vectors of all cameras share the same translation in Y and Z directions, that is, Ty and Tz are constant for every view.

- As a consequence

- Views are corrected (distortion and vertical disparity are null), so vertical position of intersection of optical axis in sensors is constant

So the matrix has the following form , where  and denote the horizontal and vertical focal length in pixels;and  the position of intersection with the optical axis in image ( is constant among cameras).

Then eq. (12) given for the general case can be simplified as,

and	(13)

The equation above is used to “warp” pixels from real views to the virtual one.

Figure 39 depicts the flow diagram of the VSRS 1D-mode.

Figure 39: Flow diagram for VSRS 1D mode.

The algorithm proceeds as follows:

- In a preliminary phase,

  - The chroma components are upsampled to 4:4:4 format (for implementation simplicity).

  - For suppressing transient depth errors, the depth maps can be temporally filtered according to the variations of the colour information if the TemporalImprovementOption is chosen.

  - The colour video may be further upsampled, if sub-pixel precision is specified in the configuration file, for example, half-pixel or quarter-pixel.

- During the warping process, the reference views and the depth maps are mapped to the target viewpoint using eq. (13), which is a 1D shifting on the samples. For each reference view, a binary mask is maintained indicating whether a pixel in the targeted map is filled or not (hole pixel). The warping procedure is also controlled by the splatting switch in configuration file. When splatting is selected, each pixel in the reference view may be mapped to two sample locations. Besides, two enhancement processing on warping (corresponding to CleanNoiseOption and WarpEnhancementOption) suppress some synthesis artefacts due to the texture-depth misalignment at object boundaries (which causes foreground pixels scattered to the background) and wrongly categorized holes in the foreground (which makes background pixels appear in the foreground). Warping of the unreliable pixels (which probably yield artefacts) is forbidden accordingly.

- Two warped images from left and right reference views are obtained from last step, which are then merged to a single image. This operation is also applied on warped depth maps and filling masks. In case of conflicts (two pixels present for the same target position), the MergingOption specified by the user is applied in the following way.

  - Z-buffer only: Take the pixel closest to camera always.

  - Averaging only: Mix colours using weights in reverse proportional to the distance of the virtual camera from the left and right reference views

  - Adaptive merging: Use either the proximity criterion (o) if depth level difference is greater than a threshold or, (o) if depth levels are too similar, uses the weighting method.

- Hole areas in the warped images are filled by propagating the background pixels into the hole along the horizontal row.

- Final view image is downsampled to original size if necessary and transformed to 4:2:0 format for output purposes.

Additionally, VSRS 1D mode can use the boundary noise removal algorithm already described as final processing step in the section dedicated to the general mode.

# Software

## Software repository

The source code for the software will be available in the MPEG SVN repository. An initial version of the software is available in the following SVN repository.

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/

For tool integration a branch for a company can be obtained by contacting:

gerhard.tech@hhi.fraunhofer.de,

kwegner@multimedia.edu.pl

## Build System

The software can be built under linux using make. For Windows, solutions for different versions of Microsoft Visual Studio are provided.

## Software Structure

The 3D-HEVC Test Model Software includes several applications and libraries for encoding, decoding and view synthesis:

- Applications:

  - TAppEncoder, executable for bit stream generation

  - TAppDecoder, executable for reconstruction.

  - TAppRenderer, executable view synthesis

  - TAppExtractor, executable for bitstream extraction

- Libraries:

  - TAppCommon, library for handling encoder, decoder and renderer options and camera parameters

  - TLibEncoder, encoding functionalities

  - TLibDecoder, decoding functionalities

  - TLibRenderer, renderer functionalities

  - TLibExtractor, bitstream extraction functionalities

  - TLibCommon, common functionalities

  - TLibVideoIO, video input/output functionalities

# 3D high efficient video coding text specification draft

The specifications made in this section are based on HEVC text specification draft 8 (JCTVC-J1003_d7) and MV-HEVC draft 1 (JCT3V-B1004_d0).

In text blocks copied from JCTVC-J1003_d7 changes are highlighted.

Modifications of HEVC specification:

In Foreword replace paragraph that start with "In this Recommendation | International Standard Annexes":

In this Recommendation | International Standard Annexes A through G contain normative requirements and are an integral part of this Recommendation | International Standard.

In 0.7, add the following paragraph after the paragraph that starts with "Annex F":

Annex G specifies multiview and depth video coding, referred to as 3D Video Coding (3DVC). The reader is referred to Annex G for the entire decoding process for 3DVC, which is specified there with references being made to clauses 1-9 and Annexes A-F.

- Annex G 	3D high efficient video coding

This annex specifies 3D high efficient video coding, referred to as 3D-HEVC.

Scope

Bitstreams and decoders conforming to the profile specified in this annex are completely specified in this annex with reference made to clauses 2-9 and Annexes A-F.

Normative references

The specifications in clause 2 apply.

Definitions

For the purpose of this annex, the following definitions apply in addition to the definitions in clause F.3. These definitions are either not present in clause F.3 or replace definitions in clause F.3.

depth view: A sequence of depth view components associated with an identical value of view order index.

depth view component: A coded representation of the depth of a view in a single access unit.

texture view: A sequence of texture view components associated with an identical value of view order index.

texture view component: A coded representation of the texture of a view in a single access unit.

view component: A coded representation of a view in a single access unit  A view component may contain a depth view component and a texture view component.

Abbreviations

The specification in clause 4 apply.

Conventions

The specification in clause 5 apply.

Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specification in clause 6 apply.

Syntax and semantics

This clause specifies syntax and semantics for coded video sequences that conform to one or more of the profiles specified in this annex.

Method of specifying syntax in tabular form

The specifications in subclause 7.1 apply.

Specification of syntax functions, categories, and descriptors

The specifications in subclause 7.2 apply.

Syntax in tabular form

NAL unit syntax

General NAL unit syntax

The specifications in subclause 7.3.1.1 apply.

NAL unit header syntax

The specifications in subclause F.7.3.1.2 apply.

Raw byte sequence payloads and RBSP trailing bits syntax

Video parameter set RBSP

The specifications in subclause F.7.3.2.1 apply.

Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| num_additional_layer_operation_points | u(8) |
| num_additional_profile_level_sets_minus1 | u(8) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| reserved_zero_4bits_num_types[ i ] | u(4) |
| reserved_zero_4bits_type[ i ] | u(4) |
| view_id[ i ] | u(8) |
| if ( i>0 ) |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ] && i; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| for( i = 0; i <= num_additional_profile_level_sets_minus1; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| for( i = 1; i <= num_additional_layer_operation_points; i++ ) { |  |
| op_point( i ) |  |
| if (num_additional_profile_level_sets) |  |
| profile_level_idx[ i ] |  |
| } |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| if ( (i ! = 0) && !( i % 2 ) ) { |  |
| multi_view_mv_pred_flag[ i ] | u(1) |
| multi_view_residual_pred_flag[ i ] | u(1) |
| } |  |
| if ( i % 2 ) { |  |
| enable_dmm_flag[ i ] | u(1) |
| use_mvi_flag[ i ] | u(1) |
| lim_qt_pred_flag[ i ] | u(1) |
| dlt_flag[ i ] | u(1) |
| if( dlt_flag[ i ] ) { |  |
| num_depth_values_in_dlt[ i ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt ; j++) { |  |
| dlt_depth_value[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Sequence parameter set RBSP syntax

The specifications in subclause 7.3.2.2 apply

Picture parameter set RBSP syntax

The specifications in subclause 7.3.2.3 apply.

Scaling list data syntax

The specifications in subclause 7.3.2.4 apply.

Scaling list syntax

The specifications in subclause 7.3.2.5 apply.

Supplemental enhancement information RBSP syntax

The specifications in subclause 7.3.2.6 apply.

Access unit delimiter RBSP syntax

The specifications in subclause 7.3.2.7 apply.

End of sequence RBSP syntax

The specifications in subclause 7.3.2.8 apply.

End of bitstream RBSP syntax

The specifications in subclause 7.3.2.9 apply.

Filler data RBSP syntax

The specifications in subclause 7.3.2.10 apply.

Slice layer RBSP syntax

The specifications in subclause 7.3.2.11 apply.

RBSP slice trailing bits syntax

The specifications in subclause 7.3.2.12 apply.

RBSP trailing bits syntax

The specifications in subclause 7.3.2.13 apply.

Byte alignment syntax

The specifications in subclause 7.3.2.14 apply.

Profile, tier and level syntax

The specifications in subclause 7.3.3 apply.

Operation point syntax

The specifications in subclause 7.3.4 apply.

Slice header syntax

General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| else if ( layer_id && !DepthFlag ) |  |
| slice_ic_enable_flag | u(1) |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pic_slice_level_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

Short-term reference picture set syntax

The specifications in subclause 7.3.5.2 apply.

Reference picture list modification syntax

The specifications in subclause 7.3.5.3 apply.

Prediction weight table syntax

The specifications in subclause 7.3.5.4 apply.

Slice data syntax

The specifications in subclause 7.3.6 apply.

Coding tree unit syntax

The specifications in subclause 7.3.7 apply.

Coding quadtree syntax

| coding_quadtree( x0, y0, log2CbSize, ctDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples && y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples && log2CbSize > Log2MinCbSize && NumPCMBlock = = 0 && !MotionInhFlag[ x0 ][ y0 ] && !predSplitCuFlag ) |  |
| split_cu_flag[ x0 ][ y0 ] | ae(v) |
| if( ( cu_qp_delta_enabled_flag && log2CbSize >= Log2MinCuQpDeltaSize ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDelta = 0 |  |
| } |  |
| if( split_cu_flag[ x0 ][ y0 ] ¦ ¦ (MotionInhFlag[ x0 ][ y0 ] && TextureCtDepth[ x0 ][ y0 ] > ctDepth )) { |  |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| coding_quadtree( x0, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_quadtree( x1, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x0, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x1, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| } else { |  |
| if( NumPCMBlock = = 0 ) |  |
| coding_unit( x0, y0, log2CbSize , ctDepth) |  |
| else |  |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } |  |
| } |  |

Coding unit syntax

General Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I && !MotionInhFlag[ x0 ][ y0 ]) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| if( !MotionInhFlag[ x0 ][ y0 ] ) { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode = = MODE_INTRA && DepthFlag ) |  |
| sdc_flag[ x0 ][ y0 ] [Ed. (GT): In software there seems to be a dependency of position of sdc_flag and slice type.] | ae(v) |
| if( sdc_flag[ x0 ][ y0 ] ) { |  |
| sdc_pred_mode | ae(v) |
| if( sdc_pred_mode = = 1 ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| if( sdc_pred_mode = = 2) { |  |
| dmm_delta_end_flag[ x0 ][ y0 ] | ae(v) |
| if( dmm_delta_end_flag[ x0 ][ y0 ] ) { |  |
| dmm_delta_end_abs_minus1[ x0 ][ y0 ] | ae(v) |
| dmm_delta_end_sign[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } else { |  |
| if( ( PredMode[ x0 ][ y0 ] ! = MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) && !predPartModeFlag) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| if ( enable_DMM_flag && log2CbSize <= Log2MaxDmmCbSize ) { |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| dmm_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if (dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| dmm_mode[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULL ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULLDELTA ) ) |  |
| wedge_full_tab_idx[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| dmm_delta_end_flag[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + i ] ) |  |
| dmm_delta_end_abs_minus1[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + i ] ) |  |
| dmm_delta_end_sign_flag[ x0 ][ y0 ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| DmmDeltaFlag[ x0 + i ][ y0 + i ] = ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WFULLDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WPREDDIRDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WPREDTEXDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_CPREDTEXDELTA ) ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0 + i ] ) |  |
| dmm_dc_1_abs[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_1_abs[ x0 + i ][ y0 + i ] != 0 ) |  |
| dmm_dc_1_sign_flag[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] ) |  |
| dmm_dc_2_abs[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_2_abs[ x0 + i ][ y0 + i ] != 0 ) |  |
| dmm_dc_2_sign_flag[ x0 + i ][ y0 + i ] | ae(v) |
| } else { |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else { |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| if( rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] = = 31 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) <= 5 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) >= 2 ) { |  |
| edge_intra_flag[ x0 + i ][ y0 + j ] | ae(v) |
| if( edge_intra_flag[ x0 + i ][ y0+ j ] ) { |  |
| edge_start_left_flag[ x0 + i ][ y0+ j ] | ae(v) |
| edge_start_position[ x0 + i ][ y0+ j ] | ae(v) |
| num_edge_codes_minus1[ x0 + i ][ y0+ j ] | ae(v) |
| for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| edge_code[ k ] | ae(v) |
| edge_dc_flag[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_flag[ x0 + i ][ y0+ j ] ) { |  |
| edge_dc_1_abs[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_1_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| edge_dc_1_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| edge_dc_2_abs[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_2_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| edge_dc_2_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if ( resPredEnableFlag ) |  |
| res_pred_flag | ae(v) |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| if ( MotionInhFlag[ x0 ][ y0 ] && TextureCtDepth[ x0 ][ y0 ] > ctDepth) { |  |
| coding_tree( x0, y0, log2CbSize, ctDepth ) |  |
| } else { |  |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag && !sdc_flag) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| if( sdc_flag ) { |  |
| sdcNumSegments = ( sdc_pred_mode = = 1 ¦ ¦ sdc_pred_mode = = 2 ) ? 2 : 1 ) |  |
| for ( i = 0; i < sdcNumSegments ; i++) { |  |
| sdc_residual_flag[ x0 ][ y0 ][ i | ae(v) |
| if( sdc_residual_flag [ x0 ][ y0 ][ i ]) { |  |
| sdc_residual_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

[Ed. (GT): DMM syntax differs from syntax used in SW, since intra prediction syntax elements moved from PU to CU in HEVC text specification draft 8]

PCM sample syntax

The specifications in subclause 7.3.9.2 apply.

Prediction unit syntax

General prediction unit syntax

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0 ) |  |
| mvp_l0_idxflag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( x0, y0, 1 ) |  |
| mvp_l1_idxflag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Motion vector difference coding syntax

The specifications in subclause 7.3.10.2 apply.

Transform tree syntax

The specifications in subclause F.7.3.11 apply.

Transform unit syntax

The specifications in subclause F.7.3.12 apply.

Residual coding syntax

The specifications in subclause F.7.3.13 apply.

Semantics

NAL unit semantics

General NAL unit semantics

The specifications in subclause F.7.4.1.1  apply.

NAL unit header semantics

The specification in subclause F.7.4.1.2 apply with the following modifications and additions.

layer_id identifies the layer the NAL unit belongs to. layer_id shall be equal to 0 for any NAL unit of the texture view components of the base view. layer_id shall be equal to 1 for any NAL unit of the depth view components of the base view.

The view order index ViewIdx and DepthFlag of each NAL unit are computed as

ViewIdx = layer_id >> 1		(G-14)

DepthFlag = layer_id % 2		(G-15)

Encapsulation of an SODB within an RBSP (informative)

The specifications in subclause 7.4.1.3 apply.

Order of NAL units and association to coded pictures, access units, and video sequences

The specifications in subclause 7.4.1.4 apply.

Raw byte sequence payloads and RBSP trailing bits semantics

Video parameter set RBSP semantics

The specifications in subclause F.7.4.2.1 apply.

Video parameter set extension semantics

The specifications in subclause F.7.4.2.1.1 apply with the following modifications and additions.

multi_view_mv_pred_flag[ i ] indicates whether inter-view motion parameter prediction is used in the decoding process of the layer with layer_id equal to i. multi_view_mv_pred_flag[ i ] equal to 0 specifies that inter-view motion parameter prediction is not used for the layer with layer_id equal to i. multi_view_mv_pred_flag[ i ] equal to 1 specifies that inter-view motion parameter prediction may be used for the layer with layer_id equal to i.

When multi_view_mv_pred_flag is not present, its value shall be inferred to be equal to 0.

multi_view_residual_pred_flag[ i ] indicates whether inter-view residual prediction is used in the decoding process of the layer with layer_id equal to i. multi_view_residual_pred_flag[ i ] equal to 0 specifies that inter-view residual prediction is not used for the layer with layer_id equal to i. multi_view_residual_pred_flag[ i ] equal to 1 specifies that inter-view residual prediction may be used for the layer with layer_id equal to i. When multi_view_residual_pred_flag[ i ] is not present, its value shall be inferred to be equal to 0.

enable_dmm_flag[ i ] equal to 1 specifies that depth map modelling modes may be in the decoding process of the layer with layer_id equal to i. enable_dmm_flag[ i ] equal to 0 specifies that depth map modelling modes are not used in the decoding process of the layer with layer_id equal to i.

use_mvi_flag[ i ] equal to 1 specifies that motion parameter inheritance may be used for depth view components with layer_id equal to i. use_mvi_flag equal to 0 specifies that motion parameter inheritance is not used for the depth view components with layer_id equal to i. If use_mvi_flag[ i ] is not present, it shall be inferred to be equal to 0.

lim_qt_pred_flag[ i ] equal to 1 specifies that prediction of a limited quadtree is used for depth view components for depth view components with layer_id equal to i.. lim_qt_pred_flag[ i ] equal to 0 specifies that prediction of a limited quadtree is not used for the depth view components with layer_id equal to i.. When lim_qt_pred_flag[ i ] is not present, it shall be inferred to be equal to 0.

dlt_flag[ i ] equal to 1 specifies that depth lookup table is used and that residual values for simplified depth coded coding units are to be interpreted as indices of the depth lookup table for depth view components with layer_id equal to i. dlt_flag[ i ] equal to 0 specifies that depth lookup table is not used and residual values for simplified depth coded coding units are not to be interpreted as indices for depth view components with layer_id equal to i. When dlt_flat[ i ] is not present, it shall be inferred to be equal to 0.

num_depth_values_in_dlt[ i ] specifies the number of different depth values and the number of elements in the depth lookup table for depth view components of the current layer with layer_id equal to i.

dlt_depth_value[ i ][ j ] specifies the j-th entry in the depth lookup table for depth view components with layer_id equal to i.

[Ed. (GT): Whether 3D-tool related flags should be signalled in the SPS or VPS requires further discussion. In the HTM-4.0 depth tool related flags are signalled in the SPS.]

Sequence parameter set RBSP semantics

The specifications in subclause 7.4.2.2 apply.

Picture parameter set RBSP semantics

The specifications in subclause 7.4.2.3 apply.

Scaling list data semantics

The specifications in subclause 7.4.2.4 apply.

Scaling list semantics

The specifications in subclause 7.4.2.5 apply.

Supplemental enhancement information RBSP semantics

General supplemental enhancement information semantics

The specifications in subclause 7.4.2.6.1 apply.

Supplemental enhancement information message semantics

The specifications in subclause 7.4.2.6.2 apply.

Access unit delimiter RBSP semantics

The specifications in subclause 7.4.2.7 apply.

End of sequence RBSP semantics

The specifications in subclause 7.4.2.8 apply.

End of bitstream RBSP semantics

The specifications in subclause 7.4.2.9 apply.

Filler data RBSP semantics

The specifications in subclause 7.4.2.10 apply.

Slice layer RBSP semantics

The specifications in subclause 7.4.2.11 apply.

RBSP slice trailing bits semantics

The specifications in subclause 7.4.2.12 apply.

RBSP trailing bits semantics

The specifications in subclause 7.4.2.13 apply.

Byte alignment semantics

The specifications in subclause 7.4.2.14 apply.

Profile, tier and level semantics

The specifications in subclause 7.4.3 apply.

Operation point semantics

The specifications in subclause 7.4.4 apply.

Slice header semantics

General slice header semantics

The specification in subclause 7.4.5.1 apply with the following modifications and additions.

five_minus_max_num_merge_cand specifies the maximum number of merging MVP candidates supported in the slice subtracted from 5. The maximum number of merging MVP candidates, MaxNumMergeCand is computed as

MaxNumMergeCand = 5 − five_minus_max_num_merge_cand + multi_view_mv_pred_flag	(G-16)

The value of five_minus_max_num_merge_cand shall be limited such that MaxNumMergeCand is in the range of 0 to (5 + multi_view_mv_pred_flag), inclusive.

slice_ic_enable_flag equal to 1 specifies illumination compensation is enabled for the current slice. slice_ic_enable_flag equal to 0 specifies that illumination compensation is disabled for the current slice, When not present, slice_ic_enable_flag is inferred to be equal to 0.

Short-term reference picture set semantics

The specifications in subclause 7.4.5.2 apply.

Reference picture list modification semantics

The specifications in subclause 7.4.5.3 apply.

Weighted prediction parameters semantics

The specifications in subclause 7.4.5.4 apply.

Slice data semantics

The specifications in subclause 7.4.6 apply.

Coding tree unit semantics

The specifications in subclause 7.4.7 apply, with the following modifications and additions.

The array TextureCtDepth is set equal to the array CtDepth of the view component with view order index equal to ViewIdx of the current slice and DepthFlag equal to 0.

The array TexturePartMode is set equal to the array CtPartMode of the view component with view order index equal to ViewIdx of the current slice and DepthFlag equal to 0.

The elements of the array MotionInhFlag of size (pic_width_in_luma_samples)x(pic_height_in_luma_samples) are initially set equal to 0.

Coding quadtree semantics

The specifications in subclause 7.4.8 apply.

[Ed. (GT): Numbering of syntax and semantics in JCTVC-J1003_d7 do not match from subclause 7.4.8 on. However, the "correct" numbering of syntax is used here assuming that JCTVC-J1003_d7 would match.]

The variable predSplitCuFlag specifying whether the split_cu_flag is predicted by inter-component prediction is derived as specified in the following.

If slice_type is not equal to I and lim_qt_pred_flag is equal to 1, predSplitCuFlag is set equal to TextureCtDepth[ x0 ][ y0 ] <= ctDepth

Otherwise (slice_type is equal to I or lim_qt_pred_flag is equal to 0), predSplitCuFlag is set equal to 0.

split_cu_flag[ x0 ][ y0 ] specifies whether a coding unit is split into coding units with half horizontal and vertical size. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When split_cu_flag[ x0 ][ y0 ] is not present the following applies.

If log2CbSize is greater than Log2MinCbSizeY and predSplitCuFlag is equal to 0 and MotionInhFlag[ x0 ][ y0 ] is equal to 0, the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 1.

Otherwise (log2CbSize is equal to Log2MinCbSizeY or predSplitCuFlag is equal to 1 or MotionInhFlag[ x0 ][ y0 ] is equal to 1), the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 0.

Coding unit semantics

General coding unit semantics

The specification in subclause F.7.4.9.1 apply with the following modifications and additions.

If slice_type is not equal to I and lim_qt_pred_flag is equal to 1, the variable predPartModeFlag specifying whether part_mode is predicted by inter-component prediction is derived as follows.

predPartModeFlag = ( TextureCtDepth[ x0 ][ y0 ] = = ctDepth ) && ( TexturePartMode[ x0 ][ y0 ] ! = PART_NxN )	(G-17)

Otherwise (slice_type is equal to I or lim_qt_pred_flag is equal to 0), predPartModeFlag is set equal to 0.

pred_mode_flag equal to 0 specifies that the current coding unit is coded in inter prediction mode. pred_mode_flag equal to 1 specifies that the current coding unit is coded in intra prediction mode. The variable PredMode[ x ][ y ] is derived as follows for x = x0..x0 + nCbS − 1 and y = y0..y0 + nCbS − 1.

If pred_mode_flag is equal to 0,

PredMode[ x ][ y ] is set to MODE_INTER.

Otherwise (pred_mode_flag is equal to 1),

PredMode[ x ][ y ] is set to MODE_INTRA.

When pred_mode_flag is not present, the variable PredMode[ x ][ y ] is derived as follows for x = x0..x0 + nCbS − 1 and y = y0..y0 + nCbS − 1.

If slice_type is equal to I,

PredMode[ x ][ y ] is inferred to be equal to MODE_INTRA

Otherwise (slice_type is equal to P or B),

If MotionInhFlag[ x ][ y ] is equal to 1, PredMode[ x ][ y ] is inferred to be equal to MODE_INTER

Otherwise, if skip_flag[ x0 ][ y0 ] is equal to 1,

PredMode[ x ][ y ] is inferred to be equal to MODE_SKIP

part_mode specifies partitioning mode of the current coding unit. The semantics of part_mode depend on PredMode[ x0 ][ y0 ]. The variables PartMode and IntraSplitFlag are derived from the value of part_mode as defined in Table 7-10.

The value of part_mode is restricted as follows.

If PredMode[ x0 ][ y0 ] is equal to MODE_INTRA, part_mode shall be equal to 0 or 1.

Otherwise (PredMode[ x0 ][ y0 ] is equal to MODE_INTER), the following applies

If log2CbSize is greater than Log2MinCbSize and amp_enabled_flag is equal to 1, part_mode shall be in the range of 0 to 2, inclusive and in the range of 4 to 7, inclusive.

Otherwise, if log2CbSize is greater than Log2MinCbSize and amp_enabled_flag is equal to 0, part_mode shall be in the range of 0 to 2, inclusive.

Otherwise, if log2CbSize is equal to 3, the value of part_mode shall be in the range of 0 to 2, inclusive.

Otherwise (log2CbSize is greater than 3), the value of part_mode shall be in the range of 0 to 3, inclusive.

When part_mode is not present, the variables PartMode and IntraSplitFlag are derived as follows.

If MotionInhFlag[ x ][ y ] is equal to 1, PartMode is inferred to be equal to PART_NxN, otherwise (MotionInhFlag[ x ][ y ] is equal to 0), PartMode is inferred to be equal to PART_2Nx2N,

IntraSplitFlag is inferred to be equal to 0.

The variable Log2MaxDmmCbSize is set equal to 5.

dmm_flag[ x0 ][ y0 ] equal to 0 specifies that depth map model modes are not used. dmm_flag[ x0 ][ y0 ] equal to 1 specifies that depth map model modes are used that the dmm_mode syntax element is present. When the dmm_flag[ x0 ][ y0 ] is not present, its value should be inferred to be equal to 0.

dmm_mode[ x0 ][ y0 ] shall be one of the values shown in Table G-1. dmm_mode is used to derive the intra prediction mode when the dmm_flag[ x0 ][ y0 ] is equal to 1.

Table G-1 – Interpretation of dmm_mode

| dmm_mode | Method identifier |
| --- | --- |
| 0 | MODE_DMM_WFULL |
| 1 | MODE_DMM_WFULLDELTA |
| 2 | MODE_DMM_WPREDTEX |
| 3 | MODE_DMM_WPREDTEXDELTA |
| 4 | MODE_DMM_CPREDTEX |
| 5 | MODE_DMM_CPREDTEXDELTA |
| 6 | MODE_DMM_WPREDDIR |
| 7 | MODE_DMM_WPREDDIRDELTA |

wedge_full_tab_idx[ x0 ][ y0 ] specifies the index of the wedgelet pattern in the corresponding pattern list.

dmm_delta_end_flag[ x0 ][ y0 ] equal to 0 specifies that dmm_delta_end_abs_minus1[ x0 ][ y0 ] and dmm_delta_end_sign[ x0 ][ y0 ] syntax elements are not present. dmm_delta_end_flag equal to 1 specifies that dmm_delta_end_abs_minus1[ x0 ][ y0 ] and dmm_delta_end_sign[ x0 ][ y0 ] syntax elements are present.

dmm_delta_end_abs_minus1[ x0 ][ y0 ] and dmm_delta_end_sign_flag[ x0 ][ y0 ] are used to derive DmmDeltaEnd[ x0 ][ y0 ] as follows:

DmmDeltaEnd[ x0 ][ y0 ] = 
( 1 − 2 *dmm_delta_end_sign_flag[ x0 ][ y0 ] ) *	( dmm_delta_end_abs_minus1[ x0 ][ y0 ] + 1)	(G-18)

dmm_dc_0_abs[ x0 ][ y0 ], dmm_dc_0_sign_flag[ x0 ][ y0 ], dmm_dc_1_abs[ x0 ][ y0 ], dmm_dc_1_sign_flag[ x0 ][ y0 ] are used to derive QuantDCOffsetP0[ x0 ][ y0 ] and QuantDCOffsetP1[ x0 ][ y0 ] values as follows:

QuantDCOffsetP0[ x0 ][ y0 ]  = ( 1 − 2 *dmm_dc_0_sign_flag[ x0 ][ y0 ] ) * dmm_dc_0_abs[ x0 ][ y0 ]	(G-19)
QuantDCOffsetP1[ x0 ][ y0 ]  = ( 1 − 2 *dmm_dc_1_sign_flag[ x0 ][ y0 ] ) * dmm_dc_1_abs[ x0 ][ y0 ]	(G-20)

edge_intra_flag[ x0 ][ y0 ] equal to 0 specifies that region boundary chain coding is not used. edge_intra_flag[ x0 ][ y0 ] equal to 1 specifies that region boundary chain coding is used.

edge_start_left_flag[ x0 ][ y0 ] equal to 0 specifies that the start point of region boundary chain coding is located on the top row boundary of the current block. edge_start_left_flag[ x0 ][ y0 ] equal to 1 specifies that the start point of region boundary chain coding is located on the left column boundary of the current block.

edge_start_position[ x0 ][ y0 ] specifies the column position of the start point of region boundary chain coding when edge_start_left_flag[ x0 ][ y0 ] is equal to 0 and specifies the row position of the start point of region boundary chain coding when edge_start_left_flag[ x0 ][ y0 ] is equal to 1.

num_edge_codes_minus1[ x0 ][ y0 ] +1 specifies the number of edges within the current block.

edge_code[ i ] shall be one of the values shown in Table G-2. edge_code[ i ] is used to derive the edge direction of the i-th edge when edge_intra_flag[ x0 ][ y0 ] is equal to 1.

[Ed. (GT): Binarization in Table G-2 should be removed and TU binarization should be specified in the parsing process.]

Table G-2 – Interpretation of edge_code

| Bin string of edge_code[i] | edge direction |
| --- | --- |
| 0 | 0° |
| 10 | 45° |
| 110 | -45° |
| 1110 | 90° |
| 11110 | -90° |
| 111110 | 135° |
| 111111 | -135° |

edge_dc_flag[ x0 ][ y0 ] equal to 0 specifies that de-quantized offset is not used. edge_dc_flag[ x0 ][ y0 ] equal to 1 specifies that de-quantized offset is used.

edge_dc_0_abs[ x0 ][ y0 ], edge_dc_0_sign_flag[ x0 ][ y0 ], edge_dc_1_abs[ x0 ][ y0 ], edge_dc_1_sign_flag[ x0 ][ y0 ] are used to derive QuantDCOffsetP0[ x0 ][ y0 ] and QuantDCOffsetP1[ x0 ][ y0 ] values as follows.

QuantDCOffsetP0[ x0 ][ y0 ] = ( 1 − 2 *edge_dc_0_sign_flag[ x0 ][ y0 ]) * edge_dc_1_abs[ x0 ][ y0 ]	(G-21)
QuantDCOffsetP1[ x0 ][ y0 ] = ( 1 − 2 *edge_dc_0_sign_flag[ x0 ][ y0 ]) * edge_dc_1_abs[ x0 ][ y0 ]	(G-22)

The variables x1 and y1 specifying luma locations relative to the top left sample of the current picture are derived as specified in the following.

x1 = x0 + 		
( ( PartMode = = PART_NxN | | PartMode = = PART_Nx2N) ? ( nCbs / 2) : 0 ) + 	
( ( PartMode = = PART_nLx2N ) ? ( nCbs / 4) : 0 ) + 		
( ( PartMode = = PART_nRx2N ) ? ( 3* nCbs / 4) : 0 ) 		(G-23)

y1 = y0 + 		
( ( PartMode = = PART_NxN | | PartMode = = PART_2NxN) ? ( nCbs / 2)  : 0 ) + 	
( ( PartMode = = PART_2NxnU ) ? ( nCbs / 4) : 0 ) + 		
( ( PartMode = = PART_2NxnD ) ? ( 3* nCbs / 4) : 0 ) 		(G-24)

With X being replaced by 0 and 1, the variable refViewIdxLX[ x ][ y ] is set equal to the view order index of RefPicListLX[ RefIdxLX[ x ][ y ] ]..

The variable anyTempRefPicFlag specifies whether one or more prediction units of the current coding unit utilize a temporal reference picture. The variable anyTempRefPicFlag is initially set equal to 0. When PredMode[ x0 ][ y0 ] is not equal to MODE_INTRA the following applies for X being replaced by 0 and 1, and Y being equal to 1−X.

anyTempRefPicFlag = anyTempRefPicFlag | |  	
(inter_pred_idc[ x0 ][ y0 ] ! = Pred_LY && refViewIdxLX[ x0 ][ y0 ] = = ViewIdx)  | |	 
(inter_pred_idc[ x0 ][ y1 ] ! = Pred_LY && refViewIdxLX[ x0 ][ y1 ] = = ViewIdx)  | |	
(inter_pred_idc[ x1 ][ y0 ] ! = Pred_LY && refViewIdxLX[ x1 ][ y0 ] = = ViewIdx)  | |	 
(inter_pred_idc[ x1 ][ y1 ] ! = Pred_LY && refViewIdxLX[ x1 ][ y1 ] = = ViewIdx) 	  (G-25)

The variable anyIvRefPicFlag specifies whether one or more prediction units of the current coding unit utilize an inter-view reference picture. The variable anyIvRefPicFlag is initially set equal to 0. When PredMode[ x0 ][ y0 ] is not equal to MODE_INTRA the following applies for X being replaced by 0 and 1, and Y being equal to 1−X.

anyIvRefPicFlag = anyIvRefPicFlag | |  	
(inter_pred_idc[ x0 ][ y0 ] ! = Pred_LY && refViewIdxLX[ x0 ][ y0 ] ! = ViewIdx)  | |	 
(inter_pred_idc[ x0 ][ y1 ] ! = Pred_LY && refViewIdxLX[ x0 ][ y1 ] ! = ViewIdx)  | |	
(inter_pred_idc[ x1 ][ y0 ] ! = Pred_LY && refViewIdxLX[ x1 ][ y0 ] ! = ViewIdx)  | |	 
(inter_pred_idc[ x1 ][ y1 ] ! = Pred_LY && refViewIdxLX[ x1 ][ y1 ] ! = ViewIdx) 	  (G-26)

ic_flag equal to 1 specifies illumination compensation is used for the current coding unit. ic_flag equal to 0 specifies illumination compensation is not used for the current coding unit. When not present, ic_flag is inferred to be equal to 0.

The variable icEnableFlag specifying whether ic_flag is present in the bitstream is derived as

icEnableFlag = slice_ic_enable_flag &&  anyIvRefPicFlag	(G-27)

res_pred_flag equal to 0 specifies that residual prediction is not used. res_pred_flag equal to 1 specifies that residual prediction is used. When res_pred_flag is not present, its value shall be inferred to be equal to 0.

[Ed. (GT). The following paragraph needs to be specified more precisely. ]

The variable residualCbfNonZero is derived by values of cbf_luma, cbf_cb, cbf_cr and PredMode of the corresponding blocks as follows: residualCbfNonZero is set to 1 if at least one of the corresponding blocks has both PredMode not equal to MODE_INTRA and any of the values of cbf_luma, cbf_cb and cbf_cr not equal to 0; otherwise, residualCbfNonZero is set equal to 0. The corresponding blocks are identified by the current PU and the disparity vector. All the corresponding blocks belong to transform units that are covered or partially covered by a corresponding rectangle area (of the current PU) in the inter-view reference view component, after shifting the PU location with a disparity vector.

The variable resPredEnableFlag specifying whether res_pred_flag is present in the bitstream is derived as

resPredEnableFlag = multi_view_residual_pred_flag && residualCbfNonZero && anyTempRefPicFlag	(G-28)

sdc_flag equal to 0 specifies that simplified depth coding is not used. sdc_flag equal to 1 specifies that simplified depth coding is used. When sdc_flag is not present, it is inferred to be equal to 0.

sdc_pred_mode shall be one of the values shown in Table G-3. sdc_pred_mode specifies the intra prediction mode used for simplified depth coding.

Table G-3 – Interpretation of sdc_pred_mode

| sdc_pred_mode | Associated Intra Prediction Mode |
| --- | --- |
| 0 | Intra_DC |
| 1 | Intra_DepthPartition( 35 ) |
| 2 | Intra_DepthPartition( 41 ) |
| 3 | Intra_Planar |

sdc_residual_flag[ x0 ][ y0 ][ i ] equal to 0 specifies that the residual is zero for segment i. sdc_residual_flag equal to 1 specifies that the residual is non-zero and the sdc_residual_sign_flag and sdc_residual_abs[ i ] syntax elements are present for segment i.

sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] and sdc_residual_sign_flag[ x0 ][ y0 ][ i ] are used to derive SdcResidual[ x0 ][ y0 ][ i ] for segment i as follows:

SdcResidual[ x0 ][ y0 ][ i ] = 
( 1 − 2 * sdc_residual_sign_flag[ x0 ][ y0 ][ i ] ) * ( sdc_residual_mag_minus1[ x0 ][ y0 ][ i ] + 1) 	(G-29)

When DepthFlag is equal to 0, for use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = x0..x0 + (1 << log2cbSize) − 1, y = y0..y0 + (1 << log2cbSize) − 1:

CtDepth[ x ][ y ] = ctDepth		(G-30)
CtPartMode[ x ][ y ] = PartMode		(G-31)

PCM sample semantics

The specifications in subclause 7.4.9.2 apply.

Prediction unit semantics

General prediction unit semantics

The specifications in subclause 7.4.10.1 apply, apply with the following modifications and additions

mvp_l0_idx[ x0 ][ y0 ] specifies the motion vector predictor index of list 0 where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When mvp_l0_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

mvp_l1_idx[ x0 ][ y0 ] has the same semantics as mvp_l0_idx, with l0 and list 0 replaced by l1 and list 1, respectively.

merge_idx[ x0 ][ y0 ] specifies the merging candidate index of the merging candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. When merge_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

When merge_idx is equal to 0 and use_mvi_flag is equal to 1, for use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = x0..x0 + nPbW − 1, y = y0..y0 + nPbH − 1:

MotionInhFlag[ x ][ y ] = 1		(G-32)

[Ed. (GT). It assumed that PartMode of CUs signalling MPI should be 2Nx2N. Hence MotionInhFlag is set equal to 1 for the whole CU].

Motion vector difference coding semantics

The specifications in subclause 7.4.10.2 apply.

Transform tree semantics

The specifications in subclause 7.4.11 apply.

Transform coefficient semantics

The specifications in subclause 7.4.12 apply.

Residual coding semantics

The specifications in subclause 7.4.13 apply.

Decoding process

General decoding process

The specifications in subclause F.8.1 apply with the following modification.

All invocations of the process specified in subclause F.8.3.2 are replaced with invocations of the process specified in subclause G.8.3.2.

All invocations of the process specified in subclause F.8.3.3.1 are replaced with invocations of the process specified in subclause G.8.3.3.1.

All invocations of the process specified in subclause F.8.3.4.1 are replaced with invocations of the process specified in subclause G.8.3.4.1

All invocations of the process specified in subclause 8.4 are replaced with invocations of the process specified in subclause G.8.3.6

All invocations of the process specified in subclause 8.5 are replaced with invocations of the process specified in subclause G.8.4.4.3

When multi_view_mv_pred_flag[ layer_id ] is equal to 1 or res_pred_flag[ layer_id ] is equal to 1, the decoding process for candidate picture list for disparity vector derivation in subclause G.8.3.5 is invoked at the beginning of the decoding process for each P or B slice

When dlt_flag is equal to 1, the decoding process for the depth lookup table in subclause G.8.3.6 is invoked at the beginning of the decoding process of first slice.

[Ed. (GT): The current Annex F uses view_order_idx instead of layer_id. This might be changed, since in Annex G ViewIdx is the same for a depth and a view component. ]

NAL unit decoding process

The specifications in subclause 8.2 apply.

Slice decoding process

The specifications in subclause 8.3 apply with the following modification.

–	All invocations of the process specified in subclause 8.3.2 are replaced with invocations of the process specified in subclause G.8.3.2.

–	All invocations of the process specified in subclause 8.3.4 are replaced with invocations of the process specified in subclause G.8.3.3.1.

Decoding process for picture order count

The specifications in subclause 8.3.1 apply.

Decoding process for reference picture set

The specification in subclause F.8.3.2 apply.

[Ed. (GT): The current Annex F uses view_order_idx instead of layer_id. This might be changed, since in Annex G ViewIdx is the same for a depth and a view component. ]

Decoding process for generating unavailable reference pictures

General decoding process for generating unavailable reference pictures

The specifications in subclause 8.3.3.1 apply with the following modification.

–	All references to the process specified in subclause 8.3.2 are replaced with references to the process specified in subclause G.8.3.2.

Generation of one unavailable picture

Decoding process for reference picture lists construction

General decoding process for reference picture lists construction

The specifications in subclause 8.3.4.1 apply with the following modification:

–	All references to the process specified in subclause 8.3.2 are replaced with references to the process specified in subclause G.8.3.2

–	All invocations of the process specified in subclause 8.3.4.2 are replaced with invocations of the process specified in subclause G.8.3.4.2.

Initialization process for reference picture lists

The specifications in subclause F.8.3.4.2 apply.

Derivation process for the candidate picture list for disparity vector derivation

[Ed. (GT): This algorithm is different from the algorithm in software. However, it seems to be equivalent. ]

The variable NumDdvCandPics is set equal to 0 and the candidate picture list DdvCandPicList with a number of NumDdvCandPics elements is constructed as follows.

When slice_temporal_mvp_enable_flag is equal to 1 the following ordered steps apply:

- DdvCandPicList[ 0 ] is set equal to RefPicListX[ collocated_ref_idx ], with X equal to 1-collocated_from_l0_flag, and NumDdvCandPics  is set equal to 1.

- The variable lowestTemporalIDRefs is set equal to 7.

- NumDdvCandPics, DdvCandPicList[ 1 ] and lowestTemporalIDRefs are derived as specified in the following.

for ( dir = 0; dir < 2 ; dir++) {
	X = dir ? collocated_from_l0_flag : (1- collocated_from_l0_flag)
	for ( i =0 ; i <= num_ref_idx_lX_default_active_minus1; i++) {
		if (	 ViewIdx of the current view component is equal to ViewIdx of RefPicListX[ i ] 
			&& (X = = collocated_from_l0_flag || i != collocated_ref_idx ) 
			&& ( NumDdvCandPics ! = 2)) {
			if (RefPicListX[ i ] is a random access view component) 	{
				DdvCandPicsList[ 1 ] = RefPicListX[ i ]
				NumDdvCandPics =2
			}
			else if ( lowestTemporalIDRefs > TemporalID of RefPicListX[ i ])
				lowestTemporalIDRefs = TemporalID of RefPicListX[ i ]
		}
	}
}

- When NumDdvCandPics is equal to 1, the following applies.

pocDistance =255
for ( dir = 0; dir < 2 ; dir++) {
	X = dir ? collocated_from_l0_flag : (1- collocated_from_l0_flag)
	for ( i =0 ; i <= num_ref_idx_lX_default_active_minus1; i++) {
		if (	ViewIdx of the current view component is equal to ViewIdx of RefPicListX[ i ]
			&& (X = = collocated_from_l0_flag || i != collocated_ref_idx )
			&& TemporalID of RefPicListX[ i ] = = lowestTemporalIDRefs 
			&& (Abs( PicOrderCntVal – PicOrderCnt( RefPicListX[ i ] ) ) < pocDistance) ){
			pocDistance = Abs( PicOrderCntVal – PicOrderCnt( RefPicListX[ i ] )
			Z = X
			idx = i
		}
 	}
}
if (pocDistance < 255) {
	DdvCandPicsList[ 1 ] = RefPicListZ[ idx ]
	NumDdvCandPics =2
}

Decoding process for a depth lookup table

This process is only invoked when dlt_flag is equal to 1.

The list elements Idx2DepthValue[ i ] specifying the depth value of the i-th index in the lookup table with i ranging from 0 to  num_depth_values_in_dlt – 1, inclusive is derived as follows.

For i = 0..num_depth_values_in_dlt –1 the elements in Idx2DepthValue are derived as follows.

Idx2DepthValue[ i ] is set equal to dlt_depth_value[ i ]

The list elements DepthValue2Idx[ d ] specifying the index of depth values d in the lookup table with d ranging from 0 to BitDepthY – 1, inclusive are derived as specified in the following.

for (d = 0; d < BitDepthY; d++) {
		idxLower = 0 
		for ( iL = 1, found = 0; iL < num_depth_values_in_dlt && !found; iL++ ) 
			if ( Idx2DepthValue[ iL ] > d ) {
				idxLower = iL – 1
				found = 1
			}
		idxUpper = num_depth_values_in_dlt – 1
		for ( iU = num_depth_values_in_dlt – 2, found = 0; iU >= 0 && !found; iU++ ) 
			if ( Idx2DepthValue[ iU ] < d ) {
				idxUpper = iU + 1
				found = 1
			}
		if ( Abs( d – Idx2DepthValue[ idxLower ] ) < Abs ( d – Idx2DepthValue[ idxUpper ] ) ) 
			DepthValue2Idx[ d ] = idxLower
		else
			DepthValue2Idx[ d ] = idxUpper

Decoding process for coding units coded in intra prediction mode

General decoding process for coding units coded in intra prediction mode

The specifications in subclause 8.4.1 apply with the following modification:

–	All invocations of the process specified in subclause 8.4.2 are replaced with invocations of the process specified in subclause G.8.4.2

–	All invocations of the process specified in subclause 8.4.4.1 are replaced with invocations of the process specified in subclause G.8.4.4

Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PbSize specifying the size of the current luma prediction block.

Table G-4 specifies the value for the intra prediction mode and the associated names.

Table G-4 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_DC |
| 2..34 | Intra_Angular |
| 35...42 | Intra_DepthPartition (used only for depth) |
| Otherwise (43, 44) | Intra_Chain (used only for depth) |

[Ed. (GT): Since the Intra_FromLuma mode has been removed in the HEVC text spec used as base for this document, the dmm mode and intra chain numbers are here decremented by 1 compared to SW.]

IntraPredMode[ xB ][ yB ] labelled 0..34 represents directions of predictions as illustrated in Figure 8-1.

If sdc_flag[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is derived as follows.

If sdc_pred_mode[ xB ][ yB ] is equal to 0, IntraPredMode[ xB ][ yB ] is set equal to 1.

Otherwise, if sdc_pred_mode[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is set equal to 35.

Otherwise, if  sdc_pred_mode[ xB ][ yB ] is equal to 2, IntraPredMode[ xB ][ yB ] is set equal to 41.

Otherwise, if sdc_pred_mode [ xB ][ yB ] is equal to 3, IntraPredMode[ xB ][ yB ] is set equal to 0.

Otherwise, if dmm_flag[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is derived as follows:

IntraPredMode[ xB ][ yB ] = dmm_mode[ xB ][ yB ] + 35	(G-33)

Otherwise (dmm_flag[ xB ][ yB ] is equal to 0 and sdc_flag[ xB ][ yB ] is equal to 0), IntraPredMode[ xB ][ yB ] is derived as the following ordered steps.

The neighbouring locations ( xBA, yBA ) and ( xBB, yBB ) are set equal to ( xB−1, yB ) and ( xB, yB−1 ), respectively.

For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

The availability derivation process for a block in z-scan order as specified in subclause 6.4.1 is invoked with the location ( xCurr, yCurr ) set equal to ( xB, yB ) and the neighbouring location ( xN, yN ) set equal to ( xBN, yBN ) as the input and the output is assigned to availableN.

The candidate intra prediction mode candIntraPredModeN is derived as follows.

If availableN is equal to FALSE, candIntraPredModeN is set equal to Intra_DC.

Otherwise, if PredMode[ xBN ][ yBN ] is not equal to MODE_INTRA, candIntraPredModeN is set equal to Intra_DC,

Otherwise, if N is equal to B and yB−1 is less than (( yB >> Log2CtbSizeY ) << Log2CtbSizeY), intraPredModeB is set equal to Intra_DC.

Otherwise, candIntraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ].

The candModeList[ x ] with x=0..2 is derived as follows:

If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

If candIntraPredModeA is less than 2 (either Intra_Planar or Intra_DC), candModeList[ x ] with x=0..2 is derived as:

candModeList[0] = Intra_Planar		(G-34)
candModeList[1] = Intra_DC		(G-35)
candModeList[2] = Intra_Angular (26)	(G-36)

Otherwise, candModeList[ x ] with x=0..2 is derived as:

candModeList[0] = candIntraPredModeA	(G-37)
candModeList[1] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(G-38)
candModeList[2] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(G-39)

Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

candModeList[0] and candModeList[1] are derived as follows:

candModeList[0] = candIntraPredModeA	(G-40)
candModeList[1] = candIntraPredModeB	(G-41)

If none of candModeList[0] and candModeList[1] is equal to Intra_Planar, candModeList[2] is set equal to Intra_Planar,

Otherwise, if none of candModeList[0] and candModeList[1] is equal to Intra_DC, candModeList[2] is set equal to Intra_DC,

Otherwise, candModeList[2] is set equal to Intra_Angular (26).

IntraPredMode[ xB ][ yB ] is derived by applying the following procedure:

If prev_intra_luma_pred_flag[ xB ][ yB ] is equal to 1, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[ mpm_idx ].

Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following ordered steps:

- The array candModeList[x], x=0..2 is modified as the following ordered steps:

  - When candModeList[0] is greater than candModeList[1], both values are swapped as follows.

( candModeList[0], candModeList[1] ) = Swap( candModeList[0], candModeList[1] )	(G-42)

  - When candModeList[0] is greater than candModeList[2], both values are swapped as follows.

( candModeList[0], candModeList[2] ) = Swap( candModeList[0], candModeList[2] )	(G-43)

  - When candModeList[1] is greater than candModeList[2], both values are swapped as follows.

( candModeList[1], candModeList[2] ) = Swap( candModeList[1], candModeList[2] )	(G-44)

- IntraPredMode[xB][yB] is derived as the following ordered steps:

  - IntraPredMode[ xB ][ yB ] = rem_intra_luma_pred_mode[ xB ][ yB ]

  - When IntraPredMode[ xB ][ yB ] is greater than or equal to candModeList[ 0 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is greater than or equal to candModeList[ 1 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is greater than or equal to candModeList[ 2 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

Derivation process for chroma intra prediction mode

The specifications in subclause 8.4.3 apply.

Decoding process for intra blocks

General decoding process for intra blocks

Inputs to this process are:

a sample location ( xB0, yB0 ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

a variable log2TrafoSize specifying the size of the current transform block,

a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

a variable intraPredMode specifying the intra prediction mode,

a variable cIdx specifying the chroma component of the current block.

Output of this process is:

a modified reconstructed picture before deblocking filtering.

The variable splitFlag is derived as follows:

If cIdx is equal to 0, splitFlag is set equal to split_transform_flag[ xB0 ][ yB0 ][ trafoDepth ].

Otherwise, if all of the following conditions are true, splitFlag is set equal to 1.

cIdx is greater than 0

split_transform_flag[ xB0 << 1 ][ yB0 << 1 ][ trafoDepth ] is equal to 1

log2TrafoSize is greater than 2

Otherwise, splitFlag is set equal to 0.

Depending splitFlag and sdc_flag, the following applies:

If splitFlag is equal to 1, the following ordered steps apply:

  - The variables xB1 and yB1 are derived as follows.

The variable xB1 is set equal to xB0 + ( ( 1 << log2TrafoSize ) >> 1 ).

The variable yB1 is set equal to yB0 + ( ( 1 << log2TrafoSize ) >> 1 ).

The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xB0, yB0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode intraPredMode, and the variable cIdx as the inputs and the output is a modified reconstructed picture before deblocking filtering.

The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xB1, yB0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode intraPredMode, and the variable cIdx as the inputs and the output is a modified reconstructed picture before deblocking filtering.

The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xB0, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode intraPredMode, and the variable cIdx as the inputs and the output is a modified reconstructed picture before deblocking filtering.

The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xB1, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode intraPredMode, and the variable cIdx as the inputs and the output is a modified reconstructed picture before deblocking filtering.

Otherwise, if (splitFlag is equal to 0) and sdc_flag is equal to 0, the following ordered steps apply:

  - The variable nT is set equal to 1 << log2TrafoSize

The general intra sample prediction process as specified in subclause G.8.4.4.2.1 is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, the transform block size nT and the variable cIdx as the inputs and the output is a (nT)x(nT) array predSamples.

The scaling and transformation process as specified in subclause 8.6.2 is invoked with the location ( xB0, yB0 ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nT as the inputs and the output is a (nT)x(nT) array resSamples.

The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the transform block location ( xB0, yB0 ), the transform block size nT, the variable cIdx, the (nT)x(nT) array predSamples, and the (nT)x(nT) array resSamples as the inputs.

Otherwise (splitFlag is equal to 0 and sdc_flag is equal to 1) the following ordered steps apply:

  - The variable nT is set equal to 1 << log2TrafoSize.

The general intra sample prediction process as specified in subclause G.8.4.4.2.1 is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, the transform block size nT and the variable cIdx as the inputs and the output is a (nT)x(nT) array predSamples.

The depth value reconstruction process as specified in subclause G.8.4.4.3 is invoked with the location ( xB0, yB0 ), the transform size trafoSize set equal to nT, the (nT)x(nT) array predSamples, and the intra prediction mode intraPredMode, as the inputs and the output is a (nT)x(nT) array resSamples.

Intra sample prediction

General intra sample prediction

The specification in subclause 8.4.4.2.1 with the following paragraphs added to the end of subclause apply:

Otherwise, if intraPredMode is equal to Intra_DepthPartition(35,36), the corresponding intra prediction mode specified in subclause G.8.4.4.2.7 is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, the sample array p and the transform block size nT as the inputs and the output are the predicted sample array predSamples.

Otherwise, if intraPredMode is equal to Intra_DepthPartition(37,38), the corresponding intra prediction mode specified in subclause G.8.4.4.2.8 is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, the sample array p and the transform block size nT as the inputs and the output are the predicted sample array predSamples.

Otherwise, if intraPredMode is equal to Intra_DepthPartition(39,40), the corresponding intra prediction mode specified in subclause G.8.4.4.2.9 is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, with the sample array p and the transform block size nT as the inputs and the output are the predicted sample array predSamples.

Otherwise, if intraPredMode is equal to Intra_DepthPartition(41,42), the corresponding intra prediction mode specified in subclause G.8.4.4.2.10 is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, with the sample array p and the transform block size nT  as the inputs and the output are the predicted sample array predSamples.

Otherwise, if intraPredMode is equal to Intra_Chain(43,44), the corresponding intra prediction mode specified in subclause G.8.4.4.2.11 is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, with the sample array p and the transform block size nT  as the inputs and the output are the predicted sample array predSamples.

Reference sample substitution process for intra sample prediction

The specifications in subclause 8.4.4.2.2 apply.

Filtering process of neighbouring samples

The specifications in subclause 8.4.4.2.3 apply.

Specification of Intra_Planar (0) prediction mode

The specifications in subclause 8.4.4.2.4 apply.

Specification of Intra_DC (1) prediction mode

The specifications in subclause 8.4.4.2.5 apply.

Specification of Intra_Angular (2..34) prediction mode

The specifications in subclause 8.4.4.2.6 apply.

Specification of Intra_DepthPartition (35, 36) prediction mode

Inputs to this process are:

a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

a variable intraPredMode specifying the intra prediction mode,

the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

a variable nT specifying the transform size,

Output of this process is:

predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 35 or 36.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as specified by the following ordered steps:

  - The variable wedgePattern[ x ][ y ] with x, y =0..nT−1, specifying a binary partition pattern is derived as.

wedgePattern = WedgePatternTable[ Log2( nT) ][ wedge_full_tab_idx[ xB ][ yB ] ]	(G-45)

The depth partition value derivation and assignment process as specified in subclause G.8.4.4.2.12 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern [ xB ][ yB ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(36) ), and the DC Offsets QuantDcOffsetP0[ xB ][ yB ], and QuantDcOffsetP1[ xB ][ yB ] as inputs and the output is assigned to predSamples[ x ][ y ].

Specification of Intra_DepthPartition (37, 38) prediction mode

Inputs to this process are:

a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

a variable intraPredMode specifying the intra prediction mode,

the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

a variable nT specifying the transform size,

Output of this process is:

predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 37 or 38.

The variable textureComp is set equal to the texture view component with view order index equal to ViewIdx and DepthFlag equal to 0. The variable texturePredMode is set equal to the PredMode[ xB ][ yB ] of textureComp. The variable textureIntraPredMode is set equal to IntraPredMode[ xB ][ yB ] of the textureComp. The variable recTexturePic is set equal to the array of the reconstructed luma picture samples textureComp.

[Ed. (GT) Consider storing texturePredMode and textureIntraPredMode explicitly.]

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived by the following ordered steps.

  - Depending on texturePredMode and textureIntraPredMode, the variable dmmWedgeletPattern[ x ][ y ] specifying a binary partition pattern is derived as specified in the following.

If texturePredMode is equal to MODE_INTRA and textureIntraPredMode is in the range of 2 to 34, the following applies.

The list candWedgeIndList specifying indices of candidate wedgelets and the variable numCandWedgeInd specifying the number of elements in candWedgeIndList are derived by

candWedgeIndList = TextModePredWedgeIndTable[ Log2( nT ) ][ texturePredMode ]	(G-46)
numCandWedgeInd = NumTextModePredWedgeInd[ Log2( nT ) ][ texturePredMode ]	(G-47)

Otherwise ( texturePredMode is not equal to MODE_INTRA or textureIntraPredMode is not in the range of 2 to 34), the following applies.

The list coarseCandWedgeIndList specifying indices of candidate wedgelets and the variable numCoarseCandWedgeInd specifying the number of elements in coarseCandWedgeIndList are derived by

coarseCandWedgeIndList = CoarseWedgeIndTable[ Log2( nT ) ]	(G-48)
numCoarseCandWedgeInd = NumCoarseWedgeInd[ Log2( nT ) ]	(G-49)

The derivation process for a texture predicted wedgelet index as specified in subclause G.8.4.4.2.8.1 is invoked with the luma location ( xB, yB ), the transform size nT, the candidate wedgelet indices coarseCandWedgeList and the number of candidate wedgelet indices numCoarseCandWedgeInd as inputs and the output is assigned to coarseWedgeIdx.

The list candWedgeIndList specifying indices of candidate wedgelets and the variable numCandWedgeInd specifying the number of elements in candWedgeIndList are derived by

candWedgeIndList = RefinedWedgeIndTable[ Log2( nT ) ][ coarseWedgeIdx ]	(G-50)
numCandWedgeInd = NumRefinedWedgeInd[ Log2( nT ) ][ coarseWedgeIdx ]	(G-51)

The derivation process for a texture predicted wedgelet index as specified in subclause XXX is invoked with the luma location ( xB, yB ), the transform size nT, the candidate wedgelet indices candWedgeIndList and the number of candidate wedgelet indices numCandWedgeInd as inputs and the output is assigned to wedgeIdx.

The variable wedgePattern [ x ][ y ] with x, y =0..nT−1, specifying a binary partition pattern is derived as.

wedgePattern = WedgePatternTable[ Log2( nT) ][ wedgeIdx]

The depth partition value derivation and assignment process as specified in subclause G.8.4.4.2.8.1 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(38) ), and the DC Offsets QuantDcOffsetP0[ xB ][ yB ], and QuantDcOffsetP1[ xB ][ yB ] as inputs and the output is assigned to predSamples[ x ][ y ].

Derivation process for a texture predicted wedgelet index

Inputs to this process are:

a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

a variable nT specifying the transform size,

a list candWedgeIndList specifying candidate wedgelet indices

a variable numCandWedgeInd specifying the number of candidate wedgelet indices

Output of this process is:

a variable wedgeIdx specifying the predicted wedgelet index

The variable wedgeIdx is set equal to −1 and the following ordered steps apply for i ranging from 0 to numCandWedgeInd −1.

  - The variable candWedgeIdx is set equal to candWedgeIndList[ i ].

  - The variable candWedgePattern[ x ][ y ] with x, y =0..nT−1, specifying a candidate binary partition pattern is derived as.

candWedgePattern = WedgePatternTable[ Log2( nT) ][ candWedgeIdx ] 	(G-52)

The derivation process for a texture wedgelet distortion as specified in subclause G.8.4.4.2.12 is invoked with the sample location ( xB, yB ), the transform size nT and the binary partition pattern candWedgePattern[ x, y ], as inputs and the output is assigned to candDist.

When wedgeIdx is equal to −1 or candDist is smaller than minDist, the variable wedgeIdx is set equal to candWedgeIdx and the variable minDist is set equal to candDist.

Derivation process for a texture wedgelet distortion

Inputs to this process are:

a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

a variable nT specifying the transform size,

a binary array candWedgePattern[ x ][ y ], with x, y =0..nT−1, specifying a partitioning of the prediction block in a partition 0 and a partition 1.

Output of this process is:

a variable candDist specifying the texture wedgelet distortion.

The variable recTextPic is set equal to the array of the reconstructed luma picture samples of the view component with view order index equal to ViewIdx and DepthFlag equal to 0.

The variable candDist is set equal to 0 and for X being 0 and 1, the following ordered steps apply.

  - The variable sumTextVal specifying the sum of the texture sample values of partition X is set equal to 0 and the variable numTextVal specifying the number of the texture samples of partition X is set equal to 0 and the following applies.

For x = 0..nT − 1 the following applies.

For y = 0..nT − 1 the following applies.

When candWedgePattern [ x ][ y ] is equal to X, the following applies.

sumTextVal += recTextPic[ xB+ x ][yB+ y ]	(G-53)
numTextVal += 1		(G-54)

The variable dcVal specifying the constant partition value for partition X is derived as follows.

dcVal = ( numTextVal ! = 0 ) ? ( sumTextVal / numTextVal ) : ( 1 << ( BitDepthY  − 1 ) )	(G-55)

The variable candDist is derived as specified in the following.

For x = 0..nT − 1 the following applies.

For y = 0..nT − 1 the following applies.

When candWedgePattern[ x ][ y ] is equal to X, the following applies.

candDist += Abs( recTextPic[ xB+ x ][yB+ y ] − dcVal )	(G-56)

Specification of Intra_DepthPartition (39, 40) prediction mode

Inputs to this process are:

a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

a variable intraPredMode specifying the intra prediction mode,

the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

a variable nT specifying the transform size,

Output of this process is:

predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 39 or 40.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as specified by the following ordered steps:

  - The variable recTextPic is set equal to the array of the reconstructed luma picture samples of the view component with view order index equal to ViewIdx and DepthFlag equal to 0.

  - The variable textThresh specifying a threshold for the segmentation of recTextPic is derived as specified in the following.

The variable sumTextPicVals is set equal to 0.

For x = 0..nT−1 the following applies

For y = 0..nT−1 the following applies

sumTextPicVals += recTextPic[ xB + x ][ yB + y ]	(G-57)

The variable textThresh is set equal to sumTextPicVals / ( nT * nT )

The variable wedgeletPattern[ x ][ y ] with x, y =0..nT−1 specifying a binary partition pattern is derived as specified in the following.

For x = 0..nT−1 the following applies

For y = 0..nT−1 the following applies

wedgeletPattern[ x ][ y ] = ( recTextPic[ xB + x ][ yB + y ] > textThresh )	(G-58)

The depth partition value derivation and assignment process as specified in subclause G.8.4.4.2.12 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgeletPattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(40) ), and the DC Offsets QuantDcOffsetP0[ xB ][ yB ], and QuantDcOffsetP1[ xB ][ yB ] as inputs and the output is assigned to predSamples[ x ][ y ].

Specification of Intra_DepthPartition (41, 42) prediction mode

Inputs to this process are:

a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

a variable intraPredMode specifying the intra prediction mode,

the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

a variable nT specifying the transform size,

Output of this process is:

predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 41 or 42.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as specified by the following ordered steps:

  - The variable wedgePattern [ x ][ y ] with x, y =0..nT−1, specifying a binary partition pattern is derived as specified in the following.

[Ed. (GT): Add description of continuous intra direction wedgelet partition prediction process]

The depth partition value derivation and assignment process as specified in subclause G.8.4.4.2.12 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(42) ), and the DC Offsets QuantDcOffsetP0[ xB ][ yB ], and QuantDcOffsetP1[ xB ][ yB ] as inputs and the output is assigned to predSamples[ x ][ y ].

Specification of Intra_Chain (43, 44) prediction mode

Inputs to this process are:

a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

a variable intraPredMode specifying the intra prediction mode,

the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

a variable nT specifying the transform size,

Output of this process is:

predicted samples predSamples[ x ][ y ], with x, y = 0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 43 or 44.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as specified in the following ordered steps.

[Ed. (GT) The process of edge reconstruction (items 1 - 3) need to be specified more in detail. In current state the specification is not precise enough.]

  - The start point of the chain in the current prediction unit is derived from edge_start_left_flag[ xB ][ yB ] and edge_start_position[ xB ][ yB ].

- If edge_start_left_flag[ xB ][ yB ] is equal to 1, the start point is set as s[ 0, yS ] where yS is equal to edge_start_position[ xB ][ yB ].

- Otherwise, the start point is set as s[ xS ][ 0 ] where xS is equal to edge_start_position[ x0 ][ y0 ].

The number of edges and the direction of each edge are derived from num_edge_codes_minus1[ xB ][ yB ] and edge_code[ k ], for k = 0.. num_edge_codes_minus1[ xB ][ yB ].

The region boundary generated by connecting each edge separates the predicted samples predSamples[ x ][ y ], with x, y = 0..nT-1 into two regions: the region rA that covers the top-left pixel (x0, y0) and the region rB that covers the remaining region.

For x, y = 0..nT-1 the binary pattern edgePattern[ x ][ y ] is derived as follows:

- If (x, y) is covered by rA, edgePattern[ x ][ y ] =  0.

- Otherwise, edgePattern[ x ][ y ] = 1.

The depth partition value derivation and assignment process as specified in subclause G.8.4.4.2.12 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern edgePattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(44) ), and the DC offsets QuantDcOffsetP0[ xB ][ yB ], and QuantDcOffsetP1[ xB ][ yB ] as inputs and the output is assigned to predSamples[ x ][ y ].

Depth partition value derivation and assignment process

Inputs to this process are:

neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

an binary array partitionPattern[ x ][ y ], with x, y =0..nT−1, specifying a partitioning of the prediction block in a partition 0 and a partition 1.

a variable nT specifying the transform size,

a flag dcOffsetAvailFlag, specifying whether DC Offset values are available

the variables quantDcOffsetP0 and quantCcOffsetP1, specifying the DC offsets for the block partitions

Output of this process is:

predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

For X being replaced by 0 and 1, the values of the prediction samples predSamples[ x][ y ] are derived as specified by the following ordered steps:

  - The variable sumNeigh specifying the sum of the neighbouring samples depth values of partition X is set equal to 0 and the variable numNeigh specifying the number of the neighbouring samples of partition X is set equal to 0 and the following applies.

For x = 0..nT − 1 the following applies.

When partitionPattern[ x ][ 0 ] is equal to X, the following applies:

sumNeigh += p[ x ][ −1 ]		(G-59)
numNeigh += 1		(G-60)

For y = 0..nT − 1 the following applies.

When partitionPattern[ 0, y ] is equal to X, the following applies:

sumNeigh += p[ −1 ][ y ]		(G-61) 
numNeigh += 1		(G-62)

The variables predDcVal specifying the predicted constant partition values for partition X is derived as follows.

predDcVal = ( numNeigh ! = 0 ) ? ( sumNeigh / numNeigh ) : ( 1 << ( BitDepthY  − 1 ) )	(G-63)

The variable deQuantDcOffset specifying the de-quantized DC offset is derived as follows.

If dcOffsetAvailFlag is equal to 1, the following applies. :

dcOffset = 	
quantDcOffsetPX * Clip3( 1, ( 1 << BitDepthY ) − 1, Round( 2 (QP’Y /10)- 2 ) )	(G-64)

Otherwise ( dcOffsetAvailFlag is equal to 0), deQuantDcOffset is set equal to 0.

The predicted sample values predSamples are derived as follows for x = 0..nT−1 and for y = 0..nT−1.

When partitionPattern[ x ][ y ] is equal to X, the following applies.

predSamples[ x ][ y ] = predDcVal + dcOffset	(G-65)

Specification of tables WedgePatternTable, CoarseWedgeIndTable, RefinedWedgeIndTable and TextModePredWedgeIndTable.

The variable WedgePatternTable[ log2BlkSize ] specifies a list of binary partition patterns of size (1 << log2BlkSize )x(1 << log2BlkSize). NumWedgePattern[ log2BlkSize ] specifies the number of binary partition patterns in list WedgePatternTable[ log2BlkSize ]. For log2BlkSize ranging from 2 to Log2MaxDmmCbSize, inclusive, WedgePatternTable[ log2BlkSize ][ wedgeIdx ] and NumWedgePattern[ log2BlkSize ] are derived as specified in the following.

[Ed. (GT) To be specified.]

The variable CoarseWedgeIndTable[ log2BlkSize ] specifies a list of indices to the list WedgePatternTable[ log2BlkSize ]. NumCoarseWedgeInd[ log2BlkSize ] specifies the number of elements of the list CoarseWedgeIndTable[ log2BlkSize ]... For log2BlkSize ranging from 2 to Log2MaxDmmCbSize, inclusive, CoarseWedgeIndTable[ log2BlkSize ][ idx ] and NumCoarseWedgeInd[ log2BlkSize ] are derived as specified in the following.

[Ed. (GT) To be specified.]

The variable RefinedWedgeIndTable[ log2BlkSize ][ coarseWedgeIdx ] specifies a list of indices to the list WedgePatternTable[ log2BlkSize ]. NumRefinedWedgeInd[ log2BlkSize ][ coarseWedgeIdx ] specifies the number of elements of the list in RefinedWedgeIndTable[ log2BlkSize ][ coarseWedgeIdx ].. For log2BlkSize ranging from 2 to Log2MaxDmmCbSize, inclusive, RefinedWedgeIndTable[ log2BlkSize ][ coarseWedgeIdx ][ idx ] and NumRefinedWedgeInd[ log2BlkSize ][ coarseWedgeIdx ] are derived as specified in the following.

[Ed. (GT) To be specified.]

The variable TextModePredWedgeIndTable[ log2BlkSize ][ texturePredMode ] specifies a list of indices to the table WedgePatternTable[ log2BlkSize ]. NumTextModePredWedgeInd[ log2BlkSize ][ texturePredMode ] specifies the number of elements of the list TextModePredWedgeIndTable[ log2BlkSize ][ texturePredMode ]. For log2BlkSize ranging from 2 to Log2MaxDmmCbSize, inclusive, TextModePredWedgeIndTable[ log2BlkSize ][ texturePredMode ] and NumTextModePredWedgeInd[ log2BlkSize ][ texturePredMode ] are derived as specified in the following.

[Ed. (GT) To be specified.]

Depth value reconstruction process

Inputs to this process are:

a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

a variable nT specifying the prediction size

predicted samples predSamples[ x ][y ], with x, y =0..nT−1

a variable intraPredMode specifying the prediction mode of the current prediction block

Output of this process is:

reconstructed depth value samples resSamples[ x ][ y ], with x, y = −1..2*nT−1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as per the following ordered steps.

  - Depending on intraPredMode the array wedgePattern[ x ][ y ] with x, y =0..nT−1 specifying the binary segmentation pattern is derived as follows.

If intraPredMode is equal to Intra_DepthPartition( 35 ), the following applies.

wedgePattern = WedgePatternTable[ Log2( nT) ][ wedge_full_tab_idx[ xB ][ yB ] ]

Otherwise, if  intraPredMode is equal to Intra_DepthPartition( 41 ), the following applies.

[Ed. (GT): Specify derivation for continuous intra direction wedgelet partition.]

Otherwise ( intraPredMode is not equal to Intra_DepthPartition( 35 ) and  intraPredMode is not equal to Intra_DepthPartition( 41 ) ), the following applies.

For x, y = 0..nT−1 wedgePattern[ x ][ y ] is set equal to 0.

For x, y = 0..nT−1, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following.

If dlt_flag is equal to 0, the following applies.

resSamples[ x ][ y ] = predSamples[ x ][ y ] + SdcResidual[ xB ][ yB ][ wedgePattern[ x ][ y ] ]

Otherwise ( dlt_flag is equal to 1 ), the following applies.

dltIdxPred = DepthValue2Idx[ predSamples[ x ][ y ] ] 	(G-66)
dltIdxResi = SdcResidual[ xB ][ yB ][ wedgePattern[ x ][ y ] ]	(G-67)
resSamples[ x ][ y ] = Idx2DepthValue[ dltIdxPred + dltIdxResi ]	(G-68)

Decoding process for coding units coded in inter prediction mode

The specifications in subclause 8.5 apply with the following modification:

–	All invocations of the process specified in subclause 8.5.1 are replaced with invocations of the process specified in subclause G.8.5.1.

Inter prediction process

The specifications in subclause 8.5.1 apply with the following modification:

–	All invocations of the process specified in subclause 8.5.2 are replaced with invocations of the process specified in subclause G.8.5.2.

Decoding process for prediction units in inter prediction mode

The specifications in subclause 8.5.2 apply with the following modification:

–	All invocations of the process specified in subclause 8.5.2.1 are replaced with invocations of the process specified in subclause G.8.5.2.1.

–	All invocations of the process specified in subclause 8.5.2.2 are replaced with invocations of the process specified in subclause G.8.5.2.2.

Derivation process for motion vector components and reference indices

Input to this process are

a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

a luma location ( xB, yB ) of the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

a variable nCS specifying the size of the current luma coding block,

variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

luma motion vectors mvL0 and mvL1

chroma motion vectors mvCL0 and mvCL1,

reference indices refIdxL0 and refIdxL1,

prediction list utilization flags predFlagL0 and predFlagL1.

Let ( xP, yP ) specify the top-left sample location of the current luma prediction block relative to the top-left luma sample of the current picture where xP = xC + xB and yP = yC + yB.

Let the variable currPic and ListX be the current picture and RefPicListX (with X being 0 or 1) of the current picture, respectively.

The function LongTermRefPic( picX, refIdx, LX ), with X being either 0 or 1, is defined as follows. If the picture with index refIdx from reference picture list LX of the picture picX was marked as "used for long term reference" at the time when picX was the current picture, LongTermRefPic( picX, refIdx, LX ) is equal to 1; otherwise LongTermRefPic( picX, refIdx, LX ) is equal to 0.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1 as well as prefFlagL0 and prefFlagL1, the following applies.

If MotionInhFlag[ xP ][ yP ] is equal to 1, following ordered steps apply.

The derivation process for a luma motion vector for motion parameter inheritance mode as specified in subclause G.8.5.2.1.19 is invoked with the luma location ( xP, yP ), and the variables nPbW, nPbH as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the prediction list utilization flags predFlagL0 and predFlagL1.

The variable ivpMvFlagLX and the variable ivpMvDispLX are derived as

ivpMvFlagLX = 0		(G-69)
ivpMvDispLX[ 0 ] = 0		(G-70)
ivpMvDispLX[ 1 ] = 0		(G-71)

Otherwise, Iif PredMode[ xC ][ yC ] is equal to MODE_SKIP, the derivation process for luma motion vectors for merge mode as specified in subclause G.8.5.2.1.1 is invoked with the luma location ( xC, yC ), the luma location ( xP, yP ), variables nCS, nPbW, nPbH and the partition index partIdx as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the disparity vectors ivpMvDispL0 and ivpMvDispL1.

Otherwise, if PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in subclause G.8.5.2.1.1 is invoked with the luma location ( xC, yC ), luma location ( xP, yP ), variables nCS, nPbW and nPbH and the partition index partIdx as inputs and the outputs being the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the disparity vectors ivpMvDispL0 and ivpMvDispL1.

Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX, ivpMvFlagLX, ivpMvDispLX , and in Pred_LX and in the syntax elements ref_idx_lX and MvdLX, the following applies.

    - The variables refIdxLX and predFlagLX are derived as follows.

If inter_pred_idc[ xP ][ yP ] is equal to Pred_LX or Pred_BI,

refIdxLX = ref_idx_lX[ xP ][ yP ]		(G-72)
predFlagLX = 1		(G-73)

Otherwise, the variables refIdxLX and predFlagLX are specified by

refIdxLX = −1		(G-74)
predFlagLX = 0		(G-75)

    - The variable mvdLX is derived as follows.

mvdLX[ 0 ] = MvdLX[ xP ][ yP ][ 0 ]		(G-76)
mvdLX[ 1 ] = MvdLX[ xP ][ yP ][ 1 ]		(G-77)

    - When predFlagLX is equal to 1, the derivation process for luma motion vector prediction in subclause G.8.5.2.1.5 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), variables nPbW and nPbH, refIdxLX, and the partition index partIdx as the inputs and the output being mvpLX.

    - When predFlagLX is equal to 1, the luma motion vector mvLX is derived as

uLX[ 0 ] = (mvpLX[ 0 ] + mvdLX[ 0 ] + 216) % 216	(G-78)
mvLX[ 0 ] = ( uLX[ 0 ] >= 215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ]	(G-79)
uLX[ 1 ] = (mvpLX[ 1 ] + mvdLX[ 1 ] + 216) % 216	(G-80)
mvLX[ 1 ] = ( uLX[ 1 ] >= 215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]	(G-81)

    - The variable ivpMvFlagLX and the variable ivpMvDispLX are derived as

ivpMvFlagLX = 0		(G-82)
ivpMvDispLX [ 0 ] = 0		(G-83)
ivpMvDispLX [ 1 ] = 0		(G-84)

NOTE – The resulting values of mvLX[ 0 ] and mvLX[ 1 ] as specified above will always be in the range of −215 to 215 − 1.

When ChromaArrayType is not equal to 0 and predFlagLX (with X being either 0 or 1) is equal to 1, the derivation process for chroma motion vectors in subclause 8.5.2.1.8 is invoked with mvLX and refIdxLX as inputs and the output being mvCLX.

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xB..xB + nPbW − 1, y = yB..yB + nPbH− 1(with X being either 0 or 1):

IvpMvFlagLX[ x ][ y ] = ivpMvFlagLX		(G-85)
IvpMvDispLX[ x ][ y ] = ivpMvDispLX		(G-86)

[Ed.(GT): Consider reordering of G.8.5.2.1.9 to G.8.5.2.1.18 to Merge: 9 15 17 AMVP: 10 11 12 18 Disparity: 13 16 14]

[Ed.(GT): The complete derivation process for inter-view motion vectors candidates has the problem (in software as well as in this draft), that it is assumed that disparity vectors are related to the base view. However, in the draft and the software there are some loops looking for an arbitrary non-base view inter-view reference picture for inter-view motion vector prediction in the reference picture lists. This works for CTC, since other inter-view reference pictures apart from the base view are not allowed. However, to enable a more general approach, this should be discussed and possibly made more general in SW and text. ]

[Ed.(CY): To fix the above mentioned problem, the derived disparity vector should be associated with a reference view index.]

Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode[ xC ][ yC ] is equal to MODE_SKIP or PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

a variable nCS specifying the size of the current luma coding block,

variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

the luma motion vectors mvL0 and mvL1,

the reference indices refIdxL0 and refIdxL1,

the prediction list utilization flags predFlagL0 and predFlagL1,

the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1,

the disparity vectors ivpMvDispL0 and ivpMvDispL1.

The variables singleMCLFlag is derived as follows.

If log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, singleMCLFlag is set to 1.

Otherwise, singleMCLFlag is set to 0.

When singleMCLFlag is equal to 1, xP is set equal to xC, yP is set equal to yC, and both nPbW and nPbH are set equal to nCS.

NOTE – When singleMCLFlag is equal to 1, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the disparity vectors ivpMvDispL0 and ivpMvDispL1 are derived as specified by the following ordered steps:

  - The derivation process for spatial merge ing candidates from neighboring prediction unit partitions as specified in subclause G.8.5.2.1.2 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the variable singleMCLFlag, the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

  - The reference index for temporal merging candidate refIdxLX (with X being 0 or 1) is set equal to 0.

  - The derivation process for temporal luma motion vector prediction in subclause G.8.5.2.1.7 is invoked with luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, and refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(G-87) 
predFlagLXCol = availableFlagLXCol		(G-88)

  - Depending on multi_view_mv_pred_flag, the following applies.

If multi_view_mv_pred_flag is equal to 0, the flags availableFlagIvMC and availableFlagIvDC are set equal to 0.

Otherwise (multi_view_mv_pred_flag is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause G.8.5.2.1.9 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPSW and nPSH, and the partition index partIdx, as the inputs and the output is assigned to the view order index of the reference view refViewIdx, the availability flags availableFlagIvMC and availableFlagIvDC, the reference indices refIdxLXIvMC and refIdxLXIvDC, the prediction list utilization flags predFlagLXIvMC and predFlagLXIvDC, and the motion vectors mvLXIvMC and mvLXIvDC (with X being 0 or 1, respectively), and the disparity vector mvDisp.

  - The merge candidate list, mergeCandList, is constructed as specified by the following ordered steps.

  - The variable numMergeCand is set equal to 0.

  - When availableFlagIvMC is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvMC and the variable numMergeCand is increased by 1.

  - When availableFlagA1 is equal to 1 and one or more of the following conditions is true,

availableFlagIvMC  = = 0,

predFlagLXIvMC ! = predFlagLXA1, (with X being replaced by 0 and 1),

mvLXIvMC ! = mvLXA1 (with X being replaced by 0 and 1),

refIdxLXIvMC ! = refIdxLXA1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to A1 and the variable numMergeCand is increased by 1.

  - When availableFlagB1 is equal to 1, and one or more of the following conditions is true ,

availableFlagIvMC = = 0,

predFlagLXIvMC ! = predFlagLXB1, (with X being replaced by 0 and 1),

mvLXIvMC ! = mvLXB1 (with X being replaced by 0 and 1),

refIdxLXIvMC ! = refIdxLXB1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to B1 and the variable numMergeCand is increased by 1.

  - When availableFlagB0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to B0 and the variable numMergeCand is increased by 1.

  - When availableFlagIvDC is equal to 1, and one or more of the following conditions is true,

availableFlagA1 = = 0,

predFlagLXA1 ! = predFlagLXIvDC, (with X being replaced by 0 and 1),

mvLXA1 ! = mvLXIvDC (with X being replaced by 0 and 1),

refIdxLXA1 ! = refIdxLXIvDC (with X being replaced by 0 and 1),

and one or more of the following conditions is true,

availableFlagB1 = = 0,

predFlagLXB1 ! = predFlagLXIvDC, (with X being replaced by 0 and 1),

mvLXB1 ! = mvLXIvDC (with X being replaced by 0 and 1),

refIdxLXB1 ! = refIdxLXIvDC (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to IvDC and the variable numMergeCand is increased by 1.

  - When availableFlagA0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to A0 and the variable numMergeCand is increased by 1.

  - When availableFlagB2 is equal to 1 and numMergeCand is less than 4 + multi_view_mv_pred_flag, the entry mergeCandList[ numMergeCand ] is set equal to B2 and the variable numMergeCand is increased by 1.

  - When availableFlagCol is equal to 1 and numMergeCand is less than 5 + multi_view_mv_pred_flag, the entry mergeCandList[ numMergeCand ] is set equal to Col and the variable numMergeCand is increased by 1.

- A1, if availableFlagA1 is equal to 1

- B1, if availableFlagB1 is equal to 1

- B0, if availableFlagB0 is equal to 1

- A0, if availableFlagA0 is equal to 1

- B2, if availableFlagB2 is equal to 1

- Col, if availableFlagCol is equal to 1

  - The variable numMergeCand and numOrigMergeCand are is set equal to numMergeCand to the number of merging candidates in the mergeCandList.

  - When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause 8.5.2.1.3 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

  - The derivation process for zero motion vector merging candidates specified in subclause 8.5.2.1.4 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

  - The variable MergeIdx is derived as follows.

If use_mvi_flag is equal to 0, MergeIdx is set equal to merge_idx[ xP][ yP ].

Otherwise (use_mvi_flag is equal to 1), MergeIdx is set equal to merge_idx[ xP][ yP ] −  1.

  - The following assignments are made with N being the candidate at position MergeIdx merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ MergeIdx merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]		(G-89)

mvLX[ 1 ] = mvLXN[ 1 ]		(G-90)

refIdxLX = refIdxLXN		(G-91)

predFlagLX = predFlagLXN		(G-92)

  - When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nPbW + nPbH ) is equal to 12, the following applies.

refIdxL1 = −1		(G-93)

predFlagL1 = 0		(G-94)

  - The disparity availability flag ivpMvFlagLX and the disparity vector ivpMvDispLX are derived as follows (with X being replace by 0 or 1).

If all of the following conditions are true, ivpMvFlagLX is set equal to 1 and ivpMvDispLX is set equal to mvDisp [Ed. (GT) There is some redundancy in draft and software since ivpMvDispLX is derived for each list, although it is always equal for both lists.]

availableFlagIvMC = = 1

MergeIdx = = 0

predFlagLXIvMC = = 1

- [Ed. (GT): PredMode[ xC ][ yC ] = = MODE_SKIP might be added here instead of testing it in the disparity vector derivation process]

Otherwise, ivpMvFlagLX is set equal to 0 and both components of ivpMvDispLX are set equal to 0.

Derivation process for spatial merge candidates

The specifications in subclause 8.5.2.1.2 apply, with the following changes:

The following paragraph

- N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

is removed.

Derivation process for combined bi-predictive merging candidates

The specifications in subclause 8.5.2.1.3 apply.

Derivation process for zero motion vector merging candidates

The specifications in subclause 8.5.2.1.4 apply.

Derivation process for luma motion vector prediction

Inputs to this process are:

a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

a variable nCS specifying the size of the current luma coding block,

variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

the reference index refIdxLX (with X being equal to 0 or 1) specifying a reference picture in the reference picture list RefPicListLX.

a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Output of this process is

the motion vector predictor mvpLX (with X being equal to 0 or 1).

The motion vector predictor mvpLX is derived as specified in the following.

If mvp_lX_idx[ xP ][ yP ] is not equal to 0 or multi_view_mv_pred_flag is equal to 0, the derivation process for a non-inter-view luma motion vector prediction as specified in subclause G.8.5.2.1.18 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the size of the current luma coding block nCS, the width and the height of the luma prediction block nPbW and nPbH, the reference index refIdxLX (with X being 0 or 1, respectively), the partition index partIdx, and the motion vector predictor index mvp_lX_idx[ xP ][ yP ] − multi_view_mv_pred_flag as the inputs and the output is the motion vector predictor mvpLX (with X being equal to 0 or 1)

Otherwise (if mvp_lX_idx[ xP ][ yP ] is equal to 0 and multi_view_mv_pred_flag is equal to 1) the following applies.

The derivation process for the inter-view motion vector predictor candidate as specified in subclause G.8.5.2.1.10 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPbW and nPbH, the partition index partIdx, the reference index refIdxLX as the inputs and the outputs are the view order index of the reference, refViewIdx, the flag availableFlagLXInterView and the motion vector mvLXInterView (with X being equal to 0 or 1).

The motion vector predictor mvpLX is derived as follows.

If availableFlagLXInterView is equal to 1, the motion vector predictor mvpLX is set equal to mvLXInterView.

Otherwise (if availableFlagLXInterView is equal to 0), the following applies;

mvpLX[ 0 ] = 0		(G-95)
mvpLX[ 1 ] = 0		(G-96)

Derivation process for motion vector predictor candidates

The specifications in subclause 8.5.2.1.6 apply.

        - Derivation process for temporal luma motion vector prediction

The specifications in subclause 8.5.2.1.7 apply with the following additions at the end of the subclause.

The function ViewId( picX ) is specified as follows.

ViewId( picX ) = view_id of the picture picX		(G-97)

NOTE – The view_id value of a picture is the view_id of the view the picture belongs to and is defined in the video parameter set extension.

The function RefViewId( picX, refIdx, LX ) returns the view_id value of reference picture with index refIdx from reference picture list LX of the picture picX and is specified as follows.

RefViewId( picX, refIdx, LX ) = ViewId( RefPicListX[ refIdx ] of the picture picX)	(G-98)

[Ed. (GT). The functions specified above might be specified in MV-HEVC when view_id is parsed ].

When availableFlagLXCol is equal to 1 and RefPicListX[ refIdxLX ] is a long-term reference picture, the following applies.

- The variable viewDistCol is set equal to ViewId( colPic ) – RefViewId( colPic, refIdxCol, listCol ).

- The variable viewDistCur is set equal to ViewId ( currPic ) – ViewId( RefPicListX[ refIdxLX ] ).

- When viewDistCur is not equal to viewDistCol and viewDistCur is not equal to 0 and viewDistCol is not equal to 0, mvLXCol is modified to be a scaled version of the motion vector mvCol as specified by equation 8-156, equation 8-157 and equation 8-158 with td and tb derived as specified in the following.

td = Clip3( −128, 127, viewDistCol)		(G-99)

tb = Clip3( −128, 127, viewDistCur )		(G-100)

Derivation process for chroma motion vectors

The specifications in subclause 8.5.2.1.8 apply.

Derivation process for inter-view merge candidates

This process is not invoked when multi_view_mv_pred_flag is equal to 0.

Inputs to this process are:

a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

a variable nCS specifying the size of the current luma coding block,

variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with X being 0 or 1, respectively)

a view order index refViewIdx specifying a reference view.

the availability flags availableFlagIvMC and availableFlagIvDC specifying whether the inter-view merge candidates are available,

the reference indices refIdxLXIvMC and refIdxLXIvDC,

the prediction list utilization flags predFlagLXIvMC and predFlagLXIvDC,

the motion vectors mvLXIvMC and mvLXIvDC,

the disparity vector mvDisp.

The derivation process for a disparity vector as specified in subclause G.8.5.2.1.13 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPSW and nPSH, and the partition index partIdx, as the inputs and the outputs are and the view order index of the reference view refViewIdx, the flag availableDV and the disparity vector mvDisp.

The temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

The derivation process for a temporal inter-view motion vector merging candidate as specified in subclause G.8.5.2.1.15 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, the disparity vector mvDisp, the prediction list indication X and the view identifier refViewIdx as the inputs and the outputs are the flag availableFlagLXIvMC, the motion vector mvLXIvMC and the reference index refIdxLXIvMC.

  - The availability flag availableFlagIvMC, and the prediction utilization flags predFlagL0IvMC and predFlagL1IvMC are derived by

availableFlagIvMC = availableFlagL0IvMC | | availableFlagL1IvMC	(G-101)
predFlagL0IvMC = availableFlagL0IvMC		(G-102)
predFlagL1IvMC = availableFlagL1IvMC		(G-103)

The disparity inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

The derivation process for the disparity inter-view motion vector merging candidate as specified in subclause G.8.5.2.1.17 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, the disparity vector mvDisp, and the prediction list indication X as the inputs and the outputs are the flag availableFlagLXIvDC, the motion vector mvLXIvDC, and the reference index refIdxLXIvDC.

  - The availability flag availableFlagIvDC, and the prediction utilization flags predFlagL0IvDC and predFlagL1IvDC are derived by

availableFlagIvDC = availableFlagL0IvDC | | availableFlagL1IvDC 	(G-104)
predFlagL0IvDC = availableFlagL0IvDC		(G-105)
predFlagL1IvDC = availableFlagL1IvDC		(G-106)

Derivation process for the inter-view motion vector predictor candidate

This process is not invoked when multi_view_mv_pred_flag is equal to 0.

Inputs to this process are:

a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

a variable nCS specifying the size of the current luma coding block,

variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

a variable partIdx specifying the index of the current prediction unit within the current coding unit.

a reference index refIdxLX (with X being equal to 0 or 1) specifying a reference picture in the reference picture list RefPicListLX.

Outputs of this process are:

–	a view order index refViewIdx specifying a reference view,

–	a flag availableFlagLXInterView specifying whether the inter-view motion vector candidate is available,

–	a motion vector candidate mvLXInterView (if availableFlagLXInterView is equal to 1).

The derivation process for a disparity vector as specified in subclause G.8.5.2.1.13 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPSW and nPSH, and the partition index partIdx as the inputs and the view order index refViewIdx, a flag availableDV and a disparity vector mvDisp as the outputs.

The variable refPicViewIdx is set equal to the variable ViewIdx of the RefPicListLX[ refIdxLX ].

[Ed. (GT): refViewIdx might be used here to further restrict the reference views]

Depending on the value of refPicViewIdx, the following applies:

If refPicViewIdx is not equal to ViewIdx, the derivation process for a disparity inter-view motion vector predictor candidate as specified in subclause G.8.5.2.1.11 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, and the disparity vector mvDisp as the inputs and the outputs are the flag availableFlagLXInterView and the motion vector candidate mvLXInterView.

Otherwise (refPicViewIdx is equal to ViewIdx), the derivation process for a temporal inter-view motion vector candidate as specified in subclause G.8.5.2.1.12 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, the reference index refIdxLX, the disparity vector mvDisp, and the reference view identifier refViewIdx as the inputs and the outputs are the flag availableFlagLXInterView and the motion vector candidate mvLXInterView.

[Ed. Note (CY): consider adding definition for inter-view disparity motion vector and disparity vector.]

Derivation process for a disparity inter-view motion vector predictor candidate

This process is not invoked when multi_view_mv_pred_flag is equal to 0.

Inputs to this process are:

a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

a disparity vector mvDisp specifying the disparity to the reference block.

Outputs of this process are:

a flag availableFlagLXInterView specifying whether the inter-view motion vector candidate is available,

a motion vector candidate mvLXInterView.

The flag availableFlagLXInterView is set equal to 1 and the motion vector candidate mvLXInterView is derived by

mvLXInterView[ 0 ] = mvDisp[ 0 ]		(G-107)
mvLXInterView[ 1 ] = 0		(G-108)

Derivation process for a temporal inter-view motion vector predictor candidate

This process is not invoked when multi_view_mv_pred_flag is equal to 0.

Inputs to this process are:

a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

a reference index refIdxLX (with X being equal to 0 or 1) specifying a reference picture in the reference picture list RefPicListLX,

a disparity vector mvDisp specifying the disparity to the reference block,

a view index refViewIdx specifying a reference view.

Outputs of this process are:

a flag availableFlagLXInterView specifying whether the inter-view motion vector candidate is available,

a motion vector candidate mvLXInterView (if availableFlagLXInterView is equal to 1).

The reference layer luma location ( xRef, yRef ) is derived by

xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 ) >> 1 ) + ( ( mvDisp[ 0 ] + 2 ) >> 2 ) )	(G-109)
yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( ( nPSH – 1 ) >> 1 ) + ( ( mvDisp[ 1 ] + 2 ) >> 2 ) )	(G-110)

Let refPU be the coding unit that covers the luma location ( xRef, yRef ) in the view component with ViewIdx equal to refViewIdx.

[Ed. (GT): In software pictures of already coded view components of the same time instant are searched in coding order to find a coding unit refCU that covers the luma location ( xRef, yRef ) and has a PredMode equal to MODE_SKIP or MODE_INTER and provides a valid temporal inter-view motion vector candidate. Moreover, the view with ViewIdx equal to 1 is skipped in this search if the current ViewIdx of the current view component is larger than 1 to avoid prediction from non-base views. This only works correctly with current CTC and should be fixed. ].

The flag availableFlagLXInterView is set equal to 0, and both components of mvLXInterView are set equal to 0.

When the variable PredMode for the coding unit refCU is equal to MODE_SKIP or MODE_INTER, the following ordered steps apply.

  - For Y being replaced by 0 and 1, the following applies.

The variable refPredFlagLY is set equal to the variable predFlagLY of the prediction unit refPU.

The variable refMvLY is set equal to mvLY of the prediction unit refPU.

The variable refRefPicListLY is set equal to the reference picture list RefPicListLY of refPU.

  - When refPredFlagL0 is equal to 1 and the picture order count of the picture refRefPicListL0[ refRefIdxL0 ] is equal to the picture order count of the picture RefPicListLX[ refIdxLX ], the flag availableFlagLXInterView is set equal to 1 and the motion vector mvLXInterView is derived by

mvLXInterView[ 0 ] = refMvL0[ 0 ]		(G-111)
mvLXInterView[ 1 ] = refMvL0[ 1 ]		(G-112)

  - When availableFlagLXInterView is equal to 0 and refPredFlagL1 is equal to 1 and the picture order count of the picture refRefPicListL1[ refRefIdxL1 ] is equal to the picture order count of the picture RefPicListLX[ refIdxLX ], the flag availableFlagLXInterView is set equal to 1 and the motion vector mvLXInterView is derived by

mvLXInterView[ 0 ] = refMvL1[ 0 ]		(G-113)
mvLXInterView[ 1 ] = refMvL1[ 1 ]		(G-114)

Derivation process for a disparity vector

Inputs to this process are:

a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

a variable nCS specifying the size of the current luma coding block,

variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

a variable partIdx specifying the index of the current prediction unit within the current coding unit.

- Outputs of this process are:

a view order index refViewIdx specifying a reference view.

a flag availableDV specifying whether the disparity vector is available,

a disparity vector mvDisp.

The flag availableDV is set equal to 0, and both components of the disparity vector mvDisp are set equal to 0.

The variable checkParallelMergeFlag is derived as follows.

If one or more of the following conditions are true, checkParallelMergeFlag is set equal to 1.

PredMode[ xC ][ yC ] is equal to MODE_SKIP

PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1.

Otherwise, checkParallelMergeFlag is set equal to 0.

The motion vector mvDisp and the availability flag availableDV are derived in the following ordered steps and the whole decoding process of this sub-clause terminates once availableDV is set equal to 1.

  - For each N being A1, B1, B0, A0, or B2, and ( xN, yN ) being ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW − 1,  yP − 1 ), ( xP + nPSW,  yP − 1 ), ( xP − 1,  yP + nPSH  ), or ( xP − 1,  yP − 1 ), the following applies.

- When yP−1 is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY), the following applies.

xB0 = ((xB0>>3)<<3) + ((xB0>>3)&1)*7		(G-115)
xB1 = ((xB1>>3)<<3) + ((xB1>>3)&1)*7		(G-116)
xB2 = ((xB2>>3)<<3) + ((xB2>>3)&1)*7		(G-117)

- The availability derivation process for a prediction block as specified in subclause 6.4.2 is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the partition index partIdx, and the luma location ( xN, yN ) as inputs and the output is assigned to the prediction block availability flag availableN.

- When all of the following conditions are true, availableA1 is set equal to 0.

N is equal to A1

PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N

partIdx is equal to 1

- When all of the following conditions are true, availableB1 is set equal to 0.

N is equal to B1

PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD

partIdx is equal to 1

- When all of the following conditions are true, availableN is set equal to 0.

checkParallelMergeFlag is equal to 1

(xP >> (log2_parallel_merge_level_minus2 + 2) ) is equal to
(xN >> (log2_parallel_merge_level_minus2 + 2) )

(yP >> (log2_parallel_merge_level_minus2 + 2) ) is equal to (yN >> (log2_parallel_merge_level_minus2 + 2) ).

- The flag availableIvpMvSearchFlagN is set equal to availableN.

- When one of the following conditions is true, availableIvpMvSearchFlagN is set equal to 0.

N is equal to B0 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

N is equal to B1 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

N is equal to B2 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

- The flags availableFlagIvpMvN0 and availableFlagIvpMvN1 are set equal to 0.

- For each X from 0 to 1, the following applies, when availableN is equal to 1 and PredFlagLX[ xN ][ yN ] is equal to 1

If RefPicListX[ RefIdxLX[ xN ][ yN ] ] is an inter-view reference picture, the following applies:

refViewIdx = ViewIdx( RefPicListX[ RefIdxLX[ xN ][ yN ] ] )	(G-118)
mvDisp = MvLXN[ xN ][ yN ]		(G-119)
availableDV = 1		(G-120)

Otherwise (RefPicListX[ RefIdxLX[ xN ][ yN ] ] is not an inter-view reference picture), the following applies.

When availableIvpMvSearchFlagN is equal to 1 and PredMode[ xN ][ yN ] is equal to MODE_SKIP and IvpMvFlagLX[ xN ][ yN ] is equal to 1, the following applies:

ivpMvDispLXN = IvpMvDispLX[ xN ][ yN ]	(G-121)
availableFlagIvpMvLXN = 1		(G-122)

The derivation process for a disparity vector from temporal neighbour block as specified in G.8.5.2.1.16 is invoked with a luma location ( xP, yP ), variables nPSW and nPSH as inputs, and flag availableDV, and disparity vector mvDisp outputs

For each N being A1, B1, B0, A0, and B2, the following applies.

For each X from 0 to 1, the following applies.

When availableFlagIvpMvLXN is equal to 1, the following applies:

mvDisp = ivpMvDispLXN		(G-123)
availableDV = 1		(G-124)

[Ed. (GT): In contrast to the previous version where refViewIdx was assumed to be equal to 0. Here refViewIdx is derived together with the motion vectors from neighbouring blocks. Although the whole inter-view motion vector prediction has still the problem as described in the editor comment in subclause G.8.5.2.1 and needs further discussion, this approach is more closely aligned to what happens in software. However, for IvpMv refViewIdx and the temporal candidate refViewIdx is not derived here. ]

Derivation process for a disparity vector in a block of a candidate picture

Inputs to this process are:

–	a candidate picture candPic,

–	a luma location ( xPCand , yPCand ) relative to the top-left luma sample of the candidate picture.

Outputs of this process are:

–	a flag availableDV specifying whether the disparity vector is available,

–	a disparity vector mvDisp.

The motion vector mvDisp and the availability flag availableDV are derived as specified by the following ordered steps and the whole decoding process of this sub-clause terminates once availableDV is set equal to 1.

  - The variable colPu is set as the prediction unit in candPic covering the position ( ( xPCand >>4) <<4 ,  (yPCand >>4) <<4 ).[Ed.(GT): Where is this reflected in the software? ] [Ed. (CY): This is related to motion field compression and its done in picture level in the HM or HTM software, similarly, TMVP has the same operation and it achieves the same effect as motion field compression. Another way of implementing this is to change G.8.5.2.1.16, similar to TMVP.]

  - The position ( xPCol, yPCol ) is set equal to the position of the top-left sample of colPu relative to the top-left luma sample of the candPic.

  - If slice_type is equal to B, the variable dir is set equal to collocated_from_l0_flag, otherwise, dir is set equal to 1 – collocated_from_l0_flag.[Ed. (GT): In software the selection of dir also includes the "Low Delay check"] [Ed. (CY): It was noted this aspect has negligible impact on the performance, it might be desiable to change the software to align with the current text. This requires further discussion.]

  - The flag availableDV is set equal to 0, and both components of mvDisp are set equal to 0.

  - For each X from dir to 1- dir, inclusive, the following applies:

The variable candPicRefPicList is set equal to the reference picture list RefPicListX of candPic.

The variable candPredFlag is set equal to the prediction list utilization flag PredFlagLX of candPic.

The variable candRefIdx is set equal to the reference indices RefIdxLX of candPic.

The variable candMV is set equal to the motion vectors mvLX of candPic.

When the ViewIdx of candPicRefPicList[ candRefIdx[ xPCol ][ yPCol ] ] is not equal to the ViewIdx of candPic and candPredFlag[ xPCol ][ yPCol ] is equal to 1 and , the following applies:

mvDisp = candMV[ xPCol ][ yPCol ]		(G-125)
availableDV = 1		(G-126)

Derivation process for a temporal inter-view motion vector merging candidate

This process is not invoked when multi_view_mv_pred_flag is equal to 0.

Inputs to this process are:

a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

a disparity vector mvDisp,

a prediction list indication X,

a reference view index refViewIdx.

Outputs of this process are:

a flag availableFlagLXInterView specifying whether the inter-view motion vector candidate is available,

a motion vector candidate mvLXInterView (if availableFlagLXInterView is equal to 1),

a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX,

The flag availableFlagLXInterView is set equal to 0, both components of mvLXInterView are set equal to 0, and refIdxLX is set equal to −1.

When X is equal to 1 and the current slice is not a B slice the whole decoding process specified in this subclause terminates.

The reference layer luma location ( xRef, yRef ) is derived by

xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 ) >> 1 ) + ( ( mvDisp[ 0 ] + 2 ) >> 2 ) )	(G-127)
yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( ( nPSH – 1 ) >> 1 ) + ( ( mvDisp[ 1 ] + 2 ) >> 2 ))	(G-128)

Let refCU be the coding unit that covers the luma location ( xRef, yRef ) in the view component with ViewIdx equal to refViewIdx.

[Ed. (GT:) In software refCU is the coding unit that covers the luma location ( xRef, yRef ) in the view component that closest (in terms of view_id) to the current view component and that is included in one of the reference lists of the current view component.]

[Ed. (CY): What is implemented in the software has no impact on the final results in common test condition, so it is preferred to have the current text and software changes may be desirable. ]

When the variable PredMode for the coding unit refCU is equal to MODE_SKIP or MODE_INTER, the following ordered steps apply:

  - The variable refPredFlagLX is set equal to the prediction utilization flag predFlagLX of the prediction unit refPU.

The variable refRefIdxLX, is set equal to the reference index refIdxLX of  the prediction unit refPU.

The variable refMvLX is set equal to the motion vector mvLX of the prediction unit refPU.

The variable refRefPicListLX, is set equal to the reference picture list RefPicListLX of the view component with ViewIdx equal to refViewIdx.

When refPredFlagLX is equal to 1, the following applies for each i from 0 to num_ref_idx_lX_active_minus1, inclusive

When availableFlagLXInterView is equal to 0, and the picture order count of the picture refRefPicListLX[ refRefIdxLX ] is equal to the picture order count of the picture RefPicListLX[ i ], the flag availableFlagLXInterView is set equal to 1 and the following applies.

mvLXInterView[ 0 ] = refMvLX[ 0 ]		(G-129)
mvLXInterView[ 1 ] = refMvLX[ 1 ]		(G-130)
refIdxLX = i		(G-131)

Derivation process for a disparity vector from temporal neighbour blocks

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH

Outputs of this process are

- the disparity vector mvDisp,

- the availability flag availableFlag.

The luma location ( xPRb , yPRb) specifying the position of the right bottom luma prediction block of the current luma prediction block and the flag availableRb specifying the availability of the right bottom luma prediction block are derived by the following ordered steps:

  - The vertical component of the position of the right bottom luma prediction block is derived as follows.

yPRb = yP + nPbH		(G-132)

The horizontal component of the position of the right bottom luma prediction block is derived as follows.

xPRb = xP + nPbW		(G-133)

When xPRb is less than pic_width_in_luma_samples and yPRb is less than pic_height_in_luma_samples, availableRb is set equal to 1. Otherwise, availableRb is set equal to 0.

[Ed.(GT): This is not reflected in the software. There availableRb is always true, even when (xPRb, yPRb) is outside the LCU or the frame. When (xPRb, yPRb) is outside the frame ( xP, yP ) is used instead.]

[Ed. (CY): Based on the previous comment, this subclause is modified to reflect the current software. Checking whether (xPRb, yPRb) is outside the LCU is currently not implemented but may be desirable.]

The luma location ( xCtr , yCtr) specifying the centre position of the current luma prediction block is derived by

xPCtr = xP + ( nPbW >> 1 )		(G-134)

yPCtr = yP + ( nPbH >> 1 )		(G-135)

The flag availableFlag is set equal to 0, and both components of mvDisp are set equal to 0.

For i from 0 to NumDdvCandPics, the following ordered steps apply and the whole decoding process of this sub-clause terminates once availableFlag is set to 1.

  - When ViewIdx of the current picture is greater than 1 and availableRb is equal to 1, the derivation process for a disparity vector in a block of a candidate picture as specified in subclause G.8.5.2.1.14 is invoked with candidate picture DdvCandPicsList[ i ], luma location ( xPRb , yPRb ) as inputs, and the flag availableFlag and the disparity vector mvDisp as outputs.

The derivation process for a disparity vector in a block of a candidate picture as specified in subclause G.8.5.2.1.14 is invoked with candidate picture DdvCandPicsList[ i ], luma location (xPCtr, yPCtr) as inputs, and the flag availableFlag, the disparity vector mvDisp as outputs.

When ViewIdx of the current picture is equal to 1 and availableRb is equal to 1, the derivation process for a disparity vector in a block of a candidate picture as specified in subclause G.8.5.2.1.14 is invoked with candidate picture DdvCandPicsList[ i ], luma location ( xPRb , yPRb ) as inputs, and the flag availableFlag, the disparity vector mvDisp as outputs.

Derivation process for a disparity inter-view motion vector merging candidate

This process is not invoked when multi_view_mv_pred_flag is equal to 0.

Inputs to this process are:

a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

a prediction list indication X,

Outputs of this process are:

a view order index refViewIdx specifying a reference view.

a flag availableFlagLXInterView specifying whether the inter-view motion vector candidate is available,

a motion vector candidate mvLXInterView (if availableFlagLXInterView is equal to 1),

a reference index refIdxLX (with X being equal to 0 or 1) specifying a reference picture in the reference picture list RefPicListLX.

[GT (Ed.) This derivation process is specified as done in software. However, it might be changed to use refViewIdx to restrict the reference view to be the base view or the view mvDisp is related to. ]

The flag availableFlagLXInterView is set equal to 0, both components of mvLXInterView are set equal to 0, and refIdxLX is set equal to −1.

When X is equal to 1 and the current slice is not a B slice the whole decoding process specified in this subclause terminates.

The following apply for each i from 0 to num_ref_idx_lX_active_minus1, inclusive

When PicOrderCntVal of the picture RefPicListX[ i ] is equal to the PicOrderCntVal or the current picture and availableFlagLXInterView is equal to 0, availableFlagLXInterView is set equal to 1 and the motion vector mvLXInterView and the reference index RefPicListLX are derived by

mvLXInterView[ 0 ] = mvDisp[ 0 ]		(G-136)
mvLXInterView[ 1 ] = 0		(G-137)
refIdxLX = i		(G-138)

Derivation process for a non-inter-view luma motion vector prediction

The specifications in subclause 8.5.2.1.5 apply with the following additional input parameter.

a motion vector candidate index mvpIdxLX (with X being equal to 0 or 1)

and the following changes.

mvp_lX_flag[ xP][ yP ] is replaced by mvpIdxLX.

All invocations of the process specified in subclause 8.5.2.1.7 are replaced with invocations of the process specified in subclause Error! Reference source not found..

Derivation process for a luma motion vector for motion parameter inheritance mode

This process is only invoked when MotionInhFlag[ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The variable textureComp is set equal to the texture view component with view order index equal to ViewIdx and DepthFlag equal to 0. The variable texturePU is set equal to the PU in textureComp that covers the luma location ( xP, yP ).

For X being replaced by 0 or and 1, the following applies.

If texturePU is coded in an intra prediction mode, the following applies.

Both components of mvLX are set equal to 0.

If slice_type is equal to B or X is equal to 0, the following applies.

refIdxLX is set equal to 0.

predFlagLX is set equal to 1.

Otherwise (slice_type is equal to P and X is equal to 1), the following applies.

refIdxLX is set equal to −1.

predFlagLX is set equal to 0.

Otherwise (texturePU is not coded in an intra prediction mode), the following applies.

mvLX is set equal to MvLX[ xP ][ yP ] of textureComp.

refIdxLX is set equal to RefIdxLX[ xP ][ yP ] of the textureComp.

predFlagLX is set equal to PredFlagLX[ xP ][ yP ] of textureComp.

[Ed. (GT): Consider storing also the PredMode explicitly to find out if motion data can be inherited. ]

Decoding process for inter prediction samples

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

–	a variable nCS specifying the size of the current luma coding block,

–	variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

–	luma motion vectors mvL0 and mvL1,

–	chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1.

Outputs of this process are:

–	a (nCSL)x(nCSL) array predSamplesL of luma prediction samples, where nCSL is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCb of chroma prediction samples for the component Cb, where nCSC is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCr of chroma residual samples for the component Cr, where nCSC is derived as specified below.

The variable nCSL is set equal to nCS and the variable nCSC is set equal to nCS >> 1.

Let predSamplesL0L and predSamplesL1L be (nPbW)x(nPbH) arrays of predicted luma sample values and predSampleL0Cb, predSampleL1Cb, predSampleL0Cr, and predSampleL1Cr be (nPbW/2)x(nPbH/2) arrays of predicted chroma sample values.

For LX being replaced by either L0 or L1 in the variables predFlagLX, RefPicListX, refIdxLX, refPicLX, and predPartLX, the following is specified.

When predFlagLX is equal to 1, the following applies.

The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with refIdxLX as input.

The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived by invoking the fractional sample interpolation process specified in subclause 8.5.2.2.2 with the luma locations ( xC, yC ), ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, the motion vectors mvLX, mvCLX, and the reference arrays with refPicLXL, refPicLXCb and refPicLXCr given as input.

If ic_flag is equal to 0, the following applies.

The array predSampleL of the prediction samples of luma component is derived by invoking the weighted sample prediction process specified in subclause 8.5.2.2.3 with the luma location ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, and the sample arrays predSamplesL0L and predSamplesL1L as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1 and cIdx equal to 0 given as input.

The array predSampleCb of the prediction samples of component Cb is derived by invoking the weighted sample prediction process specified in subclause 8.5.2.2.3 with the chroma location ( xB/2, yB/2 ), the width and the height of the current chroma prediction block nPbWCb set equal to nPbW/2, nPbHCb set equal to nPbH/2, and the sample arrays predSamplesL0Cb and predSamplesL1Cb as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 1 given as input.

The array predSampleCr of the prediction samples of component Cr is derived by invoking the weighted sample prediction process specified in subclause 8.5.2.2.3 with the chroma location ( xB/2, yB/2 ), the width and the height of the current chroma prediction block nPbWCr set equal to nPbW/2, nPbHCr set equal to nPbH/2, and the sample arrays predSamplesL0Cr and predSamplesL1Cr as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 2 given as input.

Otherwise ( ic_flag is equal to 1), the following applies.

The array predSampleL of the prediction samples of luma component is derived by invoking the illumination compensated sample prediction process specified in subclause G.8.5.2.2.4, with the luma location ( xC, yC ), the size of the current luma coding block nCS, the luma location ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, and the sample arrays predSamplesL0L and predSamplesL1L as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvL0, mvL1 and cIdx equal to 0 given as input.

The array predSampleCb of the prediction samples of component Cb is derived by invoking the illumination compensated sample prediction process specified in subclause G.8.5.2.2.4, with the luma location ( xC, yC ), the size of the current luma coding block nCS, with the chroma location ( xB/2, yB/2 ), the width and the height of the current chroma prediction block nPbWCb set equal to nPbW/2, nPbHCb set equal to nPbH/2, and the sample arrays predSamplesL0Cb and predSamplesL1Cb as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvCL0, mvCL1, and cIdx equal to 1 given as input.

The array predSampleCr of the prediction samples of component Cr is derived by invoking the illumination compensated sample prediction process specified in subclause G.8.5.2.2.4, with the luma location ( xC, yC ), the size of the current luma coding block nCS, with the chroma location ( xB/2, yB/2 ), the width and the height of the current chroma prediction block nPbWCr set equal to nPbW/2, nPbHCr set equal to nPbH/2, and the sample arrays predSamplesL0Cr and predSamplesL1Cr as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvCL0, mvCL1, and cIdx equal to 2 given as input.

When res_pred_flag is equal to 1, the inter-view residual prediction process as specified in subclause G.8.5.2.2.5 is invoked with the luma locations ( xC, yC ) and ( xB, yB ), the size of the current luma coding block nCS, the variables nPSW and nPSH, the reference view index refViewIdx set equal to 0, the prediction list utilization flags, predFlagL0 and predFlagL1, the reference indices refIdxL0 and refIdxL1, and the arrays predSamplesL, predSamplesCb, and predSamplesCr as the inputs and the outputs are modified versions of the arrays predSamplesL, predSamplesCb, and predSamplesCr.

Reference picture selection process

The specifications in subclause 8.5.2.2.1 apply.

Fractional sample interpolation process

The specifications in subclause 8.5.2.2.2 apply.

Weighted sample prediction process

The specifications in subclause 8.5.2.2.3 apply.

Illumination compensated sample prediction process

Inputs to this process are:

a location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left sample of the current picture,

the size of current luma coding block nCS,

a location ( xB, yB ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

the width and height of this prediction block, nPbW and nPbH,

two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1,

prediction list utilization flags, predFlagL0 and predFlagL1,

reference indices, refIdxL0 and refIdxL1,

motion vector mvL0 and mvL1

colour component index, cIdx,

Outputs of this process are:

the (nPbW)x(nPbH) array predSamples of prediction sample values.

Variables shift1, shift2, offset1 and offset2 are derived as follows.

The variable shift1 is set equal to 14 − bitDepth and the variable shift2 is set equal to 15 − bitDepth,

The variable offset1 is derived as follows.

If shift1 is greater than 0, offset1 set equal to 1 << ( shift1 − 1 ).

Otherwise (shift1 is equal to 0), offset1 is set equal to 0.

The variable offset2 is set equal to 1 << ( shift2 − 1 ).

The variable bitDepth is derived as follows.

If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

Otherwise (cIdx is equal to 1 or 2), bitDepth is set equal to BitDepthC.

The derivation process for illumination compensation mode availability and parameters as specified in subclause G.8.5.2.2.4.1 is invoked with the luma location ( xC, yC ), the size of the current luma coding block nCS, prediction list utilization flags, predFlagL0 and predFlagL1, reference indices refIdxL0 and refIdxL1, motion vectors mvL0 and mvL1, the bit depth of samples, bitDepth, a variable cIdx specifying colour component index as the inputs and the outputs are the flags puIcFlagL0 and puIcFlagL1, the variables icWeightL0 and icWeightL1 specifying weights for illumination compensation, the variables icOffsetL0 and icOffsetL1 specifying offsets for illumination compensation, the variables icShiftL0 and icShiftL1 specifying bit shifts for illumination compensation.

Depending on the value of predFlagL0 and predFlagL1, the prediction samples predSamples[ x ][ y ] with x = 0..(nPbW)−1 and y = 0..(nPbH)−1 are derived as follows.

If predFlagL0 is equal to 1 and predFlagL1 is equal to 0,

clipPredVal= Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL0[ x ][ y ] + offset1 ) >> shift1 )	(G-139)

predSamples[ x ][ y ] = !puIcFlagL0 ? clipPredVal : 
( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL0 ) >> icShiftL0 ) + icOffsetL0 ) 	(G-140)

Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1,

clipPredVal = Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL1[ x ][ y ] + offset1 ) >> shift1 )	(G-141)

predSamples[ x ][ y ] = !puIcFlagL1 ? clipPredVal : 
( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL1 ) >> icShiftL1 ) + icOffsetL1 ) 	(G-142)

Otherwise,

predVal0 = !puIcFlagL0 ? predSamplesL0[ x ][ y ] : 
( ( predSamplesL0[ x ][ y ] * icWeightL0 ) >> icShiftL0  ) + ( icOffsetL0 << shift1 ) ) 	(G-143)

predVal1 = !puIcFlagL1 ? predSamplesL1[ x ][ y ] : 
( ( predSamplesL1[ x ][ y ] * icWeightL1 ) >> icShiftL1 ) + ( icOffsetL1 << shift1 ) ) 	(G-144)

predSamples[ x ][ y ] = 
Clip3( 0, ( 1 << bitDepth ) − 1, ( predVal0 + predVal1 + offset2 ) >> shift2 )	(G-145)

Derivation process for illumination compensation mode availability and parameters

Inputs to this process are:

a luma location ( xCl, yCl ) specifying the top-left sample of the current coding block relative to the top left sample of the current picture,

the size of the current luma coding block, nCSl,

prediction list utilization flags, predFlagL0 and predFlagL1,

reference indices refIdxL0 and refIdxL1,

motion vectors mvL0 and mvL1

a bit depth of samples, bitDepth.

a variable cIdx specifying colour component index.

Outputs of this process are:

flags puIcFlagL0 and puIcFlagL1 specifying whether illumination compensation is enabled.

variables icWeightL0 and icWeightL1 specifying weights for illumination compensation

variables icOffsetL0 and icOffsetL1 specifying offsets for illumination compensation

variables icShiftL0 and icShiftL1 specifying bit shifts for illumination compensation.

The variables puIcFlagL0 and PuIcFlagL1 are set equal to 0, the variables icWeightL0 and icWeightL1 are set equal to 1, the variables icOffsetL0 and icOffsetL1 are set equal to 0 and the variables icShiftL0 and icShiftL1 are set equal to 0.

The variables nCS specifying the current luma or chroma coding block size, and the location ( xC, yC ) specifying the top left sample of the current luma or chroma coding block is derived as follows.

nCS = ( cIdx = = 0 ) ? nCSl : nCSl / 2 		(G-146)

( xC, yC ) =  ( cIdx = = 0 ) ? ( xC, yC ) : ( xCl /2 , yCl / 2 ) 	(G-147)

The variable availFlagCurAboveRow specifying the availability of above neighbouring row samples is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4.1 with the location ( xCurr, yCurr ) set equal to ( xCl, yCl ) and the neighbouring location ( xN, yN ) set equal to ( xCl, yCl−1 ) as the input and the output is assigned to availFlagCurAboveRow.

The variable availFlagCurLeftCol specifying the availability of left neighbouring column samples is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4.1 with the location ( xCurr, yCurr ) set equal to ( xCl, yCl ) and the neighbouring location ( xN, yN ) set equal to ( xCl−1, yCl ) as the input and the output is assigned to availFlagCurLeftCol.

[Ed. (GT) The availability derivation is specified as performed in the software. However, the check of the availability of left and above PU might not be sufficient to guarantee the availability of the whole left column or above row. A check of availability similar to that used for intra prediction might be a better solution. ]

When availFlagCurAboveRow is equal to 0 and availFlagCurLeftCol is equal to 0 the whole derivation process of this subclause terminates.

For X being replaced by 0 and 1, when predFlagLX is equal to 1, the variable puIcFlagLX is derived by the following ordered steps.

  - The variable refPicLX specifying the reference picture from reference picture list X is set equal to RefPicListX[ refIdxLX ] .

If the view order index of the picture refPicLX is not equal to ViewIdx, the variable puIvPredFlagLX specifying whether inter-view prediction from List X is utilized is set equal to 1, otherwise (predFlagLX is equal to 0 or the view order index of the picture RefPicListX[ refIdxLX ]  is equal to ViewIdx ), puIvPredFlagLX is set equal to 0.

If puIvPredFlagLX is equal to 0, the variable puIcFlagLX is set equal to 0, otherwise (puIvPredFlagLX is equal to 1 ) the following applies.

The luma location (xRLX, yRLX) specifying the top-left sample of the reference block in refPicLX is derived as

xRLX = xC + ( mvLX[ 0 ] >> (2 + (cIdx ? 1 : 0)) )	(G-148)

yRLX = yC + ( mvLX[ 1 ] >> (2 + (cIdx ? 1 : 0)) ) 	(G-149)

The variable availFlagAboveRowLX specifying whether the above neighbouring row samples of the current block and the reference block are available is derived as specified in the following.

availFlagAboveRowLX = ( yRLX > 0 ) && availFlagCurAboveRow 	 (G-150)

The variable availFlagLeftColLX specifying whether the left neighbouring column samples of the current block and the reference block are available is derived as specified in the following.

availFlagLeftColLX = ( xRLX > 0 ) && availFlagCurLeftCol	(G-151)

The variable puIcFlagLX is derived as follows.

puIcFlagLX = availFlagAboveRowLX | | availFlagLeftColLX 	 (G-152)

Depending on the colour component cIdx, the variable curRecSamples specifying the reconstructed picture samples of the current picture is derived as

curRecSamples = ( !cidx ) ? RecSamplesL : ( ( icdx == 1 ) ? RecSamplesCb : RecSamplesCr) 	(G-153)

[Ed. (GT). The reconstructed samples before deblocking filter RecSamplesL, RecSamplesCb and RecSamplesCr as used above although not explicitly defined. However, they should be defined in the base spec. ]

For X being replaced by 0 and 1, when puIcFlagLX is equal to 1, the variables icWeightLX, icOffsetLX and icShiftLX are derived by the following ordered steps.

  - Depending on the colour component cIdx, the variable refRecSamples specifying the reconstructed picture samples of the reference picture is derived as specified in the following.

If cIdx is equal to 0, refRecSamples is set equal to reconstructed picture sample array SL of picture refPicLX.

Otherwise, if cIdx is equal to 1, refRecSamples is set equal to the reconstructed chroma sample array SCb of picture refPicLX.

Otherwise (cIdx is equal to 2), refRecSamples is set equal to the reconstructed chroma sample array SCr of picture refPicLX.

The lists curNeighSampleListLX and refNeighSampleListLX specifying the neighbouring samples in the current picture and the reference picture are derived as specified in the following.

The variable numNeighSamplesLX specifying the number of elements in curNeighSampleListLX and in refNeighSampleLX is set equal to 0.

The variable leftNeighOffLX specifying the offset of the left neighbouring samples in curNeighSampleListLX and refNeighSampleLX is derived as

leftNeighOffLX = availFlagAboveRowLX ? 0 : nCS 	(G-154)

For i ranging from 0 to nCS−1, inclusive the following applies.

When availFlagAboveRowLX is equal to 1 the following applies.

curNeighSampleListLX[ i ] = curRecSamples[ xC + i][ yC − 1 ] 	(G-155)

refNeighSampleListLX[ i ] = refRecSamples[ xRLX + i ][ yRLX − 1 ]	(G-156)

numNeighSamplesLX += 1		(G-157)

When availFlagLeftColLX is equal to 1 the following applies

curNeighSampleListLX[ i + leftNeighOffLX ] = curRecSamples[ xC − 1][ yC + i ] 	(G-158)

refNeighSampleListLX[ i + leftNeighOffLX ] = refRecSamples[ xRLX − 1][ yRLX + i ] 	(G-159)

numNeighSamplesLX += 1		(G-160)

The derivation process for illumination compensation parameters as specified in subclause G.8.5.2.2.4.2 is invoked, with the list of neighbouring samples in the current picture curNeighSampleList, the list of neighbouring samples in the reference picture refNeighSample list, the number of neighbouring samples numNeighSamlesLX and the size of the current luma coding block nCSl as inputs and the illumination parameters icWeightLX, icOffsetLX and icShiftLX as outputs.

Derivation process for illumination compensation parameters

Inputs to this process are:

a list curSampleList specifying the current samples, ,

a list refSampleList specifying the reference samples,

a variable numSamples specifying the number of elements in curSampleList and refSampleList.

a bit depth of samples, bitDepth.

the size of the current luma coding block nCSl

Outputs of this process are:

a variable icWeight specifying a weight for illumination compensation,

a variable icOffset specifying a offset for illumination compensation,

a variable icShift specifying a bit shift for illumination compensation.

The variable icWeight is set equal to 1, the variable icOffset is set equal to 0, and the variable icShift is set equal to 0.

The variables sumRef, sumCur, sumRefSquare and sumProdRefCur are set equal to 0 and the following applies for i ranging from 0 to numSamples − 1, inclusive.

sumRef += refSampleList[ i ]		(G-161)

sumCur += curSampleList[ i ]		(G-162)

sumRefSquare += ( refSampleList[ i ] << 1 ) 		(G-163)

sumProdRefCur += ( refSampleList[ i ] * curSampleList[ i ] ) 	(G-164)

The variables precShift and precOffset specifying the bit shift and offset needed to restrict precision to 16 bit are derived as

precShift = Max( 0, bitDepth + Log2( nCSl) − 14 )		(G-165) 
precOffset =  1 << ( precShift − 1 )		(G-166)

The variables psSumRef, psSumCur, psSumRefSquare and psSumProdRefCur are derived as

psSumRef = ( precShift > 0 ) ? (sumRef + precOffset) >> precShift: sumRef	(G-167)

psSumCur = ( precShift > 0 ) ? (sumCur + precOffset ) >> precShift : sumCur	(G-168)

psSumRefSquare = ( precShift > 0 ) ? (sumRefSquare + precOffset ) >> precShift : sumRefSquare	(G-169)

psSumProdRefCur = ( precShift > 0 ) ? (sumProdRefCur + precOffset ) >> precShift : sumProdRefCur	(G-170)

The variable avgShift and avgOffset specifying the bit shift and offset needed for averaging are derived as

avgShift = Ceil( Log2( numSamples ) ) − precShift		(G-171)

avgOffset = 1 << ( avgShift − 1 )		(G-172)

When avgShift is equal to 0 the whole derivation process specified in this subclause terminates..

The variables numerDiv and denomDiv specifying numerator and denominator of a following divisions are derived as.

numerDiv= ( psSumProdRefCur <<  avgShift ) – psSumRef * psSumCur	(G-173)

denomDiv= ( psSumRefSquare <<  avgShift ) – psSumRef * psSumRef	(G-174)

The variables psShiftNumer and psShiftDenom specifying the bit shifts to restrict the precision of numerDiv and denomDiv to 15 bit and 6 bit, respectively are derived as

psShiftNumer = Max( 0, Ceil( Log2( Abs( numerDiv ) ) ) − 15)	(G-175)

psShiftDenom = Max( 0, Ceil( Log2( Abs( numerDiv ) ) ) − 6)	(G-176)

The variables psNumerDiv and psDenomDiv are derived as

psNumerDiv = numerDiv >> psShiftNumer 		(G-177)

psDenomDiv = denomDiv >> psShiftDenom 		(G-178)

The variable psIcWeight specifying the shifted weight for illumination compensation is derived as specified in the following.

If psDenomDiv is greater than 0, the following applies,

The value of variable divCoeff  is derived from Table G-5 depending on psDenomDiv.

The value of psIcWeight is derived as

psIcWeight = psNumerDiv * divCoeff 		(G-179)

Otherwise( psDenomDiv is less or equal to 0), psIcWeight is set equal to 0.

The variable icShift specifying a bit shift for illumination compensation is set equal to 13.

The variable invPsShift specifying the number of bits needed to shift psIcWeight back to a range of 16 bit precision is derived as

invPsShift = psShiftDenom – psShiftNumer + 15 – icShift	(G-180)

The variable invPsIcWeight specifying a weight for illumination compensation with 16 bit precision is derived as specified in the following

If invPsShift is less than 0, the following applies,

invPsIcWeight = Clip3( invPsShift << ( Abs( invPsShift ) ), −215, 215 )	(G-181)

Otherwise, ( invPsIcWeight is greater than or equal to 0), the following applies than 0

invPsIcWeight = Clip3( invPsShift << ( Abs( invPsShift ) ), −215, 215 )	(G-182)

The variable icWeight specifying a weight for illumination compensation with 7 bit precision is derived as specified in the following

If invPsIcWeight is greater than or equal to −26 and less than 26, the following applies.

icWeight = invPsIcWeight 		(G-183)

Otherwise, ( invPsIcWeight is less than −26 or greater than or equal to 26 ), the following applies.

decIcShift = Max( 0, Ceil(Log2( icWeight ) − 6 ) )	(G-184)

[Ed (GT): In software a function counting leading zero ones is utilized to derive decIcShift. Does this match with draft text?]

icWeight = invPsIcWeight >> decIcShift		(G-185)

icShift −= decIcShift		(G-186)

The variable icOffset specifying an offset for illumination compensation is derived as

- icOffset = ( psSumCur – ( ( icWeight*psSumRef ) >>  icShift ) + avgOffset ) >>  avgShift	(G-187)

Table G-5 – Specification of divCoeff depending on psDenomDiv

| psDenomDiv | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| divCoeff | 32768 | 16384 | 10923 | 8192 | 6554 | 5461 | 4681 | 4096 | 3641 | 3277 | 2979 | 2731 | 2521 |
| psDenomDiv | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| divCoeff | 2341 | 2185 | 2048 | 1928 | 1820 | 1725 | 1638 | 1560 | 1489 | 1425 | 1365 | 1311 | 1260 |
| psDenomDiv | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| divCoeff | 1214 | 1170 | 1130 | 1092 | 1057 | 1024 | 993 | 964 | 936 | 910 | 886 | 862 | 840 |
| psDenomDiv | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 |
| divCoeff | 819 | 799 | 780 | 762 | 745 | 728 | 712 | 697 | 683 | 669 | 655 | 643 | 630 |
| psDenomDiv | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 |  |
| divCoeff | 618 | 607 | 596 | 585 | 575 | 565 | 555 | 546 | 537 | 529 | 520 | 512 |  |

Inter-view residual prediction process

The process is only invoked if res_pred_flag is equal to 1.

Inputs to this process are:

a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

a variable nCS specifying the size of the current luma coding block,

variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,prediction list utilization flags, predFlagL0 and predFlagL1,

reference indices refIdxL0 and refIdxL1,

a (nPSW)x(nPSH) array predSamplesL of luma prediction samples,

two (nPSW / 2)x(nPSH / 2) arrays predSamplesCb and predSamplesCr of chroma prediction samples.

Output of this process are:

a variable refViewIdx specifying the view order index of the reference view disparity derivation,

a modified version of the (nPSW)x(nPSH) array predSamplesL,

a modified versions of the (nPSW / 2)x(nPSH / 2) arrays predSamplesCb and predSamplesCr.

The derivation process for a disparity vector as specified in subclause G.8.5.2.1.13 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPSW and nPSH, and the partition index partIdx as the inputs and the outputs are the view order index refViewIdx, the flag availableDV and the disparity vector mvDisp.[Ed. (GT) partIdx is missing as input to G.8.5.2.2.5].

[Ed. (GT). In software refViewIdx is set equal to the view order index of the view with minimal view_id difference compared to the view_id of the current view].

Let refResSamplesL be the (PicWidthInSamplesL)x(PicHeightInSamplesL) array of constructed luma residual samples for inter-coded coding units for the view component with ViewIdx equal to refViewIdx. Let refResSamplesCb and refResSamplesCr be the (PicWidthInSamplesL / 2)x(PicHeightInSamplesL / 2) arrays of constructed Cb and Cr residual samples, respectively, for inter-coded coding units for the view component with ViewIdx equal to refViewIdx.

When the flag availableDV is equal to 0 the whole decoding process of this sub-clause terminates.

The variable log2resPredDenom is set equal to 0 and the following ordered steps apply.

  - When predFlagL0 is equal to 1 and ViewIdx is not equal to the view order index of RefPicListL0[ refIdxL0 ], log2resPredDenom is set equal to log2resPredDenom + 1.

When predFlagL1 is equal to 1 and ViewIdx is not equal to the view order index of RefPicListL1[ refIdxL1 ], log2resPredDenom is set equal to log2resPredDenom + 1.

The variable log2MaxResPredDenom is derived by

log2MaxResPredDenom = ( ( predFlagL0 = = 1 ) && ( predFlagL1 == 1 ) ? 1 : 0 )	(G-188)

When log2resPredDenom is greater than log2MaxResPredDenom the whole decoding process of this sub-clause terminates.

For y proceeding over the values 0..(nPSH – 1) and x proceeding over the values 0..(nPSW – 1), the following ordered steps apply.

  - The variables xR0, xR1, w0, and w1 are derived by

xR0 = Clip3( 0, PicWidthInSamplesL – 1, xP + x + (mvDisp[ 0 ] >> 2 ) )	(G-189)
xR1 = Clip3( 0, PicWidthInSamplesL – 1, xP + x + (mvDisp[ 0 ] >> 2 ) + 1 )	(G-190)
w0 = 4 – mvDisp[0] + ( ( mvDisp[0] >> 2 ) << 2 )	(G-191)
w1 = mvDisp[0] − ( ( mvDisp[0] >> 2 ) << 2 )		(G-192)

The sample predSamplesL[ x, y ] is modified by

deltaL = ( w0 * refResSamplesL[ xR0, y ] + w1 * refResSamplesL[ xR1, y ] + 4 ) >> 3	(G-193)
predSamplesL[ x, y ] = predSamplesL[ x, y ] + ( deltaL >> log2resPredDenom )	(G-194)

For y proceeding over the values 0..(nPSH / 2 – 1) and x proceeding over the values 0..(nPSW / 2 – 1), the following ordered steps are specified:

  - The variables xR0, xR1, w0, and w1 are derived by

xR0 = Clip3( 0, PicWidthInSamplesL / 2 – 1, xP / 2 + x + (mvDisp[0] >> 3 ) )	(G-195)
xR1 = Clip3( 0, PicWidthInSamplesL / 2 – 1, xP / 2 + x + (mvDisp[0] >> 3 ) + 1 )	(G-196)
w0 = 8 – mvDisp[0] + ( (mvDisp[0] >> 3 ) << 3 )	(G-197)
w1 = mvDisp[0] − ( (mvDisp[0] >> 3 ) << 3 )		(G-198)

The sample predSamplesCb[ x, y ] is modified by

deltaCb = ( w0 * refResSamplesCb[ xR0, y ] + w1 * refResSamplesCb[ xR1, y ] + 8 ) >> 4	(G-199)
predSamplesCb[ x, y ] = predSamplesCb[ x, y ] + ( deltaCb  >> log2resPredDenom )	(G-200)

The sample predSamplesCr[ x, y ] is modified by

deltaCr = ( w0 * refResSamplesCr[ xR0, y ] + w1 * refResSamplesCr[ xR1, y ] + 8 ) >> 4	(G-201)
predSamplesCr[ x, y ] = predSamplesCr[ x, y ] + ( deltaCr  >> log2resPredDenom )	(G-202)

Decoding process for the residual signal of coding units coded in inter prediction mode

The specifications in subclause 8.5.3 apply.

Scaling, transformation and array construction process prior to deblocking filter process

The specifications in subclause 8.6 apply.

In-loop filter process

The specifications in subclause 8.7 apply.

Parsing process

The specifications in clause 9 apply with the following modifications

–	All references to the process specified in subclause 7.3 are replaced with references to the process specified in subclause G.7.3 .

–	All invocations of the process specified in subclause 9.1 are replaced with invocations of the process specified in subclause G.9.1.

Parsing process for Truncated Unary codes

The specifications in subclause 9.1 apply with the following modifications.

–	All references to the process specified in subclause 7.3 are replaced with references to the process specified in subclause G.7.3.

Parsing process for 0-th order Exp-Golomb codes

The specifications in subclause 9.2 apply with the following modifications .

–	All references to the process specified in subclause 7.3 are replaced with references to the process specified in subclause G.7.3.

–	All invocations of the process specified in subclause 9.2.1 are replaced with invocations of the process specified in subclause G.9.2.1.

Mapping process for signed Exp-Golomb codes

The specifications in subclause 9.2.1 apply with the following modifications.

–	All references to the process specified in subclause 9.2 are replaced with references to the process specified in subclause G.9.2

CABAC parsing process for slice data

The specifications in subclause 9.3 apply with the following modifications.

–	All references to the process specified in subclauses 7.3.6 to 7.3.13 are replaced with references to the process specified in subclauses G.7.3.6 to G.7.3.13

–	All invocations of the process specified in subclause 9.3.1 are replaced with invocations of the process specified in subclause G.9.3.1.

–	All invocations of the process specified in subclause 9.3.1.2 are replaced with invocations of the process specified in subclause G.9.3.1.2.

–	All invocations of the process specified in subclause 9.3.1.3 are replaced with invocations of the process specified in subclause G.9.3.1.3.

–	All invocations of the process specified in subclause 9.3.1.4 are replaced with invocations of the process specified in subclause G.9.3.1.4.

–	All invocations of the process specified in subclause 9.3.2 are replaced with invocations of the process specified in subclause G.9.3.2.

Initialization process

The specifications in subclause 9.3.1 apply with the following modifications.

–	All references to the process specified in subclauses 7.3.6 to 7.3.13 are replaced with references to the process specified in subclauses G.7.3.6 to G.7.3.13

–	All invocations of the process specified in subclause 9.3.1.1 are replaced with invocations of the process specified in subclause G.9.3.1.1.

–	All invocations of the process specified in subclause 9.3.1.2 are replaced with invocations of the process specified in subclause G.9.3.1.2.

Initialization process for context variables

The specifications in subclause 9.3.1.1 apply with the following modifications.

–	All references to the process specified in subclauses 7.3.6 to 7.3.13 are replaced with references to the process specified in subclauses G.7.3.6 to G.7.3.13

–	Table G-6 is appended to the end of Table 9-4.

–	Table G-7 to Table G-19 are appended to the end of the subclause.

Table G-6 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit() | dmm_flag | Table G-7 | 0 | 1 | 2 |
|  | dmm_mode | Table G-8 | 0 | 1 | 2 |
|  | wedge_full_tab_idx | Table G-9 | 0 | 1 | 2 |
|  | dmm_delta_end_flag dmm_delta_end_abs_minus1 | Table G-10 | 0 | 1 | 2 |
|  | dmm_dc_1_abs dmm_dc_2_abs | Table G-11 | 0 | 1 | 2 |
|  | res_pred_flag | Table G-12 | 0 | 1 | 2 |
|  | ic_flag | Table G-13 |  | 0 | 1 |
|  | mvp_l0_idx mpv_l1_idx | Table G-14 |  | 0..1 | 2..3 |
|  | sdc_flag | Table G-15 | 0..2 | 3..5 | 6..8 |
|  | sdc_residual_flag | Table G-16 | 0..1 | 2..3 | 4..5 |
|  | sdc_residual_sign_flag | Table G-17 | 0 | 1 | 2 |
|  | sdc_residual_abs_minus1 | Table G-18 | 0..19 | 20..39 | 40..59 |
|  | sdc_pred_mode | Table G-19 | 0..2 | 3..5 | 6..8 |

[Ed (GT). In software one table with different offsets is used for wedge_full_tab_idx, dmm_delta_end_flag, dmm_delta_end_abs_minus1, dmm_dc_1_abs and dmm_dc_2_abs ].

[Ed (FJ). In software the tables for sdc_residual_flag, sdc_sign_flag, sdc_residual_data and sdc_pred_mode are larger, but many entries are not used. Therefore, only used entries are defined here. Needs software fix. ].

Table G-7 – Values of variable initValue for dmm_flag ctxIdx

| Initialization variable | dmm_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table G-8 – Values of variable initValue for dmm_mode ctxIdx

| Initialization variable | dmm_mode |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table G-9 – Values of variable initValue for wedge_full_tab_idx ctxIdx

| Initialization variable | wedge_full_tab_idx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table G-10 – Values of variable initValue for dmm_delta_end_flag and dmm_delta_end_abs_minus1 ctxIdx

| Initialization variable | dmm_delta_end_flag dmm_delta_end_abs_minus1 |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table G-11 – Values of variable initValue for dmm_dc_1_abs and dmm_dc_2_abs ctxIdx

| Initialization variable | dmm_dc_1_abs dmm_dc_2_abs |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table G-12 – Values of variable initValue for res_pred_flag ctxIdx

| Initialization variable | res_pred_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table G-13 – Values of variable initValue for ic_flag ctxIdx

| Initialization variable | ic_flag |  |
| --- | --- | --- |
|  | 0 | 1 |  |
| initValue | 197 | 197 |  |

Table G-14 – Values of variable initValue for mvp_l0_idx and mvp_l1_idx ctxIdx

| Initialization variable | mvp_l0_idx and mvp_l1_idx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| initValue | 168 | 168 | 154 | 154 |

Table G-15 – Values of variable initValue for sdc_flag ctxIdx

| Initialization variable | sdc_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 0 | 0 | 64 | 0 | 0 | 0 | 64 | 0 | 0 |

Table G-16 – Values of variable initValue for sdc_residual_flag ctxIdx

| Initialization variable | sdc_residual_flag |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 251 | 255 | 0 | 0 | 56 | 0 |

Table G-17 – Values of variable initValue for sdc_residual_sign_flag ctxIdx

| Initialization variable | sdc_residual_sign_flag |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 255 | 255 | 0 | 252 | 55 | 0 |

Table G-18 – Values of variable initValue for sdc_residual_abs_minus1 ctxIdx

| Initialization variable | sdc_residual_abs_minus1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| initValue | 255 | 255 | 64 | 0 | 2 | 0 | 64 | 0 | 6 | 0 |
|  | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| initValue | 67 | 0 | 8 | 0 | 61 | 0 | 7 | 0 | 47 | 0 |
|  | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| initValue | 243 | 255 | 12 | 0 | 14 | 0 | 33 | 0 | 243 | 255 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| initValue | 2 | 0 | 66 | 0 | 0 | 0 | 63 | 0 | 1 | 0 |
|  | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| initValue | 12 | 0 | 50 | 0 | 14 | 0 | 27 | 0 | 0 | 0 |
|  | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| initValue | 239 | 255 | 14 | 9 | 27 | 0 | 0 | 0 | 239 | 255 |

Table G-19 – Values of variable initValue for sdc_pred_mode ctxIdx

| Initialization variable | sdc_pred_mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 9 | 255 | 70 | 0 | 9 | 255 | 70 | 0 | 9 |

Memorization process for context variables

The specifications in subclause 9.3.1.2 apply with the following modifications

–	All references to the process specified in subclauses 7.3.6 to 7.3.13 are replaced with references to the process specified in subclauses G.7.3.6 to G.7.3.13

Synchronization process for context variables

The specifications in subclause 9.3.1.3 apply with the following modifications

–	All references to the process specified in subclauses 7.3.6 to 7.3.13 are replaced with references to the process specified in subclauses G.7.3.6 to G.7.3.13

Initialization process for the arithmetic decoding engine

The specifications in subclause 9.3.1.4 apply.

Binarization process

The specifications in subclause 9.3.2 apply with the following modifications.

–	Table G-20 is appended to the end of Table 9-37.

Table G-20 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| --- | --- | --- | --- | --- | --- |
| dmm_flag | 0 | FL, cMax = 1 | 0 | Table G-7 | 0 |
|  | 1 |  | 0 | Table G-7 | 1 |
|  | 2 |  | 0 | Table G-7 | 2 |
| dmm_mode | 0 | FL, cMax = 7 | 0 | Table G-8 | 0 |
|  | 1 |  | 0 | Table G-8 | 1 |
|  | 2 |  | 0 | Table G-8 | 2 |
| wedge_full_tab_idx | 0 | FL, cMax = dmmWedgeletPatternListSize [Ed. (GT): Definition missing] | 0 | Table G-9 | 0 |
|  | 1 |  | 0 | Table G-9 | 1 |
|  | 2 |  | 0 | Table G-9 | 2 |
| dmm_delta_end_flag | 0 | FL, cMax = 1 | 0 | Table G-10 | 0 |
|  | 1 |  | 0 | Table G-10 | 1 |
|  | 2 |  | 0 | Table G-10 | 2 |
| dmm_delta_end_abs_minus1 | 0 | FL, cMax = 3 | 0 | Table G-10 | 0 |
|  | 1 |  | 0 | Table G-10 | 1 |
|  | 2 |  | 0 | Table G-10 | 2 |
| dmm_delta_end_sign_flag | na | FL, cMax = 1 | na | na | na, (Bypass) |
| dmm_dc_1_abs dmm_dc_2_abs | 0 | UEG0 [Ed. (GT) To be specified] | 0 | Table G-11 | 0 |
|  | 1 |  | 0 | Table G-11 | 1 |
|  | 2 |  | 0 | Table G-11 | 2 |
| dmm_dc_1_sign_flag dmm_dc_2_sign_flag | na | FL, cMax = 1 | na | na | na, (Bypass) |
| res_pred_flag | 0 | FL, cMax = 1 | 0 | Table G-12 | 0 |
|  | 1 |  | 0 | Table G-12 | 1 |
|  | 2 |  | 0 | Table G-12 | 2 |
| ic_flag | 1 | FL, cMax = 1 | 0 | Table G-13 | 0 |
|  | 2 |  | 0 | Table G-13 | 1 |
| mvp_l0_idx mvp_l1_idx | 1 | FL, cMax = 1 + multi_view_mv_pred_flag | 1 | Table G-14 | 0 |
|  | 2 |  | 1 | Table G-14 | 2 |
| sdc_flag | 0 | FL, cMax = 1 | 0 | Table G-15 | 0 |
|  | 1 |  | 0 | Table G-15 | 3 |
|  | 2 |  | 0 | Table G-15 | 6 |
| sdc_residual_flag | 0 | FL, cMax = 1 | 0 | Table G-16 | 0 |
|  | 1 |  | 0 | Table G-16 | 2 |
|  | 2 |  | 0 | Table G-16 | 4 |
| sdc_residual_sign_flag | 0 | FL, cMax = 1 | 0 | Table G-17 | 0 |
|  | 1 |  | 0 | Table G-17 | 2 |
|  | 2 |  | 0 | Table G-17 | 4 |
| sdc_residual_abs_minus1 | 0 | FL, cMax = dltFlag ? Floor( Log2( num_depth_values_in_dlt ) : BitDepthY | 9 | Table G-18 | 0 |
|  | 1 |  | 9 | Table G-18 | 20 |
|  | 2 |  | 9 | Table G-18 | 40 |
| sdc_pred_mode | 0 | U [Ed. (GT): in implementation 0 and 1 are switched. ] [Ed. (GT): Could be TU, cMax = 3] | 0 | Table G-19 | 0 |
|  | 1 |  | 0 | Table G-19 | 3 |
|  | 2 |  | 0 | Table G-19 | 6 |

Unary (U) binarization process

The specifications in subclause 9.3.2.1 apply.

Truncated unary (TU) binarization process

The specifications in subclause 9.3.2.2 apply.

Truncated Rice (TR) binarization process

The specifications in subclause 9.3.2.3 apply.

k-th order Exp-Golomb (EGk) binarization process

The specifications in subclause 9.3.2.4 apply.

Fixed-length (FL) binarization process

The specifications in subclause 9.3.2.5 apply.

Binarization process for part_mode

The specifications in subclause 9.3.2.6 apply.

Binarization process for coeff_abs_level_remaining

The specifications in subclause 9.3.2.7 apply.

Binarization process for intra_chroma_pred_mode

The specifications in subclause 9.3.2.8 apply.

Binarization process for inter_pred_idc

The specifications in subclause 9.3.2.9 apply.

Decoding process flow

The specifications in subclause 9.3.3 apply with the following modifications.

–	All references to the process specified in subclause 9.3.2 are replaced with references to the process specified in subclause  G.9.3.2.

Derivation process for ctxIdx

The specifications in subclause 9.3.3.1 apply with the following modifications.

–	Table G-21 is appended to the end of Table 9-37.

Table G-21 – Assignment of ctxIdxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| dmm_flag | 0 | na | na | na | na | na |
| dmm_mode | 0 | 0 | 0 | 0 | 0 | 0 |
| wedge_full_tab_idx | 0 | 0 | 0 | 0 | 0 | 0 |
| dmm_delta_end_flag | 0 | na | na | na | na | na |
| dmm_delta_end_abs_minus1 | 0 | 0 | 0 | 0 | 0 | 0 |
| dmm_dc_1_abs dmm_dc_2_abs | 0 | 0 | 0 | 0 | 0 | 0 |
| res_pred_flag | 0 | na | na | na | na | na |
| ic_flag | 0 | na | na | na | na | na |
| mvp_l0_idx, mvp_l1_idx | 0 | 1 | na | na | na | na |
| sdc_flag | 0,1,2 (subclause G.9.3.3.1.1) | na | na | na | na | na |
| sdc_residual_flag | 0,1 (subclause G.9.3.3.1.7) | na | na | na | na | na |
| sdc_residual_sign_flag | 0,1 (subclause G.9.3.3.1.7) | na | na | na | na | na |
| sdc_residual_abs_minus1 | 0,10 (subclause G.9.3.3.1.7) |  |  |  |  |  |
| sdc_pred_mode | 0 | 1 | 2 | na | na | na |

Derivation process of ctxIdxInc using left and above syntax elements

The specifications in subclause 9.3.3.1.1 apply with the following modifications.

–	Table G-22 is appended to the end of Table 9-38.

Table G-22 – Specification of ctxIdxInc using left and above syntax elements

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| sdc_flag | sdc_flag [ xL ][ yL ] | sdc_flag[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |

.Derivation process of ctxIdxInc for the syntax elements last_significant_coeff_x_prefix and last_significant_coeff_y_prefix

The specifications in subclause 9.3.3.1.2 apply.

Derivation process of ctxIdxInc for the syntax element coded_sub_block_flag

The specifications in subclause 9.3.3.1.3 apply.

Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

The specifications in subclause 9.3.3.1.4 apply.

Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag

The specifications in subclause 9.3.3.1.5 apply.

Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater2_flag

The specifications in subclause 9.3.3.1.6 apply.

Derivation process of ctxIdxInc using segment index

Input to this process is the luma location ( xC, yC ) specifying the top-left luma sample of the current luma coding block relative to the top-left sample of the current picture.

Output of this process is ctxIdxInc.

If IntraPredMode[ xC ][ yC ] is not equal to 0 and IntraPredMode[ xC ][ yC ] is not equal to 1, ctxIdxInc depends on the SdcSegmentIndex = 0..1, as specified in Table G-23.

Otherwise, the ctxIdxInc is equal to 0.

Table G-23 – Specification of ctxIdxInc using segment index

| Syntax element | IntraPredMode | SdcSegmentIndex | ctxIdxInc |
| --- | --- | --- | --- |
| sdc_residual_flag | IntraPredMode[ xC ][ yC ] == 0 ¦¦ IntraPredMode[ xC ][ yC ] == 1 | 0 | 0 |
|  | IntraPredMode[ xC ][ yC ] != 0 && IntraPredMode[ xC ][ yC ] != 1 | 0 | 0 |
|  |  | 1 | 1 |
| sdc_residual_sign_flag | IntraPredMode[ xC ][ yC ] == 0 ¦¦ IntraPredMode[ xC ][ yC ] == 1 | 0 | 0 |
|  | IntraPredMode[ xC ][ yC ] != 0 && IntraPredMode[ xC ][ yC ] != 1 | 0 | 0 |
|  |  | 1 | 1 |
| sdc_residual_abs_minus1 | IntraPredMode[ xC ][ yC ] == 0 ¦¦ IntraPredMode[ xC ][ yC ] == 1 | 0 | 0 |
|  | IntraPredMode[ xC ][ yC ] != 0 && IntraPredMode[ xC ][ yC ] != 1 | 0 | 0 |
|  |  | 1 | 10 |

Arithmetic decoding process

The specifications in subclause 9.3.3.2 apply with the following modifications.

–	All references to the process specified in subclause 9.3.3.1 are replaced with references to the process specified in subclause  G.9.3.1.1.

Arithmetic encoding process (informative)

The specifications in subclause 9.3.4 apply with the following modifications.

All references to the process specified in subclause 9.3.3.2 are replaced with references to the process specified in subclause  G.9.3.3.1.7.

Specification of bitstream subsets

The specifications in clause 10 apply.

Profiles and levels

The specifications in Annex A apply, with the following modification:

TBD

Byte stream format

The specifications in Annex B apply.

3D-HEVC hypothetical reference decoder

The specifications in Annex F.13 apply.

3D-HEVC SEI messages

The specifications in Annex D apply, with the following modification :

TBD

Video usability information

The specifications in Annex E apply.

