| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B1006 |
| --- | --- |

| Title: | Work Plan in 3D Standards Development |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Work Plan |  |  |
| Author(s) or Contact(s): | Jens-Rainer Ohm Dmytro Rusanovskyy Anthony Vetro Karsten Müller | Email: | ohm@ient.rwth-aachen.de |
| Source: | JCT-3V |  |  |

_____________________________

# Abstract

Several 3D video coding methods are currently developed by the Joint Collaborative Team on 3D Video Extension Development (JCT-3V), a joint working group of MPEG and ITU-T VCEG. This section provides a summary of them, expected timeline and references for further readings.

# 3D video with AVC-based coding technology

## MVC compatible extension including depth (MVC+D)

The main target of this work item is to enable 3D enhancements while maintaining MVC stereo compatibility.  The project is targeting to encapsulate coded texture and depth maps into a single bitstream with minimal changes to MVC specification.  Macroblock-level changes to the AVC or MVC syntax, semantics and decoding processes are not considered.

The texture data coding to be compatible with MVC profiles (MVC High and Stereo High) and a new profile (profile id 138) is to be introduced for coding of multiview depth map data. Multiview texture and depth data are coded independently, and no restrictions on strict association between texture and depth map is imposed. In addition to this, texture and depth map data can be coded at different spatial resolution, mostly reduced resolution for depth map data is considered, see Figure 1.

Fig. 1: Visualization of MVD data with depth map at reduced resolution,

In a current stage of the development, most of the development efforts is targeting the 3D application specific SEI message design and streamlining syntax changes.

MVC+D data format is expected to be used in applications that require a backward compatibility with MVC.

The specification of this extension (which is 14496-10:2012/Amd.2 as ISO/IEC standard), has reached DAM status by July 2012 and is expected to be finalized in January 2013 (FDAM).

## AVC compatible video-plus-depth extension (3D-AVC)

Another ongoing JCT-3V development is a 3D video extension of H.264/AVC, referred here to as 3D-AVC. A goal of this development is a 3DV coding system that is able to benefit from a wide deployment of H.264/AVC-based 2D video services and from widely available hardware and software implementations of H.264/AVC, and therefore would be able to facilitate the rapid deployment of 3D video services in a short time. Thus, specification requires that only the base texture view is compatible with H.264/AVC however, the compatibility of dependent texture views to MVC may optionally be provided.

Similarly to MVC+D, 3D-AVC supports coding of texture and depth map data at different spatial resolutions, particularly with reduced resolution for depth map comparing to the texture. In addition to this, 3D-AVC exploits inter-component dependencies between texture and depth and introduces joint coding of texture and depth data. Inter-component dependency is exploited in two directions, as shown in Figure 2. Depth map of the base view is coded with information extracted from the coded texture view, whereas dependent texture views are coded with information available from associated depth which is coded prior to the texture.

Fig. 2: High Level flow-chart of the 3D-AVC encoder

The technology studied in respect to the 3D-AVC seems to provide a significant improvement in coding efficiency in comparison to the MVC+D, recently reported gain is about 23% of dBR on the total bitrate.

A PDAM specification has been issued in October 2012, DAM is expected by April 2013 and finalization of the extension (FDAM) is planned for November 2013.

# 3D video with HEVC-based coding technology

High-efficiency video coding (HEVC) is a new video coding standard for which the base version (2D) is currently under development in JCT-VC. A primary usage of HEVC is seen in the area of high and ultra-high definition (UHD) video. Many HD displays already provide the capability for stereo rendering, and it can be expected that the increased resolution and display size of UHD displays makes them even more suitable for such purposes. Beyond that, the improved compression capability of HEVC (expected half bit rate with same quality compared to AVC High profile) makes it attractive for the introduction of stereo; for example, it can be expected that by using mechanisms that exploit the redundancy between views, HEVC would be able to encode full resolution stereo at significantly lower rates than AVC would be capable just for one (monoscopic) view at the same quality and resolution.

Similarly to AVC-based projects, JCT-3V is conducting study of two 3DV solutions that are using the HEVC coding technology. The first is multi-view extension of HEVC, so called MV-HEVC and another is depth enhanced HEVC-based full 3DV codec, 3D-HEVC. This section provides a short review on these developments.

## Multiview extension of HEVC (MV-HEVC)

To achieve higher compression efficiency than simulcast coding of multiview texture data, HEVC-based multi-view coding is currently investigated in  JCT-3V. Similarly to the MVC extension of AVC, MV-HEVC targets to utilize a redundancy between different texture views captured from the same scene. It is assumed that a base view of MV-HEVC will be compatible to monoscopic HEVC and can thus be interpreted by legacy HEVC decoders. Such an approach could be defined by extending the high-level syntax appropriately, and by rearrangement of decoded picture buffers to store the reference pictures as needed, without any changes to the core of the coding layer below the level of coded tree blocks (CTB). Under the current specification, MV-HEVC is not indented to include coding of depth data. If however justified by market requirements, carriage of depth data could be included in a similar fashion as in AVC’s MVC depth extension (i.e. carrying depth maps as an independent monochrome channel without change of low-level coding tools).

Currently, MV-HEVC is at Working Draft stage.

The PDAM specification for MV-HEVC is planned for January 2013, DAM is expected by July 2013 and the finalization of the extension (FDAM) to be available by January 2014.

### Multiview plus depth coding with HEVC-based technology (3D-HEVC)

With the advancement of UHD display technology and the capability to present stereoscopic views with higher resolution, the demand for higher compression capability is again expected to arise together with advanced display features supported by depth maps. This could again be achieved by joint compression of video texture and depth maps.

Similarly to 3D-AVC concept described above, the 3D-HEVC design exploits inter-component dependencies between texture and depth and introduces joint coding of texture and depth data. However only texture-to-depth dependency is allowed, thus coded texture information may be utilized for coding of depth map data, but not wise versa. See a flowchart of 3D-HEVC coding design in Figure 3. In addition to this, this development is concentrated on development of more sophisticated depth map coding deployed in the sub-CTB level of the core codec.

Similar to the requirements for 3D-AVC, it is anticipated that the market adoption of such an approach would require providing about 25% of total bit-rate reduction in comparison to MV-HEVC.

The launch of a draft amendment for closely integrated video plus depth solutions will hardly happen before early 2014, e.g. a PDAM specification for January 2014, DAM by July 2014 and finalization of the extension (FDAM) could be expected for January 2015.

Fig. 3: High level flowchart of the 3D-HEVC encoder

## Scalable stereo / Multiview

For monoscopic video coding, plans exist to develop a suite of tools for scalable coding for HEVC (which will be executed by JCT-VC, the joint team that is currently defining the base spec for HEVC). Here, the application of particularly view scalability and spatial scalability are considered highly beneficial in the evolution of 3D services, as they allow for backward-compatible extensions for more views, and/or enhancing the resolution of views in a way that decoding by legacy devices is possible. Therefore, it is likely that tools which JCT-VC will develop for scalable coding of monoscopic video are similarly applicable to frame-compatible stereo and MV-HEVC. If specific additional definitions are necessary for this purpose, this will be conducted by JCT-3V in close coordination with JCT-VC. A timeline for this activity would need to follow the scalable HEVC extension, such that finalization could not be expected earlier than end of 2014 or beginning of 2015.

For AVC, the possibility for a spatial scalable extension of frame-compatible formats (Multi-resolution Frame Compatible Stereo, “MFC”) is currently investigated by MPEG and intended to be standardized in JCT-3V (upon negotiation by the parent bodies). The intended timeline is PDAM by January 2013, DAM by April 2013 and FDAM by November 2013.