| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 October 2012 | Document: JCT3V-B1103 |
| --- | --- |

| Title: | Description of Core Experiment 3 on Inter-Component Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 92130 ISSY LES MOULINEAUX France | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange.com |
|  | S. Kamp Monzastraße 4c 63225 Langen Germany | Tel: Email: | +49 6103 766271 steffen.kamp@eu.panasonic.com |
| Source: | CE coordinators |  |  |

# Introduction

This Core Experiment relates to inter-component prediction. During the 2nd JCT-3V meeting, contributions have been proposed taking benefit of inter-component predictions. Two of them were considered for further study in a Core Experiment (JCT3V-B0085 and JCT3V-B0156). The goal of this CE is to evaluate them in the context of the 3D-HTM 5.0 under the common test conditions.

# Participants List

| Participant | Person, Email address | Status (P/C/-) |
| --- | --- | --- |
| Intel | Y.-J. Chiu (yi-jen.chiu@intel.com) Z. Deng (zhipin.deng@intel.com) W. Zhang (wenhao.zhang@intel.com) | ? |
| MediaTek | Y.-W. Chen (yiwen.chen@mediatek.com) J.-L. Lin (jl.lin@mediatek.com) Y.-W. Huang (yuwen.huang@mediatek.com) S. Lei (shawmin.lei@mediatek.com) | P |
| Orange Labs | J. Jung (joelb.jung@orange.com) E. Mora (elie.mora@orange.com) | C |
| Panasonic | S. Kamp (steffen.kamp@eu.panasonic.com) | C |
| Samsung | B. T. Oh (byung.oh@samsung.com) K.-J. Oh (kwanjung.oh@samsung.com) | P |

Table 1: participant list

# Experiment description

JCT3V-B0085 - [MediaTek] - Removal of picture buffers for motion parameter inheritance

In the contribution JCT3V-B0085, it is proposed to replace motion parameter inheritance (MPI) by adding one additional candidate, texture merging candidate, into the merge candidate set for the depth merge/skip mode coding.

Similar to the concept of inter-view merging candidate, the proposed texture merging candidate directly reuse the motion vectors (MVs) and reference index of the corresponding texture block as a merging candidate in depth coding.

As shown in Figure 1, the corresponding texture block is selected as the 4x4 block (C3 block) located to the right bottom of the center of the current PU in the corresponding texture picture.

The goal of this CE experimentation is to evaluate this method in the latest 3D-HTM version. Different texture blocks used to derive texture merging candidate will also be further studied.

Figure 1. The derivation of corresponding texture block

Table 2: Summary of the experiments

| Experiment | Tool involved |  | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- |
|  | MPI | Texture Merging candidate |  |  |
| Anchor | ON | OFF |  |  |
| M1 | OFF | ON (C3 block) | MediaTek | Orange Labs |
| M2 | OFF | ON (Different texture blocks) | MediaTek | Xx |

JCT3V-B0156 - [Samsung] - 3D-CE3.h related results on Improved Motion Parameter Inheritance

3D-HTM MPI description:

MPI targets the motion parameter sharing between texture and depth CU. Its detailed functionality differs according to the following three cases:

- Case 1 : Size of Texture CU = size of Depth CU

- Case 2 : Size of Texture CU > size of Depth CU

- Case 3 : Size of Texture CU < size of Depth CU

In case 1 and 2, only the motion data are inherited from its corresponding texture signal.

In case 3, motion data + CU/PU subdivisions are inherited.

MPI is turned on only when the corresponding texture CU has motion information.

Proposed method 1:

In case 1, 2 and 3, only the motion data are inherited from its corresponding texture signal. CU/PU subdivisions are no more inherited.

Proposed method 2:

MPI is turned on even when one of the subCUs is intra coded.

It is proposed to derive the motion data from other subCU in depth map, for example, left or upper subCU, which are already derived from its corresponding texture subCU. If all subCU are intra-coded, then MPI mode is off.

When the corresponding texture subCU has no motion data:

- Step 1: If available, derive the motion data from its left subCU in depth map.

- Step 2: If step 1 does not hold, then derive the motion data from its upper subCU in depth map.

- Step 3: If step 2 does not hold, then find out the first available motion data by scanning subCUs in texture with raster-scanning order.

Table 3: Summary of the experiments

| Experiment | Proposed Method |  | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- |
|  | Modified MPI | Handle subCUs with no motion |  |  |
| Anchor | OFF | OFF |  |  |
| M1 | ON | OFF | Samsung | Orange Labs |
| M2 | ON | ON | Samsung | Xx |

# Test Sequences / Test Points / SW Configuration

Simulation results will be generated for sequences and coding conditions specified in the document JCT3V-A1100. 3D-HTM 5.1 will be used for evaluation.

# Definition of Performance Measurement Criteria

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-Rate).

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the default configuration)

- Decoding time (compared to the default configuration)

# Time-line

| T1 | Nov. 2nd | Final description of proposals sent to the coordinators |
| --- | --- | --- |
| T2 | Nov. 9th | Upload of Final CE description document |
| T3 | Availability of 3D-HTM5.1 Dec. 3rd | Start of tool integration |
| T4 | Sept. 28th | Completion of tool integration, test configuration announcement Start of testing |
| T5 | Oct. 8th | Testing complete Reporting of all results to CE3 participants complete |
| T6 | Official document submission deadline Oct. 9th | Input documents uploaded |
| T7 | T6 + 3 days Oct. 12th | CE3 summary report uploaded |

Table 4: Time-line