| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012 | Document: JCT3V-B1106 |
| --- | --- |

| Title: | Description of Core Experiment 6 (CE6) on Depth Intra Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Philipp Merkle | Email: | philipp.merkle@hhi.fraunhofer.de |
| Source: | CE coordinator |  |  |

_____________________________

# Abstract

This document defines Core Experiment 6 on depth intra coding (CE6) to be performed for the 3rd JCT-3V meeting.

# Introduction

The goal of this Core Experiment (CE) is to investigate the methods for depth intra coding proposed to the 2nd JCT-3V meeting. This CE covers AVC-based proposals under sub-experiment CE.A and HEVC-based proposals under sub-experiment CE.H. All tools under test for the 3rd JCT-3V meeting are in sub-experiment CE6.H. The corresponding test model is described in JCT3V-B1005, including intra prediction modes and residual coding methods specific to depth, such as DMM, region boundary chain, and SDC.

The objective of tools under test in CE6.H is simplification of both the implementation and processing complexity of existing depth intra coding methods.

# Participants

| Participant | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| HHI | P. Merkle | philipp.merkle@hhi.fraunhofer.de |  | X |
| Aachen Univ. | F. Jäger | jaeger@ient.rwth-aachen.de | X | X |
| Qualcomm | X. Zhao | xinzhao@qti.qualcomm.com | X | X |
| LGE | H. Liu | hongbin.liu@lge.com | X | X |
| JDL | S. Wang | sqwang@jdl.ac.cn | X | X |
| GIST | Y. Song | ysong@gist.ac.kr | X | X |
|  | Y.-S. Ho | hoyo@gist.ac.kr |  |  |
| MediaTek | Y.-W. Chen | yiwen.chen@mediatek.com |  |  |
|  | Y.-L. Chang | yulin.chang@mediatek.com |  |  |

(P=proponent, C=cross checker)

# Tools under Test for sub-experiment CE6.H

## Simplified Depth Coding with an optional Depth Lookup Table [JCT3V-B0036]

Contribution JCT3V-B0036 proposes two methods for depth intra coding. One is SDC (simplified depth coding), which adds an alternative coding path for each CU and is restricted to four prediction modes (DC; planar, DMM 1 and 2). Moreover, the resulting residual is not coded using transform and quantization for SDC modes, but a depth offset value in the pixel domain is coded for each segment of a block. The second method is DLT (depth lookup table), which maps depth values to valid depth values of the original depth map. The DLT is constructed based on an initial analysis of the input depth map and is then coded in the SPS.

In this CE the following modifications to the SDC and DLT methods will be tested:

- signalling of DLT in the slice header instead of the SPS.

- reduction of contexts used for SDC signalling.

## Distortion calculation simplification for Depth Modeling Mode (DMM) 3 [JCT3V-B0053]

Contribution JCT3V-B0053 proposes further simplifications of Depth Modeling Mode (DMM) 3. During the decoder search in DMM 3, for each given Wedgelet pattern, the cost is no longer derived by the SAD of reconstructed co-located texture block and the predicted texture block generated based on the Wedgelet pattern. Therefore, the process to generate the predicted texture block based on the Wedgelet pattern is removed. Instead, the intersections between the Wedgelet pattern partition line and the current depth PU boundaries are used to derive two depth reference sample pairs. The cost of the Wedgelet pattern is calculated by only the differences of these two depth reference sample pairs.

In this CE the described modification of DMM mode 3 will be tested.

## Simplification of Depth Modeling Mode 3 [JCT3V-B0064]

Contribution JCT3V-B0064 proposes to further simplify Depth Modeling Mode (DMM) 3. Based on the method of JCT2-A0098, this tool further simplifies the Wedgelet pattern search at decoder by signalling the best pattern in the predefined search range. In this way, the decoder does not need to perform any Wedgelet pattern search.

In this CE the described modification of DMM mode 3 will be tested.

## Simplified wedgelet search for DMM mode 3 [JCT3V-B0122]

JCT3V-B0122 proposes to reduce the number of Wedgelet distortion measures required in DMM mode 3. This tool applies a 1D filter at each side (top, left, bottom and right) of the co-located texture luma block (CTLB). The coordinates where intensity changes are maximum can be found, denoted as difference points. They are adjusted to take Wedgelet resolution into account. With the estimated four difference points, six Wedgelets with distinct orientations can be defined. Distortion calculation is performed only on Wedgelets where start/end positions are difference points.

In this CE, the tool will be tested as an alternative approach to DMM mode 3 simplifications adopted from JCT3V-B0039 as follows:

- JCT3V-B0122 enabled / JCT3V-B0039 DMM mode 3 modifications disabled.

# Core Experiment Conditions

## Software

Sub-experiment CE6.H will use the 3D-HTM-5.0 software that is recommended in JCT3V-B1100. Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

## Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configuration and conditions that are recommended in JCT3V-B1100. Moreover, proponents and cross checkers are required to provide simulation results for the following two configurations:

- Random access configuration as specified in JCT3V-B1100.

- Intra-only configuration (configuration files to be provided by CE coordinator).

## Evaluation of CE Results

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this core experiment. A corresponding Excel sheet for reporting the simulation results will be provided by the CE coordinator.

Coding Performance Measurements: PSNR values shall be computed for the decoded texture views relative to original texture views and for the synthesized views relative to the synthesized views based on uncompressed texture and depth. The 4-point BD-rate according to common test conditions is used to report the results for random access as well as intra-only configuration.

Complexity measurements: For the complexity measurement, the reference software and the reference software with the proposed method implemented will be executed on the same machine with the same configuration and the computational time will be measured. A time ratio will then be calculated between the reference software and the reference software with the proposed method implemented.

## Timeline

2012/11/09		Final description of CE and proposals available
2012/11/19		Reference software 3D-HTM-5.0 available 
2012/01/03		Make source code, simulation results and draft text available for all proponents and cross-					checks
2012/01/10		Document registration and upload deadline (register documents and upload simulation 					and cross check results for CE and related proposals)
2013/01/15		CE summary report available