| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0010 |
| --- | --- |

| Title: | JCT-3V AHG Report: Inter Component Dependencies |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | J. Jung J.-L. Lin (co-chairs) | Email: | joelb.jung@orange.com jl.lin@mediatek.com |
| Source: | Coordinators |  |  |

_____________________________

# Abstract

This report summarizes the activities of the Inter Component Dependencies ad hoc group (AHG10) between the 2nd JCT-3V meeting and the 3rd JCT-3V meeting, and lists the related input documents submitted to this meeting according to their topics.

# Mandates

- Identify current inter component dependencies in the 3D-ATM and 3D-HTM softwares

- Evaluate the impact of these dependencies on the current design

# Reflector discussions

AHG4 used the main JCT-VC reflector: jct-3v@lists.rwth-aachen.de.

No relevant email discussions have taken place on the email reflector.

# Related contributions

The list of AHG10 related documents is as follows:

| JCT3V-C0023 | A+H | 3D-CE3: Summary report of Core Experiment 3 on Inter-Component Prediction | CE coordinator |
| --- | --- | --- | --- |
| JCT3V- C0091 | A | Improved inside view motion prediction for 3D-AVC | Samsung, Qualcomm |
| JCT3V-C0095 | H | 3D-CE3.h results on Improved Motion Parameter Inheritance | Samsung |
| JCT3V-C0136 | A | 3D-CE3.a related: Completed inside-view motion prediction in 3D video coding | MediaTek |
| JCT3V-C0137 | H | 3D-CE3.h results on removal of parsing dependency and picture buffers for motion parameter inheritance | MediaTek |
| JCT3V-C0170 | H | CE2.h-related: Flexible Coding Order (FCO) and depth-first-based disparity derivation | Nokia |

Cross-checks:

| JCT3V- C0176 | H | 3D-CE3.h: Cross check on removal of parsing dependency and picture buffers for motion parameter inheritance (JCT3V-C0137) | Qualcomm |
| --- | --- | --- | --- |
| JCT3V- C0177 | H | 3D-CE3.h: Cross check on Improved Motion Parameter Inheritance (JCT3V-C0095) | Qualcomm |
| JCT3V- C0186 | A | 3D-CE3.a related: Crosscheck results on Improved inside-view motion prediction (JCT3V-C0091) | NTT |

# 3D-ATM Study

## Identify tools with inter component dependencies and impact on the design (such as additional buffers, delay, parsing changes, etc)

Tool 1: View Synthesis Prediction – VSP

Description:

In VSP, the decoded texture and depth information from the independent base view are used to compute a prediction signal for the texture and depth component of the dependent views. Forward warping VSP uses the depth map and texture from other view to synthesize the reference of current view. Backward warping VSP uses the depth map in the current view and the texture from other view to synthesize the reference of current view.

Impact:

Dependency: the depth map is directly used to synthesize the VSP reference with the texture.

Tool 2: Inside View Motion Prediction – IMVP

Description:

In IVMP, the motion information, including mb_type, sub_mb_type, reference indices and motion vectors of the co-located MB in texture view component is inherited by the depth view component of the same view. IVMP is enabled when all of the corresponding four texture macroblocks are coded as skip or inter 16x16 or inter 16x8 or inter 8x16.

Impact:

Dependency: motion information of the depth is directly used for the texture.

Tool 3: Direction-separate Motion Vector Prediction in Inter Mode

Description:

Direction-separate motion vector prediction is applied in Inter mode.

If the target reference picture is a temporal prediction picture, the motion vector prediction is derived as the median of the motion vectors of the adjacent blocks A, B, and C.

If the target reference picture is an inter-view prediction picture, the inter-view motion vectors of the neighboring blocks are employed for the inter-view prediction. If an inter-view motion vector is unavailable, a disparity vector which is derived from the maximum depth value of four corner depth samples within the associated depth block is used. The motion vector predictor is then derived as the median of the inter-view motion vector of the adjacent blocks A, B, and C.

Impact:

Dependency: disparity vector computed from depth samples to predict texture MV.

Tool 4: Inter-view Motion Vector Prediction in Skip/Direct Mode

Description:

In Skip and Direct mode, a MVP candidate is derived based on a predefined derivation order: inter-view candidate and the three spatial candidates derived from the neighboring blocks A, B, and C. The central point of the current block in the dependent view and its disparity vector are used to find the corresponding point in the base view. After that, the MV of the corresponding block is used as the inter-view candidate of the current block. The disparity vector (DV) can be derived from the neighboring blocks or the depth value of the central point. Specifically, if one of the neighboring blocks has a DV, the DV is used as the disparity. Otherwise, the depth-based disparity vector which is derived using the depth value of the central point and camera parameters is used.

Impact:

Dependency: disparity vector computed from depth samples to get the inter-view candidate for the texture

## Total improvement provided by this package of tools

Not evaluated. Tool 3 cannot be disabled by a macro.

# 3D-HTM Study

## Identify tools with inter component dependencies and impact on the design (such as additional buffers, delay, parsing changes, etc)

Tool 1: Intra DMM Mode 3: Inter-component prediction of Wedgelet partitions

Description:

The basic principle of this mode is to predict the Wedgelet partition from a texture reference block, namely the co-located block of the associated video picture.

Impact:

Dependency: texture wedgelet partition used to predict depth wedgelet partition.

Tool 2: Intra DMM Mode 4: Inter-component prediction of Contour partitions

Description:

The basic principle of this mode is to predict a Contour partition from a texture reference block by inter-component prediction. Like for the inter-component prediction of a Wedgelet partition pattern, the reconstructed luminance signal of the co-located block of the associated video picture is used as a reference,

Impact:

Dependency: contour partition used to predict depth contour partition.

Tool 3: Motion Parameter Inheritance

Description:

This mode allows inheritance of the treeblock subdivision into CUs and PUs and their corresponding motion parameters from the video signal. If the MPI mode is indicated at a higher level of the depth map coding tree, corresponding to a CU size that is larger than the CU size that is used for the video signal, the CU/PU subdivision, together with the corresponding motion data, is inherited from the video signal.

Impact:

Dependency: motion information and quadtree subdivision of the texture used for the depth.

Tool 4: QuadTree limitation and predictive coding

Description:

3D-HTM splits a given CTB into smaller CUs, based on RD optimized decisions. A corresponding quadtree (QT) is obtained for the texture, and another one for the depth. This tool performs a prediction of the depth QT, using the texture QT as the predictor, assuming that the level of partitioning of the depth is lower or equal to the level of partitioning of the texture, for any CU. The output of this prediction is transmitted.

Impact:

Dependency: between the parsing of the texture and the parsing of the depth. The syntax elements corresponding to the QT of the texture are stored and used during the depth parsing process.

## Total improvement provided by this package of tools

Reference: 3D-HTM without inter-component tools.

Tested: reference with inter-component tools enabled.

### Results reported in the new template

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | 0,0% | 0,0% | -2,7% | -0,9% | -1,5% | 75,3% | 105,9% | 92,1% |
| Kendo | 0,0% | 0,0% | 0,0% | 0,0% | -3,8% | -3,0% | -3,2% | 76,3% | 108,4% | 93,9% |
| Newspapercc | 0,0% | 0,0% | 0,0% | 0,0% | -3,7% | -0,9% | -1,8% | 71,2% | 106,0% | 99,5% |
| GhostTownFly | 0,0% | 0,0% | 0,0% | 0,0% | -1,9% | -1,9% | -1,9% | 84,5% | 105,0% | 97,3% |
| PoznanHall2 | 0,0% | 0,0% | 0,0% | 0,0% | -3,0% | -2,0% | -2,3% | 84,8% | 104,6% | 98,4% |
| PoznanStreet | 0,0% | 0,0% | 0,0% | 0,0% | -1,7% | -1,3% | -1,5% | 87,6% | 99,3% | 90,0% |
| UndoDancer | 0,0% | 0,0% | 0,0% | 0,0% | -2,3% | -2,0% | -2,1% | 83,2% | 105,4% | 97,0% |
| 1024x768 | 0,0% | 0,0% | 0,0% | 0,0% | -3,4% | -1,6% | -2,2% | 74,2% | 106,8% | 95,1% |
| 1920x1088 | 0,0% | 0,0% | 0,0% | 0,0% | -2,2% | -1,8% | -1,9% | 85,0% | 103,6% | 95,6% |
| Average HTM5 | 0,0% | 0,0% | 0,0% | 0,0% | -2,7% | -1,7% | -2,0% | 80,2% | 104,9% | 95,4% |
| Average HTM4 | 0,0% | 0,0% | 0,0% | 0,0% | -0,6% | -1,8% | -1.4% | 105,7% | 107,3% | 102,6% |

Disabling the inter-component tools comes with an increase of the encoding runtime and a loss in BD rate for both video and synthesized views. These gains have been significantly improved between HTM4 and HTM5.

# Recommendations

The Inter Component Dependencies ad hoc group recommends:

- To review all related input contributions.

- To encourage further work for improving existing tools or propose new ones.