| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0021 |
| --- | --- |

| Title: | 3D-CE1 Summary Report on View Synthesis Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Shinya Shimizu Fabian Jäger | Tel: Email: | shimizu.shinya@lab.ntt.co.jp jaeger@ient.rwth-aachen.de |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

This document is the summary report of Core Experiment 1 (CE1) on view synthesis prediction. This Core Experiment investigates coding methods based on view synthesis prediction, which uses reconstructed texture of the independent view and depth information of the independent or the dependent view to construct a synthesized prediction signal for coding of the dependent views.

# Participants

| Participant | Contact | Email | Proponent |
| --- | --- | --- | --- |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp | P/C |
| Mitsubishi Electric | Dong Tian | tian@merl.com | P/C |
| RWTH Aachen University | Fabian Jäger | jaeger@ient.rwth-aachen.de |  |
| MediaTek | Yu-Lin Chang Yi-Wen Chen Yu-Wen Huang | yulin.chang@mediatek.com yiwen.chen@mediatek.com yuwen.huang@mediatek.com | P/C |
| Qualcomm | Xin Zhao Jewon Kang | xinzhao@qualcomm.com jewonk@qti.qualcomm.com | P/C |
| Nokia | Dmytro Rusanovskyy | Dmytro.Rusanovskyy@nokia.com | P/C |
| Zhejiang University | Yin Zhao | zhaoyin@zju.edu.cn | P/C |
| LG Electronics | Sehoon Yea | sehoon.yea@lge.com |  |
| Samsung | Jin Young Lee | jinyoung79.lee@samsung.com |  |
| ETRI | Gun Bang | gbang@etri.re.kr |  |
| KyungHee University | Kyung Yong Kim | kimky@khu.ac.kr |  |

(P = proponent, C = cross checker)

# Tools under Investigation

The tools to be investigated in this Core Experiment use the decoded texture and depth information from the independent view and/or the already coded dependent view to generate prediction signals for the texture and depth component of the dependent views.

For the case of view synthesis prediction (VSP), the view synthesis algorithm is part of the coding loop and therefore a normative coding tool.

## CE1.a: View Synthesis Prediction based on 3D-ATM

3D-ATM supports a view synthesis prediction using a backward warping, where displacement shift is determined for each 2x2 block and disparity compensation is applied.

The tools to be investigated can be categorized into the following 5 categories. TABLE I summarizes the combinations of the tools to be studied.

At the Shanghai meeting, a sub-MB level VSP skip signaling proposed in JCT3V-B0054 was adopted into 3D-AVC. However, during the integration, VSP reference pictures were accidentally removed from the reference picture lists.

- Block size for warping (JCT3V-B0094)

- The disparity vector derivation method from a depth block

- Removal of VSP picture from the reference picture list (JCT3V-B0054)

- Skipping motion vector coding (JCT3V-A0107)

- Block-based VSP using forward warping (JCT3V-B0121)

TABLE I: Combinations of the tools under investigation.

|  | VSP block size | Disparity vector derivation | VSP ref. pic. in RefPicList | MV coding | Warping direction | Sub-MB VSP skip/direct | Input contributions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor | 2x2 |  | Y | Skipped | Bwd | N | ATM v6.1 with QC_GVSP_B0054=0 |
| Test 0 | 2x2 |  | Y | Skipped | Bwd | Y | JCT3V-C0053 |
| Test 1 | 4x4 | Max of 4 corners | Y | Skipped | Bwd | Y | JCT3V-C0130 |
| Test 2 | 4x4 | Others | Y | Skipped | Bwd | Y | JCT3V-C0130 |
| Test 3 | 2x2 |  | N | n/a | Bwd | Y | JCT3V-C0053 |
| Test 4 | 2x2 |  | Y | Coded | Bwd |  |  |
| Test 5 (1+3) | 4x4 | Max of 4 corners | N | n/a | Bwd | Y | JCT3V-C0130 |
| Test 6 (1+4) | 4x4 | Max of 4 corners | Y | Coded | Bwd |  |  |
| Test 7 (2+3) | 4x4 | Others | N | n/a | Bwd | Y | JCT3V-C0130 |
| Test 8 (2+4) | 4x4 | Others | Y | Coded | Bwd |  |  |
| Test 9 | 2x2 |  | Y | Skipped | Fwd |  |  |
| Test 10 | 4x4 | Max of 4 corners | Y | Skipped | Fwd |  |  |

### Input contributions:

A total of 12 input contributions were submitted in this category, among which were 2 CE proposals and 3 CE-related proposals. JCT3V-C0053 provides CE results on Test 0 and 3, and JCT3V-C0130 provides those on Test1, 2, 5 and 7 as presented in TABLE I.

Regarding the related contributions, JCT3V-C0063 addresses several concerns on the current design of view synthesis prediction in 3D-AVC, JCT3V-C0114 proposes a disparity vector derivation process, JCT3V-C0169 proposes a simplification of BVSP, and JCT3V-C0200 provides a study result on the view synthesis prediction in 3D-AVC.

## CE1.h: View Synthesis Prediction based on 3D-HTM

3D-HEVC have no VSP although the basic VSP scheme proposed in JCT3V-B0102 was adopted as SW. The basic VSP scheme was implemented on the top of HTM 5.1 and released via the SVN server. The SW supports a forward warping based view synthesis, VSP signaling using reference picture index, VSP skip using the merging candidate list, and sequence/slice level signaling.

There were two proposals on the complexity reduction at the Shanghai meeting. JCT3V-B0103 proposed that a backward projection based VSP and JCT3V-B121 proposed that a forward warping based approach.

Disparity vector derivation using depth information is also considered as a part of VSP. JCT3V-B0090 proposed to enhance the NBDV by extracting a more accurate disparity vector from the coded depth map.

### Input contributions:

A total of 14 input contributions were submitted in this category, among which were 5 CE proposals and 4 CE-related proposals. The input contributions can be roughly categorized as follows:

- VSP framework

C0104 (NTT/MERL), C0171 (Nokia) – Basic framework

C0104 (NTT/MERL), C0163 (HHI) – VSP skip

- Simplification

C0087 (Zhejiang University) – Forward projection based

C0100 (NTT), C0152 (MERL), C0171 (Nokia) – Backward projection based

- Disparity vector derivation

C0112 (LG), C0131 (MediaTek) – For inter-view motion/residual prediction

C0132 (MediaTek) – For disparity vector prediction

# Summary of Proposals & Results

## CE1.a

Test 0: Sub-MB level VSP skip/direct flag (adopted at the Shanghai meeting)

Contribution: JCT3V-C0053 (Qualcomm) (Cross-Checked by JCT3V-C0203)

TABLE II: Test 0 results (Calculated from the data in JCT3V-C0053)

Test 1: Test 0 + 4x4 block basis warping using maximum depth value

Contribution: JCT3V-C0130 (MediaTek) (Cross-Checked by JCT3V-C0173)

TABLE III: Test 1 results

Test 2: Test 0 + 4x4 block basis warping using average depth value

Contribution: JCT3V-C0130 (MediaTek) (Cross-Checked by JCT3V-C0173)

TABLE IV: Test 2 results

Test 3: Test 0 + removal of VSP reference pictures from the reference picture lists

Contribution: JCT3V-C0053 (Qualcomm) (Cross-Checked by JCT3V-C0173 and JCT3V-C0203)

TABLE V: Test 3 results

Test 5: Test 3 + 4x4 block basis warping using maximum depth value

Contribution: JCT3V-C0130 (MediaTek) (Cross-Checked by JCT3V-C0173 and JCT3V-C0182)

TABLE VI: Test 5 results

Test 7: Test 3 + 4x4 block basis warping using average depth value

Contribution: JCT3V-C0130 (MediaTek) (Cross-Checked by JCT3V-C0182)

TABLE VII: Test 7 results

TABLE VIII: Summary of the results on CE1.a

|  | Warping block size | Disparity vector derivation | VSP ref. pic. in RefPicList | Sub-MB skip/direct flag | Coded PSNR, Total bitrate | Synthesis PSNR, Total bitrate | Decoder time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor | 2x2 |  | Y | N |  |  |  |
| JCT3V-C0053 | 2x2 |  | Y | Y | 0.02% | -0.01% |  |
| JCT3V-C0130 | 4x4 | Maximum | Y | Y | -0.03% | -0.05% | 97.48% |
| JCT3V-C0130 | 4x4 | Average | Y | Y | 0.04% | 0.07% | 96.84% |
| JCT3V-C0053 | 2x2 |  | N | Y | -0.17% | -0.16% |  |
| JCT3V-C0130 | 4x4 | Maximum | N | Y | -0.19% | -0.14% | 95.25% |
| JCT3V-C0130 | 4x4 | Average | N | Y | 0.44% | 0.58% | 98.94% |

## CE1.a related

- JCT3V-C0063 [Qualcomm] Comments on view synthesis prediction in 3D-AVC (No Cross-Check)

This document includes discussions on the following aspects related to view synthesis prediction in 3D-AVC: 1) constraining the smallest view synthesis blocks to 4x4, 2) sizes for bi-directional predicted blocks, 3) constraining the number of VSP blocks in level definitions, 4) synthetic view component, and 5) signaling of view synthesis prediction. On the last aspect, it is proposed to consider a trade-off between flexibility and implementation friendliness.

- JCT3V-C0114 [Zhejiang Univ.] 3D-CE1.a and CE2.a related: synthesized disparity vectors for BVSP and DMVP (Cross-Checked by JCT3V-C0183)

In this proposal, a 4x4-block-based disparity vector field is synthesized from the depth map of a reference view to provide the disparity vectors required by BVSP and DMVP. More specifically, for each 4x4 texture block in the reference view, one depth value X is obtained as the maximum of the depth values associated with four corners of the block. The corresponding 4x4 block in the current-view texture is then located by the disparity vector D converted from X, which results in a 4x4-block-based disparity vector field. The synthesis process utilizes two low-complexity techniques, 1) one-loop warping and hole filling and 2) directional warping without z-buffering. For BVSP, the disparity vector for each 4x4 texture block in current view is derived as the synthesized disparity vector of the block (i.e., BVSP uses 4x4-based disparity compensation). For DMVP, the disparity vector for an MxN texture block is derived as the maximum of the disparity vectors of four (or fewer) 4x4 corner blocks (if MxN<8x8).

- JCT3V-C0169 [Nokia] CE1.a-related: Simplification of BVSP in 3DV-ATM (No Cross-Check)

This contribution presents simulation results produced with 3DV-ATM v6.1 and configurable block sizes of BVSP. The BVSP implemented with 4x4 blocks produced identical average results as the 2x2 implementation (-0.01% of dBR), whereas BVSP with 8x8 block sizes reduced coding performance by 0.15% of dBR on average. In the tested configurations, a single disparity vector was derived for a target block size through the normative 3D-AVC disparity derivation method (i.e., the maximum out of 4 corner samples). As a result of this study, it is proposed to adopt BVSP implementation with the 8x8 block size in the 3D-AVC specification and in the common test conditions.

- JCT3V-C0200 [Samsung] Study on view synthesis prediction in 3D-AVC (Cross-Checked by JCT3V-C0212)

At the last meeting, generalized view synthesis prediction (GVSP) mode (JCT3V-B0054) and 4x4 VSP skip mode (JCT3V-B0094) were proposed. Both the methods achieved about 0.5~0.6% bit rate reduction. The GVSP mode allows sub-block level signaling of VSP and removes VSP reference form the reference picture list. The 4x4 VSP skip mode increases the VSP block size from 2x2 to 4x4 and it does not require the reference picture list index as in the GVSP mode. It was discussed that probably the gain for both the methods are mainly coming from not sending the reference picture index. This contribution reports how much gain comes from the sub-block level signaling of VSP in JCT3V-B0054 and the VSP block size increase to 4x4 in JCT3V-B0094. The results demonstrate that benefits of the sub-block level signaling of VSP and 4x4 block size in the VSP skip mode are negligible in terms of the performance.

## CE1.h

- JCT3V-C0087 [Zhejiang Univ.] 3D-CE1.h: Forward Warping Block-based View Synthesis Prediction (Cross-Checked by JCT3V-C0180)

This contribution describes the implementation of the Forward warping Block-based View Synthesis Prediction (FBVSP) on HTM. FBVSP estimates a window in base view, i.e., reference block, by only using the dependent view depth map. Then pixels in the reference block are warped to a target block in dependent view to be prediction samples. The overall bitrate savings on the coded and synthesized views are +0.4% for XGA sequences and -1.6% for HD sequences compared to HTM-5.1 and the decoding time is 106.5% for all sequences on average, where VSP is turned on for all frames of dependent-view texture and depth. The maximum bitrate savings of texture on dependent left and right views are -15.0% and -13.9%, respectively. In comparison to HTM-5.1-VSP, the overall bitrate savings on the coded and synthesized views are 0.0% for XGA sequences and +0.1% for HD sequences while the decoding time is 55.0% of HTM-5.1-VSP.

- JCT3V-C0100 [NTT] CE1.h: Backward Projection based View Synthesis Prediction using Derived Disparity Vector (Cross-Checked by JCT3V-C0144)

This contribution proposes a backward projection based View Synthesis Prediction (B-VSP). B-VSP allows generating synthetic picture only for VSP coded blocks. The depth map for VSP coded block is generated by disparity compensation prediction from a reference view, where disparity vectors are derived from the neighbouring already-coded blocks. The average bitrate savings for the coded views are +0.5% for XGA sequence, -1.0% for HD sequences, and -0.4% for all sequences. The decoding time increase is about 3% on average for all sequences. The maximum bitrate savings of texture on the dependent left and right views are -10.8% and -7.9%, respectively, for Undo_Dancer sequence.

- JCT3V-C0104 [NTT/MERL] CE1.h: View Synthesis Prediction using Forward Warping (Cross-Checked in the other CE proposals)

View synthesis prediction (VSP) is a technique to remove interview redundancies among video signal from different viewpoints, in which synthetic pictures are first generated and then used as references to predict a current picture. A forward warping based VSP scheme has been studied and was adopted as a branch version of HTM. In this document, we presented the VSP scheme which is implemented on the top of HTM 5.1. The overall bitrate savings are -0.8% and -0.7% for coded and synthesized views, respectively. The maximum bitrate savings of texture on the dependent left and right views are -14.2% and -13.4%, respectively, for Undo_Dancer sequence.

- JCT3V-C0131 [MediaTek] CE1.h: Depth-oriented Neighboring Block Disparity Vector (DoNBDV) with virtual depth retrieval (Cross-Checked by JCT3V-C0181)

In current HTM, the neighboring block disparity vector (NBDV) mode is used to replace the original predicted depth map (PDM) for inter-view motion prediction. In this contribution, a new estimated disparity vector – depth oriented neighboring block disparity vector (DoNBDV) is proposed to enhance the accuracy of the NBDV by utilizing the coded depth map. By referring to the NBDV and the coded depth information, the inter-view information can be predicted more accurately without maintaining a whole frame buffer like the predicted depth map. The experimental results reportedly show that 1.6% and 1.8% BD-Rate gains are achieved for video 1 and video 2, 0.5% BD-Rate gains is achieved for coded and synthesized views while applying the DoNBDV in the AMVP and the merge mode.

- JCT3V-C0152 [MERL] CE1.h: Backward View Synthesis Prediction using Neighbouring Blocks (Cross-Checked by JCT3V-C0172)

View synthesis prediction (VSP) is a technique that warps a picture from a neighboring viewpoint to the current viewpoint for prediction purposes. Depth information is used to perform the warping. In order to avoid the high complexity introduced by forward-warping VSP (FVSP), a backward-warping VSP (BVSP) approach is proposed. The proposed BVSP approach uses the neighboring blocks to derive a depth block to perform the backward warping operation. It is proposed that a new merging candidate indicating the BVSP mode be added in the merging candidate list. The contribution reports an average bitrate saving of 1.2% for video PSNR vs. video bitrate, and 1.0% for coded & synthesis PSNR vs. total bitrate, with a 3% increase in decoding time.

## CE1.h related

- JCT3V-C0112 [LG] CE1.h related: Adaptive method for Depth Oriented Neighboring Block Disparity Vector (No Cross-Check)

In the last meeting, Depth Oriented Neighboring Block Disparity Vector (DoNBDV) was proposed. The disparity vector candidates for merge and AMVP could be refined by the method. In the proposal, they were generated from the maximum value of the coded depth. However, It is not clear why the maximum value of the depth was used for the derivation of disparity. In this contribution, the Most Frequent Disparity (MFD) value is used adaptively for some cases instead of the disparity derived from the maximum value of the depth block. These methods are applied to the AMVP mode and the merge mode. Compared to DoNBDV, experimental results reportedly show -0.6% and 0.1% BD-Rate changes for video 1 and video 2, -0.1% BD-Rate gains is achieved for coded and synthesized views. Compared to HTM-5.0.1, experimental results reportedly show -2.0% and -1.5% BD-Rate changes for video 1 and video 2 with encoding time 102.0% and decoding time 102.8%.

- JCT3V-C0132 [MediaTek] 3D-CE1.h related: Depth-oriented disparity vector predictor (DoDVP) (Cross-Checked by JCT3V-C0217)

In current HTM, the neighboring block disparity vector (NBDV) mode is used to replace the original predicted depth map (PDM) for inter-view motion prediction. In this contribution, a depth oriented disparity vector predictor (DoDVP) is proposed to enhance the accuracy of the estimated disparity vector by utilizing the coded depth map in the same view while flexible coding order is enabled. By referring to the coded depth information, the inter-view information can be predicted more accurately. The experimental results reportedly show that 1.8% and 2.4% BD-Rate gains are achieved for video 1 and video 2, 0.7% BD-Rate gains is achieved for coded views over video bitrate while applying the DoDVP in the AMVP and the merge mode.

- JCT3V-C0163 [HHI] 3D-CE1.h related: Results on the VSP-mode (No Cross-Check)

This contribution proposes a view synthesis prediction (VSP) mode for dependent texture views which is signaled by a flag without coding additional information for the VSP mode. It is reported that the coding gain for non-base texture views is -25.9% BD-rate and -25.3% BD-rate, for Video PSNR versus total bitrate it is -0.9% BD-rate, for synthesized PSNR versus total bitrate it is -4.5% BD-rate and for coded and synthesized PSNR versus total bitrate it is -3.4% BD-rate. The reported encoding time is 98.7% and the decoding time is 201.5%.

- JCT3V-C0171 [Nokia] CE1.h-related: Backward VSP for 3D-HEVC (Cross-Checked by JCT3V-C0153)

This document proposes to utilize the disparity information derived from the depth of the same view for Backward View Synthesis Prediction utilized for texture coding. Simulation results show that the proposed scheme reduces the bitrate for coded texture by 1.4% of dBR on average and up-to 4% on average for synthetic sequences. The gain of block-based implementation of BVSP (4x4 block size) is 1.1% of dBR for the coded texture. However, it should be noted that the reconfiguration of the coding order leads to a change in the VSO settings, which in turn introduced a penalty on depth coding and penalized the results for synthesised views. As a result, proposed scheme produces 0.7% of dBR increase for synthesised sequences, and 0.1% of dBR increase of C2 case. It is believed that a proper adjustment of VSO parameters would resolve this problem. Considering the negligible computational complexity and the observed gain for texture coding, the contribution proposes to integrate the BVSP solution to 3D-HTM for its FCO configuration.

Summary of CE1.h and CE1.h-related

There are two different kinds of techniques although they have the same concept that depth information is utilized as disparity information in order to improve coding efficiency. The first category is to predict pixel information like the other prediction modes. TABLE IX gives a rough summary of the proposals in this category.

The second category is to improve the prediction of disparity vectors which are utilized for merge modes including inter-view motion prediction, AMVP, and possibly inter-view residual prediction.

The same virtual depth map derivation process is proposed in JCT3V-C0100, JCT3V-C0152, JCT3V-C0131, and JCT3V-C0112.

TABLE IX: Summary of the results as pixel prediction on CE1.h and CE1.h-related

|  | Warping Direction | Block-based | Derivation of depth | Usage of VSP | video PSNR /total bitrate | synth PSNR /total bitrate | Dec time | Anchor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JCT3V-C0087 | Fwd | Y |  | Merge/RefIdx | -0.8% | -0.7% | 106.5% | CTC |
| JCT3V-C0100 | Bwd | Y | NBDV | Merge/RefIdx | -0.4% | 0.6% | 102.9% | CTC |
| JCT3V-C0104 | Fwd | N |  | Merge/RefIdx | -0.8% | -0.7% | 150.8% | CTC |
| JCT3V-C0152 | Bwd | Y | NBDV | Merge/RefIdx | -1.0% | -0.9% | 103.3% | CTC |
| JCT3V-C0163 | Fwd | N |  | New mode | -0.9% | -4.5% | 201.5% | Non-CTC |
| JCT3V-C0171 | Bwd | Y | FCO (Co-located) | Merge/RefIdx | 1.4% | 0.7% |  | Non-CTC |

TABLE X: Summary of the results as disparity vector prediction on CE1.h and CE1.h-related

|  | Derivation of depth | Representative depth | video PSNR /total bitrate | synth PSNR /total bitrate | Dec time | Anchor |
| --- | --- | --- | --- | --- | --- | --- |
| JCT3V-C0131 | NBDV | Maximum | -0.6% | -0.5% | 102.8% | CTC |
| JCT3V-C0112 | NBDV | Adaptive | -0.5% | -0.4% | 99.5% | CTC |
| JCT3V-C0132 | Co-located (FCO) | Maximum | 1.9% | 1.2% | 101.1% | Non-CTC |

Note: Anchor is HTM v5.1 except for JCT3V-C0112 where anchor is HTM v5.0.1

# Recommendations

...