| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0023 |
| --- | --- |

| Title: | 3D-CE3: Summary report of Core Experiment 3 on Inter-Component Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Core Experiment Report |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 92130 ISSY LES MOULINEAUX France | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange.com |
|  | S. Kamp Monzastraße 4c 63225 Langen Germany | Tel: Email: | +49 6103 766271 steffen.kamp@eu.panasonic.com |
| Source: | CE Coordinators |  |  |

_________________________

# Introduction

This document summarizes the activities of the Core Experiment CE3 on Inter-Component Prediction. During the 2nd JCT-3V meeting, contributions have been proposed taking benefit of inter-component predictions. Two of them were considered for further study in a Core Experiment (JCT3V-B0085 and JCT3V-B0156). The goal of this CE is to evaluate them in the context of the 3D-HTM 5.0 under the common test conditions.

The experiments relate to 3D-HTM only. 7 companies were involved in the CE (2 of them as contributors).

# Document List

|  | Tester 1 | Tester 2 (cross-check) | x-check |
| --- | --- | --- | --- |
| M1 | MediaTek – JCT3V-C0137 | Qualcomm – JCT3V-C0176 | ? |
| M2 & M3 | Withdrawn |  |  |
| S1 | Samsung – JCT3V-C0095 | Qualcomm – JCT3V-C0177 | ? |
| S2 | Withdrawn |  |  |

Table 1: document list and x-check status.

# Experiments and results

JCT3V-C0137 - [MediaTek] - Removal of parsing dependency and picture buffers for motion parameter inheritance

This contribution is related to previous contribution JCT3V-B0085.

It is proposed to replace motion parameter inheritance (MPI) by adding one additional candidate, called texture merging candidate, into the merge candidate, set for the depth merge/skip mode coding.

Similar to the concept of inter-view merging candidate, the proposed texture merging candidate directly reuse the motion vectors (MVs) and reference index of the corresponding texture block as a merging candidate in depth coding.

As shown in Figure 1, the corresponding texture block is selected as the 4x4 block (C3 block) located to the right bottom of the center of the current PU in the corresponding texture picture.

Figure 1. The derivation of corresponding texture block

Table 2 shows the proposed merge candidate set for texture and depth.

| Merge candidate set in the proposed scheme |  |
| --- | --- |
| Texture | Depth |
| 1. Inter-view candidate 2. Spatial candidate 3. Temporal candidate 4. Additional candidate | 1. Texture candidate 2. Spatial candidate 3. Temporal candidate 4. Additional candidate |

Table 2: proposed merge candidate set.

JCT3V-C0095 - [Samsung] - 3D-CE3.h results on Improved Motion Parameter Inheritance

This contribution is related to previous contribution JCT3V-B0156.

3D-HTM MPI description:

MPI targets the motion parameter sharing between texture and depth CU. Its detailed functionality differs according to the following three cases:

- Case 1 : Size of Texture CU = size of Depth CU

- Case 2 : Size of Texture CU > size of Depth CU

- Case 3 : Size of Texture CU < size of Depth CU

In case 1 and 2, only the motion data are inherited from its corresponding texture signal.

In case 3, motion data + CU/PU subdivisions are inherited.

MPI is turned on only when the corresponding texture CU has motion information.

Proposed method:

It is proposed to disable CU/PU subdivisions inheritance in case 3.

| Experiment | video 0 | video 1 | video 2 | video only | video PSNR / total bitrate (New Template) | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M1 | 0,0% | 0,0% | 0,0% | 0,0% | -0,2% | -0,2% | -0,2% | 97,4% | 98,1% | 99,3% |
|  |  |  |  |  |  |  |  | x | x | X |
| S1 | 0,0% | 0,0% | 0,0% | 0,0% | 0.0% | -0,1% | 0,0% | 98,5% | 99,9% | 99,7% |
|  |  |  |  |  |  |  |  | x | x | x |

Table 3: Summary of the experiments (second line corresponds to the cross-check).

# Conclusion

The CE recommends reviewing both contributions.