| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0027 |
| --- | --- |

| Title: | 3D-CE7: Summary report on Coded Depth Representation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Krzysztof Wegner | Tel: Email: | +48 61 665 3893 kwegner@multimedia.edu.pl |
| Source: | JCT-3V CE Coordinators |  |  |

_____________________________

# Abstract

This document is the summary report of Core Experiment 7: Coded Depth Representation. CE7 was established on the 2nd JCT-3V Meeting in Shanghai to investigate the methods improved for coded depth representation (linear and non-linear) in 3D video coding [1]. Only one proposal was made during this meeting period. Summary of the proposed tool and the cross check results are reported.

# Participants

| Participant | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| Samsung | I. Lim | ilsoon.lim@samsung.com | X |  |
| Nokia | D. Rusanovskyy | Dmytro.Rusanovskyy@nokia.com |  | X |
| Poznan | K. Wegner | kwegner@multimedia.edu.pl |  | X |
| MediaTek | Yu-Lin Chang Jian-Liang Lin | yulin.chang@mediatek.com jl.lin@mediatek.com |  | X |

(P=proponent, C=crosss checker)

# List of input documents

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| Samsung | JCT3V-C0094 | CE7.a Improved Nonlinear Depth Representation | Proposal |
| Poznan | JCT3V-C0166 | 3D-CE7.a Cross check of Improved Nonlinear Depth Representation by Poznan University of Technology | Cross check |

# Summary Proposals

## JCT3V-C0094 [Samsung] Improved Nonlinear Depth Representation 
   Cross-checked by Poznan (JCT3V-C0166)

In the proposal the nonlinear depth representation (NDR) adopted in the 3DV-ATM is modified. Although the present NDR has good coding gain, there is still chance to improve error-resilience and coding gain. Currently, to decide whether NDR is enable or not, analysis of the first frame of depth map is made and such decision is final and fixed to the whole sequence. Such one-time on/off decision could suffer from a kind of scene change that often happens and is not efficient throughout a sequence. In the proposal, the nonlinear_depth_representation_enable_flag was added, which indicates on/off decision on every frame basis. Thus, decision whether NDR is enabled or not and the on/off decision is explicitly signaled to the decoder every slice. In sub-proposal 1, the on/off signaling was written inside the slice header (SH) but the LUT tables related to the nonlinear representation is written in SPS (EHP profile) or in SEI (HP profile) as before. In sub-proposal 2, the on/off signaling was written inside the depth parameter set(DPS) but the LUT tables related to the nonlinear representation is written in SPS (EHP profile) or in SEI (HP profile) as before.

The current NDR is prune to center disparity fluctuation. Because it checks just one time at the beginning of sequence and then on/off decision is fixed for the whole sequence. This proposal, aims to make NDR more resilient to every frame by explicitly signalling frame by frame.

# Simulation Results

Overview of the simulation results for proposed method JCT3V-C0094 in two variants are presented in Table 1. All of the results was cross checked by Poznan and results matched.

Table 1. Overview of the simulation results for Improved Nonlinear Depth Representation

|  |  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| SH Variant | EHP profile | -0,11 | 0,00 | -0,66 | 0,08 | -0,80 | 0,04 | -0,32 | 0,01 |
|  | HP profile | 0,00 | 0,00 | -0,61 | 0,09 | -0,81 | 0,04 | -0,36 | 0,01 |
| DSP variant | EHP profile | -0,10 | 0,00 | -0,41 | 0,07 | -0,81 | 0,04 | -0,27 | 0,01 |
|  | HP profile | 0,00 | 0,00 | -0,61 | 0,09 | -0,81 | 0,04 | -0,36 | 0,01 |

# Conclusions

The modified nonlinear depth representation was implemented in two variants. The best variant archives gains of 0.80% on decoded views and 0.32% on rendered views in EHP Profile and 0.81% on decoded views and 0.36% on rendered views in HP Profile in comparison with 3DV-ATM ver. 6.0 under the common test conditions [3] without the encoding and decoding time increase.

# References

[1]	K. Wegner, “Description of Core Experiment 7 (CE7) on Coded Depth Representation” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 Doc. JTC3V-B1107, 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012.

[2]	I. Lim, H.-C. Wey, D.-S. Park, “3D-CE7.a Improved Nonlinear Depth Representation” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 Doc. JTC3V-C0094, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

[3]	K. Wegner, O. Stankiewicz, “3D-CE7.a Cross check of Improved Nonlinear Depth Representation by Poznan University of Technology” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 Doc. JTC3V-C0166, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

[4]	D. Rusanovskyy, K. Mueller, A. Vetro, "Common Test Conditions of 3DV Core Experiments", Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 Doc. JTC3V-B1100, 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012.