| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0031r1 |
| --- | --- |

| Title: | MVC+D: on target output views |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela and Dmytro Rusanovskyy Nokia Research Center Visiokatu 3, 33720 Tampere, Finland Ying Chen and Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +358 7180 08000 firstname.lastanme@nokia.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Nokia Corporation and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to indicate whether a depth view or a texture view or both for each target output view is included in the 3DVC operation point. It is asserted that such indication requires syntax changes in the sequence parameter set 3DVC extension, the video usability information extension, the 3DVC view scalability information SEI message, and the 3DVC scalable nesting SEI message. Related changes are proposed for the main-level decoding process and for the sub-bitstream extraction process.

In addition, the contribution proposes the following changes:

- It is proposed to remove the capability of signalling texture-only operation points in the 3DVC view scalability information SEI message.

- It is proposed to add the information whether a texture view is coded with AVC/MVC NAL unit header and if a depth view is coded with AVC/MVC NAL unit header in the operation point signalling of the 3DVC view scalability information SEI message.

Revision 1 attempts to integrate the proposed text from JCT3V-0057 to the sub-bitstream extraction proposed in this contribution as decided in the JCT-3V meeting on 19th Januery, 2013.

# Introduction

MVC+D supports the unpaired multiview-video-plus-depth (MVD) data format, where either a depth view or a texture view or both can be present for a certain viewpoint identified by a specific view_id value.  One advantage of the unpaired MVD format is that it requires less memory and processing (hence potentially a lower level) than the conventional MVD format for the same views.

At the moment, MVC+D does not support signalling of operation points where the target output views would have only one of the texture and depth views present (if the bitstream contained both). Signalling of such unpaired MVD operation points could be useful for many use cases, including:

- Content encoded previously is adapted in the sender or in a middle-box for example as response to:

  - Received information on the receiver’s display configuration revealing that not all depth views are needed for efficient depth-image-based rendering (DIBR).

  - Bitrate adaptation due to prevailing network conditions.

- Live or pre-encoded content is adapted differently for multiple receivers due to different display configurations in receivers and/or due to different channel characteristics available for the receivers. The adaptation may be performed by the sender and/or by a middle-box.

It is therefore proposed to add the indication of whether a depth view or a texture view or both for each target output view is included in the 3DVC operation point.

# Overview of the proposal

We reviewed the MVC+D specification text and found that the following syntax structures need changes to indicate whether a depth view or a texture view or both is included in a target output view of an operation point:

- Sequence parameter set 3DVC extension (which specifies operation points for level signalling).

- Video usability information extension (which specifies operation points for HRD parameters signalling)

- 3DVC view scalability information SEI message (which specifies operation points for associating different signalled characteristics with them).

- 3DVC scalable nesting SEI message (which specifies operation points to which  the nested SEI messages apply).

For each of the above-mentioned syntax structures, the proposed changes are similar: it is proposed to add two flags, one to indicate the inclusion of the depth view and another to indicate the inclusion of the texture view, for each view_id value that is used to specify the target output views and/or the operation point. At least one of the flags for a particular view_id value is required to be equal to 1. The approach to use two flags was already used to indicate the presence of texture and depth views in the sequence parameter set 3DVC extension structure.

As the VUI extension is proposed to be changed compared to that of MVC, there a new syntax structure, 3DVC VUI parameters extension, is proposed.

The specification text is proposed to be changed throughout to indicate that a target output view may include a depth view, a texture view, or both the depth view and the texture view.

In the main-level decoding process (subclause I.8) the requirement on having the same set of view order indexes present in each access unit is proposed to be removed. As depth view components can be coded as frames, when texture view components are coded as fields, some access units containing a texture view component for a complementary view component pair may not have depth view components present. If for a certain view order index only depth view is present, then that view order index might not be present in each access unit.

The sub-bitstream extraction process is proposed to be modified to be able to take a list of view_id values for depth views as input (referred to as viewIdDepthTargetList). If the inputs to the sub-bitstream extraction process are depthPresentFlagTarget equal to 1 and viewIdDepthTargetList is not present, then the sub-bitstream extraction process is “paired”, i.e. viewIdTargetList is used for both texture and depth views. If the inputs to the sub-bitstream extraction process are depthPresentFlagTarget equal to 1 and viewIdDepthTargetList is present as input, then the sub-bitstream extraction process is “unpaired”, i.e. viewIdTargetList is used for texture views and viewIdDepthTargetList is used for depth views.

It is proposed to remove the capability of signalling texture-only operation points in the 3DVC view scalability information SEI message. This signalling has been indicated with op_with_depth_flag[ i ] equal to 0, which seems unnecessary, as the view scalability information SEI message could be used instead and would provide better compatibility with existing MVC implementations.

It is proposed to add the information whether a texture view is coded with AVC/MVC NAL unit header and if a depth view is coded with AVC/MVC NAL unit header in the operation point signalling of the 3DVC view scalability information SEI message. Syntax- and semantics-wise the SEI message could also be used for bitstreams conforming to the upcoming 3D-AVC amendment. Sub-bitstreams of 3D-AVC bitstreams may also conform to MVC+D, MVC, or AVC. However, the SEI message includes the profile information, from which the use of AVC/MVC NAL unit header could be concluded, only optionally. Hence, it may be useful for middle-boxes to know whether a texture view conforms to AVC/MVC or AVC-3D and whether a depth view conforms to AVC/MVC or AVC-3D.

# Proposed specification text changes

## Definitions

I.3.18	3DVC operation point: A 3DVC operation point is an operation point, for which each target output view includes both texture and depth a texture view or a depth view or both a texture view and a depth view.

## Sequence parameter set and VUI related changes

| seq_parameter_set_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| num_level_values_signalled_minus1 | 0 | ue(v) |
| for( i = 0; i <= num_level_values_signalled_minus1; i++ ) |  |  |
| level_idc[ i ] | 0 | u(8) |
| num_applicable_ops_minus1[ i ] | 0 | ue(v) |
| for( j = 0; j <= num_applicable_ops_minus1[ i ]; j++ ) |  |  |
| applicable_op_temporal_id[ i ][ j ] | 0 | u(3) |
| applicable_op_num_target_views_minus1[ i ][ j ] | 0 | ue(v) |
| for( k = 0; k <= applicable_op_num_target_views_minus1[ i ][ j ]; k++ ) { |  |  |
| applicable_op_target_view_id[ i ][ j ][ k ] | 0 | ue(v) |
| applicable_op_depth_flag[ i ][ j ][ k ] | 0 | u(1) |
| applicable_op_texture_flag[ i ][ j ][ k ] | 0 | u(1) |
| } |  |  |
| applicable_op_num_texture_views_minus1[ i ][ j ] | 0 | ue(v) |
| applicable_op_num_depth_views[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| } |  |  |
| 3dvc_vui_parameters_present_flag | 0 | u(1) |
| if( 3dvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) | 0 |  |
| 3dvc_vui_parameters_extension( ) | 0 |  |
| texture_vui_parameters_present_flag | 0 | u(1) |
| if( texture_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) | 0 |  |
| } |  |  |

- applicable_op_depth_flag[ i ][ j ][ k ] equal to 0 indicates that the depth view with view_id equal to applicable_op_target_view_id[ i ][ j ][ k ] is not included in the j-th operation point. applicable_op_depth_flag[ i ][ j ][ k ] equal to 1 indicates that the depth view with view_id equal to applicable_op_target_view_id[ i ][ j ][ k ] is included in the j-th operation point.

- applicable_op_texture_flag[ i ][ j ][ k ] equal to 0 indicates that the texture view with view_id equal to applicable_op_target_view_id[ i ][ j ][ k ] is not included in the j-th operation point. applicable_op_texture_flag[ i ][ j ][ k ] equal to 1 indicates that the texture view with view_id equal to applicable_op_target_view_id[ i ][ j ][ k ] is included in the j-th operation point. When applicable_op_depth_flag[ i ][ j ][ k ] is equal to 0, applicable_op_texture_flag[ i ][ j ][ k ] shall be equal to 1.

- I.7.4.1.2.1	Order of 3DVC sequence parameter set RBSPs and picture parameter set RBSPs and their activation

…

- For any active 3DVC sequence parameter set or active view 3DVC sequence parameter set, part of the syntax elements in the MVC sequence parameter set extension applies only to the depth views referring to this sequence parameter set, while the some other parts of the syntax elements in the 3DVC sequence parameter set extension collectively apply to both the depth views referring to this sequence parameter set and the corresponding texture views. More specifically, the view dependency information of the 3DVC sequence parameter set extension applies only to the depth views, and the level definitions collectively apply to operation points, each of which contains both depth views and their corresponding texture views. Moreover, the mvc_vui_parameters_extension( ) included when 3dvc_vui_parameters_present_flag is equal to 1the 3dvc_vui_parameters_extension( ) applies collectively to both the depth views referring to this 3DVC sequence parameter set and the corresponding texture views. The vui_parameters( ) included in the sequence parameter set data syntax structure, if present, apply collectively to both the depth views referring to this sequence parameter set and the corresponding texture views, except for the aspect ratio information and the bitstream restriction information, if present, which apply only to the depth views referring to this 3DVC sequence parameter set. The aspect ratio information and the bitstream restriction information for the texture views may be present in the vui_parameters( ) syntax structure included in an MVC sequence parameter set.

- I.14	Video usability information

The specifications in Annex H.14 apply with substituting 3DVC sequence parameter set for MVC sequence parameter set. The mvc_vui_parameters_extension() contained in 3DVC sequence parameter set apply for the 3DV operation points.

- I.14.1	3DVC VUI parameters extension syntax

[Ed. (MH): the syntax elements in red below are those that indicate the difference compared to the mvc_vui_parameters_extension syntax structure.]

| 3dvc_vui_parameters_extension( ) { | C | Descriptor |
| --- | --- | --- |
| vui_3dvc_num_ops_minus1 | 0 | ue(v) |
| for( i = 0; i <= vui_3dvc_num_ops_minus1; i++ ) { |  |  |
| vui_3dvc_temporal_id[ i ] | 0 | u(3) |
| vui_3dvc_num_target_output_views_minus1[ i ] | 5 | ue(v) |
| for( j = 0; j <= vui_3dvc_num_target_output_views_minus1[ i ]; j++ ) { |  |  |
| vui_3dvc_view_id[ i ][ j ] | 5 | ue(v) |
| vui_3dvc_depth_flag[ i ][ j ] | 5 | u(1) |
| vui_3dvc_texture_flag[ i ][ j ] | 5 | u(1) |
| } |  |  |
| vui_3dvc_timing_info_present_flag[ i ] | 0 | u(1) |
| if( vui_3dvc_timing_info_present_flag[ i ] ) { |  |  |
| vui_3dvc_num_units_in_tick[ i ] | 0 | u(32) |
| vui_3dvc_time_scale[ i ] | 0 | u(32) |
| vui_3dvc_fixed_frame_rate_flag[ i ] | 0 | u(1) |
| } |  |  |
| vui_3dvc_nal_hrd_parameters_present_flag[ i ] | 0 | u(1) |
| if( vui_3dvc_nal_hrd_parameters_present_flag[ i ] ) |  |  |
| hrd_parameters( ) | 0 |  |
| vui_3dvc_vcl_hrd_parameters_present_flag[ i ] | 0 | u(1) |
| if( vui_3dvc_vcl_hrd_parameters_present_flag[ i ] ) |  |  |
| hrd_parameters( ) | 0 |  |
| if( vui_3dvc_nal_hrd_parameters_present_flag[ i ] ¦ ¦ vui_3dvc_vcl_hrd_parameters_present_flag[ i ] ) |  |  |
| vui_3dvc_low_delay_hrd_flag[ i ] | 0 | u(1) |
| vui_3dvc_pic_struct_present_flag[ i ] | 0 | u(1) |
| } |  |  |
| } |  |  |

- I.14.2	3DVC VUI parameters extension semantics

[Ed. (MH): the text in red below attempt to indicate the most essential differences compared to the mvc_vui_parameters_extension semantics.]

The 3DVC VUI parameters extension specifies VUI parameters that apply to one or more operation points for the coded video sequence. In Annex Error: Reference source not found it is specified which of the HRD parameter sets specified in the 3DVC VUI parameters extension are used for conformance checking. All 3DVC VUI parameters extensions that are referred to by a coded video sequence shall be identical.

Some texture and depth views identified by vui_3dvc_view_id[ i ][ j ] may not be present in the coded video sequence. Some temporal subsets identified by vui_3dvc_temporal_id[ i ] may not be present in the coded video sequence.

vui_3dvc_num_ops_minus1 plus 1 specifies the number of operation points for which timing information, NAL HRD parameters, VCL HRD parameters, and the pic_struct_present_flag may be present. The value of vui_3dvc_num_ops_minus1 shall be in the range of 0 to 1023, inclusive.

vui_3dvc_temporal_id[ i ] indicates the maximum value of temporal_id for all VCL NAL units in the representation of the i-th operation point.

vui_3dvc_num_target_output_views_minus1[ i ] plus one specifies the number of target output views for the i-th operation point. The value of vui_3dvc_num_target_output_views_minus1[ i ] shall be in the range of 0 to 1023, inclusive.

vui_3dvc_view_id[ i ][ j ] indicates the j-th target output view in the i-th operation point. The value of vui_3dvc_view_id[ i ] shall be in the range of 0 to 1023, inclusive.

vui_3dvc_depth_flag[ i ][ j ] equal to 0 specifies that no depth view with view_id equal to vui_3dvc_view_id[ i ][ j ] is included in the j-th operation point. vui_3dvc_depth_flag[ i ][ j ] equal to 1 specifies that the depth view with view_id equal to vui_3dvc_view_id[ i ][ j ] is included in the j-th operation point.

The value of vui3dvcOpDepthPresent[ i ] is derived as follows:

vui3dvcOpDepthPresent[ i ] = 0
for( k = 0; k < vui_3dvc_num_target_output_views_minus1[ i ]; k++ )
   vui3dvcOpDepthPresent[ i ] = vui3dvcOpDepthPresent[ i ] | vui_3dvc_depth_flag[ i ][ k ]

vui_3dvc_texture_flag[ i ][ j ] equal to 0 specifies that no texture view with view_id equal to vui_3dvc_view_id[ i ][ j ] is included in the j-th operation point. vui_3dvc_depth_flag[ i ][ j ] equal to 1 specifies that the texture view with view_id equal to vui_3dvc_view_id[ i ][ j ] is included in the j-th operation point. When vui_3dvc_depth_flag[ i ][ j ]  is equal to 0, vui_3dvc_texture_flag[ i ][ j ] shall be equal to 1.

- The following syntax elements apply to the coded video sequence that is obtained by the sub-bitstream extraction process as specified in subclause I.8.5.3 with tIdTarget equal to vui_3dvc_temporal_id[ i ], viewIdTargetList containing vui_3dvc_view_id[ i ][ j ] for all j in the range of 0 to vui_3dvc_num_target_output_views_minus1[ i ], inclusive, for which vui_3dvc_texture_flag[ i ][ j ] is equal to 1, depthPresentFlagTarget equal to vui3dvcOpDepthPresent[ i ], and, if vui3dvcOpDepthPresent[ i ] is equal to 1, viewIdDepthTargetList containing vui_3dvc_view_id[ i ][ j ] for all j in the range of 0 to vui_3dvc_num_target_output_views_minus1[ i ], inclusive, for which vui_3dvc_depth_flag[ i ][ j ] is equal to 1 as the inputs and the i-th sub-bitstream as the output.

- vui_3dvc_timing_info_present_flag[ i ] equal to 1 specifies that vui_3dvc_num_units_in_tick[ i ], vui_3dvc_time_scale[ i ], and vui_3dvc_fixed_frame_rate_flag[ i ] for the i-th sub-bitstream are present in the 3DVC VUI parameters extension. vui_3dvc_timing_info_present_flag[ i ] equal to 0 specifies that vui_3dvc_num_units_in_tick[ i ], vui_3dvc_time_scale[ i ], and vui_3dvc_fixed_frame_rate_flag[ i ] for the i-th sub-bitstream are not present in the 3DVC VUI parameters extension.

- The following syntax elements for the i-th sub-bitstream are specified using references to Annex Error: Reference source not found. For these syntax elements the same semantics and constraints as the ones specified in Annex Error: Reference source not found apply, as if these syntax elements vui_3dvc_num_units_in_tick[ i ], vui_3dvc_time_scale[ i ], vui_3dvc_fixed_frame_rate_flag[ i ], vui_3dvc_nal_hrd_parameters_present_flag[ i ], vui_3dvc_vcl_hrd_parameters_present_flag[ i ], vui_3dvc_low_delay_hrd_flag[ i ], and vui_3dvc_pic_struct_present_flag[ i ] were present as the syntax elements num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag, respectively, in the VUI parameters of the active 3DVC sequence parameter sets for the i-th sub-bitstream.

vui_3dvc_num_units_in_tick[ i ] specifies the value of num_units_in_tick, as specified in subclause Error: Reference source not found, for the i-th sub-bitstream.

vui_3dvc_time_scale[ i ] specifies the value of time_scale, as specified in subclause Error: Reference source not found, for the i-th sub-bitstream.

vui_3dvc_fixed_frame_rate_flag[ i ] specifies the value of fixed_frame_rate_flag, as specified in subclause Error: Reference source not found, for the i-th sub-bitstream.

vui_3dvc_nal_hrd_parameters_present_flag[ i ] specifies the value of nal_hrd_parameters_present_flag, as specified in subclause Error: Reference source not found, for the i-th sub-bitstream.

When vui_3dvc_nal_hrd_parameters_present_flag[ i ] is equal to 1, NAL HRD parameters (subclauses Error: Reference source not found and Error: Reference source not found) for the i-th sub-bitstream immediately follow the flag.

The variable VuiMvcNalHrdBpPresentFlag[ i ] is derived as follows: [Ed. the variable name might need to be changed depending on how the specification of levels is written.]

–	If any of the following is true, the value of VuiMvcNalHrdBpPresentFlag[ i ] shall be set equal to 1:

–	vui_3dvc_nal_hrd_parameters_present_flag[ i ] is present in the bitstream and is equal to 1,

–	for the i-th sub-bitstream, the need for presence of buffering periods for NAL HRD operation to be present in the bitstream in buffering period SEI messages is determined by the application, by some means not specified in this Recommendation | International Standard.

–	Otherwise, the value of VuiMvcNalHrdBpPresentFlag[ i ] shall be set equal to 0.

vui_3dvc_vcl_hrd_parameters_present_flag[ i ] specifies the value of vcl_hrd_parameters_present_flag, as specified in subclause Error: Reference source not found, for the i-th sub-bitstream.

When vui_3dvc_vcl_hrd_parameters_present_flag[ i ] is equal to 1, VCL HRD parameters (subclauses Error: Reference source not found and Error: Reference source not found) for the i-th sub-bitstream immediately follow the flag.

The variable VuiMvcVclHrdBpPresentFlag[ i ] is derived as follows:

–	If any of the following is true, the value of VuiMvcVclHrdBpPresentFlag[ i ] shall be set equal to 1:

–	vui_3dvc_vcl_hrd_parameters_present_flag[ i ] is present in the bitstream and is equal to 1,

–	for the i-th sub-bitstream, the need for presence of buffering periods for VCL HRD operation to be present in the bitstream in buffering period SEI messages is determined by the application, by some means not specified in this Recommendation | International Standard.

–	Otherwise, the value of VuiMvcVclHrdBpPresentFlag[ i ] shall be set equal to 0.

The variable VuiMvcCpbDpbDelaysPresentFlag[ i ] is derived as follows:

–	If any of the following is true, the value of VuiMvcCpbDpbDelaysPresentFlag[ i ] shall be set equal to 1:

–	vui_3dvc_nal_hrd_parameters_present_flag[ i ] is present in the bitstream and is equal to 1,

–	vui_3dvc_vcl_hrd_parameters_present_flag[ i ] is present in the bitstream and is equal to 1,

–	for the i-th sub-bitstream, the need for presence of CPB and DPB output delays to be present in the bitstream in picture timing SEI messages is determined by the application, by some means not specified in this Recommendation | International Standard.

–	Otherwise, the value of VuiMvcCpbDpbDelaysPresentFlag[ i ] shall be set equal to 0.

vui_3dvc_low_delay_hrd_flag[ i ] specifies the value of low_delay_hrd_flag, as specified in subclause Error: Reference source not found, for the i-th sub-bitstream.

vui_3dvc_pic_struct_present_flag[ i ] specifies the value of pic_struct_present_flag, as specified in subclause Error: Reference source not found, for the i-th sub-bitstream.

## Changes to the main-level decoding process

- I.8	3DVC decoding process

This subclause specifies the decoding process for an access unit of a coded video sequence conforming to one or more of the profiles specified in Annex I. Specifically, this subclause specifies how the decoded picture with multiple texture view components and multiple depth view components is derived from syntax elements and global variables that are derived from NAL units in an access unit when the decoder is decoding the operation point identified by the target temporal level and the target output texture and depth views.

The decoding process is specified such that all decoders shall produce numerically identical results for the target output texture and depth views. Any decoding process that produces identical results for the target output texture and depth views to the process described here conforms to the decoding process requirements of this Recommendation | International Standard.

Unless stated otherwise, the syntax elements and derived upper-case variables that are referred to by the decoding process specified in this subclause and all child processes invoked from the process specified in this subclause are the syntax elements and derived upper-case variables for the current access unit.

The target output texture and depth views are either specified by external means not specified in this Specification, or, when not specified by external means, there shall be one target output texture view which is the base texture view. Let OutputVOIdxList be the list of VOIdx values, in increasing order of VOIdx, of all the target output views in one access unit. The list OutputVOIdxList shall not change within a coded video sequence.

NOTE – The association of VOIdx values to view_id values according to the decoding process of clause I.8 may differ from that of the decoding process of clause H.8.

A target output view may includes either only a texture view, when no depth view with the same value of view_id is present, only a depth view, or both the texture view and the depth view, which have the same view_id value.

All sub-bitstreams that can be derived using the sub-bitstream extraction process with depthPresentFlagTarget equal to 0 or 1, pIdTarget equal to any value in the range of 0 to 63, inclusive, tIdTarget equal to any value in the range of 0 to 7, inclusive, viewIdTargetList consisting of any one or more viewIdTarget's identifying the views in the bitstream as inputs as specified in subclause I.8.5 shall result in a set of coded video sequences, with each coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found, Annex Error: Reference source not found and Annex I.

Let vOIdxList be a list of integer values specifying the VOIdx values of the view components of the access unit. The variable VOIdxMax is set equal to the maximum value of the entries in the list vOIdxList, and the variable vOIdxMin is set to the minimum value of the entries in the list vOIdxList. VOIdxMax shall be the same for all access units within a coded video sequence. vOIdxMin shall be the same for all anchor access units within a coded video sequence. When the current access unit is an anchor access unit, the variable VOIdxMin is set to vOIdxMin.

The 3DVC video decoding process specified in this subclause is repeatedly invoked for each texture and depth view component with VOIdx from vOIdxMin to VOIdxMax, inclusive, which is present in the list vOIdxList, in increasing order of VOIdx and in decoding order of texture or depth view components as specified in subclause Error: Reference source not found.

Outputs of the 3DVC video decoding process are decoded samples of the current primary coded picture including all decoded texture and depth view components of the target output texture and depth views.

For each texture view component and each depth view component, the specifications in clause H.Error: Reference source not found apply, with the decoding processes for picture order count, reference picture lists construction and decoded reference picture marking being modified in subclauses I.8.1, I.8.2 and I.8.3, respectively. The 3DVC inter prediction and inter-view prediction process is specified in subclause I.8.4.

## Changes to the sub-bitstream extraction process

- I.8.5	Specification of bitstream subsets

The specifications of subclause H.8.5 apply.

- I.8.5.1	Derivation process for required anchor view components

The specification of subclause H.8.5.1 apply with substituting "view component" with either "depth view component" or "texture view component" and "view" with either "depth view" or "texture view" and substituting “required for anchor” with “required for anchor depth”.

The specification of subclause H.8.5.1 apply with substituting "view component" with "texture view component" and "view" with "texture view" and substituting “required for anchor” with “required for anchor texture”.

- I.8.5.2	Derivation process for required non-anchor view components

The specification of subclause H.8.5.2 apply with substituting "view component" with either "depth view component" or "texture view component" and "view" with either "depth view" or "texture view",  and substituting “required for anchor” with “required for anchor depth”.

The specification of subclause H.8.5.2 apply with substituting "view component" with "depth view component" with either "depth view", and substituting “required for anchor” with “required for anchor texture”.

- I.8.5.3	Sub-bitstream extraction process

It is requirement of bitstream conformance that any sub-bitstream that is the output of the process specified in this subclause with depthPresentFlagTarget equal to 0 or 1, pIdTarget equal to any value in the range of 0 to 63, inclusive, tIdTarget equal to any value in the range of 0 to 7, inclusive, viewIdTargetList consisting of any one or more values of viewIdTarget identifying the views in the bitstream, shall be conforming to this Recommendation | International Standard.

NOTE 1 – A conforming bitstream contains one or more coded slice NAL units with priority_id equal to 0 and temporal_id equal to 0.

NOTE 2 – It is possible that not all operation points of sub-bitstreams resulting from the sub-bitstream extraction process have an applicable level_idc or level_idc[ i ]. In this case, each coded video sequence in a sub-bitstream must still conform to one or more of the profiles specified in Annex Error: Reference source not found, Annex Error: Reference source not found and Annex I, but may not satisfy the level constraints specified in subclauses Error: Reference source not found, Error: Reference source not found and I.10.2, respectively.

Inputs to this process are:

–	a variable depthPresentFlagTarget (when present),

–	a variable pIdTarget (when present),

–	a variable tIdTarget (when present),

–	a list viewIdTargetList consisting of one or more values of viewIdTarget (when present),

–	a list viewIdDepthTargetList consisting of one or more value of viewIdDepthTarget (when present).

Outputs of this process are a sub-bitstream and a list of VOIdx values VOIdxList.

When depthPresentFlagTarget is not present as input to this subclause, depthPresentFlagTarget is inferred to be equal to 0.

When pIdTarget is not present as input to this subclause, pIdTarget is inferred to be equal to 63.

When tIdTarget is not present as input to this subclause, tIdTarget is inferred to be equal to 7.

When viewIdTargetList is not present as input to this subclause, there shall be one value of viewIdTarget inferred in viewIdTargetList and the value of viewIdTarget is inferred to be equal to view_id of the base view.

When viewIdDepthTargetList is not present as input to this subclause, the viewIdDepthTargetList is inferred to be identical to viewIdTargetList. viewIdDepthTargetList shall not be present as input if depthPresentFlagTarget is equal to 0.

The sub-bitstream is derived by applying the following operations in sequential order:

- Let VOIdxList be empty and minVOIdx be the VOIdx value of the base view.

- For each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause I.8.5.1 for texture views with the viewIdTarget as input.

- If depthPresentFlagTarget is equal to 1, for each value of viewIdTarget included in viewIdDepthTargetList viewIdTargetList, invoke the process specified in subclause I.8.5.1 for depth views with the viewIdTarget as input.

- For each value of viewIdTarget included in viewIdTargetList, invoke the process specified in subclause I.8.5.2 for texture views with the value of viewIdTarget as input.

- If depthPresentFlagTarget is equal to 1, for each value of viewIdTarget included in viewIdDepthTargetList viewIdTargetList, invoke the process specified in subclause I.8.5.2 for depth views with the viewIdTarget as input.

- Mark all VCL NAL units of texture view components and filler data NAL units for which any of the following conditions are true as "to be removed from the bitstream":

–	priority_id is greater than pIdTarget,

–	temporal_id is greater than tIdTarget,

–	anchor_pic_flag is equal to 1, nal_unit_type is not equal to 21 and view_id is not marked as "required for anchor texture",

–	anchor_pic_flag is equal to 0, nal_unit_type is not equal to 21 and view_id is not marked as "required for non-anchor texture",

–	anchor_pic_flag is equal to 1, nal_unit_type is equal to 21 and view_id is not marked as "required for anchor depth",

–	anchor_pic_flag is equal to 0, nal_unit_type is equal to 21 and view_id is not marked as "required for non-anchor depth",

–	nal_unit_type is not equal to 21, nal_ref_idc is equal to 0 and inter_view_flag is equal to 0 and view_id is not equal to any value in the list viewIdTargetList.

–	nal_unit_type is equal to 21, nal_ref_idc is equal to 0 and inter_view_flag is equal to 0 and view_id is not equal to any value in the list viewIdDepthTargetList.

–	nal_unit_type is equal to 21 and depthPresentFlagTarget is equal to 0.

- Remove all access units for which all VCL NAL units are marked as "to be removed from the bitstream".

- Remove all VCL NAL units and filler data NAL units that are marked as "to be removed from the bitstream".

…

## Changes to SEI messages

Motiovation for the proposed changes: op_with_depth_flag[ i ] equal to 0 seems unnecessary, as the view scalability information SEI message could be used instead and would provide better compatibility with existing MVC implementations.

| 3dvc_view_scalability_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| num_operation_points_minus1 | 5 | ue(v) |
| for( i = 0; i <= num_operation_points_minus1; i++ ) { |  |  |
| operation_point_id[ i ] | 5 | ue(v) |
| priority_id[ i ] | 5 | u(5) |
| temporal_id[ i ] | 5 | u(3) |
| num_target_output_views_minus1[ i ] | 5 | ue(v) |
| op_with_depth_flag[ i ] | 5 | u(1) |
| for( j = 0; j <= num_target_output_views_minus1[ i ]; j++ ) { |  |  |
| view_id[ i ][ j ] | 5 | ue(v) |
| 3dvc_op_view_info( ) |  |  |
| } |  |  |
| profile_level_info_present_flag[ i ] | 5 | u(1) |
| bitrate_info_present_flag[ i ] | 5 | u(1) |
| frm_rate_info_present_flag[ i ] | 5 | u(1) |
| if( !num_target_output_views_minus1[ i ] ) |  |  |
| view_dependency_info_present_flag[ i ] | 5 | u(1) |
| parameter_sets_info_present_flag[ i ] | 5 | u(1) |
| bitstream_restriction_info_present_flag[ i ] | 5 | u(1) |
| if ( profile_level_info_present_flag[ i ] ) |  |  |
| op_profile_level_idc[ i ] | 5 | u(24) |
| if( bitrate_info_present_flag[ i ] ) { |  |  |
| avg_bitrate[ i ] | 5 | u(16) |
| max_bitrate[ i ] | 5 | u(16) |
| max_bitrate_calc_window[ i ] | 5 | u(16) |
| } |  |  |
| if( frm_rate_info_present_flag[ i ] ) { |  |  |
| constant_frm_rate_idc[ i ] | 5 | u(2) |
| avg_frm_rate[ i ] | 5 | u(16) |
| } |  |  |
| if( view_dependency_info_present_flag[ i ] ) { |  |  |
| num_directly_dependent_views[ i ] | 5 | ue(v) |
| for( j = 0; j < num_directly_dependent_views[ i ]; j++ ) { |  |  |
| directly_dependent_view_id[ i ][ j ] | 5 | ue(v) |
| 3dvc_op_view_info( ) |  |  |
| if( op_with_depth_flag[ i ] ) { |  |  |
| num_directly_dependent_depth_views[ i ] | 5 | ue(v) |
| for( j = 0; j < num_directly_dependent_views[ i ]; j++ ) { |  |  |
| directly_dependent_depth_view_id[ i ][ j ] | 5 | ue(v) |
| } |  |  |
| } else |  |  |
| view_dependency_info_src_op_id[ i ] | 5 | ue(v) |
| if( parameter_sets_info_present_flag[ i ] ) { |  |  |
| num_seq_parameter_set_minus1[ i ] | 5 | ue(v) |
| for( j = 0; j <= num_seq_parameter_set_minus1[ i ]; j++ ) |  |  |
| seq_parameter_set_id_delta[ i ][ j ] | 5 | ue(v) |
| num_subset_seq_parameter_set_minus1[ i ] | 5 | ue(v) |
| for( j = 0; j <= num_subset_seq_parameter_set_minus1[ i ]; j++ ) |  |  |
| subset_seq_parameter_set_id_delta[ i ][ j ] | 5 | ue(v) |
| num_pic_parameter_set_minus1[ i ] | 5 | ue(v) |
| for( j = 0; j <= num_init_pic_parameter_set_minus1[ i ]; j++ ) |  |  |
| pic_parameter_set_id_delta[ i ][ j ] | 5 | ue(v) |
| } else |  |  |
| parameter_sets_info_src_op_id[ i ] | 5 | ue(v) |
| if( bitstream_restriction_info_present_flag[ i ] ) { |  |  |
| motion_vectors_over_pic_boundaries_flag[ i ] | 5 | u(1) |
| max_bytes_per_pic_denom[ i ] | 5 | ue(v) |
| max_bits_per_mb_denom[ i ] | 5 | ue(v) |
| log2_max_mv_length_horizontal[ i ] | 5 | ue(v) |
| log2_max_mv_length_vertical[ i ] | 5 | ue(v) |
| num_reorder_frames[ i ] | 5 | ue(v) |
| max_dec_frame_buffering[ i ] | 5 | ue(v) |
| } |  |  |
| } |  |  |
| } |  |  |

| 3dvc_op_view_info( ) { | C | Descriptor |
| --- | --- | --- |
| vinfo_depth_view_present_flag | 5 | u(1) |
| if( vinfo_depth_view_present_flag ) |  |  |
| depth_avc_or_mvc_nalu_header_flag | 5 | u(1) |
| vinfo_texture_view_present_flag | 5 | u(1) |
| if( vinfo_texture_view_present_flag ) |  |  |
| texture_avc_or_mvc_nalu_header_flag | 5 | u(1) |
| } |  |  |

vinfo_depth_view_present_flag equal to 0 specifies that the depth view is not included in the operation point for the view for which the 3dvc_op_view_info( ) syntax structure is present. vinfo_depth_view_present_flag equal to 1 specifies that the depth view is included in the operation point for the view for which the 3dvc_op_view_info( ) syntax structure is present.

depth_avc_or_mvc_nalu_header_flag equal to 0 specifies that some VCL NAL units for the depth view for the view for which the 3dvc_op_view_info( ) syntax structure is present may have nal_unit_type not equal to 21 or reserved_zero_bit not equal to 0. depth_avc_or_mvc_nalu_header_flag equal to 1 indicates that all VCL NAL units for the depth view for the view for which the 3dvc_op_view_info( ) syntax structure is present have nal_unit_type equal to 21 and reserved_zero_bit equal to 0.

vinfo_texture_view_present_flag equal to 0 specifies that the texture view is not included in the operation point for the view for which the 3dvc_op_view_info( ) syntax structure is present. vinfo_depth_view_present_flag equal to 1 specifies that the texture view is included in the operation point for the view for which the 3dvc_op_view_info( ) syntax structure is present. When vinfo_depth_view_present_flag is equal to 0, vinfo_texture_view_present_flag shall be equal to 1.

texture_avc_or_mvc_nalu_header_flag equal to 0 specifies that some VCL NAL units for the texture view for the view for which the 3dvc_op_view_info( ) syntax structure is present may have nal_unit_type not among the nal_unit_type values specified for VCL NAL units according to Annex A, Annex G or Annex H as specified with Table 7-1. texture_avc_or_mvc_nalu_header_flag equal to 1 indicates that all VCL NAL units for the texture view for the view for which the 3dvc_op_view_info( ) syntax structure is present have nal_unit_type among the nal_unit_type values specified for VCL NAL units according to Annex A, Annex G or Annex H as specified with Table 7-1.

| 3dvc_scalable_nesting( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| operation_point_flag | 5 | u(1) |
| if ( !operation_point_flag ) { |  |  |
| all_view_components_in_au_flag | 5 | u(1) |
| if( !all_view_components_in_au_flag ) { |  |  |
| num_view_components_minus1 | 5 | ue(v) |
| for( i = 0; i <= num_view_components_minus1; i++ ) { |  |  |
| sei_view_id[ i ] | 5 | u(10) |
| sei_view_applicability_flag[ i ] | 5 | u(1) |
| } |  |  |
| } |  |  |
| } else { |  |  |
| sei_op_texture_only_flag | 5 | u(1) |
| num_view_components_op_minus1 | 5 | ue(v) |
| for( i = 0; i <= num_view_components_op_minus1; i++ ) { |  |  |
| sei_op_view_id[ i ] | 5 | u(10) |
| if( !sei_op_texture_only_flag ) { |  |  |
| sei_op_depth_flag[ i ] | 5 | u(1) |
| sei_op_texture_flag[ i ] | 5 | u(1) |
| } |  |  |
| } |  |  |
| sei_op_temporal_id | 5 | u(3) |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| sei_nesting_zero_bit /* equal to 0 */ | 5 | f(1) |
| sei_message( ) | 5 |  |
| } |  |  |

sei_op_depth_flag[ i ] equal to 0 specifies that the depth view with view_id equal to sei_op_view_id[ i ] is not included in the operation point to which the nested SEI message applies. sei_op_depth_flag[ i ] equal to 1 specifies that the depth view with view_id equal to sei_op_view_id[ i ] is included in the operation point to which the nested SEI message applies. If sei_op_depth_flag[ i ] is not present, it is inferred to be equal to 1.

sei_op_texture_flag[ i ] equal to 0 specifies that the texture view with view_id equal to sei_op_view_id[ i ] is not included in the operation point to which the nested SEI message applies. sei_op_texture_flag[ i ] equal to 1 specifies that the texture view with view_id equal to sei_op_view_id[ i ] is included in the operation point to which the nested SEI message applies. If sei_op_texture_flag[ i ] is not present, it is inferred to be equal to 1. When sei_op_depth_flag[ i ] is equal to 0, sei_op_texture_flag[ i ] shall be equal to 1.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).