| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0033 |
| --- | --- |

| Title: | AhG8: Multiview video + warp coding with a MVC+D-like MV-HEVC-based coding system |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nikolce Stefanoski Aljoscha Smolic | Tel: Email: | stefanos@disneyresearch.com |
| Source: | Disney Research Zurich |  |  |

_____________________________

# Abstract

In this contribution, a MVC+D-like MV-HEVC-based coding system is proposed for multiview video + warp coding. Warps are coded with help of MV-HEVC. QPs are used which match as close as possible the warp bit rate with the bit rate of the corresponding depth which is provided as part of the current HTM 5.1 anchors. The goal of matching the warp bit rate with the depth bit rate is to compare and evaluate view synthesis results obtained with similar bit budgets. MV-HEVC encoded anchor video and MV-HEVC encoded warp data are used for IDW-based view synthesis. Also corresponding synthesis results achieved with VSRS 1D-Fast and HTM 5.1 anchor video and depth at similar  bit rate are provided. These results and synthesis data are provided as a response to mandate 3 of the AhG8 on Alternative 3D Formats, i.e. to investigate suitable configurations for MV-HEVC and to perform coding experiments with multiview video + warp data with settings based on CTC. It is recommended to have an informal viewing during the meeting to examine the synthesis quality using coded warps and to derive conclusions on how to proceed in this matter.

# Introduction

Warp data, which was computed from multiview video + depth data as well as the corresponding Image-Domain-Warping-based (IDW) synthesis software are provided in JCT3V-B0074 and JCT3V- A003, respectively. During the 2nd  JCT-3V meeting it was confirmed that the warp data, which was provided in JCT3V-B0074, is of sufficiently high quality to be used as test data for warp coding experiments and further analysis in the scope of the JCT-3V activity (JCT3V-B0225). In this document, a MVC-like MV-HEVC-based coder for multiview video + warp (MVW) data is proposed and coding results are reported. The anticipated MVW coding system is shown in Figure 1 and Figure 2. Warps are encoded with MV-HEVC with QPs which match as close as possible the depth bit rate of the 2-view+depth HTM 5.1 anchors.

Figure 1. Proposed multiview video + warp coder.

Figure 2. Proposed multiview video + warp decoder.

Please note that different to the 3D-HEVC technology, the proposed coding technology is a MVC-like HEVC-based coding technology which doesn’t exploit dependencies between texture and warp (or texture and depth).

Section 2 shows details on the warp coding method. Proposed quantization parameters and warp coding results are shown in 3. The document ends with a summary, discussion and recommendation.

# Warp Coding

We propose a warp coding method, which encodes and decodes multiview video and warps according to the system shown in Figure 1 and Figure 2. To encode warps, a Warp Quantizer is used to convert warps into a YUV representation. YUV representations of warps are then encoded with MV-HEVC (HTM 5.1). At the decoder side, YUV representations of warps are decoded with MV-HEVC and warps are reconstructed in the Warp Dequantizer.

## 2.1	Warp Representation

A warp W specifies a mapping of vertices to vertices  (Error: Reference source not found). Vertices are arranged on a uniform quad grid of resolution  which is overlaid on an image. Error: Reference source not found illustrates the uniform grid with vertices and shows how the warp deforms this grid (and the image) by mapping vertices  to vertices.   For an image with pixel resolution and a warp with a grid resolution, the vertex position  is computed as where and.

Figure 3. Example of warping an image of pixel resolution 4x4 with a warp of resolution of 3x3.

Obviously, a warp is used to map image content contained in a quad to a new quad with different position and/or shape. Since vertices are arranged on an uniform quad grid, a warp is completely described by 2D positions  and the image and warp resolutions and , respectively.

Figure 4. Visualization of a warp which is overlaid on an image.

## 2.2	Warp Quantization and Coding with MV-HEVC

Warps of each view are quantized. For each warp of each view Warp Quantization first computes the deviations. These are then uniformly quantized to 8-bit resolution and stored in the y-channel of a YUV image (Fig. 1). Since all 3D video test sequences are captured with parallel camera setups, warps deform images only in horizontal direction. Thus, only the x-component of deviationsis quantized and stored in the y-channel of a YUV image. The u- and v-channel contain a constant value. Finally a multiview video representation of the warp data in YUV format is obtained and MV-HEVC is then used to encode this data.

Fig. 1: YUV representations of warps for sequence Balloons.

# Coding and Synthesis Results

HTM 5.1 is used to encode quantized warps with MV-HEVC. QPs are selected to match the bit rates of the 2-view depth that is part of the HTM 5.1 anchors. In , total anchor video and anchor depth bit-rate as well as warp bitrate with corresponding QPs used for warp coding are shown. The selected QPs for warp coding lead to deviations between -6.7% and +6.5% with respect to the depth bit rate. Assuming that the coded video is the same as in the 2-view HTM 5.1 anchors, a substitution of depth data with warp data leads to a deviation between -0.64% and +0.76% with respect to the overall anchor video + depth bit rate.

The raw warp data used for coding, the warp quantizer, the config files used for warp coding with HTM 5.1, as well as the decoded and dequantized warps used for view synthesis can be found here:

http://zurich.disneyresearch.com/videodata/niko/MPEG/JCT3V-C0033/

Based on decoded anchor video and decoded warps, intermediate views are synthesized and stereo sequences are generated for all 2-view sequences as specified in the CTC. Stereo sequences will be brought to the meeting to examine the synthesis quality.

Table 1. Comparison of warp bit rate with 2-view anchor data rates.

# Summary, Discussion and Recommendation

In this document, a MVC+D-like MV-HEVC-based system for multiview-view video + warp coding was proposed. Warps are encoded at similar bit rates as HTM 5.1 anchor depth. Note that the goal of matching the warp bit rate with the depth bit rate is to compare and evaluate view synthesis results obtained with similar bit budget. Nevertheless, we consider the currently used warp coding QPs as not optimal and expect that warps can be encoded at lower bit rates without visually reducing the synthesis quality [4]. To further examine the synthesis quality achieved by IDW-based view synthesis using coded warps in combination with anchor video, it is recommended to have a viewing during the meeting and to derive conclusions on how to proceed in this matter.

# References

- JCT3V-A0003, “3DV: Test data and view synthesis software for warp coding and IDW-based view synthesis experiments”, Nikolce Stefanoski, Aljoscha Smolic, JCT-3V, July 2012, Stockholm, Sweden

- JCT3V-B0074, “AHG8: New warp data for IDW-based view synthesis experiments”, Nikolce Stefanoski, Aljoscha Smolic, JCT-3V, Oct 2012, Shanghai, China

- JCT3V-B0225, “Report: Subjective testing results on comparing synthesis quality of IDWR and VSRS 1D Fast”, N. Stefanoski, JCT-3V, Oct 2012, Shanghai, China

- “Image Quality vs Rate Optimized Coding of Warps for View Synthesis in 3D Video Applications”, N Stefanoski, M. Lang, A. Smolic, ICIP 2012

# Patent rights declaration(s)

Disney Research Zurich may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).