| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0035 |
| --- | --- |

| Title: | MB skip flag coding in 3D-ATM optimized for 3D video compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Igor Kovliga Alexey Fartukov Mikhail Mishurovskiy JinYoung Lee | Tel: Email: | +7-495-7972500 3765 i.kovliga@samsung.com +7-495-7972500 3756 a.fartukov@samsung.com +7-495-7972500 3760 mike.mishky@samsung.com +82-31-280-6629 jinyoung79.lee@samsung.com |
| Source: | Samsung R&D Russian Institute, Samsung Electronics Co. LTD |  |  |

_____________________________

Abstract

This document describes 3D-based coding tool for improving coding efficiency of skip flags in dependent views of texture and all views of depth implemented in the 3D-ATM 6.1 reference test model. The proposed tool is intended for texture and depth coding of B slices which belong to views/depths of a 3D video sequence. Due-to excessive amount of skip flags in dependent views and all depths caused by higher quantization factors of B-slices comparing with ones from a base view, conventional mb_skip_flag has probability very close to 1 that is no longer processed efficiently by the CABAC engine. In order to improve coding efficiency without any modifications of the CABAC engine, it is proposed to use run-length coding approach for compact representation of mb_skip_flag series in dependent views and all depths of a 3D video sequence which is realized by introducing a new syntax element. Encoding of the new syntax element is tuned for peculiarities of 3D data such that no parsing dependency between base view and supplementary views is introduced.

Measurements have been accomplished in accordance with CTC (except that 3D-ATM 6.1 was used for integration and as a reference) and the following results are reported: total compression gain (coded PSNR): -1.21% BD-Rate; total compression gain (Synthetic PSNR): -1.23% BD-Rate; Compression Gain (Depended views): -4.16% BD-Rate, Maximum Compression Gain (dependent view): -9.96% BD-Rate for left view, S01. Decoder Complexity is estimated as low as 99.6%.

# Problem Statement

It is known that so-called skip mode is one of effective encoding modes that is extensively used in video and 3D video coding and is supported by recent 3D-ATM [4]. In this mode no motion and residual information is transmitted, which however is derived based on already available motion information around current macroblock and predefined derivation rules. Hierarchical B prediction is known as yet another effective approach to improve coding efficiency of a whole 3D video sequence that is ensured by B-predicted slice encoding provided that quantization factors of B-slices are gradually increased with increasing hierarchy level.

On the other hand, 3D video coding implemented by 3D-ATM adopts additional increase of quantization parameters of dependent views in order to improve coding gain furthermore (in particular inter-view QPOffset for texture  is equal to 4). Further increase of skipped macroblocks in B-Slices is achieved by the tool recently adopted into 3D-ATM [2] that defines derivation rules of motion information based on already decoded base view and depths information. As a result, a huge amount of macroblocks encoded in the skip mode is expected to appear while encoding dependent views and depths.

Consequently it is worth considering how efficient skip mode is signaled (encoded) in the bitstream and may this process be improved assuming peculiarities of 3D video.

Let’s go to CABAC coding engine that is one of key components of 3D-ATM. As it defined in [1] the supported range of probability values is defined aspσ∈[0.01875,0.5]. It means that coding efficiency of symbols with LPS probability less than 0.01875 may be decreased. It’s also known that some syntax elements in 3D-ATM are encoded using available context information, namely states of the syntax elements to the left and above the current one. mb_skip_flag is also encoded conditionally. Some statistical research was conducted for mb_skip_flag and the results are presented below in the table where LPS probabilities are shown for mb_skip_flag for B-slices of all test sequences defined in CTC [3] for QP values: 26,31,36,41, coded first 25 frames.

| ctxIdxInc (mb_skip_flag) | skip flag | Base view |  | Depended views |  | Base depths |  | Depended depths |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Quantity of realizations | Probability of LPS | Quantity of realizations | Probability of LPS | Quantity of realizations | Probability of LPS | Quantity of realizations | Probability of LPS |
| 2 | 0 | 75172 | 0.0185 | 87015 | 0.0099 | 8806 | 0.0081 | 18515 | 0.0084 |
|  | 1 | 3981146 |  | 8700005 |  | 1075923 |  | 2174913 |  |
| 1 | 0 | 260992 | 0.4794 | 186360 | 0.3988 | 39575 | 0.4550 | 66568 | 0.4926 |
|  | 1 | 283457 |  | 280990 |  | 33042 |  | 64629 |  |
| 0 | 0 | 139816 | 0.2914 | 87977 | 0.2459 | 33238 | 0.2118 | 61554 | 0.1728 |
|  | 1 | 57498 |  | 28678 |  | 8933 |  | 12861 |  |

It might be concluded that LPS of mb_skip_flag is strongly skewed to zero with the probability significantly less than minimum level defined in CABAC if ctxIdxInc is appeared to be equal 2. Having mentioned all this, it might be concluded that mb_skip_flag is encoded suboptimal for dependent views and all depths if ctxIdxInc is appeared to be equal 2.

Straightforward approach for solving this issue is to modify probability tables that are used in the CABAC. But this table is used for all syntax elements. Optimization of this table seems to be too complex.

Another way is to modify an alphabet so that a probability of new symbols becomes closer to the probabilities of the original AVC table. Therefore it is proposed to use run length technique for series of mb_skip_flag on condition that ctxIdxInc is equal to 2. Prior-art analysis shows that this approach is quite useful for decrease the number of symbols that are passed through an entropy coder.

# Description of proposed tool

## Syntax in tabular form

In this section, additional syntax elements introduced in the proposal are described. Syntax elements which are added or extended in comparison to AVC or 3D ATM v.6.1 are marked in yellow.

### Sequence parameter set 3DVC extension syntax

| seq_parameter_set_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| for( i = 0; i <= num_level_values_signalled_minus1; i++ ) |  |  |
| for( j = 0; j <= num_applicable_ops_minus1[ i ]; j++ ) |  |  |
| applicable_op_num_depth_views_minus1[ i ][ j ] |  | ue(v) |
| depth_info_present_flag | 0 | u(1) |
| if( depth_info_present_flag ) { |  |  |
| 3dv_acquisition_idc | 0 | ue(v) |
| for( i = 0; i < num_views_minus1; i++ ) |  |  |
| view_id_3dv[ i ] | 0 | ue(v) |
| if( 3dv_acquisition_idc ) { |  |  |
| depth_ranges( 2, 0 ) |  |  |
| if( profile_idc = = 139 &&num_views_minus1 ) |  |  |
| vsp_param( 2, 0 ) |  |  |
| } |  |  |
| if ( profile_idc = = 139 ) |  |  |
| for( i = 0; i<= num_views_minus1; i++ ) |  |  |
| depth_preceding_texture_flag[ i ] | 0 | u(1) |
| } |  |  |
| if( profile_idc = = 139 && depth_info_present_flag ) { |  |  |
| reduced_resolution_flag | 0 | u(1) |
| slice_header_prediction_flag | 0 | u(1) |
| inside_view_mvp_flag | 0 | u(1) |
| disp_flag | 0 | u(1) |
| psip_flag | 0 | u(1) |
| nonlinear_depth_representation_num | 0 | ue(v) |
| for( i = 1; i <= nonlinear_depth_representation_num; i++ ) |  |  |
| nonlinear_depth_representation_model[ i ] | 0 | ue(v) |
| seq_view_synthesis_flag | 0 | u(1) |
| } |  |  |
| if( profile_idc = = 139 && !depth_info_present_flag ) |  |  |
| enable_alc_encoder_flag | 0 | u(1) |
| if( profile_idc = = 139 ) |  |  |
| enable_rle_skip_flag | 0 | u(1) |
| } |  |  |

### Slice data syntax

| slice_data( ) { | C | Descriptor |
| --- | --- | --- |
| if( entropy_coding_mode_flag ) |  |  |
| while( !byte_aligned( ) ) |  |  |
| cabac_alignment_one_bit | 2 | f(1) |
| CurrMbAddr = first_mb_in_slice * ( 1 + MbaffFrameFlag ) |  |  |
| moreDataFlag = 1 |  |  |
| prevMbSkipped = 0 |  |  |
| RunLength = 0 |  |  |
| do { |  |  |
| if( slice_type != I && slice_type != SI ) |  |  |
| if( !entropy_coding_mode_flag ) { |  |  |
| mb_skip_run | 2 | ue(v) |
| prevMbSkipped = ( mb_skip_run > 0 ) |  |  |
| for( i=0; i<mb_skip_run; i++ ) |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && mb_skip_run > 0 && VspRefExist ) |  |  |
| mb_skip_type_flag | 2 | u(1) ¦ ae(v) |
| if( mb_skip_run > 0 ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| } else { |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && VspRefExist && leftMbVSSkipped && upMbVSSkipped ) { |  |  |
| mb_vsskip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_vsskip_flag |  |  |
| if( !mb_vsskip_flag ) { |  |  |
| mb_skip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_skip_flag |  |  |
| } |  |  |
| } else { |  |  |
| RLECtx = RLE_skip_context( ) |  |  |
| if( RLECtx && RunLength == 0) { |  |  |
| mb_skip_run_type | 2 | u(1) ¦ ae(v) |
| if (mb_skip_run_type = = 1) |  |  |
| RunLength = 16 |  |  |
| else { |  |  |
| number_of_skipped_macroblocks | 2 | u(4) ¦ ae(v) |
| RunLength= number_of_skipped_macroblocks+1 |  |  |
| } |  |  |
| } else |  |  |
| if( !RLECtx && RunLength != 0) |  |  |
| RunLength = 0 |  |  |
| if( RLECtx) { |  |  |
| if(!mb_skip_run_type && RunLength == 1) |  |  |
| mb_skip_flag = 0 |  |  |
| else |  |  |
| mb_skip_flag = 1 |  |  |
| RunLength = RunLength - 1 |  |  |
| } else |  |  |
| mb_skip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_skip_flag |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && VspRefExist && !mb_skip_flag ) { |  |  |
| mb_vsskip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_vsskip_flag |  |  |
| } |  |  |
| } |  |  |
| if ( enable_alc_encoder_flag && nal_unit_type == 21 && |  |  |
| slice_type == P && !DepthFlag && |  |  |
| !mb_vsskip_flag && mb_skip_flag == 1) { |  |  |
| mb_alc_skip_flag | 2 | u(1) ¦ ae(v) |
| } |  |  |
| } |  |  |
| if( moreDataFlag ) { |  |  |
| if( MbaffFrameFlag && ( CurrMbAddr % 2 = = 0 ¦ ¦ ( CurrMbAddr % 2 = = 1 && prevMbSkipped ) ) ) |  |  |
| mb_field_decoding_flag | 2 | u(1) ¦ ae(v) |
| macroblock_layer( ) | 2 ¦ 3 ¦ 4 |  |
| } |  |  |
| if( !entropy_coding_mode_flag ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| else { |  |  |
| if( slice_type != I && slice_type != SI ) |  |  |
| prevMbSkipped = mb_skip_flag ¦ ¦ mb_vsskip_flag |  |  |
| if( MbaffFrameFlag && CurrMbAddr % 2 = = 0 ) |  |  |
| moreDataFlag = 1 |  |  |
| else { |  |  |
| if (RunLength == 0 ¦¦ CurrMbAddr == (PicSizeInMbs-1) ¦¦ !mb_skip_run_type && RunLength == 1 ¦¦ mb_skip_run_type && NonSkipNextUpMb()) |  |  |
| end_of_slice_flag | 2 | ae(v) |
| else |  |  |
| end_of_slice_flag = 0 |  |  |
| moreDataFlag = !end_of_slice_flag |  |  |
| } |  |  |
| } |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| } while( moreDataFlag ) |  |  |
| } |  |  |

## Semantics

### Sequence parameter set 3DVC extension semantics

The semantics specified in subclause J.7.4.2.1.5 apply with the following changes and additions.

enable_rle_skip_flag equal to 0 specifies that mb_skip_run_type and number_of_skipped_macroblocks are not present. enable_rle_skip_flag equal to 1 specifies that mb_skip_run_type and number_of_skipped_macroblocks may be present. If enable_rle_skip_flag is not present in the bitstream, it shall be inferred to be equal to 0.

### Slice data semantics

The semantics specified in subclause J.7.4.4 apply.

Syntax function RLE_skip_context( ) is specified as follows:

- The derivation process for neighbouring macroblocks specified in subclause 6.4.11.1 is invoked and the output is assigned to mbAddrA and mbAddrB;

- If (enable_rle_skip_flag is equal to 1) and (nal_unit_type is equal to 21) and (slice_type is equal to B) and (mbAddrA is unavailable or mb_skip_flag for the macroblock mbAddrA is equal to 1) and (mbAddrB is unavailable or mb_skip_flag for the macroblock mbAddrB is equal to 1), the return value of RLE_skip_context( ) is equal to TRUE;

- Otherwise, the return value of RLE_skip_context( ) is equal to FALSE.

Syntax function NonSkipNextUpMb( ) is specified as follows:

- mbAddrNextUpCtx value is calculated as NextMbAddress(CurrMbAddr) – PicWidthInMbs;

- If (mbAddrNextUpCtx is available and mb_skip_flag for the macroblock mbAddrNextUpCtx is equal to 0), the return value of NonSkipNextUpMb() is equal to TRUE;

- Otherwise, the return value of NonSkipNextUpMb() is equal to FALSE.

mb_skip_run_type specifies how the number of consecutive skipped macroblocks is derived for which, when decoding a B Slice and enable_rle_skip_flag is equal 1, the following is provided:

If mb_skip_run_type is equal to 1, the following is specified:

- For each consecutive macroblock in the current slice starting from the macroblock at which last mb_skip_run_type was parsed while (RLE_skip_context function returns TRUE) and (CurrMbAddr is less than 16 plus MbAddr of the macroblock at which last mb_skip_run_type was parsed) the following is specified:

  - mb_skip_flag shall not be parsed from the bitstream and shall be inferred to be 1 for the current macroblock;

  - end_of_slice_flag shall not be parsed from the bitstream unless (CurrMbAddr is equal to PicSizeInMbs-1) or (CurrMbAddr is equal to 15 plus MbAddr of the macroblock at which last mb_skip_run_type was parsed) or (the output of the NonSkipNextUpMb() is FALSE).

If mb_skip_run_type is equal to 0, the following is specified for the current slice:

- for subsequent number_of_skipped_macroblocks macroblocks including the macroblock at which last mb_skip_run_type was parsed, mb_skip_flag shall not be read from the bitstream and shall be inferred to be 1;

- for subsequent (number_of_skipped_macroblocks – 1) macroblocks including the macroblock at which last mb_skip_run_type was parsed, end_of_slice_flag shall not be parsed from the bitstream;

- if CurrMbAddr is equal to 1 plus number_of_skipped_macroblocks plus MbAddr of the macroblock at which last mb_skip_run_type was parsed, mb_skip_flag shall not be parsed from the bitstream and shall be inferred to be 0.

number_of_skipped_macroblocks explicitly specifies the number of consecutive skipped macroblocks, processed if mb_skip_run_type is equal to zero. The value of number_of_skipped_macroblocks shall be in the range of 0 to 15, inclusive.

## Parsing process

The specifications in clause 9 are applied. Additionally, the following modifications related to the syntax elements mb_skip_run_type and number_of_skipped_macroblocks are specified and added.

### Alternative CABAC parsing process for slice data

Subclause 2.3.1.1 specifies the initialisation process for the alternative CABAC parsing process for slice data according to proposed tool.

Subclause 2.3.1.2 specifies the binarization process for the alternative CABAC parsing process for slice data according to proposed tool.

Subclause Error: Reference source not found specifies the decoding process flow for the alternative CABAC parsing process for slice data according to proposed tool.

#### Initialization process

Outputs of this process are the initialised CABAC context variables indexed by ctxIdx.

Table I-7 contains the values of the variables n and m used in the initialisation of context variables that are assigned to syntax element mb_skip_run_type. The initialization process for two variables pStateIdx and valMPS is the same as other syntax elements, as defined in Equation 9-5. For all other syntax elements in subclause 7.3.4 the initialisation process of context variables as specified in subclause 9.3.1 applies.

In Table I-2, the ctxIdx for which initialisation is needed for B slices are listed. Also listed is the table number that includes the values of m and n needed for the initialisation.

- Table I-2 – Association of ctxIdx and syntax elements in the initialisation process

| Syntax element |  | Table |  | Slice type |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | I |  | P |  | B |
| mb_skip_run_type | Error: Reference source not found-7 |  |  |  |  |  | 1049 |  |

- Table I-7 – Values of variables m and n for ctxIdx 1049

| Initialisation variables | ctxIdx |
| --- | --- |
|  | 1049 |
| m | 18 |
| n | 64 |

#### Binarization process

Input to this process is a request for a syntax element.

Output of this process is the binarization of the syntax element, maxBinIdxCtx, ctxIdxOffset, and bypassFlag.

Associated with each binarization or binarization part of a syntax element is a specific value of the context index offset (ctxIdxOffset) variable and a specific value of the maxBinIdxCtx variable as given in Table I-5.

The use of the DecodeBypass process and the variable bypassFlag is derived as follows:

– If no value is assigned to ctxIdxOffset for the corresponding binarization or binarization part in Table I-5 labelled as "na", all bins of the bit strings of the corresponding binarization or of the binarization prefix/suffix part are decoded by invoking the DecodeBypass process as specified in subclause 9.3.3.2.3. In such a case, bypassFlag is set equal to 1, where bypassFlag is used to indicate that for parsing the value of the bin from the bitstream the DecodeBypass process is applied.

– Otherwise, for each possible value of binIdx up to the specified value of maxBinIdxCtx given in Table I-5, a specific value of the variable ctxIdx is further specified in subclause 9.3.3. bypassFlag is set equal to 0.

- Table I-5 – Syntax elements and associated types of binarization, maxBinIdxCtx, and ctxIdxOffset

| Syntax element | Type of binarization | maxBinIdxCtx | ctxIdxOffset |
| --- | --- | --- | --- |
| mb_skip_run_type (B slices only) | FL, cMax=1 | 0 | 1049 |
| number_of_skipped_macroblocks (B slices only) | FL, cMax=15 | 0 | na, (uses DecodeBypass) |

# Testing results

The tool was implemented in 3D-ATM v. 6.1 and results of measurements are presented below:

| Compression Gain |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seq. | Texture Coding |  | Depth Coding |  | Total Coding ( coded PSNR) |  | Total Coding ( synthesized PSNR) |  |
|  | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB | DBR, % | dPSNR, dB |
| S01 | -2.60 | 0.08 | -8.45 | 0.45 | -3.37 | 0.10 | -3,38 | 0,11 |
| S02 | -1.56 | 0.05 | -5.28 | 0.23 | -1.96 | 0.06 | -1,98 | 0,06 |
| S03 | -0.29 | 0.01 | -1.81 | 0.14 | -0.44 | 0.01 | -0,48 | 0,01 |
| S04 | -0.25 | 0.01 | -2.98 | 0.15 | -0.55 | 0.02 | -0,53 | 0,02 |
| S05 | -0.22 | 0.01 | 1.85 | -0.09 | -0.37 | 0.02 | -0,33 | 0,01 |
| S06 | -0.70 | 0.03 | -0.41 | 0.02 | -0.92 | 0.04 | -0,98 | 0,04 |
| S08 | -0.77 | 0.03 | -0.45 | 0.02 | -0.86 | 0.04 | -0,92 | 0,03 |
| Average | -0.91 | 0.03 | -2.51 | 0.13 | -1.21 | 0.04 | -1,23 | 0,04 |

# Integration description – additional parameters in the encoder configuration file

- RLESkip

Class: Numeric (Integer)

Description: Enable 3D Extended Skip flag Coding Tool in P slices for non-base texture views or all depth views (1: enable, 0 disable/default).

Note: If RLESkip is set to 1 in texture configuration file (3dv_enc_25Hz_texture_EHP.cfg or 3dv_enc_30Hz_texture_EHP.cfg) then 3D Extended Skip flag Coding Tool is enabled for non-base texture views.

If RLESkip is set to 1 in depth configuration file (3dv_enc_25Hz_depth_EHP.cfg or 3dv_enc_30Hz_depth_EHP.cfg) then 3D Extended Skip flag Coding Tool is enabled for all depth views.

# Conclusion

This document provides motivation and brief description of 3D-based coding tool for improving coding efficiency of skip flags in dependent views of texture and all views of depth implemented in the 3D-ATM 6.1 reference test model. It was shown that the probability of mb_skip_flag in B-slices of dependent views encoded with 3D-ATM is substantially skewed that leads to potential coding efficiency loss of this syntax element provided that ctxIdxInc is equal to 2. In order to solve this problem without any modifications of the CABAC engine, run-length coding of mb_skip_flag series has been proposed for dependent views and all depths video streams. Proposed approach has been optimized for peculiarities of 3D data such that no parsing dependency between base view and supplementary views is introduced. Working draft text of the standard is provided as well.

Measurements have been accomplished in accordance with CTC (except that 3D-ATM 6.1 was used for integration and as a reference) and the following results are reported: total compression gain (coded PSNR): -1.21% BD-Rate; total compression gain (Synthetic PSNR): -1.23% BD-Rate; Compression Gain (Depended views): -4.16% BD-Rate, Maximum Compression Gain (dependent view): -9.96% BD-Rate for left view, S01. Decoder Complexity is estimated as low as 99.6%.

Based on this, Samsung Moscow Research Center suggests adopting this tool into subsequent revision of 3D-ATM.

# Reference

- Detlev Marpe, Heiko Schwarz, and Thomas Wiegand, “Context-Based Adaptive Binary Arithmetic Coding in the H.264/AVC Video Compression Standard”, IEEE TRANSACTIONS ON CIRCUITS AND SYSTEMS FOR VIDEO TECHNOLOGY, VOL. 13, NO. 7, JULY 2003.

- Jian-Liang Lin, Yi-Wen Chen, Yu-Lin Chang, Yu-Pao Tsai, Yu-Wen Huang, and Shawmin Lei “3D-CE5.a results on the motion vector competition-based Skip/Direct mode with explicit signaling”, JCT2-A0045, 1st Meeting: Stockholm, SE, 16–20 July 2012.

- Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, „Common Test Conditions of 3DV Core Experiments“, JCT2-A1100, 1st Meeting: Stockholm, SE, 16–20 July 2012.

- 3D-AVC Draft Text 4, JCT3V-B1002, October 2012

# atent rights declaration(s)

Samsung Moscow Research Center and Samsung Electronics Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).