Information technology – Coding of audio-visual objects – Part 10: Advanced Video Coding, AMENDMENT 3: MVC extensions for inclusion of MFC (multi-resolution frame compatible)

In 7.3.2.1.1, replace the syntax table with:

| seq_parameter_set_data( ) { | C | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | u(8) |
| constraint_set0_flag | 0 | u(1) |
| constraint_set1_flag | 0 | u(1) |
| constraint_set2_flag | 0 | u(1) |
| constraint_set3_flag | 0 | u(1) |
| constraint_set4_flag | 0 | u(1) |
| constraint_set5_flag | 0 | u(1) |
| reserved_zero_2bits /* equal to 0 */ | 0 | u(2) |
| level_idc | 0 | u(8) |
| seq_parameter_set_id | 0 | ue(v) |
| if( profile_idc = = 100 ¦ ¦ profile_idc = = 110 ¦ ¦ profile_idc = = 122 ¦ ¦ profile_idc = = 244 ¦ ¦ profile_idc = = 44 ¦ ¦ profile_idc = = 83 ¦ ¦ profile_idc = = 86 ¦ ¦ profile_idc = = 118 ¦ ¦ profile_idc = = 128 ¦ ¦ profile_idc = = 134 ) { |  |  |
| chroma_format_idc | 0 | ue(v) |
| if( chroma_format_idc = = 3 ) |  |  |
| separate_colour_plane_flag | 0 | u(1) |
| bit_depth_luma_minus8 | 0 | ue(v) |
| bit_depth_chroma_minus8 | 0 | ue(v) |
| qpprime_y_zero_transform_bypass_flag | 0 | u(1) |
| seq_scaling_matrix_present_flag | 0 | u(1) |
| if( seq_scaling_matrix_present_flag ) |  |  |
| for( i = 0; i < ( ( chroma_format_idc != 3 ) ? 8 : 12 ); i++ ) { |  |  |
| seq_scaling_list_present_flag[ i ] | 0 | u(1) |
| if( seq_scaling_list_present_flag[ i ] ) |  |  |
| if( i < 6 ) |  |  |
| scaling_list( ScalingList4x4[ i ], 16, UseDefaultScalingMatrix4x4Flag[ i ]) | 0 |  |
| else |  |  |
| scaling_list( ScalingList8x8[ i − 6 ], 64, UseDefaultScalingMatrix8x8Flag[ i − 6 ] ) | 0 |  |
| } |  |  |
| } |  |  |
| log2_max_frame_num_minus4 | 0 | ue(v) |
| pic_order_cnt_type | 0 | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |  |
| log2_max_pic_order_cnt_lsb_minus4 | 0 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |  |
| delta_pic_order_always_zero_flag | 0 | u(1) |
| offset_for_non_ref_pic | 0 | se(v) |
| offset_for_top_to_bottom_field | 0 | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | 0 | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |  |
| offset_for_ref_frame[ i ] | 0 | se(v) |
| } |  |  |
| max_num_ref_frames | 0 | ue(v) |
| gaps_in_frame_num_value_allowed_flag | 0 | u(1) |
| pic_width_in_mbs_minus1 | 0 | ue(v) |
| pic_height_in_map_units_minus1 | 0 | ue(v) |
| frame_mbs_only_flag | 0 | u(1) |
| if( !frame_mbs_only_flag ) |  |  |
| mb_adaptive_frame_field_flag | 0 | u(1) |
| direct_8x8_inference_flag | 0 | u(1) |
| frame_cropping_flag | 0 | u(1) |
| if( frame_cropping_flag ) { |  |  |
| frame_crop_left_offset | 0 | ue(v) |
| frame_crop_right_offset | 0 | ue(v) |
| frame_crop_top_offset | 0 | ue(v) |
| frame_crop_bottom_offset | 0 | ue(v) |
| } |  |  |
| vui_parameters_present_flag | 0 | u(1) |
| if( vui_parameters_present_flag ) |  |  |
| vui_parameters( ) | 0 |  |
| } |  |  |

In 7.3.2.1.3, replace the syntax table with:

| subset_seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_data( ) | 0 |  |
| if( profile_idc = = 83 ¦ ¦ profile_idc = = 86 ) { |  |  |
| seq_parameter_set_svc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| svc_vui_parameters_present_flag | 0 | u(1) |
| if( svc_vui_parameters_present_flag = = 1 ) |  |  |
| svc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 118 ¦ ¦ profile_idc = = 128 ¦ ¦ profile_idc = = 134 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| mvc_vui_parameters_present_flag | 0 | u(1) |
| if( mvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } |  |  |
| additional_extension2_flag | 0 | u(1) |
| if( additional_extension2_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| additional_extension2_data_flag | 0 | u(1) |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

In H.7.3.2.1.4, replace the syntax table with:

| seq_parameter_set_mvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| num_views_minus1 | 0 | ue(v) |
| for( i = 0; i <= num_views_minus1; i++ ) |  |  |
| view_id[ i ] | 0 | ue(v) |
| for( i = 1; i <= num_views_minus1; i++ ) { |  |  |
| num_anchor_refs_l0[ i ] | 0 | ue(v) |
| for( j = 0; j < num_anchor_refs_l0[ i ]; j++ ) |  |  |
| anchor_ref_l0[ i ][ j ] | 0 | ue(v) |
| num_anchor_refs_l1[ i ] | 0 | ue(v) |
| for( j = 0; j < num_anchor_refs_l1[ i ]; j++ ) |  |  |
| anchor_ref_l1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| for( i = 1; i <= num_views_minus1; i++ ) { |  |  |
| num_non_anchor_refs_l0[ i ] | 0 | ue(v) |
| for( j = 0; j < num_non_anchor_refs_l0[ i ]; j++ ) |  |  |
| non_anchor_ref_l0[ i ][ j ] | 0 | ue(v) |
| num_non_anchor_refs_l1[ i ] | 0 | ue(v) |
| for( j = 0; j < num_non_anchor_refs_l1[ i ]; j++ ) |  |  |
| non_anchor_ref_l1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| num_level_values_signalled_minus1 | 0 | ue(v) |
| for( i = 0; i <= num_level_values_signalled_minus1; i++ ) { |  |  |
| level_idc[ i ] | 0 | u(8) |
| num_applicable_ops_minus1[ i ] | 0 | ue(v) |
| for( j = 0; j <= num_applicable_ops_minus1[ i ]; j++ ) { |  |  |
| applicable_op_temporal_id[ i ][ j ] | 0 | u(3) |
| applicable_op_num_target_views_minus1[ i ][ j ] | 0 | ue(v) |
| for( k = 0; k <= applicable_op_num_target_views_minus1[ i ][ j ]; k++ ) |  |  |
| applicable_op_target_view_id[ i ][ j ][ k ] | 0 | ue(v) |
| applicable_op_num_views_minus1[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| } |  |  |
| if ( profile_idc = = 134 ) { |  |  |
| rpu_type | 0 | u(6) |
| rpu_format | 0 | u(6) |
| if ( rpu_type = = 0 && ( rpu_format = = 0 ¦ ¦ rpu_format = = 1 ) ) { |  |  |
| default_grid_position_flag | 0 | u(1) |
| if( !default_grid_position_flag ) { |  |  |
| view0_grid_position_x | 0 | u(4) |
| view0_grid_position_y | 0 | u(4) |
| view1_grid_position_x | 0 | u(4) |
| view1_grid_position_y | 0 | u(4) |
| } |  |  |
| } |  |  |
| if ( !frame_mbs_only_flag ) |  |  |
| rpu_field_processing_flag | 0 | u(1) |
| } |  |  |
| } |  |  |

In subclause H.3, add the following definition and note:

H.3.46	Reference Processing Unit: A unit which pre-processes an inter-view prediction reference before it is used for inter-view prediction in the decoding process of subsequent view components in decoding order.

In subclause H.4, add the following abbreviation:

MFC           multi-resolution frame compatible

RPU            reference processing unit

In subclause H.7.4.2.1.4, add the following note after the semantics of num_views_minus1:

NOTE 3 – When profile_idc is equal to 134, num_views_minus1 equal to 0 indicates that the bitstream is a frame compatible bitstream. num_views_minus1 equal to 1 indicates that the bitstream comprises a frame compatible base layer bitstream and a full resolution enhancement layer bitstream.

NOTE4 – When profile_idc is equal to 134, the base layer is identified by the base view with VOIdx equal to 0, and the enhancement layer is identified by the view with VOIdx equal to 1.

In subclause H.7.4.2.1.4, add the following semantics:

rpu_type specifies the prediction type for the inter-view prediction reference signal. The value of rpu_type shall be in the range of 0 to 63, inclusive. When not present, rpu_type shall be inferred to be equal to 0.

rpu_format specifies the frame packing arrangement type for the frame compatible base layer bitstream. Given the rpu_type, rpu_format shall be used when processing the inter-view prediction reference for decoding the full resolution enhancement layer bitstream. The value of rpu_format shall be in the range of 0 to 63, inclusive. When not present, rpu_format shall be inferred to be equal to 0. The frame packing arrangement type for the frame compatible base layer bitstream and the enhancement layer bitstream are specified in Table H-1 given the syntax element rpu_type and rpu_format.

Table H-2 – Association between frame packing arrangement type and syntax elements

| rpu_type | rpu_format | frame packing arrangement type for the base layer | frame packing arrangement type for the enhancement layer |
| --- | --- | --- | --- |
| 0 | 0 | frame_packing_arrangement_type equal to 3 and quincunx_sampling_flag equal to 0: side-by-side | frame_packing_arrangement_type equal to 4 and quincunx_sampling_flag equal to 0: top-bottom |
| 0 | 1 | frame_packing_arrangement_type equal to 4 and quincunx_sampling_flag equal to 0: top-bottom | frame_packing_arrangement_type equal to 3 and quincunx_sampling_flag equal to 0: side-by-side |
| 0 | 2-63 | Reserved | Reserved |
| 1-63 | 0-63 | Reserved | Reserved |

default_grid_position_flag equal to 0 indicates that the syntax view0_grid_position_x, view0_grid_position_y, view1_grid_position_x and view1_grid_position_y shall be explicitly signaled. default_grid_position_flag equal to 1 indicates that view0_grid_position_x, view0_grid_position_y, view1_grid_position_x and view1_grid_position_y are not present. When not present, default_grid_position_flag shall be inferred to be equal to 1. When default_grid_position_flag is equal to 1, view0_grid_position_x, view0_grid_position_y, view1_grid_position_x and view1_grid_position_y shall be inferred as follows:

–	If rpu_type equal to 0,

–	If rpu_format equal to 0,

–	view0_grid_position_x is inferred to be equal to 4,

–	view0_grid_position_y is inferred to be equal to 8,

–	view1_grid_position_x is inferred to be equal to 12,

–	view1_grid_position_y is inferred to be equal to 8.

–	Otherwise if rpu_format equal to 1,

–	view0_grid_position_x is inferred to be equal to 8,

–	view0_grid_position_y is inferred to be equal to 4,

–	view1_grid_position_x is inferred to be equal to 8,

–	view1_grid_position_y is inferred to be equal to 12.

–	Otherwise (rpu_format greater than 1), the values of view0_grid_position_x, view0_grid_position_y, view1_grid_position_x and view1_grid_position_y are undefined.

–	Otherwise (rpu_type not equal to 0), the values of view0_grid_position_x, view0_grid_position_y, view1_grid_position_x and view1_grid_position_y are undefined.

view0_grid_position_x has the same semantics as specified in subclause D.2.25 for the frame0_grid_position_x syntax element. The value shall be 4, 8 or 12.

view0_grid_position_y has the same semantics as specified in subclause D.2.25 for the frame0_grid_position_y syntax element. The value shall be 4, 8 or 12.

view1_grid_position_x has the same semantics as specified in subclause D.2.25 for the frame1_grid_position_x syntax element. The value shall be 4, 8 or 12.

view1_grid_position_y has the same semantics as specified in subclause D.2.25 for the frame1_grid_position_y syntax element. The value shall be 4, 8 or 12.

rpu_field_processing_flag equal to 0 indicates that an inter-view prediction reference shall be processed as a frame. rpu_field_processing_flag equal to 1 indicates that the two fields of an inter-view prediction reference shall be processed separately. When not present, rpu_field_processing_flag shall be inferred to be equal to zero.

In subclause H.8, adding the following paragraph after the last paragraph:

When profile_idc is equal to 134, subclause H.8.4.1 specifies the additional processing for an inter-view prediction reference, and subclause H.8.6 recommends the full resolution picture reconstruction process.

In subclause H.8.4, adding the following sentence after the last sentence of the first paragraph:

When profile_idc is equal to 134, subclause H.8.4.1 specifies the additional processing for an inter-view prediction reference.

Add a new subclause H.8.4.1

- H.8.4.1	Processing of the luma and chroma components of an inter-view prediction reference

This process is invoked to modify each colour component of an inter-view prediction reference used in decoding the enhancement layer bitstream in an MFC bitstream. The modification steps include one-dimensional downsampling filtering followed by one-dimensional upsampling filtering. The process is used to convert an inter-view prediction reference from the base layer frame compatible format to the enhancement layer frame compatible format.

The derivation of the modified inter-view prediction reference shall obey the following rules:

- frame_crop_left_offset shall be equal to 0 and frame_crop_top_offset shall be equal to 0,

- The processing shall be limited within either the left or the right view only,

- During filtering, samples outside the view boundary shall be replicated with the same value as the boundary pixel.

Inputs of this process are:

- two variables refW and refH specifying the width and height, respectively, of the inter-view prediction reference picture sample array,

- a (refW)x(refH) inter-view prediction reference picture sample array refPicture for either the luma or a chroma component,

Output of this process is a (refW)x(refH) modified inter-view prediction reference picture sample array RpuPicture for either the luma or a chroma component.

Depending on rpu_field_processing_flag, the following applies:

- If rpu_field_processing_flag is equal to 1, the inter-view prediction reference picture refPicture is a field, and the modified inter-view prediction reference picture RpuPicture is a field. The inter-view prediction reference modification is applied to the individual inter-view prediction reference fields separately.

- Otherwise (rpu_field_processing_flag is equal to 0), the inter-view prediction reference picture refPicture is a frame, and the modified inter-view prediction reference picture RpuPicture is a frame. The inter-view prediction reference modification is applied to the inter-view prediction reference frame.

The mathematical function Clip1( ) is defined with Clip1( ) being substituted with Clip1Y( ) for the luma component and Clip1( ) being substituted with Clip1C( ) for the chroma components, respectively.

The variables refW and refH are derived as follows:

- For the luma component, refW is set equal to PicWidthInSamplesL and refH is set equal to PicHeightInSamplesL.

- For the chroma components, refW is set equal to PicWidthInSamplesC and refH is set equal to PicHeightInSamplesC.

The variables subW and subH are derived as follows:

- For the luma component, subW is set equal to SubWidthC and subH is set equal to SubHeightC.

- For the chroma components, subW is set equal to 1 and subH is set equal to 1.

The variable viewIOffsetX (with I being 0 or 1) is derived as follows:

- If viewI_grid_position_x is equal to 4 or 8, viewIOffsetX is set equal to 0,

- Otherwise (viewI_grid_position_x is equal to 12), viewIOffsetX is set equal to 1.

The variable viewIOffsetY (with I being 0 or 1) is derived as follows:

- If viewI_grid_position_y is equal to 4 or 8, viewIOffsetY is set equal to 0,

- Otherwise (viewI_grid_position_y is equal to 12), viewIOffsetY is set equal to 1.

The variable RpuW specifying the width of an active area of the reference picture is derived as specified by the following ordered steps:

- rightOffset = frame_crop_right_offset * subW

- RpuW = refW – rightOffset 	(H-1)

The variable RpuH specifying the height of an active area of the reference picture is derived as specified by the following ordered steps:

- botOffset = frame_crop_bottom_offset * subH * ( 2 − frame_mbs_only_flag )

- if ( rpu_field_processing_flag )

botOffset = botOffset >> 1

- RpuH = refH – botOffset 	(H-2)

The variable SbsV is set equal to RpuW >> 1.

The variable TabV is set equal to RpuH >> 1.

Let tempPicture[ x, y ] be a (SbsV)x(TabV) array of samples with x = 0..SbsV − 1 and y = 0..TabV – 1.

- If rpu_format is equal to 0,

Let tempRefPicture[ x, y ] be a (SbsV)x(RpuH) array of samples with x = 0..SbsV − 1 and y = 0..RpuH – 1.

Let tempRpuPicture[ x, y ] be a (RpuW)x(TabV) array of samples with x = 0..RpuW − 1 and y = 0..TabV − 1.

- Otherwise (rpu_format is equal to 1),

Let tempRefPicture[ x, y ]  be a (RpuW)x(TabV) array of samples with x = 0..RpuW − 1 and y = 0..TabV − 1.

Let tempRpuPicture[ x, y ] be a (SbsV)x(RpuH) array of samples with x = 0..SbsV − 1 and y = 0..RpuH − 1.

The filtered samples of picture sample array RpuPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..RpuH − 1 are derived as specified by the following ordered steps:

- If rpu_format is equal to 0,

- a variable gMinI is set equal to − 2 − ViewIOffsetX, with I being 0 or 1,

- RpuPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..TabV − 1 is derived from the input of the array refPicture[ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH − 1 as specified by the following ordered steps:

- tempRefPicture[ x, y ] = refPicture[ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH – 1	(H-3)

- Apply one-dimensional downsampling process:

tempPicture[ x, y ] = Clip1 ( ( 4 * tempRefPicture[ x, 2 * y − 2 ] + 7 * tempRefPicture[ x, 2 * y − 1 ] +
                                          10 * tempRefPicture[ x, 2 * y ] + 7 * tempRefPicture[ x, 2 * y + 1 ] +
                                              4 * tempRefPicture[ x, 2 * y + 2 ] + 32 ) >> 6) 	(H-14)

with x = 0..SbsV − 1 and y = 0..TabV – 1

- Apply one-dimensional upsampling process:

tempRpuPicture[ 2 * x + viewIOffsetX, y ] = tempPicture[ x, y ],	(H-15)

tempRpuPicture[ 2 * x + ( 1 – viewIOffsetX ), y ] = Clip1( ( 3 * tempPicture[ x + gMinI, y ] −
 17 * tempPicture[ x + gMinI + 1, y ] + 78 * tempPicture[ x + gMinI + 2, y ] +
 78 * tempPicture[ x + gMinI + 3, y ] − 17 * tempPicture[ x + gMinI + 4, y ] +
 3 * tempPicture[ x + gMinI + 5, y ] + 64) >> 7)	(H-16)

with I being set to 0, x = 0..SbsV − 1 and y = 0..TabV – 1

- RpuPicture[ x, y ] = tempRpuPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..TabV – 1	(H-17)

- RpuPicture[ x, y ] with x = 0..RpuW − 1 and y = TabV..RpuH − 1 is derived from the input of the array refPicture[ x, y ] with x = SbsV..RpuW − 1 and y = 0..RpuH − 1 following the same process as specified in Step 2 with the following changes:

- In 2a, apply change:  tempRefPicture[ x, y ] = refPicture[ x + SbsV, y ],

- In 2c, apply change: with I set to 1.

- In 2d, apply change: RpuPicture[ x, y + TabV ] = tempRpuPicture[ x, y ]

- Otherwise ( rpu_format is equal to 1),

- a variable gMinI is set equal to − 2− ViewIOffsetY, with I being 0 or 1,

- RpuPicture[ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH − 1 is derived from the input of the array refPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..TabV − 1 as specified by the following ordered steps:

- tempRefPicture[ x, y ] = refPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..TabV – 1	(H-18)

- Apply one-dimensional downsampling process:

tempPicture[ x, y ] = Clip1 ( ( 4 * tempRefPicture[ 2 * x – 2, y ] + 7 * tempRefPicture[2 * x – 1, y ] +
                                          10 * tempRefPicture[ 2 * x, y ] + 7 * tempRefPicture[ 2 * x + 1, y ] +
                                              4 * tempRefPicture[ 2 * x + 2, y ] + 32 ) >> 6) 	(H-19)

with x = 0..SbsV − 1 and y = 0..TabV – 1

- Apply one-dimensional upsampling process:

tempRpuPicture[ x, 2 * y + viewIOffsetY ] = tempPicture[ x, y ],	(H-20)

tempRpuPicture[ x, 2 * y + ( 1 – viewIOffsetY ) ] ] = Clip1( ( 3 * tempPicture[ x, y + gMinI ] −
 17 * tempPicture[ x, y + gMinI + 1 ] + 78 * tempPicture[ x, y + gMinI + 2 ] +
 78 * tempPicture[ x, y + gMinI + 3 ] − 17 * tempPicture[ x, y + gMinI + 4 ] +
 3 * tempPicture[ x, y + gMinI + 5 ] + 64 ) >> 7)	(H-21)

with I being set to 0, x = 0..SbsV − 1 and y = 0..TabV – 1

- RpuPicture[ x, y ] = tempRpuPicture[ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH − 1	(H-22)

- RpuPicture[ x, y ] with x = SbsV..RpuW − 1 and y = 0..RpuH − 1 is derived from the input of the array refPicture[ x, y ] with x = 0..RpuW − 1 and y = TabV..RpuH − 1, following the same process as specified in Step 2 with the following changes:

- In 2a, apply change: tempRefPicture[ x, y ] = refPicture[ x, y + TabV ].

- In 2c, apply change: with I being set to 1.

- In 2d, apply change: RpuPicture[ x + SbsV, y ] = tempRpuPicture[ x, y ].

The filtered samples of picture sample array RpuPicture[ x, y ] with x = RpuW..refW − 1 or y = RpuH..refH − 1 are derived as specified by the following ordered steps:

- RpuPicture[ x, y ] = RpuPicture[ RpuW − 1, y ] with x = RpuW..refW − 1 and y = 0..RpuH − 1	(H-23)

- RpuPicture[ x, y ] = RpuPicture[ x, RpuH − 1 ] with x =  0..refW − 1 and y = RpuH..refH − 1	(H-24)

NOTE – If the base layer picture is a side-by-side frame compatible picture, the inter-view reference picture is first vertically downsampled and then horizontally upsampled in a conversion to the top-bottom format. If the base layer is a top-bottom frame compatible picture, the inter-view reference picture is first horizontally downsampled and then vertically upsampled in a conversion to the side-by-side format.

Add a new subclause H.8.6 “MFC full resolution picture reconstruction”

H.8.6	MFC full resolution picture reconstruction

This subclause does not form an integral part of this Recommendation | International Standard.

This informative subclause describes the process for reconstructing the full resolution stereo views in MFC systems. The process is applied to each colour component independently.

Inputs of this process are:

- a (refW)x(refH) modified inter-view prediction reference picture sample array RpuPicture derived in subclause H.8.4.1,

- a (RpuW)x(RpuH) cropped decoded picture sample array decBasePicture for a luma or a chroma component of the base view component with VOIdx equal to 0 (base layer),

- a (RpuW)x(RpuH) cropped decoded picture sample array decEnhPicture for a luma or a chroma component of the view component with VOIdx equal to 1 (enhancement layer).

Outputs of this process are:

- a (RpuW)x(RpuH) full resolution picture sample array leftPicture for a luma or a chroma component of the left view,

- a (RpuW)x(RpuH) full resolution picture sample array rightPicture for a luma or a chroma component of the right view.

Depending on rpu_field_processing_flag, the following applies:

- If rpu_field_processing_flag is equal to 1, the modified inter-view prediction reference picture RpuPicture is a field, the cropped decoded base layer picture decBasePicture is a field, the cropped decoded enhancement layer picture decEnhPicture is a field, the full resolution left view picture leftPicture is a field, and the full resolution right view picture rightPicture is a field. The full resolution picture reconstruction is applied to the individual fields separately.

- Otherwise (rpu_field_processing_flag is equal to 0), the modified inter-view prediction reference picture RpuPicture is a frame, the cropped decoded base layer picture decBasePicture is a frame, the cropped decoded enhancement layer picture decEnhPicture is a frame, the full resolution left view picture leftPicture is a frame, and the full resolution right view picture rightPicture is a frame. The full resolution picture reconstruction is applied to the frame.

The mathematical function Clip1( ) is defined with Clip1( ) being substituted with Clip1Y( ) for the luma component and Clip1( ) being substituted with Clip1C( ) for the chroma components, respectively.

The variable tVal is set equal to ( 1 << ( BitDepthY − 1 ) ) for the luma component and tVal is set equal to ( 1 << ( BitDepthC − 1 ) ) for a chroma component, respectively.

Let upBasePicture[ x, y ] be an (RpuW)x(RpuH) array of samples with x = 0..RpuW − 1 and y = 0..RpuH – 1.

Let resPicture[ x, y ] be an (RpuW)x(RpuH) array of samples with x = 0..RpuW − 1 and y = 0..RpuH – 1.

Let upResPicture[ x, y ] be an (RpuW)x(RpuH) array of samples with x = 0..RpuW − 1 and y = 0..RpuH – 1.

- If rpu_format is equal to 0,

Let tempDecBasePic[ x, y ] be a (SbsV)x(RpuH) array of samples with x = 0..SbsV − 1 and y = 0..RpuH – 1.

Let tempDecEnhPic[ x, y ] be a (RpuW)x(TabV) array of samples with x = 0..RpuW − 1 and y = 0..TabV − 1.

Let tempRpuPic[ x, y ] be a (RpuW)x(TabV) array of samples with x = 0..RpuW − 1 and y = 0..TabV − 1.

- Otherwise (rpu_format is equal to 1),

Let tempDecBasePic[ x, y ] be a (RpuW)x(TabV) array of samples with x = 0..RpuW − 1 and y = 0..TabV − 1.

Let tempDecEnhPic[ x, y ] be a (SbsV)x(RpuH) array of samples with x = 0..SbsV − 1 and y = 0..RpuH − 1.

Let tempRpuPic[ x, y ] be a (SbsV)x(RpuH) array of samples with x = 0..SbsV − 1 and y = 0..RpuH − 1.

The samples of full resolution picture sample array for the left view leftPicture[ x, y ] and the right view rightPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..RpuH − 1 are derived as follows:

- If rpu_format is equal to 0,

- a variable gMinI is set equal to − 2 − ViewIOffsetX, with I being 0 or 1,

- leftPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..RpuH − 1 is derived from the input of the arrays decBasePicture[ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH − 1, decEnhPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..TabV − 1 and RpuPicture [ x, y ] with x = 0..RpuW − 1 and y = 0..TabV − 1 as specified by the following ordered steps:

- tempDecBasePic[ x, y ] = decBasePicture[ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH – 1.

- tempDecEnhPic[ x, y ] = decEnhPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..TabV – 1.

- tempRpuPic[ x, y ] = RpuPicture[ x, y ] with x = 0..RpuW− 1 and y = 0..TabV – 1.

- Apply one-dimensional upsampling process:

upBasePicture[ 2 * x + viewIOffsetX, y ] = tempDecBasePic[ x, y ],	(H-25)

upBasePicture[2 * x + ( 1 – viewIOffsetX ), y ] = Clip1( ( 3 * tempDecBasePic[ x + gMinI, y ] −
 17 * tempDecBasePic[ x + gMinI + 1, y ] + 78 * tempDecBasePic[ x + gMinI + 2, y ] +
 78 * tempDecBasePic[ x + gMinI + 3, y ] − 17 * tempDecBasePic[ x + gMinI + 4, y ] +
 3 * tempDecBasePic[ x + gMinI + 5, y ] + 64 ) >> 7)	(H-26)

with I being set to 0, x = 0..SbsV − 1 and y = 0..RpuH – 1

- resPicture[ x, y ] = Clip3( − tVal, tVal − 1, ( tempDecEnhPic[ x, y ] − tempRpuPic[ x, y ] ) )	(H-4)

with x = 0..RpuW − 1 and y = 0..TabV – 1

- Apply one-dimensional upsampling process:

upResPicture[ x, 2 * y] = resPicture[ x, y ]	(H-5)

upResPicture[ x, 2 * y + 1 ] = Clip3( − tVal, tVal − 1, ( 3 * resPicture[ x, y – 2 ] −
 17 * resPicture[ x, y – 1 ] + 78 * resPicture[ x, y ] + 78 * resPicture[ x, y + 1 ] −
 17 * resPicture[ x, y + 2 ] + 3 * resPicture[ x, y + 3 ] + 64 ) >> 7 ) )	(H-6)

with x = 0..RpuW − 1 and y = 0..TabV – 1

- leftPicture[ x, y ] = Clip1( upBasePicture[ x, y ] + upResPicture[ x, y ] )	(H-30)

with x = 0..RpuW − 1 and y = 0..RpuH – 1

- rightPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..RpuH − 1 is derived from the input of the arrays decBasePicture[ x, y ] with x = SbsV..RpuW − 1 and y = 0..RpuH − 1, decEnhPicture[ x, y ] with x = 0..RpuW − 1 and y = TabV..RpuH − 1 and RpuPicture [ x, y ] with x = 0..RpuW − 1 and y = TabV..RpuH − 1 following the same process as specified in Step 2 with the following changes:

- In 2a, apply change: tempDecBasePic[ x, y ] = decBasePicture[ x+ SbsV, y ].

- In 2b, apply change: tempDecEnhPic[ x, y ] = decEnhPicture[ x, y+ TabV ].

- In 2c, apply change: tempRpuPic[ x, y ] = RpuPicture[ x, y + TabV ].

- In 2d, apply change: with I being set to 1.

- In 2g, change variable leftPicture to rightPicture.

- Otherwise ( rpu_format is equal to 1 ),

- a variable gMinI is set equal to  − 2 − ViewIOffsetY, with I being 0 or 1,

- leftPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..RpuH − 1 is derived from the input of the arrays decBasePicture[ x, y ] with x = 0..RpuW − 1 and y = 0..TabV − 1, decEnhPicture[ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH − 1 and RpuPicture [ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH − 1 as specified by the following ordered steps:

- tempDecBasePic[ x, y ] = decBasePicture[ x, y ] with x = 0..RpuW − 1 and y = 0..TabV – 1.

- tempDecEnhPic[ x, y ] = decEnhPicture[ x, y ] with x = 0..SbsV − 1 and y = 0..RpuH – 1.

- tempRpuPic[ x, y ] = RpuPicture[ x, y ] with x = 0..SbsV− 1 and y = 0..RpuH – 1.

- Apply one-dimensional upsampling process:

upBasePicture[ x, 2 * y + viewIOffsetY ] = tempDecBasePic[ x, y ],	(H-31)

upBasePicture[ x , 2 * y + ( 1 – viewIOffsetY ) ] = Clip1( ( 3 * tempDecBasePic[ x, y + gMinI ] −
 17 * tempDecBasePic[ x, y + gMinI + 1 ] + 78 * tempDecBasePic[ x, y + gMinI + 2 ] +
 78 * tempDecBasePic[ x, y + gMinI + 3 ] − 17 * tempDecBasePic[ x, y + gMinI + 4 ] +
 3 * tempDecBasePic[ x, y + gMinI + 5 ] + 64) >> 7)	(H-32)

with I being set to 0, x = 0..RpuW − 1 and y = 0..TabV – 1.

- resPicture[ x, y ] = Clip3( − tVal, tVal − 1, ( tempDecEnhPic[ x, y ] − tempRpuPic[ x, y ] ) )	(H-7)

with x = 0..SbsV − 1 and y = 0..RpuH – 1.

- Apply one-dimensional upsampling process:

upResPicture[ 2 * x, y ] = resPicture[ x, y ]	(H-8)

upResPicture[ 2 * x + 1, y ] = Clip3(− tVal, tVal − 1, ( 3 * resPicture[ x – 2, y ] –
 17 * resPicture[ x – 1, y ] + 78* resPicture [ x, y ] + 78* resPicture [ x + 1, y ] −
 17 * resPicture[ x + 2, y ] + 3* resPicture [ x + 3, y ] + 64 ) >> 7 ) )	(H-9)

with x = 0..SbsV − 1 and y = 0..RpuH – 1.

- leftPicture[ x, y ] = Clip1( upBasePicture[ x, y ] + upResPicture[ x, y ] )	(H-10)

with x = 0..RpuW − 1 and y = 0..RpuH – 1

- rightPicture[ x, y ] with x = 0..RpuW − 1 and y = 0..RpuH − 1 is derived from the input of the arrays decBasePicture[ x, y ] with x = 0..RpuW − 1 and y = TabV..RpuH − 1, decEnhPicture[ x, y ] with x = SbsV..RpuW − 1 and y = 0..RpuH − 1 and RpuPicture [ x, y ] with x = SbsV..RpuW − 1 and y = 0..RpuH − 1 following the same process as specified in Step 2 with the following changes:

- In 2a, apply change: tempDecBasePic[ x, y ] = decBasePicture[ x, y+ TabV ].

- In 2b, apply change: tempDecEnhPic[ x, y ] = decEnhPicture[ x + SbsV, y ].

- In 2c, apply change: tempRpuPic[ x, y ] = RpuPicture[ x + SbsV, y ].

- In 2d, apply change: with I being set to 1.

- In 2g, change variable leftPicture to rightPicture.

Add a new subclause H.10.1.3 “MFC High profile”

H.10.1.3	MFC High profile

Bitstreams conforming to the MFC High profile shall obey the following constraints:

–	The base view bitstream as specified in subclause H.8.5.4 shall obey all constraints of the High profile specified in subclause A.2.4 and all active sequence parameter sets shall fulfil one of the following conditions:

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1,

–	profile_idc is equal to 100.

–	Only I, P, and B slice types may be present.

–	NAL unit streams shall not contain nal_unit_type values in the range of 2 to 4, inclusive.

–	Arbitrary slice order is not allowed.

–	Picture parameter sets shall have num_slice_groups_minus1 equal to 0 only.

–	Picture parameter sets shall have redundant_pic_cnt_present_flag equal to 0 only.

–	MVC sequence parameter sets shall have chroma_format_idc in the range of 0 to 1 inclusive.

–	MVC sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

–	MVC sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

–	MVC sequence parameter sets shall have qpprime_y_zero_transform_bypass_flag equal to 0 only.

–	When num_views_minus1 is present in an MVC sequence parameter set, its value shall be less than 2.

–	For each access unit, the value of level_idc for all active view MVC sequence parameter set RBSPs shall be the same as the value of level_idc for the active MVC sequence parameter set RBSP.

–	The level constraints specified for the Stereo High profile in subclause H.10.2 shall be fulfilled.

Conformance of a bitstream to the MFC High profile is indicated by profile_idc being equal to 134.

Decoders conforming to the MFC High profile at a specific level shall be capable of decoding all bitstreams in which both of the following conditions are true:

- All active MVC sequence parameter sets have one or more of the following conditions fulfilled:

–	profile_idc is equal to 134,

–	profile_idc is equal to 128,

–	profile_idc is equal to 100,

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1.

- All active MVC sequence parameter sets have one or more of the following conditions fulfilled:

–	level_idc or (level_idc and constraint_set3_flag) represent a level less than or equal to the specific level,

–	level_idc[ i ] or (level_idc[ i ] and constraint_set3_flag) represent a level less than or equal to the specific level.