| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0037 |
| --- | --- |

| Title: | Unification of Upsampling Filters in MFC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Yin, Hariharan Ganapathy, Taoran Lu, Tao Chen, Walt Husak 432 Lakeside Drive, Sunnyvale, CA, 94085 USA | Tel: Email: | +1 408 330 3251 {pyin, wjh}@dolby.com |
| Source: | Dolby Laboratories Inc. |  |  |

_____________________________

# Abstract

In the MFC CfP submission, two upsampling filters (one 6-tap and one 4-tap) were initially supported in the RPU process and the 6-tap upsampling filter was used in the full resolution reconstruction process. Later in a simplification contribution, the upsampling filter used in the RPU process was fixed to the 4-tap and the upsampling filter in the reconstruction process remained unchanged. This might cause confusion and extra effort in filter implementation. In this contribution, it is proposed to unify the upsampling filters in both RPU process and full resolution reconstruction. Simulation results suggest that the use of the 6-tap upsampling filter has better performance, which is very close to the results in the MFC CfP submission. It is recommended to adopt the 6-tap upsampling filter in the MFC specification.

# Introduction

In the MFC CfP contribution [1], a set of downsampling and upsampling filters was initially suggested as shown in Table 1 for the RPU process within the coding loop. In the CfP bitstream submission, the filters of higher complexity, OM_RPU_DOWN_F1 (9-tap) and OM_RPU_UP_F0 (6-tap), were used.  OM_RPU_UP_F0 was also used in the full resolution reconstruction process. Later, it was suggested to simplify the combinations and keep the filters of lower complexity, OM_RPU_DOWN_F0 (5-tap) and OM_RPU_UP_F1 (4-tap), in the RPU process to reduce the complexity for the codec (normative part). But OM_RPU_UP_F0 was still used in the non-normative full resolution reconstruction process [2]. In this contribution, we propose to unify the upsampling filters in both RPU process and full resolution reconstruction process to avoid any potential confusion in implementation. In the following simulations, the downsampling filter in the RPU process is fixed to OM_RPU_DOWN_F0 (5-tap) as in [2].

Table 1: RPU Filters originally proposed in the CfP submission

| filter_name | filter_1D_tap | filter_dynamic_range | filter_offset | filter_1D_tap_length | cutoff freq |
| --- | --- | --- | --- | --- | --- |
| OM_RPU_DOWN_F0 | [4 7 10 7 4] | 6 | 32 | 5 | 0.29 |
| OM_RPU_DOWN_F1 | [2 8 17 24 26 24 17 8 2] | 8 | 128 | 9 | 0.21 |
| OM_RPU_UP_F0 | [3 -17 78 78 -17 3] | 7 | 64 | 6 | 0.5 |
| OM_RPU_UP_F1 | [-11 75 75 -11] | 7 | 64 | 4 | 0.5 |

# Simulation Results

The simulation follows all the test conditions defined in the MFC CfP. All the quantization parameters are exactly the same as those used in the CfP submission. The CfP submission results are used as an anchor for BD rate and BD PSNR computation. Table 2 shows the BD results with the 6-tap OM_RPU_UP_F0 used in both RPU process and full resolution reconstruction process. Table 3 shows the BD results with the 4-tap OM_RPU_UP_F1 used in both RPU process and reconstruction process. The OM_RPU_UP_F0 results are very close to the results in the CfP submission where more complex filters were used for downsampling. The OM_RPU_UP_F1 results are not as good, and in some cases it shows a 4% loss in comparison with the results in the CfP submission. The difference in complexity between the 6-tap filter to and the 4-tap filter is not significant and such a complexity reduction does not justify the measured loss in coding efficiency. In addition, the performance of the OM_RPU_UP_F0 results is very similar to [2]. Therefore, we recommend using OM_RPU_UP_F0 as the unified upsampling filter for both RPU process and full resolution reconstruction process.

Table 2: BD result of 6-tap upsampling filter compared to CfP results

|  | 25% overhead |  | 50% overhead |  |
| --- | --- | --- | --- | --- |
|  | BD rate | BD PSNR | BD rate | BD PSNR |
| Billiards | -0.02 | 0.00 | 0.27 | -0.01 |
| NewsRoom | -0.42 | 0.01 | -0.48 | 0.01 |
| Amelia | 0.14 | 0.00 | 0.37 | -0.01 |
| LivingRoomQuestion | 0.54 | -0.01 | 1.55 | -0.04 |
| Avg. | 0.06 | 0.00 | 0.43 | -0.01 |

Table 3: BD result of 4-tap upsampling filter compared to CfP results

|  | 25% overhead |  | 50% overhead |  |
| --- | --- | --- | --- | --- |
|  | BD rate | BD PSNR | BD rate | BD PSNR |
| Billiards | 1.08 | -0.03 | 1.56 | -0.05 |
| NewsRoom | 1.75 | -0.05 | 1.99 | -0.06 |
| Amelia | 3.37 | -0.05 | 4.00 | -0.06 |
| LivingRoomQuestion | 2.94 | -0.08 | 4.16 | -0.11 |
| Avg. | 2.28 | -0.05 | 2.93 | -0.07 |

# Attachment

The following attachments are included in this proposal:

•	Specification text with the suggested change. (All changes relative to JCT3V-C0038 [4] are highlighted in yellow).

•	Two excel files containing all the simulation results.

•	Updated MFC software source code and software manual.

We would like to point out that the attached software is aligned with the MFC working draft [3] [4]. The only change is that OM_RPU_UP_F0 is used instead of OM_RPU_UP_F1 for the RPU upsampling process. In addition, PAFF and MBAFF are now supported in MFC software.

# Crosscheck

We would like to thank InterDigital Communications, LLC for crosschecking the encoded bitstreams [5].

# Acknowledgement

We would like to thank Fons Bruls from Philips for pointing out the issue of using different upsampling filters.

# References

- P. Yin, T. Lu, T. Chen, H. Ganapathy, G. Lakshminarayanan and W. Husak, “Description of MFC Video Coding Technology Proposal by Dolby”, 102th MPEG Meeting, Shanghai, China, October, 2012, Doc.m26661

- P. Yin, T. Lu, T. Chen, and W. Husak, “Revision of m26661”, 102th MPEG Meeting, Shanghai, China, October, 2012, Doc.m27249

- “WD on MVC extensions for inclusion of MFC (Multi-resolution Frame Compatible)”, 102th MPEG Meeting, Shanghai, China, October, 2012, Doc.N13137

- P. Yin, H. Ganapathy, T. Lu, T. Chen, and W. Husak, “Editorial Improvements on WD for MVC extensions for inclusion of MFC (multi-resolution frame compatible)”, JCT3V-C0038, JCT3V 3rd Meeting, Geneva, CH, 17–23 Jan. 2013

- Y. He and Y. Ye, “Crosscheck of JCT3V-C0037: Unification of Upsampling Filters in MFC”, JCT3V-C0040, JCT3V 3rd Meeting, Geneva, CH, 17–23 Jan. 2013

# Patent rights declaration(s)

Dolby Laboratories Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).