| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0048r1 /M27783r1 |
| --- | --- |

| Title: | AHG8: Draft Text for Signaling of Alternative 3D Format in ISO/IEC 14496-10 (ITU-T H.264) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Yasuyuki Ichihashi, Hisayuki Sasaki, Kenji Yamamoto Nukui-Kitamachi 4-2-1, Koganei, Tokyo 184-8795, Japan Masayuki Tanimoto, Kazuyoshi Suzuki Zehn 101, Kawana-cho 2-64, Showa-ku, Nagoya, 466-0856, Japan Nicolce Stefanoski Zurich, Switzerland | Tel: Email: | +81-42-327-7262 {senoh, y-ichihashi, sasaki, k.yamamoto}@nict.go.jp +81-52-223-6639 tanimoto@nagoya-u.ac.jp, kazuyoshi-suzuki@cb3.so-net.ne.jp stefanos@disneyresearch.com |
| Source: | National Institute of Information and Communications Technology(NICT) Nagoya Industrial Science Research Institute(NISRI) |  |  |

_____________________________

# Abstract

This contribution reports discussions and conclusions on the AHG8 mandate,

- Investigate compact signaling (SEI / VUI) and descriptions required to identify different types of 3D data formats (GVD / Warp), as usable to interpret decoded 3D data by non-normative processing after decoding.

It investigates on compact signalling and description methods for alternative 3D format (GVD / Warp) in ISO/IEC 14496-10 / ITU-T H.264 :201X(E). Two options were discussed.

Option 1: signaling with depth_type in a SEI message.

Option 2: signaling with depth_type in a VUI parameter.

Since the depth_type is required by view synthesis after decoding but before displaying multi-view images and depth maps, SEI message is more appropriate than VUI parameter for the signalling.

In addition to this option, definition of a new profile_idc for alternative 3D format can solve the incompatible view synthesis problem for current 3DV tracks.

Another additional option is the one_byte0 (NAL_unit_type=0) insertion after start code, for the depth data to be carried in MVC streams.

AHG8 recommends to integrate SEI signaling description in the current 3DV tracks.

# Introduction

At the 2nd JCT-3V meeting (October 2012), AHG8 proposed signaling methods, which is required to identify the type of 3D data format (e.g. global view/depth, warp) and to interpret such decoded 3D data by non-normative processing after decoding. The first proposal was specifying a new NAL_unit_type in ISO/IEC 14496-10 / ITU-T H.264 which covers all components under the new NAL_unit_type. Since this proposal was not following the MVC+Depth or 3D-AVC extension, requiring a new Annex, however it was not approved but advised to follow the existing MVC+Depth or 3D-AVC specifications. According to this advice, revised two options are proposed this time.

The first option specifies a new payload type in SEI message, which signals view synthesis method for alternative 3D format. All other components of alternative 3D format conform to MVC+Depth or 3D-AVC specidications. The merit of this option is that the specifications for ISO/IEC 14496-10 / ITU-T H.264 keep their consistency, since SEI messages specify many depth- and view synthesis-related parameters such as multiview_acquisition_info, 3dvc_view_scalability_info, depth_acquisition_info, depth_representation_information, etc.

The second option specifies a new VUI parameter in Subset SPS to signal the alternative 3d format. This signalling method might confuse users, since VUI parameters are mainly related to display information such as sample aspect_ratio, over_scan info., colour_primaries, etc. but not related to the depth or view synthesis information.

Another requirement for these signalling methods is to keep the compatibility with the conforming decoders and to avoid malfunction of view syntesizers. Since the global view and depth or warp data are encoded just like ordinary multi-view and single/multi-depth streams having existing NAL_unit_types, ordinary decoders output decoded incompatible views or depth maps to ordinary view synthesizers. In such case, synthesized 3D images are not correct.

One method to keep the compatibility and to avoid the malfunction of view synthesizer is to define a new profile idc for Alternative 3D format bitstreams. However, this is not mandatory.

Yet another option is proposed when alternative 3D format is integrated in MPEG-4 MVC streams. In this case, besides depth_type, another signalling method for depth streams must be specified, since depth streams look identical to the view streams. For that purpose, insertion of one_byte_zero just after the start code of depth streams is proposed. Since NAL_unit_type=0 is unspecified, this byte can be used for the depth stream signalling.

This contribution describes the option 1 and option 2 signalling methods for the Alternative 3D format together with optional new profile_idc and optional insertion of one_byte_zero (NAL_unit_type=0) to MVC streams, which keep the compatibility and to avoid the malfunction of decoders and/or view synthesizers.

# Proposed Syntax and Semantics

# (option 1: SEI signaling)

In 0.6, add the following paragraph after the last paragraph that starts with “A multiview video coding extension with depth information is specified in Annex J.”:

Alternative 3D format signaling is specified in Annex D: Supplemental Enhancement Information. Associated non-normative view synthesis process is specified in its semantics specifications allowing the decoding of bitstreams that represent Alternative 3D format based on the ISO/IEC 14496-10 (ITU-T Rec. H.264) specifications. Since the Alternative 3D format represents both Global View and Depth format and Warp format, a depth_type is specified in the SEI message to identify the decoded data for view synthesis process.

In Table D.1, add the following after “constrained_depth_parameter_set_identifier( payloadSize )”:

| else if (payloadType = = 54) |  |  |
| --- | --- | --- |
| depth_type_info ( payloadSize ) |  |  |

After J.13.1.1 Constrained depth parameter set identifier SEI message syntax, add the following syntax Table:

    - Depth type information SEI message syntax

| depth_type_info ( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_type | 5 | u (3) |
| } |  |  |

After J.13.2.1 Constrained depth parameter set identifier SEI message semantics, add the following:

    - Depth type information SEI message semantics

Refer section 3 Depth type semantics

- (option 2: VUI)

In 0.6, add the following paragraph after the last paragraph that starts with“A multiview video coding extension with depth information is specified in Annex J.”:

Alternative 3D format signaling is specified in Annex E: Video usability information. Associated non-normative view synthesis process is specified in its semantics specifications allowing the decoding of bitstreams that represent Alternative 3D format based on the ISO/IEC 14496-10 (ITU-T Rec. H.264) specifications. Since the Alternative 3D format represents both Global View and Depth format and Warp format, a depth_type is specified in the VUI parameter to identify the decoded data for view synthesis process.

In E.1.1 VUI parameters syntax, add the following syntax table after max_dec_frame_buffering }:

| - depth_type_info_present_flag | - 0 | - u(1) |
| --- | --- | --- |
| - if(depth_type_info_present_flag) |  |  |
| depth_type | 0 | u(3) |

In E.2.1 VUI parameter semantics, add the following text after the paragraph on max_dec_fram_buffering:

depth_type_info_present_flag equal to 1 specifies that signaling of alternative 3D format by depth_type is present. Depth_type_info_present_flag equal to 0 specifies that the signaling is not present.

depth_type refer section 3 Depth type semantics.

- (common)

In A.2, add the following section after section A.2.11:

    - Alternative 3D format profile

- Conformance of a bitstream to the alternative 3D format profile is indicated by profile_idc being equal to 140, which requires the view synthesis conformance. Decoders conforming to the alternative 3D format profile shall be capable of decoding all bitstreams and view synthesis in which profile_idc is equal to 140.

- NOTE1 –Profile_idc = 140 may not be used if failing of non-conforming view synthesis is allowed. In such case, profile_idc = 138 for MVC+Depth or = 139 for 3D-MVC are used.

# Depth type semantics

depth_type specifies the view synthesis type for Alternative 3D format for Multiview and depth video coding (Annex I and Annex J). The depth_type value is defined in Table D-10 (Table E-7).

- Table D-10 (Table E-7) – Definition of depth_type

| - depth_type | - interpretation |
| --- | --- |
| - 0 | - Global depth for 2 view |
| - 1 | - Global depth and residual depth for 2 view |
| - 2 | - Global depth for 3 view |
| - 3 | - Global depth and residual depth for 3 view |
| - 4 | - Reserved for Warp |
| - 5…6 | - Unspecified |
| - 7 | - Reserved for future extension |

- When depth_type = 0 or 2, target view is synthesized as follows. First, global depth is up-sampled by 2x2 horizontally and vertically. During the up-sampling, if the difference of contiguous two depth samples are smaller than th (=10, decoder dependent), an average sample is inserted. Otherwise, a larger sample is repeated. Then, 2D median filter of 5x5 is applied to smooth the shape of depth edge. Then, the up-sampled depth map is projected to the target view-point. If depth_type = 0 (two-view case), global depth is in the middle of two views. If depth_type = 2 (three-view case), global depth is at the center view. After the forward projection, holes are inpainted with a smaller sample at the left or right edge of the hole. Then, the base view is inversely projected to the target views.

- Before the view projection, occlusion holes must be detected by checking the depth values D(x, y) with the following equation first. Here, B is the baseline length between the target view and the base view. th is 5 (view synthesizer dependent).

- for(0≤δ≤fBZnear)if Δ=(D(x+δ,y)−D(x,y))≥th, (x,y) might be in an occlusion hole.	(3.1)

- Then, the disparity differenceεis calculated from the depth difference Δ as follows. Where k = 0.6 (view synthesizer dependent) is a margin for the mismatch between depth map and texture.

- ε=fB255(1Znear−1Zfar)Δ+fB255∗1Zfarif[Δ≥δ, or D(x+ε,y)−D(x,y)≥k∗Δ], (x,y) is in an occlusion hole.	(3.2)

- If (x, y) is in an occlusion hole, the pixel (x, y) is not projected but registered to an occlusion hole mask. Equation (3.1) and (3.2) are for two-view case and include some tolerance for depth error. If the target view is between the base view and right residual view, x-δ instead of x+δ is used in equation (3.1) and (3.2). The target view is projected in sub-pixel accuracy by averaging two contiguous pixels in the reference view weighted by the mantissa of disparity value calculated from the depth value. The area which is not covered by the base view is additionally registered to the hole mask. Then, the occlusion hole mask is dilated by two pixels allowing the mismatch between depth map and texture. Then, the decoded left residual view pixels and right residual view pixels (NULL for two view case), which reside in upper left quarter and lower left quarter in the residual view respectively, are up-sampled by 2x2 horizontally and vertically with a two-tap (0.5, 0.5) filter and projected to the occlusion hole in the target view. During the projection, occlusion hole in the residual view is detected and registered to a common hole mask in the same way as the base view projection. Then, the common hole is inpainted by the surrounding pixels. Then, horizontally two pixels around the hole edge are filtered by three-tap filter(0.25, 0.5, 0.25) if their difference is smaller than a threshold (=20, view synthesizer dependent).

- When depth_type = 1 or 3, target view is synthesized as follows.

First, base depth is up-sampled by 2x2 horizontally and vertically. During the up-sampling, if the difference of contiguous two depth samples are smaller than th (=10, decoder dependent), an average sample is inserted. Otherwise, a larger sample is repeated. Then, 2D median filter of 5x5 is applied to smooth the shape of depth edge. Then, the up-sampled depth map is projected to the target view-point for base view projection. In addition, the up-sampled depth map is projected to left or right view-point for generating residual hole masks. If depth_type = 1 (two-view case), base depth is at the right one of two views. If depth_type = 3 (three-view case), base depth is at the center view. Next, the decoded left residual depth and right residual depth (NULL for two view case), which reside in the upper half and the lower half of the residual depth, are up-sampled by 2x2 horizontally and vertically in the same way as the base depth. Then, residual area of the up-sampled residual depth are projected to the target view-point by using the residual hole masks. After that, remaining holes in the target depth are inpainted with a smaller sample at the left or right edge of the hole. Then, the base view is inversely projected to the target views. Before the base view projection, occlusion holes must be detected by the same way as that for depth_type = 0 or 2. Then, the occlusion hole mask is dilated by two pixels allowing the mismatch between depth map and texture. Then, the decoded left residual view pixels and right residual view pixels (NULL for two view case), which reside in upper left quarter and lower left quarter in the residual view respectively, are up-sampled by 2x2 horizontally and vertically with a two-tap (0.5, 0.5) filter and backwardly projected to the occlusion hole in the target view. During the projection, occlusion hole in the projected residual view is detected and registered to a common hole mask in the same way as the base view projection. Then, the common hole is inpainted by the surrounding pixels. Then, horizontally two pixels around the hole edge are filtered by three-tap filter(0.25, 0.5, 0.25) if their difference is smaller than a threshold (=20, view synthesizer dependent).

- NOTE1 –In case NAL unit type = 0 (or one byte “00”) is detected just after start code = 00 00 01, extract following depth streams by deleting the one byte “ 00” in such NAL unit. Extracted depth stream is decodable by MVC (Annex H) decoders.

Further details are described in the following references.

- ISO/IEC JTC1/SC29/WG11 M22573, Technical description of 3DV proposal P-27

- JCT2-A00072 | ISO/IEC JTC1/SC29/WG11 M26104, Proposal on Simple Test Model for 3DV Coding based on CE7.

- JCT3V-B0075| ISO/IEC JTC1/SC29/WG11 M26809

- JCT3V-C0058| ISO/IEC JTC1/SC29/WG11 M27793, AHG08: Technical Description of residual data generation and target view synthesis in GVD (Global View and Depth) 3D Format

# Patent rights declaration(s)

NICT (National Institute of Information and Communications Technology) and NISRI (Nagoya Industrial Science Research Institute) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).