| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0048r4 /M27783r4 |
| --- | --- |

| Title: | AHG8: Draft Text for Signaling of Alternative 3D Format in ISO/IEC 14496-10 (ITU-T H.264) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Yasuyuki Ichihashi, Hisayuki Sasaki, Kenji Yamamoto Nukui-Kitamachi 4-2-1, Koganei, Tokyo 184-8795, Japan Masayuki Tanimoto, Kazuyoshi Suzuki Zehn 101, Kawana-cho 2-64, Showa-ku, Nagoya, 466-0856, Japan Nicolce Stefanoski Zurich, Switzerland | Tel: Email: | +81-42-327-7262 {senoh, y-ichihashi, sasaki, k.yamamoto}@nict.go.jp +81-52-223-6639 tanimoto@nagoya-u.ac.jp, kazuyoshi-suzuki@cb3.so-net.ne.jp stefanos@disneyresearch.com |
| Source: | National Institute of Information and Communications Technology(NICT) Nagoya Industrial Science Research Institute(NISRI) |  |  |

_____________________________

# Abstract

This contribution describes a compact signaling (SEI) and descriptions required to identify different types of 3D data formats (GVD / Warp), as usable to interpret decoded 3D data by non-normative processing after decoding, for ISO/IEC 14496-10 / ITU-T H.264 :201X(E).

# Proposed Syntax

In 0.6, add the following paragraph after the last paragraph that starts with “A multiview video coding extension with depth information is specified in Annex J.”:

Alternative 3D format signaling is specified in Annex J.13: 3D-AVC SEI message. Associated non-normative view synthesis process is specified in its semantics specifications allowing the decoding of bitstreams that represent Alternative 3D format based on the ISO/IEC 14496-10 (ITU-T Rec. H.264) specifications. Since the Alternative 3D format represents both Global View and Depth format and Warp format, a depth_view_info is specified in the 3D-AVC SEI message to identify the decoded data for view synthesis process.

In Table D.1, add the following after “constrained_depth_parameter_set_identifier( payloadSize )”:

(this and following naming shall be replaced by one for payloadType=54.)

| else if (payloadType = = 55) |  |  |
| --- | --- | --- |
| depth_view _info ( payloadSize ) |  |  |

After J.13.1.1 Constrained depth parameter set identifier SEI message syntax, add the following syntax Table:

    - Depth view information SEI message syntax

| - depth_view_info ( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_type | 5 | u(2) |
| If(depth_type==0){ |  |  |
| num_residual_texture_views | 5 | u(3) |
| num_residual_depth_views | 5 | u(3) |
| } |  |  |
| } |  |  |

# Proposed Semantics

After J.13.2.1 Constrained depth parameter set identifier SEI message semantics, add the following:

    - Depth view information SEI message semantics

- The syntax elements in the depth view information SEI message for alternative 3D format specifies various depth type, number of residual texture views and number of residual depth views for the purpose of processing decoded texture and depth view components prior to rendering on a 3D display, such as view synthesis. The depth view information SEI message, when present, is associated with an IDR access unit for the purpose of random access. The information signaled in the SEI message applies to all the access units from the access unit the SEI message is associated with to the next access unit, in decoding order, containing an SEI message of the same type, exclusively, or to the end of the coded video sequence, whichever is earlier in decoding order.

- Alternative 3D format data is encoded in two texture view streams and one or two depth view streams. They are base texture view stream, non-base texture view stream, base depth view stream and non-base depth view stream. One alternative 3D format contains up to five texture views and five depth views in the coded two texture view bitstreams and coded two depth view bitstreams. They are signalled as follows.

- depth_type specifies the depth type of global depth or warp as specified in Table J.13-1. If depth_type=0, the depth type is global depth and alternative 3D format contains up to five texture views and up to five depth views. The sampling ratio between the texture view and the depth view is 1:1/4 (1:1/2x1/2). Among five texture views, base view is encoded in the base texture view bitstream and the other up to four texture views are subsampled, packed in a picture and encoded in a non-base texture view bitstream. Among five depth views, base depth view is encoded in the base depth view bitstream and the other up to four depth views are subsampled, packed in a depth picture and encoded in a non-base depth view bitstream. If depth_type=1, the depth type is warp and two texture views are encoded in the base texture view stream and the non-base texture view stream. Tow warp data are encoded in the base depth view stream and the non-base depth view stream.

num_residual_texture_views specifies the number of residual texture views which are subsampled, packed in a picture and encoded in the non-base texture view bitstream. The base texture view is encoded in the base texture view bitstream. The sampling ratio between the base texture view and the residual texture views is 1:1/4 (1:1/2x1/2). The value of num_residual_texture_views shall be in the range of 0 to 4, inclusive. The location order of residual texture views in the non-base texture view bitstream is, according to the num_residual_texture_views, from top-left (nearest left residual texture view to the base texture view), bottom-left (nearest right residual texture view to the base texture view), top-right (second near left texture view to the base texture view) and bottom-right (second near right texture view to the base texture view) in a picture. If num_residual_texture_views <4, the left over area in the non-base texture view is NULL (filled with the average color of the texture).

num_residual_depth_views specifies the number of residual depth views which are subsampled, packed in a depth picture and encoded in the non-base depth view bitstream. The base depth view is encoded in the base depth view bitstream. The sampling ratio between the base depth view and residual depth views is 1:1/4 (1:1/2x1/2). The value of num_residual_depth_views shall be in the range of 0 to 4, inclusive. The location order of residual depth views in the non-base depth view bitstream is, according to the num_residual_depth_views, from top-left (nearest left residual depth view to the base depth view), bottom-left (nearest right residual depth view), top-right (second near left residual depth view to he base depth view) and bottom-right (second near right residual depth view to the base depth view) in a picture. If num_residual_depth_views <4, the left over area in the non-base depth view is NULL (filled with the average color of the depth).

- Table J.13-1 – Definition of depth_type

| - depth_type | - interpretation |
| --- | --- |
| - 0 | - Global depth |
| - 1 | - Reserved (Warp) |
| - 2…7 | - Reserved |

-If num_residual_depth_views =0 and num_residual_texture_views = odd, the base depth view is not associated with the base texture view, but resides in the center of texture views. Otherwise, the base depth view is associated with the base texture view.

-Alternative 3D format can be applied to multivews and depth maps sequences which have more than five views. In such case, alternative 3D format is repeated over the views, starting from the farthest residual view in the previous group.

- View synthesis

- When num_residual_depth_views = 0, target view is synthesized as follows. First, base depth is up-sampled horizontally and vertically to the size of the base texture view. During the up-sampling, if the difference of contiguous two depth samples are smaller than th (=10, decoder dependent), an average sample is inserted. Otherwise, a larger sample is repeated. Then, 2D median filter of 5x5 is applied to smooth the shape of depth edge. Then, the up-sampled depth map is projected to the target view-point. If num_residual_depth_views = 0 and num_residual_texture_views = odd, base depth view is in the center of the texture views. Otherwise, base depth view is associated with the base texture view. After the forward projection, holes are inpainted with a smaller sample at the left or right edge of the hole. Then, the base texture view is inversely projected to the target views.

- Before the base texture view projection, occlusion holes must be detected by checking the target depth values D(x, y) with the following equation first. Here, B is the baseline length between the target view and the base view. th is 5 (view synthesizer dependent).

- for(0≤δ≤fBZnear)if Δ=(D(x+δ,y)−D(x,y))≥th, (x,y) might be in an occlusion hole.	(J.13-1)

- Then, the disparity differenceεis calculated from the depth difference Δ as follows. Where k = 0.6 (view synthesizer dependent) is a margin for the mismatch between depth map and texture.

- ε=fB255(1Znear−1Zfar)Δ+fB255∗1Zfarif[Δ≥δ, or D(x+ε,y)−D(x,y)≥k∗Δ], (x,y) is in an occlusion hole.	(J.13-2)

- If (x, y) is in an occlusion hole, the pixel (x, y) is not projected but registered to an occlusion hole mask. Equation (J.13-1) and (J.13-2) are for the case where target view resides on the left hand side of the base view. They include some tolerance for depth error. If the target view resides on the right hand side of the base view, x-δ instead of x+δ is used in equation (J.13-1) and (J.13-2). The target texture view is synthesized in sub-pixel accuracy by averaging two contiguous pixels in the base texture view, weighted by the mantissa of disparity value calculated from the target depth value. The area which is not covered by the base texture view is additionally registered to the occlusion hole mask. Then, the occlusion hole mask is dilated by two pixels allowing the mismatch between the target depth map and target texture. Then, the decoded residual texture which is closest to the target view but locating at the opposite side of the target view comparing to the base view in the non-base texture view is up-sampled to the same size as the base texture view with a two-tap (0.5, 0.5) filter horizontally and vertically. Then, it is projected to the occlusion hole in the target texture view. During the projection, occlusion hole in the residual texture view is detected and registered to a common hole mask in the same way as the base texture view projection. Then, the common hole area in the target view is inpainted by the surrounding pixels. Then, horizontally two pixels around the hole edge are filtered by three-tap filter(0.25, 0.5, 0.25) horizontally, if their difference is smaller than a threshold (=20, view synthesizer dependent).

- When num_residual_depth_views != 0, target view is synthesized as follows.

First, base depth view is up-sampled horizontally and vertically to the same size as the base texture view. During the up-sampling, if the difference of contiguous two depth samples are smaller than th (=10, decoder dependent), an average sample is inserted. Otherwise, a larger sample is repeated. Then, 2D median filter of 5x5 pixels is applied to smooth the shape of the depth edge. The base depth view is associated with the base texture view. Then, the up-sampled base depth is projected to the target view-point for the base view projection. In addition, the up-sampled base depth is projected to the nearest residual texture view-point, which resides in the opposite side of the target view comparing to the base view, for generating residual hole masks. Next, the extracted residual depth view, which is closest to the target view but residing in the opposite side of the target view comparing to the base view, is up-sampled to the same size as the base texture view in the same way as the base depth was done. Then, residual area of the up-sampled residual depth is projected to the target view-point by using the generated residual hole masks. After that, remaining holes in the target depth are inpainted with a smaller sample at the left or right edge of the hole. Then, the base texture view is inversely projected to the target view. Before the base texture view projection, occlusion holes must be detected by the same way as that for num_residual_depth_views = 0 case. Then, the occlusion hole mask is dilated by two pixels allowing the mismatch between depth map and texture. Then, the extracted residual texture from the non-base texture view, which reside closest to the target view but residing in the opposite side of the target view comparing to the base texture view, is up-sampled to the same size as the base texture view with a two-tap (0.5, 0.5) filter horizontally and vertically. Then, it is backwardly projected to the occlusion hole in the target view. During the projection, occlusion hole in the projected residual view is detected and registered to a common hole mask in the same way as the base view projection. Then, the common hole is inpainted by the surrounding pixels. Then, horizontally two pixels around the hole edge are filtered by three-tap filter(0.25, 0.5, 0.25) horizontally, if their difference is smaller than a threshold (=20, view synthesizer dependent).

Further details are described in the following references.

- ISO/IEC JTC1/SC29/WG11 M22573, Technical description of 3DV proposal P-27

- JCT2-A00072 | ISO/IEC JTC1/SC29/WG11 M26104, Proposal on Simple Test Model for 3DV Coding based on CE7.

- JCT3V-B0075| ISO/IEC JTC1/SC29/WG11 M26809

- JCT3V-C0058| ISO/IEC JTC1/SC29/WG11 M27793, AHG08: Technical Description of residual data generation and target view synthesis in GVD (Global View and Depth) 3D Format

# Patent rights declaration(s)

NICT (National Institute of Information and Communications Technology) and NISRI (Nagoya Industrial Science Research Institute) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).