| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0052_proposed_text |
| --- | --- |

| Title: | Proposed text for JCT3V-C0052 based on 3D-HEVC Test Model 2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-5776-0696 xinzhao@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document provides the draft text changes of contribution JCT3V-C0052. Draft changes are identified in red texts and newly introduced texts are highlighted in green.

- G. 8.4.4.2.8.1 Derivation process for a texture predicted wedgelet index

Inputs to this process are:

- a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable nT specifying the transform size,

- a list candWedgeIndList specifying candidate wedgelet indices

- a variable numCandWedgeInd specifying the number of candidate wedgelet indices

- Output of this process is:

- a variable wedgeIdx specifying the predicted wedgelet index

The variable wedgeIdx is set equal to 0, and the following ordered steps apply for i ranging from 0 to numCandWedgeInd −1.

  - The variable candWedgeIdx is set equal to candWedgeIndList[ i ].

  - The variable candWedgePattern[ x ][ y ] with x, y =0..nT−1, specifying a candidate binary partition pattern is derived as.

      - candWedgePattern = WedgePatternTable[ Log2( nT) ][ candWedgeIdx ] 	(G-1)

  - The derivation process for a texture wedgelet distortion as specified in subclause Error: Reference source not found is invoked with the sample location ( xB, yB ), the transform size nT and the binary partition pattern candWedgePattern[ x, y ], as inputs and the output is assigned to candDist.

  - When wedgeIdx is equal to 0 or candDist is smaller larger than minDist, the variable wedgeIdx is set equal to candWedgeIdx and the variable minDist is set equal to candDist.

- G. 8.4.4.2.8.2 Derivation process for a texture wedgelet distortion

Inputs to this process are:

- a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable nT specifying the transform size,

- a binary array candWedgePattern[ x ][ y ], with x, y =0..nT−1, specifying a partitioning of the prediction block in a partition 0 and a partition 1.

Output of this process is:

- a variable candDist specifying the texture wedgelet distortion.

The variable recTextPic is set equal to the array of the reconstructed luma picture samples of the view component with view order index equal to ViewIdx and DepthFlag equal to 0.

The variable candDist is set equal to 0, and for X being 0 and 1, the following ordered steps apply for x from 0 to nT-2, inclusive.

  - The variable sumTextVal specifying the sum of the texture sample values of partition X is set equal to 0 and the variable numTextVal specifying the number of the texture samples of partition X is set equal to 0 and the following applies.

    - For x = 0..nT − 1 the following applies.

      - For y = 0..nT − 1 the following applies.

        - When candWedgePattern [ x ][ y ] is equal to X, the following applies.

- sumTextVal += recTextPic[ xB+ x ][yB+ y ]	(G-2)
numTextVal += 1		(G-3)

  - The variable dcVal specifying the constant partition value for partition X is derived as follows.

      - dcVal = ( numTextVal ! = 0 ) ? ( sumTextVal / numTextVal ) : ( 1 << ( BitDepthY  − 1 ) )	(G-4)

  - The variable candDist is derived as specified in the following.

    - For x = 0..nT − 1 the following applies.

      - For y = 0..nT − 1 the following applies.

        - When candWedgePattern[ x ][ y ] is equal to X, the following applies.

            - candDist += Abs( recTextPic[ xB+ x ][yB+ y ] − dcVal )	(G-5)

    - When candWedgePattern[ x ][ 0 ] is not equal to candWedgePattern[ x+1 ][ 0 ], candDist += Abs( p[ x+1 ][ -1 ] – p[ x ][ -1 ] )

    - When candWedgePattern[ 0 ][ x ] is not equal to candWedgePattern[ 0 ][ x+1 ], candDist += Abs( p[ -1 ][ x+1 ] – p[ -1 ][ x ] )

    - When candWedgePattern[ x ][ nT-1 ] is not equal to candWedgePattern[ x+1 ][ nT-1 ], candDist += Abs( p[ -1 ][ x+nT+1 ] – p[ -1 ][ x+nT ] )

    - When candWedgePattern[ nT-1 ][ x ] is not equal to candWedgePattern[ nT-1 ][ x+1 ], candDist += Abs( p[ x+nT+1 ][ -1 ] – p[ x+nT ][ -1 ] )

- Note that p is derived as specified in 8.4.4.2.1 and also G.8.4.4.2.1.