| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0052 |
| --- | --- |

| Title: | 3D-CE6.h: Results on distortion calculation simplification for Depth Modeling Mode (DMM) 3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-5776-0696 xinzhao@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes further simplification on Depth Modeling Mode (DMM) 3 in depth intra coding. In DMM 3, each Wedgelet candidate is evaluated by a complex cost derived by the following steps: 1) generating a prediction block for the co-located texture luma block (CTLB), and 2) calculation of the block-based Sum of Absolute Difference (SAD) between the CTLB and prediction block. The one with the smallest cost will be chosen as the final Wedgelet for current prediction unit. As a follow-up of JCT3V-B0053, in this contribution, it is proposed that the cost is simply derived by the sum of the absolute differences of two neighboring depth reference sample pairs, and the above two steps are totally removed to simplify the decoding process of DMM 3. It is reported that only negligible BD Rate difference is observed using this proposed method, while about 3%~5% of runtime saving is achieved for both encoder and decoder.

# Introduction

In 3D-HEVC, DMM is utilized for intra coding of depth map. In DMM, a depth prediction unit (PU) is partitioned into two regions using either pre-defined Wedgelet or generated Contour partitioning pattern, and each region is predicted by a constant value. As shown in Figure 1, each pixel in the PU is classified into either partition 0 or partition 1, denoted by P0 and P1, respectively. Four sub-modes are defined in DMM, and one of them, namely DMM 3, identifies a Wedgelet pattern instead of signalling the index at the decoder side based on the reconstructed co-located texture luma block (CTLB). Given a list of Wedgelet pattern candidates, the DMM 3 will evaluate each candidate by the following steps:

- The Wedgelet pattern candidate is applied on the CTLB, then the average value of all the components in CTLB in partition 0 is calculated as P0, and the average value of all the components in CTLB in partition 1 is calculated as P1. Then a predicted CTLB is generated with components in partition 0 equal to P0, and components in partition 1 equal to P1.

- The blocked-based SAD between CTLB and predicted CTLB is calculated.

Then the Wedgelet pattern with least SAD value will be selected.

Figure 1: Example of a Wedgelet pattern

Although fast Wedgelet search method [1] has been included in 3D-HEVC and only a subset of Wedgelet patterns are involved in the process of identifying the optimal Wedgelet pattern. The DMM 3 still needs to generate a predicted CTLB which needs division operations and access to the reconstructed texture buffer, also a block-based SAD calculation is performed for each Wedgelet pattern candidate. Therefore the DMM 3 is still relatively high-complexity for decoder.

# Proposed Solution

To simplify the cost calculation for each Wedgelet pattern candidate, in JCT3V-B0053 [2], it is proposed to use the neighboring depth reference samples to identify the Wedgelet pattern for DMM 3.

## Mapping of reference samples

The extended reference samples are mapped to the right and bottom prediction unit (PU) boundaries. An example of the reference sample mapping for a 4×4 PU is illustrated in Figure 2. While the top and left (boundary) reference samples (denoted as P0,-1 to P3,-1 and P-1,0 to P-1,3) are always available, the right and bottom (boundary) reference samples are not available when coding current PU. As denoted by the dotted arrows in Figure 1, the extended reference samples in the top/left (denoted as P4,-1 to P7,-1 and P-1,4 to P-1,7) are rotated clockwise 90/-90 degrees to form the new boundary reference samples for the right/bottom PU boundary.

Figure 2: Mapping of right/bottom reference samples to the boundary of a depth PU

It is noted that only a conversion of the coordinates is involved in the above reference sample mapping process.

## Cost calculation using reference samples

For a Wedgelet pattern, initialize a cost D as 0, then the following steps are applied,

- If the two partition value (partition 0 or 1) changes at PU top/bottom boundary from Ci, j to Ci+1, j, then the cost D is calculated as follows:

D += | Pi, k – Pi+1, k |,

where k equals to -1 and the height of PU corresponding to the partition value changing at top and bottom boundary, respectively.

- If the two partition value (partition 0 or 1) changes at PU left/right boundary from Ci, j to Ci, j+1, then the cost D is calculated as follows:

D += | Pk, i – Pk, i+1 |,

where k equals to -1 and the width of PU corresponding to the partition value changing at left and right boundary, respectively.

The Wedgelet pattern with the maximum value of D is chosen for the current PU which is coded with DMM 3.

For example, in Figure 2, given the Wedgelet pattern, the partition value changes at PU boundary from C3, 0 to C3, 1 and from C0, 3 to C1, 3, then the distortion of this given Wedgelet pattern is calculated as D = | P4, 0 - P4, 1 | + | P0, 4 – P1, 4 |.

It is noted that the partition value change location at PU boundary used in the cost calculation can be derived in two alternatives:

- By checking the partition value of each boundary sample and locate the change position as specified in the text changes.

- By directly checking the start and end positions of the Wedgelet pattern which are available in current 3D-HEVC.

The differences between the two alternatives only involve editorial changes.

# Experimental Results

The proposed method is implemented on top of HTM-5.0.1, and simulations were performed under “CTC” [3] and “Intra-only” [4].

The results are summarized in Table 1 and 2, respectively. As it is reported, for “CTC” and “Intra-only”, the proposed method introduces 0.04% and 0.01% BD-rate increase for coded and synthesized views, respectively.

Table 1: BD rate results for 3-view case under CTC

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.06% | 0.06% | 97.0% | 95.6% | 94.8% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.07% | -0.04% | 94.7% | 91.4% | 96.8% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.02% | 0.01% | 93.7% | 92.0% | 94.2% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | -0.07% | -0.04% | 97.0% | 97.0% | 97.5% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | 95.5% | 93.0% | 96.8% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.02% | 0.01% | 102.9% | 102.6% | 100.3% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.44% | 0.29% | 90.5% | 93.6% | 97.1% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.01% | 0.01% | 95.1% | 93.0% | 95.3% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.09% | 0.07% | 96.4% | 96.5% | 97.9% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.06% | 0.04% | 95.8% | 95.0% | 96.8% |

Table 2: BD rate results for 3-view case under Intra-only

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.01% | 0.00% | 94.3% | 96.8% | 98.9% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | 97.8% | 97.2% | 96.0% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.01% | 0.01% | 96.3% | 95.7% | 99.4% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.11% | 0.08% | 96.5% | 95.6% | 98.7% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 97.0% | 97.3% | 101.7% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 99.8% | 98.7% | 102.4% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.03% | 0.03% | 97.1% | 95.9% | 96.2% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 96.1% | 96.6% | 98.1% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.04% | 0.03% | 97.6% | 96.8% | 99.7% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.02% | 0.01% | 97.0% | 96.7% | 99.0% |

# References

- P. Merkle, K. Mueller, X. Zhao, Y. Chen, L. Zhang, M. Karczewicz, “3D-CE6.h: results on simplified Wedgelet search for DMM modes 1 and 3” JCT3V-B0039, Shanghai, CN, 13–19 Oct. 2012.

- X. Zhao, L. Zhang, Y. Chen, M. Karczewicz, “3D-CE6.h related: Distortion calculation simplification for Depth Modeling Mode (DMM) 3” JCT3V-B0053, Shanghai, CN, 13–19 Oct. 2012.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-B1100, Shanghai, CN, 13–19 Oct. 2012.

- P. Merkle, “Common Test Conditions of 3DV Core Experiments,” JCT3V-B1106, Shanghai, CN, 13–19 Oct. 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).