| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0053 |
| --- | --- |

| Title: | 3D-CE1.a: Results on GVSP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-5776-0696 xinzhao@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution provides a bug-fixed version of Generalized View Synthesis Prediction (GVSP) to reflect the adoption of the previous proposal JCT3V-B0054. With the bug-fixed version, the VSP reference is treated as a conventional reference picture at the encoder side where it may be selected based on the rate-distortion criteria. Moreover, the benefit of removal of VSP reference from the reference picture list is tested. Based on the current 3D-ATM, it is reported that by removing VSP reference from the reference list, average 0.21% BD rate gain is achieved for texture coding compared to that kept in the reference list, while some decoding time saving is also achieved.

# Introduction

In 3DV-ATM, in-loop View Synthesis Prediction (VSP), which generates the prediction samples using a block-based backward warping process [1], is utilized for enhanced texture coding. To further improve the coding performance of VSP, a sub-block VSP signaling method is proposed in JCT3V-B0054 [2] and incorporated in 3DV-ATM version 6.0.

With the integration of JCT3V-B0054 which is first released in ATM version 6.0, the VSP reference is kept in the reference list, but will not be selected by the encoder. Therefore, the VSP reference can still be regarded as being disabled by the integration of JCT3V-B0054 which is not aligned with the meeting notes. To resolve this problem, in this contribution, a bug-fixed version is released and enabled for both the anchor and proposed method in this test. In the software released for this contribution, the bug-fix is encapsulated by macro QC_VSP_SIG_BUGFIX which is enabled by setting the value as 1. After that, VSP can be signaled with either sub-block vsp_flag [2] or VSP reference index, and the selection is based on the mode decision process at encoder.

# Proposal

In this contribution, the removal of VSP reference from reference list is further tested on top of the latest 3DV-ATM version 6.1. With JCT3V-B0054 enabled, comparison of VSP signaling with and without the removal of VSP reference from the reference picture list is described below, and changes are highlighted in yellow,

- ATM version 6.1 with JCT3V-B0054 enabled

- When an inter-coded MB is a non-skip MB, a flag is introduced to indicate whether the whole MB or each MB partition is predicted using B-VSP or not.

- If a MB or MB partition is predicted from B-VSP, no reference index is transmitted and residual information such as CBP and coded coefficients are signaled as usual. Otherwise, the reference index will still be transmitted and could be a VSP reference index.

- ATM version 6.1 with JCT3V-B0054 enabled and VSP reference removed

- When an inter-coded MB is a non-skip MB, a flag is introduced to indicate whether the whole MB or each MB partition is predicted using B-VSP or not.

- If a MB or MB partition is predicted from B-VSP, no reference index is transmitted and residual information such as CBP and coded coefficients are signaled as usual. Otherwise, the reference index will still be transmitted but could not be a VSP reference index.

# Experimental results

The proposed method is implemented based on 3DV-ATM v6.1 software. Simulations are performed under the common test condition (CTC) [3] with QC_GVSP_B0054 enabled. The anchor is ATM v6.1 with both QC_GVSP_B0054 and QC_VSP_SIG_BUGFIX enabled, and the tested method is the anchor with the VSP reference index removed from reference list.

Table 1 provides the results of the proposed method compared with 3D-ATM anchor. The overall average bitrate reduction is 0.21%, 0.19%, 0.15% for decoded texture views, total coded views and synthesized views, respectively.

Table 1: Coding gain with respect to 3D-ATM anchor for 3-view case

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Syn. PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR % | dPSNR dB | dBR % | dPSNR dB | dBR % | dPSNR dB | dBR % | dPSNR dB | Enc. Time | Dec. Time | Ren. Time |
| S01 | -0.04 | 0.00 | 0.00 | 0.00 | -0.04 | 0.00 | -0.09 | 0.00 | 1.04 | 0.93 | 1.02 |
| S02 | -0.18 | 0.01 | 0.00 | 0.00 | -0.17 | 0.01 | -0.15 | 0.00 | 1.04 | 0.85 | 0.99 |
| S03 | -0.26 | 0.01 | 0.00 | 0.00 | -0.24 | 0.01 | -0.19 | 0.01 | 1.03 | 0.87 | 1.06 |
| S04 | -0.29 | 0.01 | 0.00 | 0.00 | -0.28 | 0.01 | -0.21 | 0.01 | 1.01 | 0.90 | 0.95 |
| S05 | -0.28 | 0.01 | 0.00 | 0.00 | -0.26 | 0.01 | -0.15 | 0.01 | 0.93 | 0.93 | 1.04 |
| S06 | -0.23 | 0.01 | 0.00 | 0.00 | -0.22 | 0.01 | -0.17 | 0.01 | 0.99 | 0.93 | 1.06 |
| S08 | -0.16 | 0.01 | 0.00 | 0.00 | -0.15 | 0.01 | -0.11 | 0.00 | 0.93 | 0.96 | 0.94 |
| Aver | -0.21 | 0.01 | 0.00 | 0.00 | -0.19 | 0.01 | -0.15 | 0.01 | 0.99 | 0.91 | 1.01 |

# References

- W. Su, D. Rusanovskyy, M. M. Hannuksela, “3DV-CE1.a: Block-based View Synthesis Prediction for 3DV-ATM,” JCT3V-A0107, Stockholm, SE, 16–20 July 2012.

- X. Zhao, L. Zhang, Y. Chen, “3D-CE1.a: Generalized view synthesis prediction (GVSP) mode” JCT3V-B0054, Shanghai, CN, 13–19 Oct. 2012.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-B1100, Shanghai, CN, 13–19 Oct. 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).