| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0056 |
| --- | --- |

| Title: | Asymmetric spatial resolutions for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Ying Chen Li Zhang Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-5776-0696 xinzhao@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a new asymmetric coding scheme for 3D-HEVC which enables stereoscopic video coding with different spatial resolutions of base and non-base views. Typically, in this contribution, the scheme with full-resolution base view and half-resolution non-base views is investigated. To enable inter-view prediction from full-resolution base view to half-resolution non-base views, a downsampled version of the reconstructed reference picture from base view is stored in the reference picture buffer as an additional inter-view reference picture. Texture and depth views in the same view are coded with the same resolution. It is asserted that the proposed scheme is beneficial to reduce the total bitrate and complexity of both encoder and decoder. Simulation results show that, on 3D-HTM version 4.1 with several enhancement coding tools disabled, the proposed scheme, although not mature enough, is able to achieve some noticeable BD Rate reduction with significant runtime savings for both encoder and decoder, especially for the encoder.

# Introduction

Asymmetric coding scheme enables stereoscopic video coding with different spatial resolutions. A typical scenario of asymmetric scheme is coding a subset of views with original full resolution and the remaining views with a lower resolution. Practical schemes of asymmetric coding have been proposed for multiview video coding in [1], [2] and [3], and one typical method is to downsample the inter-view reference picture as follows:

As shown in Figure 1, the inter-view reference pictures of the higher resolution are downsampled to the same resolution of the current view [1], such that conventional motion compensation can be applied without changes. With this downsampling-based solution, either the downsampled picture is stored in the reference picture buffer or an on-the-fly downsampling process is performed during the motion compensation.

Figure 1: Interview prediction in asymmetric coding based on inter-view reference downsampling.

Since a subset of the input views are with low-resolution, the asymmetric coding scheme has a direct impact on the reduction of total bitrate and is able to achieve significant computation complexity reduction.

In the scenario of 3D video coding, to generate virtual views between coded views, the low-resolution coded views are upsampled, such that the view synthesis algorithm, i.e., depth image based rendering (DIBR), can be applied.

# Motivation

A 3D-HEVC decoder needs to process multiple texture and depth views and this might become a burden for real-time decoding, e.g., on mobile devices. Although complexity reduction can be achieved by reducing the picture resolution for, e.g., all the texture views or all the depth views, it might be desirable to maintain a relatively high quality for the middle view, especially for auto-stereoscopic display.

For the purpose of auto-stereoscopic representation of 3D video content, view synthesis is always assumed. In this case, by coding a view with lower-resolution but displaying it in higher-resolution and using it to synthesize a high-resolution view, the sub-sampling error might not be linearly propagated to the overall system.

# Proposal

To further investigate the performance of asymmetric coding scheme on 3D-HTM codec which is under development, in this contribution, the downsampling-based solution is integrated on 3DV-HTM version 4.1 with additional optimizations of both the quantization parameter settings and upsampling process for view synthesis.

In a typical case of three coded views, assume the original resolution of each view is W×H. With proposed asymmetric coding scheme, the base view is kept as full resolution (W×H) and two non-base views are input as half resolution (0.5W×H), as shown in Figure 3.

(a) Left non-base view                                         (b) Base view                                           (c) Right non-base view

Figure 3: Asymmetric spatial resolution setting for 3-view case

The half-resolution non-base views are generated by the MPEG-4 13-tap downsampling filter [2, 0, −4, −3, 5, 19, 26, 19, 5, -3, -4, 0, 2]/64, and the downsampling process is performed outside the coding loop.

## Reference downsampling and selection

Once a picture in the base view is coded, a downsampled version (0.5W×H) with the MPEG-4 13-tap filter is generated and stored together with the original full-resolution version in the decoded picture buffer.

During the motion compensation process, given a reference index, when the corresponding reference picture has a different resolution of current view (i.e., an inter-view reference picture), the downsampled version of the reference picture is selected as the reference picture. In this way, the reference picture always has the same spatial resolution with the current view, and conventional motion compensation can be applied without change.

## Adaptive upsampling

To generate the synthesized views using decoded views of asymmetric spatial resolutions, the decoded non-base views need to be upsampled to the original full resolution, such that the current reference view synthesis software can be used. The efficiency of the upsampling algorithm is important since the upsampling process has a direct impact on the quality of upsampled full-resolution non-base views, which are also used to generate synthesized views.

In this contribution, a picture-level adaptive upsampling filtering (AUF) method is used. The AUF method enables picture-level adaptive selection between a pre-defined fixed upsampling filter and an adaptive FIR filter. The adaptive filter coefficients are signalled and included in the total bitrate for coding gain calculation.

# Experimental results

The proposed method is implemented on top of HTM-4.1, and simulations were performed under “CTC” [4] with all enhancement tools disabled, such as VSO, DMM, MVI, multiview motion vector prediction and interview residual prediction. Note the QP settings are changed for the proposed method to roughly match the bitrates of anchor data.

The results are summarized in Table 1. As it is reported, for “CTC” with the above enhancement tools disabled, the proposed method could achieve average -1.6% BD Rate reduction for “Balloons”, “Kendo”, “Newspaper”, “PoznanHall2” and “PoznanStreet”. Considerable encoder runtime saving, around 40%, is also achieved and around 9% decoder runtime saving is observed. Note that the PSNR of the non-base views are calculated based on the upsampled pictures.

In the presented results, the results for “GhostTownFly”, “UndoDancer” are not included, it is noticed that the proposed method might not work well for these two sequences at this stage.

There is some minor difference for the coding of video 0, since the QP for base view texture coding is slightly changed.

Table 1: BD rate results for 3-view case under CTC

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.1% | -16.7% | -17.0% | 3.0% | -3.2% | -0.7% | 59.2% | 88.5% | 100.8% |
| Kendo | 0.0% | -15.9% | -16.0% | 2.6% | -3.4% | -1.2% | 60.5% | 92.7% | 103.2% |
| Newspapercc | -0.3% | -13.6% | -9.2% | 7.9% | 0.8% | 4.8% | 60.6% | 94.3% | 102.9% |
| PoznanHall2 | -0.1% | -9.7% | -11.8% | 12.9% | -2.3% | 2.8% | 57.5% | 88.4% | 99.2% |
| PoznanStreet | -0.1% | -23.2% | -20.1% | 11.9% | 0.2% | 3.8% | 59.8% | 90.4% | 99.0% |
| 1024x768 | -0.1% | -15.4% | -14.1% | 4.5% | -1.9% | 1.0% | 60.1% | 91.8% | 102.3% |
| 1920x1088 | -0.1% | -16.5% | -16.0% | 12.4% | -1.0% | 3.3% | 58.7% | 89.4% | 99.1% |
| average | -0.1% | -15.8% | -14.8% | 7.7% | -1.6% | 1.9% | 59.5% | 90.8% | 101.0% |

# Conclusions and discussions

Asymmetric coding scheme is proposed for 3D-HEVC in this proposal. By downsampling theinter-view references, the proposed scheme is able to achieve coding of base and non-base views with asymmetric spatial resolutions. It is reported that -1.6% BD Rate reduction for synthesized views is observed using the proposed scheme on HTM-4.1 with several enhancement tools disabled, while about 40% and 9% of average runtime are saved for encoder and decoder, respectively.

Moreover, the proposed method can be turned on and off for difference sequences and for different target bitrates, such that the high-end applications do not necessarily sacrifice from the potential quality degradation and noticeable improvements can be achieved in scenarios requiring less computations or with a limited bandwidth using the this method.

Although other normative coding tools are not fully opened, preliminary tests show that these tools can be straightforwardly applied to the proposed asymmetric 3D-HEVC scheme.

Potential further improvements in the following aspects might be possible:

- Based on the characteristics of the content, the downsampling may be applied in the horizontal direction, if there is more horizontal high frequency for a given video sequence.

- The adaptive filter may be also improved. Currently, the adaptive filter provides minor coding gain compared to the fixed filter.

- More optimal bit allocation for different texture or depth views.

- Encoder side optimization, e.g., rate distortion optimization.

- Downsampling of the base view requires more memory. However, it is possible that direct motion compensation from the high resolution view can be done as described in [3].

# References

- H. Kimata, S. Shimizu, “Inter-view prediction with down-sampled reference pictures”, Joint Video Team, JVT-W079, San Jose, California, USA, 21–27 April, 2007.

- H. Kimata, S. Shimizu, “MVC experimental results for down-sampled reference pictures”, Joint Video Team, JVT-X077, Geneva, CH, 29 June – 5 July, 2007.

- Y. Chen, S. Liu, Y.-K. Wang, M. M. Hannuksela, H. Li, “Low complexity asymmetric multiview video coding”, Joint Video Team, JVT-Y054, Shenzhen, CN, 21–26 October, 2007.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-B1100, Shanghai, CN, 13–19 Oct. 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).