| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0060r1 |
| --- | --- |

| Title: | AHG7: Reference picture list initialization for MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ying Chen Li Zhang Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

A reference picture list initialization method is proposed that signals the starting position of the inter-view reference pictures in the initial reference picture list. Signalling the starting position in the initial list for inter-view reference pictures is reported to avoid signalling reference picture list modification syntax in most cases. The proposal also reports modest BD-rate decrease under common test conditions.

# Introduction

Initialization of reference picture lists in MV-HEVC follows a particular order by which the process traverses the short-term, long-term and inter-view reference pictures to construct the list. In most cases in the common test conditions, the default position of the inter-view reference pictures after initialization is not the desired location of those pictures in the final reference picture list. The final lists are obtained only by performing reference picture list modification whose signalling may require many bits.

This document proposes an efficient method of initialization of reference picture lists for MV-HEVC where the desired initial position of the inter-view reference pictures is signalled in the slice header. The initialization process of the reference picture lists is modified so that the inter-view reference pictures are present in the signalled starting position in the initial reference picture list.

# Proposal

The proposed changes to the syntax elements, semantics and the decoding process are given below, with the changes highlighted in yellow. The rest of the syntax elements and the decoding process remain the same as defined in the current specification.

## Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| … |  |
| if( slice_header_extension_present_flag ) { // should always be true in MV-HEVC |  |
| slice_header_extension_length | ue(v) |
| if( slice_type != I_SLICE ) |  |
| inter_view_ref_start_position_plus1 | ue(v) |
| … |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

## Slice header semantics

inter_view_ref_start_position_plus1 specifies the starting position of the inter-view reference pictures in reference picture list 0 after reference picture list initialization. inter_view_ref_start_position_plus1 is in the range of 0 to min (num_ref_idx_l0_active_minus1 + 1, NumPocStCurrBefore + NumPocStCurrAfter + NumPocLtCurr), inclusive. When inter_view_ref_start_position_plus1 is equal to zero, the interview reference pictures are present in the default position in the reference picture list. For other non-zero values, inter_view_ref_start_position_plus1 minus 1 denotes the starting position of inter-view reference pictures in the initial reference picture list. When not present, inter_view_ref_start_position_plus1 is inferred to be the default value, which equals to NumPocStCurrBefore + NumPocStCurrAfter + NumPocLtCurr + 1.

The variable IvRefStartPos is derived as follows.

if( !inter_view_ref_start_position_plus1 )
	IvRefStartPos = NumPocStCurrBefore + NumPocStCurrAfter + NumPocLtCurr
else
	IvRefStartPos = inter_view_ref_start_position_plus1 - 1

## Decoding process

Decoding process for reference picture lists construction

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

cIdx = 0
while( cIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore && cIdx < NumRpsCurrTempList0; cIdx++, i++ )
		RefPicListTemp0[ cIdx ] = RefPicSetStCurrBefore[ i ] 
	for( i = 0;  i < NumPocStCurrAfter && cIdx < NumRpsCurrTempList0; cIdx++, i++ ) 	(F-1)
		RefPicListTemp0[ cIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr && cIdx < NumRpsCurrTempList0; cIdx++, i++ )
		RefPicListTemp0[ cIdx ] = RefPicSetLtCurr[ i ]
	for( i = 0; i < NumIvCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetIvCurr[ i ]
}

// Shift the pictures in the list to create space for inter-view reference pictures, if necessary
for( cIdx = NumRpsCurrTempList0 - 1; cIdx >= IvRefStartPos + NumPocIvCurr; cIdx -- )
	RefPicListTemp0[ cIdx ] = RefPicListTemp0[ cIdx - NumPocIvCurr ]

// Include the inter-view reference pictures
for( i=0, cIdx = IvRefStartPos; i < NumPocIvCurr; cIdx++, i++ )
	RefPicListTemp0[ cIdx ] = RefPicSetIvCurr[ i ]

- The list RefPicList0 is constructed as follows:

for( cIdx = 0; cIdX ≤ num_ref_idx_l0_active_minus1; cIdx++)			(F-2)
		RefPicList0[ cIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ cIdx ] ] :
																			RefPicListTemp0[ cIdx ]

The derivation of RefPicList1 is the same as defined in the current specification.

# Results

The proposed changes were evaluated using the common test conditions. The anchor used for comparison was version HTM5.1, where the 3D tools were disabled. The proposed changes provide BD-rate decrease of 0.05% for the configuration with 3 views. The attached Excel file provides the bit rates and the PSNR for the common test conditions. For the common test conditions, using the proposed method did not require the use of reference picture list modification comments.

Table 1: BD-rate comparison of proposed method and HTM5.1 (anchor) for MV-HEVC common test conditions

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | -0.1% | -0.1% |
| Kendo | 0.0% | -0.1% | -0.1% | -0.1% |
| Newspapercc | 0.0% | -0.2% | -0.1% | -0.1% |
| GhostTownFly | 0.0% | -0.1% | -0.1% | 0.0% |
| PoznanHall2 | 0.0% | -0.2% | -0.2% | -0.1% |
| PoznanStreet | 0.0% | -0.1% | -0.1% | 0.0% |
| UndoDancer | 0.0% | -0.1% | -0.1% | 0.0% |
| 1024x768 | 0.0% | -0.1% | -0.1% | -0.1% |
| 1920x1088 | 0.0% | -0.1% | -0.1% | 0.0% |
| average | 0.00% | -0.12% | -0.12% | -0.05% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).